DCR Q-Metre

Title of  the specification and in Para 1.1 :

Change to read “RF and Power Coils, Fixed”

Para 9.3.1.1 Inductance

Delete para 9.3.1.1 in toto. Replace by :

“Inductance shall be measured with a suitable instrument (Impedance Analyser or equivalent). The measurement shall be performed at the test frequency with or without the DC Rated Current as requested by the Detail Specification.

The Test Frequency to be used for testing shall be as follows :”

| INDUCTANCE RANGE (µH) |  | TEST FREQUENCY (MHz) |
| --- | --- | --- |
| Less than | 0.018 | 150 |
| 0.022 to | 0.082 | 100 |
| 0.1 to | 0.390 | 50 |
| 0.47 to | 1.0 | 25 |
| 1.2 to | 10 | 7.9 |
| 12 to | 100 | 2.5 |
| 120 to | 1000 | 0.79 |
| 1200 to | 10000 | 0.25 |
| 12000 to | 100000 | 0.079 |

Para 9.3.1.2 Q Factor

Delete para 9.3.1.2 in toto and replace by :

“Suitable instrument shall be used for testing Q Factor. Frequencies to be used for testing the various ranges of inductance shall be as specified in Para 9.3.1.1.”

Para 9.3.1.3 Self-resonant Frequency

Delete para 9.3.1.3 in toto and replace by :

“The self-resonnant frequency shall be determined as follows :

Starting at the lowest frequency within the specified frequency range, the frequency shall be increased and the impedance observed. The frequency at which the impedance is maximum shall be recorded as the self-resonant frequency.

Zr = Impedance of the inductor at self-resonant frequency.”

Para 9.3.1.6 Insulation Resistance

In sub para (b) second alinea, replace Figure VIII by Figure I.

FIGURES :

Delete FIGURES I to VII, and renumber FIGURE VIII as FIGURE I.