TRANSISTORS, POWER, MOSFET, N-CHANNEL, RAD-HARD

BASED ON TYPE STRH8N10

ESCC Detail Specification No. 5205/023

TABLE OF CONTENTS

1. 	GENERAL

1.1 	Scope 										4

1.2 	Applicable Documents 								4

1.3 	Terms, Definitions, Abbreviations, Symbols and Units 				4

1.4 	The ESCC Component Number and Component Type Variants 			4

1.4.1 	The ESCC Component Number 							4

1.4.2 	Component Type Variants 							4

1.5 	Maximum Ratings 								5

1.6 	Handling Precautions 								7

1.7 	Physical Dimensions and Terminal Identification 					7

1.8 	Functional Diagram 								8

1.9 	Materials and Finishes 								8

2. 	REQUIREMENTS

2.1 	General 									8

2.1.1 	Deviations from the Generic Specification 					9

2.1.1.1	Deviations from Screnning Tests – Chart F3					9

2.1.1.2 	Deviations from Qualification and Periodic Tests - Chart F4 			9

2.2	Wafer Lot Acceptance

2.3 	Marking 									9

2.4 	Terminal Strength 								9

2.5 	Electrical Measurements at Room, High and Low Temperatures 			9

2.5.1 	Room Temperature Electrical Measurements 					10

2.5.2 	High and Low Temperatures Electrical Measurements 				11

2.5.3 	Notes to Room,High and Low Electrical Measurement Tables			11

2.6 	Parameter Drift Values 								12

2.7 	Intermediate and End-Point Electrical Measurements 				12

2.8 	High Temperature Reverse Bias Burn-in Conditions 				13

2.9 	High Temperature Forward Bias Burn-in conditions				13

2.10	Operating Life Conditions 							13

2.11	Total Dose Radiation Testing							14

2.11.1	Bias Conditions and Total Dose Level for Total Dose Radiation Testing		14

2.11.2   Electrical Measurements for Total Dose Radiation Testing                                      14

APPENDIX ‘A’

1. GENERAL

1.1 SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

1.2 APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

(a) ESCC Generic Specification No. 5000

(b) MIL-STD-750, Test Methods and Procedures for Semiconductor Devices

1.3 TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

1.4 THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

1.4.1 The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 520502301F

• Detail Specification Reference: 5205023

• Component Type Variant Number: 01 (as required)

• Total Dose Radiation Level Letter:F (as required)

1.4.2 Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Based on Type | Case | Lead/Terminal Material and Finish | Weight max g | Total Dose Radiation Level Letter |
| --- | --- | --- | --- | --- | --- |
| 01 | STRH8N10 | SMD.5 | Q14 | 2 | F[70kRAD/Si] |

The lead/terminal material and finish shall be in accordance with the requirements of ESCC Basic Specification No. 23500.

Total dose radiation level letters are defined in ESCC Basic Specification No. 22900. If an alternative radiation test level is specified in the Purchase Order the letter shall be changed accordingly.

1.5 MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the applicable ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Drain-Source Voltage | VDS | 100 | V | Over Top VGS=0V Note 2 |
| Gate-Source Voltage | VGS | ±20 | V | Over Top |
| Drain Current | IDS | 6 | A | Continuous At Tcase ≤ +25°C Note 1 |
|  |  | 4.1 | A | Continuous At Tcase >+100°C Note 1 |
| Drain Current (pulsed) | IDM | 24 | A | Note 2 |
| Power Dissipation | PTOT | 25 | W | AtTcase ≤ +25°C Note 1 |
| Avalanche Energy (Single Pulse) | EAS | 457 134 | mJ | VDS=50V IAS=4A Tj=+25±3°C Tj=+110 (+0 -5°C) |
| Avalanche Energy (Repetitive Pulse) | EAR | 4.3 1.4 | mJ | VDS=50V IAR=4A f=100kHz Duty Cycle= 10% Tj=+25±3°C Tj=+110 (+0 -5°C) |
| Operating Temperature Range | Top | -55 to +150 | °C | Note 3 |
| Junction Temperature | Tj | +150 | °C |  |
| Storage Temperature Range | Tstg | -55 to +150 | °C | Note 3 |
| Soldering Temperature | Tsol | +260 | °C | Note 4 |
| ThermalResistance, Junction-to-Heat Sink | Rth(J-S) | 5 2 | °C/W | Note 5 |
| ThermalResistance, Junction-to-Ambient | Rth(J-A) | 175 52 | °C/W |  |

NOTES:

- IDS and Ptot ratings are in accordance with Rth(j-s). The maximum theoretical ID limit at Tcase > +25°C can be obtained by using the following formula (ID is limited by the package and device construction):

Where (rDS(on) at Tj(max) = 720mΩ

For Tcase  > +25°C, the power Dissipation derates linearly to 0W at Tcase = +150°C

- Safe Operating Area applies as follows:

Maximun Safe Operating Area

- For Variants with hot solder dip lead finish all testing performed at Tamb>+125°C shall be carried out in a 100% inert atmosphere.

- Duration 10 seconds maximum at a distance of not less than 1.5mm from the device body and the same lead shall not be resoldered until 3 minutes have elapsed.

- Package mounted on infinite heatsink..

1.6 PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION

Metal can Package (TO39) - 3 Lead

NOTES:

1. Terminal identification is specified by reference to the tab position where Lead 1 = emitter, Lead 2 = base and Lead 3 = collector.

2. Applies to all leads.

3. Øb2 applies between L1 and L2. Øb applies between L1 and 12.7mm from the seating plane. Diameter is uncontrolled within L1 and beyond 12.7mm from the seating plane.

4. Measured from the maximum diameter of the actual device.

5. This zone is controlled for automatic handling. The variation in actual diameter within this zone shall

not exceed 0.254mm.

6. The details of outline in this zone are optional.

7. Measured from the tab centreline.

1.7 FUNCTIONAL DIAGRAM

1.8 MATERIALS AND FINISHES

Materials and finishes shall be as follows:

- Case

For the metal can package, the case shall be hermetically sealed and have a metal body with hard glass seals.

For the surface mount package the case shall be hermetically sealed and have a ceramic body with a Kovar lid.

- Leads/Terminals

As specified in Component Type Variants.

2. REQUIREMENTS

2.1 GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

2.1.1 Deviations from the Generic Specification

2.1.1.1 Deviations from Screening Tests - Chart F3

- Verification of Safe Operating Area

The Safe Operating Area shall be verified by performing the VSD test specified in Room Temperature Electrical Measurements (Thermal Resistance, Junction-to-Heat Sink).

- A High Temperature Forward Bias test shall be performed instead of Power Burn-in.

2.1.1.2 Deviations from Qualification and Periodic Tests - Chart F4

(a) Constant Acceleration is omitted.

(b) For SMD.5 Terminal Strength is not applicable

2.2 WAFER LOT ACCEPTANCE

A SEM inspection shall be performed as defined in Chart F2 and Para 5.2.2 of the ESCC Generic Specification.

2.3 MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component shall be:

(a) The ESCC Qualified components symbol (for ESCC qualified components only).

(b) The ESCC Component Number.

(c) Traceability information (Date Code).

2.4 TERMINAL STRENGTH

The test conditions for terminal strength, tested as specified in the ESCC Generic Specification, shall be as follows:

For T039, Test Condition: E, lead fatigue.

Terminal strength is not applicable for the SMD.5

2.5 ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures. Consolidated notes are given after the tables.

2.5.1 Room Temperature Electrical Measurements

The measurements shall be performed at Tamb=+22 3°C.

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Drain-to-Source Breakdown Voltage | V(BR)DSS | 3407 | VGS=0V ID=1mA Bias Condition C | 100 | - | V |
| Gate-to-Source Leakage Current | IGSSF1 | 3411 | VGS=+20V Bias Condition C, VDS=0V | - | +100 | nA |
|  | IGSSR1 | 3411 | VGS=-20V Bias Condition C, VDS=0V | -100 | - | nA |
| Drain Current | IDSS | 3413 | VDS=80% of rated BVDSS Bias Condition C, VGS=0V | - | 10 | µA |
| Gate-Source-Voltage (threshold) | VGS(th) | 3403 | VGS=VDS ID= 1mA | 2 | 4.5 | V |
| Static Drain-to-Source on resistance | rDS(ON) | 3421 | ID=4A VGS=12V Note 1 | - | 0.3 |  |
| Forward Voltage | VSD | 4011 | ISD=8A VGS=0V Note 1 | - | 1.5 | V |
| Thermal Resistance Junction-to-Heat Sink | Rth(j-s) | 3161 | Note 2 | - | 5 | °C/W |
| Input capacitance | Ciss | 3431 | VGS=+0V VDS=25V f=1MHz | 527 | 791 | pF |
| Output capacitance | Coss | 3453 |  | 76 | 114 | pF |
| Reverse transfert capacitance | Crss | 3433 |  | 31 | 47 | pF |
| Total Gate Charge | Qg | 3471 | VDD=50V ID=4A VGS=12V | 15 | 22 | nC |
| Gate to Source charge | Qgs |  |  | 3 2.5 | 5 4.5 | nC nC |
| Gate to Drain charge | Qgd |  |  | 4 3.5 | 7 6 | nC nC |
| Turn-on delay time | td(on) | 3472 | VDD=50V ID=4A RG=4.7 VGS=12V | 6 5 | 15 10 | ns ns |
| Rise time | tr |  |  | 4 2 | 17 9 | ns ns |
| Turn-off delay time | td(off) |  |  | 13 | 30 | ns |
| Fall time | tf |  |  | 3 2.5 | 8 7.5 | ns ns |
| Reverse Recovery Time | trr | 3473 | VDS=50V ISD=8A di/dt=100A/µs Tj=+25 ±3°C | 196 | 294 | ns |

2.5.2 High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions Note 3 | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Gate-to-Source Leakage Current | IGSSF1 | 3411 | VGS=+20V Bias Condition C, VDS=0V Tcase= +125 (+0-5)°C | - | +200 | nA |
|  | IGSSR1 | 3411 | VGS=-20V Bias Condition C, VDS=0V Tcase= +125 (+0-5)°C | -200 | - | nA |
| Drain Current | IDSS | 3413 | VDS=80% of rated BVDSS Bias Condition C, VGS=0V Tcase= +125 (+0-5)°C | - | 100 | µA |
| Gate-Source Voltage (threshold) | VGS(th) | 3403 | VGS=VDS ID= 1mA Tcase= +125 (+0-5)°C | 1.5 | 3.7 | V |
|  |  |  | VGS=VDS ID= 1mA Tcase= -55 (+5-0)°C | 2.1 | 5.5 | V |
| Static Drain-to-Source on resistance | rDS(ON) | 3421 | ID=4A VGS=12V Note 1 Tcase= +125 (+0-5)°C |  | 0.72 |  |
| Forward Voltage | VSD | 4011 | ID=8A VGS=0V Note 1 Tcase= +125 (+0-5)°C |  | 1.275 | V |

2.5.3 Notes to Electrical Measurement Tables

- Pulse Width≤300µs, Duty Cycle≤2%.

- The Rth(j-s) limit is guaranteed by performing a ΔVSD (go-no-go) test. The following test conditions and limits shall apply:

  - VDS = 6V

  - ID = 4.26A

  - Ical = 5mA

  - tpulse = 20ms

  - tcal = 50µs

  - VSD = 100mV minimum, 190mV maximum.

- VDS = 6V

- ID = 4.26A

- Ical = 5mA

- tpulse = 20ms

- tcal = 50µs

- VSD = 140mV minimum, 250mV maximum.

- Read and record measurements shall be performed on a sample of 5 components with 0 failures allowed. Alternatively a 100% inspection may be performed.

2.6 PARAMETER DRIFT VALUES

Unless otherwise specified, the measurements shall be performed at Tamb=+22 3°C.

The test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The drift values () shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value  | Absolute |  |  |
|  |  |  | Min | Max |  |
| Gate-to-Source Leakage Current | IGSSF1 | ± 50 or (1) ± 100% | - | +100 | nA |
|  | IGSSR1 | ± 50 or (1) ± 100% | -100 | - | nA |
| Drain Current | IDSS | 4 or (1) 100% | - | 10 | µA |
| Gate-Source Voltage (threshold) | VGS(th) | ± 5% | 2 | 4.5 | V |
| StaticDrain-Source on resistance | RDS(ON) | ± 10% | - | 0.3 |  |

NOTES:

1. Whichever is the greater referred to the initial value.

2.7 INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb=+22 3°C.

The test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Drain Current | IDSS | 3413 | VDS=80% of rated BVDSS Bias Condition C, VGS=0V | - | 10 | µA |
| Gate-Source Voltage (threshold) | VGS(th) | 3403 | VGS=VDS ID= 1mA | 2 | 4.5 | V |
| StaticDrain-Source on resistance | rDS(ON) | 3421 | ID=4A VGS=12V Note 1 |  | 0.3 |  |

2.8 HIGH TEMPERATURE REVERSE BIAS CONDITIONS

HTRB Burn-in shall be performed in accordance with MIL-STD-750, Test Metho 1042, Test Condition A with the following conditions:

| Characteristics | Symbols | Test Conditions MIL-STD-750 Test Method 1042 Cond A | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 | °C |
| Drain to Source | VDS | +80 | V |
| Duration | - | 240 | hrs |

2.9 HIGH TEMPERATURE FORWARD BIAS CONDITIONS.

HTFB Burn-in shall be performed in accordance with MIL-STD-750, Test Metho 1042, Test Condition B with the following conditions:

| Characteristics | Symbols | Test Conditions MIL-STD-750 Test Method 1042 Cond B | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 | °C |
| Gate to Source | VGS | +16 | V |
| Duration | - | 48 | hrs |

2.10 OPERATING LIFE CONDITIONS

Operating Life shall be performed in accordance with MIL-STD-750, Test Metho 1042, Test Condition A (HTRB) and B (HTFB) with the following conditions:

HIGH TEMPERATURE REVERSE BIAS CONDITIONS (HTRB)

| Characteristics | Symbols | Test Conditions MIL-STD-750 Test Method 1042 Cond A | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 | °C |
| Drain to Source | VDS | +80 | V |
| Duration | - | 1000 | hrs |

HIGH TEMPERATURE FORWARD BIAS CONDITIONS (HTFB or HTGB)

| Characteristics | Symbols | Test Conditions MIL-STD-750 Test Method 1042 Cond B | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 | °C |
| Gate to Source | VGS | +16 | V |
| Duration | - | 1000 | hrs |

2.11 	TOTAL DOSE RADIATION TESTING

All lots shall be irradiated in accordance with ESCC Basic Specification No.22900, standard dose rate (window 1: 3.6kRAD to 36kRAD per hour).

2.11.1	Bias Conditions and Total Dose Level for Total Dose Radiation Testing

The following bias condition (worst-case) shall be used for Total Dose Radiation Testing at Tamb=22±3°C:

With VGS bias = +15V and VDS=0V during irradiation.

The total dose level applied shall be as specified in the component type variant information herein or in the Purchase order.

2.11.2	Electrical Measurements for Total Dose Radiation Testing

Prior to irradiation testing the devices shall have successfully met Room Temperature Electrical Measurements specified herein.

Unless otherwise stated the measurements shall be performed at Tamb=22 ± 3°C.

Unless otherwise specified the test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The parameters to be measured during and on completion of irradiation testing, after 24 hours anneal at Room Temperature and after 168 hours anneal at +100±3°C are shown below.

| Characteristics | Symbols | Limits |
| --- | --- | --- |
|  |  | Drift Values (∆) |
| Drain-to-source Voltage (Note 1) | VDSS | -25 % |
| Gate-to-Source Leakage Current 1 | IGSS1 | +/- 1.5 nA |
| Gate-to-Source Leakage Current 2 | IGSS2 | +/- 1.5 nA |
| Drain Current | IDSS | + 1µA |
| Gate-to-Source Threshold Voltage | VGS(th) | -60 % / + 30 % |
| Static Drain-to-Source On Resistance (TO-3) | rDS(on) | +/- 10% |
| Source-to-Drain Diode Forward Voltage | VSD | +/- 2 % |
| Total Gate Charge (a) | Qg | -5% / +40% |
| Gate-to-Source Charge (a) | Qgs | +/- 35% |
| Gate-to-Drain Charge (a) | Qgd | -5% / +130% |

- Parameter not measured after irradiation but guaranteed by the results obtained during the evaluation phase that proves this parameter is directly correlated to the VGSth shift.

APPENDIX ‘A’

AGREED DEVIATIONS FOR STMICROELECTRONICS (F)

| ITEMS AFFECTED | DESCRIPTION OF DEVIATIONS |
| --- | --- |
| Deviations from Room Temperature Electrical Measurements | The AC characteristics Ciss, Coss, Crss, Qg, Qgs, Qgd, td(on), tr, td(off), tf and trr may be considered guaranteed but not tested if successful pilot lot testing has been performed on the wafer lot in accordance with STMicroelectronics procedure 8212069, which includes AC (Ciss, Coss, Crss, Qg, Qgs, Qgd, td(on), tr, td(off), tf and trr) characteristic measurements per the Detail Specification. A summary of the pilot lot testing shall be provided if required by the Purchase Order. |
| Deviations from Electrical Measurements for Total Dose Radiation Testing | The AC characteristics Qg, Qgs and Qgd need not be measured because they are guaranteed by the results obtained by STMicroelectronics during the evaluation phase which proved these characteristics are directly correlated to the VGS(th) shift. |
| Deviations from Screening Tests –Chart F3 | Solderability is not applicable unless specifically stipulated in the Purchase Order. |

ADDITIONAL DATA – STMICROELECTRONICS (F)

Heavy ions characterization has been done on STRH100N10 part. But STRH8N10 is based on the same technology and same epitaxy. The results obtained on STRH100N10 are transposable to the STRH8N10.

- Derating for Space Application

These components are susceptible to Single Event Gate Rupture if operated in a space environment

unless the following derating is applied. The derating for space applications was originally obtained

on STRH100N10 devices under the following test conditions. The testing was performed in a

vacuum at UCL (Louvain-la-Neuve, Belgium):

Ion used = Kr

LET = 32 (MeV / (mg/cm2))

Energy = 768 MeV

Range = 94 m

VDS ≤ 100V	When	VGS = 0V

VDS ≤ 80V	When	VGS = -2V

VDS ≤ 60V	When	VGS = -5V

VDS ≤ 30V	When	VGS = -10V

VDS ≤ 10V	When	VGS = -20V

Single Event Effect Safe Operating Area (for STRH100N10)

Justification:

Delete the TO-39, this package does not meet of customer expectations (no space market demand). ST has not manufactured and sold any products in this version (TO-39).

A Contrary the SMD.5 is requested, the ESCC qualification has been done according this package and also from new silicon for which the thickness of the gate oxide is 470Å instead of 350Å on the previous one meaning a review of the limits (mainly AC test parameters).