|  | Type | 2N3810 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | REF SPEC ESCIES | 5207/005 |  |  |  |
| 1.4.2 Component Type Variants |  |  |  |  |  |
| Variant Number | Based on Type | Case | Lead/Terminal Material and/or Finish | Weight max g |  |
| 1 | 2N3810 | TO-78 | D2 | 0.95 |  |
| 2 | 2N3810 | TO-78 | D3 or D4 | 0.95 |  |
| 5 | 2N3810 | TO-78 | D7 | 0.95 |  |
| 7 | 2N3810 | CCP | 2 | 0.2 |  |
| 9 | 2N3810 | CCP | 4 | 0.2 |  |
| 10 | 2N3810 | FP-8 | G2 | 0,7 |  |
| 11 | 2N3810 | FP-8 | G4 | 0,7 |  |
| 1.5 MAXIMUM RATINGS |  |  |  |  |  |
| Power Dissipation (One Section) |  | PtotO1 | 0.5 W | At Tamb ≤ +25°C |  |
| For FP-8 |  |  |  |  |  |
| Power Dissipation (Both Sections) |  | PtotB1 | 0.6 W | At Tamb ≤ +25°C |  |
| For FP-8 |  |  |  |  |  |
| Soldering Temperature |  | Tsol | 265°C | Note 3 |  |
| For FP-8 |  |  |  |  |  |
| 1.6 PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION |  |  |  |  |  |
| 1.6.2 | Flat Package (FP) - 8 Pin |  |  |  |  |
| 1.7 FUNCTIONAL DIAGRAM |  |  |  |  |  |
| Notes | 3 For FP-8, THE lid is connected to 4 and 5 terminals |  |  |  |  |
| 1.8 MATERIALS AND FINISHES |  |  |  |  |  |
| a) | For the chip carrier package the case shall be hermetically sealed and have a ceramic body with a Kovar lid. |  |  |  |  |
| 2.3 TERMINAL STRENGTH |  |  |  |  |  |
| For FP-8 | Test condition E, lead fatigue 3 ounce weight :t: 3 bends of 15 degrees |  |  |  |  |

1.6 PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION

1.6.3	Flat Package (FP) - 8 Pin

Note 1

| Symbols | Dimension mm |  | Notes |
| --- | --- | --- | --- |
|  | MIN | MAX |  |
| A | 2.24 | 2.64 |  |
| b | 0.38 | 0.48 |  |
| c | 0.10 | 0.16 |  |
| D | 6.35 | 6.61 |  |
| E | 6.35 | 6.61 |  |
| E2 | 4.32 | 4.58 |  |
| E3 | 0.88 | 1.14 |  |
| e |  |  |  |
| L | 6.51 | 7.38 |  |
| Q | 0.66 | 0.92 |  |
| S1 | 0.92 | 1.32 |  |
| N | 08 Typical |  | 2 |

NOTES:

1. Terminal identification is specified, when viewing the top side of the package, by reference to a black

ink dot adjacent to terminal 1 = collector 1, terminal 2 = base 1, terminal 3 = emitter 1, terminal 6 = emitter 2

terminal 7 = base 2, terminal 8 = collector 2, terminals 4 and 5 are connected to seal ring and lid

2. Applies to all terminals.