‘Modifications’ to DCR845:

------------------------------------------

The original contents and attachment of this DCR845 are replaced with the following in order to clarify the exact changes requested by Microspire:

Original Wording Section: replace with the following:

See ESCC3201/008 issue 5, Page 6 & 7, Table 1a – Range of Components - 10k Series, column 7: Max DC Resistance Rdc (ohm)(see also original ‘was’ value below).

Proposed Wording Section: replace with the following:

Table 1a – Range of Components - 10k Series, column 7: Max DC Resistance Rdc (ohm). Amend the Rdc limits in column 7 for all items in the 10K series to be in accordance with the following list:

| (1) Type number | (2) L (µH) | (7) Max. DC Resistance Rdc (Ω) | For information purposes: Original Rdc value |
| --- | --- | --- | --- |
| L010K | 0.01 | 0.025 | (No Change) |
| L012K | 0.012 | 0.025 | (No Change) |
| L015K | 0.015 | 0.025 | (was: 0.04) |
| L018K | 0.018 | 0.025 | (was: 0.04) |
| L022K | 0.022 | 0.025 | (was: 0.04) |
| L027K | 0.027 | 0.03 | (was: 0.04) |
| L033K | 0.033 | 0.04 | (was: 0.05) |
| L039K | 0.039 | 0.05 | (was: 0.07) |
| L047K | 0.047 | 0.06 | (was: 0.08) |
| L056K | 0.056 | 0.065 | (was: 0.09) |
| L068K | 0.068 | 0.085 | (was: 0.1) |
| L082K | 0.082 | 0.115 | (was: 0.11) |
| L10K | 0.1 | 0.07 | (was: 0.11) |
| L12K | 0.12 | 0.07 | (was: 0.11) |
| L15K | 0.15 | 0.075 | (was: 0.12) |
| L18K | 0.18 | 0.095 | (was: 0.14) |
| L22K | 0.22 | 0.155 | (was: 0.2) |
| L27K | 0.27 | 0.18 | (was: 0.25) |
| L33K | 0.33 | 0.195 | (was: 0.3) |
| L39K | 0.39 | 0.21 | (was: 0.45) |
| L47K | 0.47 | 0.225 | (was: 0.5) |
| L56K | 0.56 | 0.245 | (was: 0.55) |
| L68K | 0.68 | 0.265 | (was: 0.58) |
| L82K | 0.82 | 0.29 | (was: 0.6) |
| 1L0K | 1 | 0.34 | (was: 0.65) |
| 1L2K | 1.2 | 0.55 | (was: 0.75) |
| 1L5K | 1.5 | 0.73 | (was: 1.1) |
| 1L8K | 1.8 | 0.92 | (was: 1.2) |
| 2L2K | 2.2 | 1.1 | (was: 1.3) |
| 2L7K | 2.7 | 1.25 | (was: 1.5) |
| 3L3K | 3.3 | 1.7 | (was: 1.8) |
| 3L9K | 3.9 | 2 | (No Change) |
| 4L7K | 4.7 | 2.3 | (No Change) |
| 5L6K | 5.6 | 2.5 | (No Change) |
| 6L8K | 6.8 | 2.6 | (No Change) |
| 8L2K | 8.2 | 2.8 | (No Change) |
| 100K | 10 | 2.8 | (was: 3.3) |