| No. | Characteristic | Symbol | ESCC 3401 Test Method | Test Condition | Limits |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Min | Max |  |
| 4 | Contact Resistance (Low Level Current) | Rcl | Para. 9.1.1.3 | Para. 9.1.1.3 |  |  | mΩ |
|  |  |  |  |  | - | 6 (2) |  |
|  |  |  |  |  | - | 25 (3) |  |
| 5 | Contact Resistance (Rated Current) | Rcr | Para. 9.1.1.3 | Table 1(b) |  |  | mΩ |
|  |  |  |  |  | - | 5 (2) |  |
|  |  |  |  |  | - | 25 (3) |  |

NOTES:

Applicable to mated connectors with grounding option.

For all termination types except FR136, FR136A, FR139.

For termination types FR136, FR136A, FR139. Measurement of Contact Resistance shall include all parts of the contact including the body and the wire.