ESCC Detail Specification		PAGE 21

No. 3401/031				ISSUE 6

4.4.3 	Guide Posts

Guide posts shall be made from passivated stainless steel, Type 303.

4.4.4	 Latching

Clip and spring shall be made of passivated stainless steel.

4.4.5 	Insulated Wires

Wire materials and finishes shall be in accordance with the requirements specified in Para. 4.4 of ESCC Detail Specification No. 3901/013.

4.4.6 	Uninsulated Solid Wires

Uninsulated solid wires shall be made of copper alloy in accordance with Type 'S' as specified in QQ-W-343. They shall be gold-plated in accordance with Class ΦΦ, Grade C or D, as specified in MIL-G-45204.

Uninsulated solid wire material and finish shall either be copper alloy in accordance with Type S as specified in QQ-W-343, gold-plated in accordance with Class ØØ Grade C or D as specified in MIL-DTL-45204.

4.4.7 	Rear Potting

Rear potting shall be made of epoxy resin.

4.5 	MARKING

4.5.1 	General

The marking of components delivered to this specification shall be in accordance with the

requirements of ESCC Basic Specification No. 21700 and the following paragraphs. When the

component is too small to accommodate all of the marking specified, as much as space permits shall be marked and the marking information, in full, shall accompany the component in its primary package.

The information to be marked and the order of precedence, shall be as follows:

(a)	 The ESCC Component Number.

(b)	 Characteristics.

(c)	 Traceability Information.

4.5.2 	Contact Identification

Not applicable.

4.5.3 	The ESCC Component Number

ESCC Component Number shall be constituted and marked as follows:

Example: 340103101B

	Detail Specification Number: 3401031

	Type Variant (see Table 1(a)): 01

	Testing level: B