## APPENDIX ‘A’

AGREED DEVIATIONS FOR ISABELLENHUETTE (D)

| Items Affected | Description of Deviations |  |  |
| --- | --- | --- | --- |
| Deviations from Generic Specification: Production Control (Chart F2) | Para. 5.2.1, Dimension Check: A 100% inspection may be performed. |  |  |
| Deviations from Generic Specification: Screening Tests (Chart F3) | Para. 8.1, Overload: For Variant 02, any holes or cracks identified during visual examination that do not fail the criteria of Para. 8.6, as below, shall not be considered as evidence of damage. |  |  |
|  | Para. 8.6, External Visual Inspection: For Variant 02, the specific requirements of ESCC Basic Specification No. 2054000 shall apply with the following deviations: - Para. 3.1, Magnification: All items shall be examined with a stereoscopic microscope with a magnification of between 5 and 20 power (x5 to x20). - Paras. 4.4(b) & 4.4(c): These reject criteria are not applicable. - Add new reject criteria to Para. 4.4 as follows: - Moulding material showing holes or cracks whose total surface area exceeds 3% of the total surface area of the moulding or coating material. - Any holes or cracks in the moulding material that bridge the space between any 2 terminals. |  |  |
| Deviations from Generic Specification: Qualification and Periodic Tests (Chart F4) | Para. 8.6, External Visual Inspection: As for Screening Tests above. |  |  |
|  | Paras. 8.8, 8.9, 8.11 & 8.12, Rapid Change of Temperature, Vibration, Robustness of Terminations & Resistance to Soldering Heat: For Variant 02, any holes or cracks identified during visual examination that do not fail the criteria of Para. 8.6, as above, shall not be considered as evidence of damage. |  |  |
|  | Para. 8.8, Rapid Change of Temperature: a high temperature extreme of Tamb = +150 (+0 -3)°C may be applied during testing. |  |  |
|  | Para. 8.10, Climatic Sequence: - Para. 8.10.5, Low Air Pressure: Not applicable. - Para. 8.10.7, DC Load: Not applicable. |  |  |
|  | Para. 8.15, Permanence of Marking: Not applicable. |  |  |
| Resistance Value Characteristic Code | Resistance Value shall be expressed by means of the following codes within the ESCC Component Number. The unit quantity shall be ohm (): |  |  |
|  | Resistance Value () | Code |  |
|  | 0.00XX | R00XX |  |
|  | 0.0XX | R0XX |  |
|  | 0.XXX | RXXX |  |
|  | X.XX | XRXX |  |
| Component Type Variants and Range of Components | For all Variants, additional resistance values, Rn, from within the specified Resistance Range, but outside of the specified Value Series, are available upon request from Isabellenhuette. |  |  |
| Marking of Tolerance on Resistance Value | The tolerance on resistance value may be marked using the actual numeric value (0.5% or 1%) instead of the specified code letter. |  |  |
| Marking of Traceability Information | The manufacturing date code may be marked using a three-digit code where the first digit is a letter to indicate the year of manufacture in accordance with the 20-year cycle code per Table 10a of IEC Publication No. 60062, and the last two digits indicate the week of the year (i.e. 01 to 52) during which encapsulation or the final production process occurred. For example: H41 for week 41 of 2016 |  |  |