Page 1 of 26

SAW FILTERS, HERMETICALLY SEALED, SURFACE MOUNT, FREQUENCY RANGE 10MHz - 4GHz

ESCC Detail Specification No. 3502/002

| Issue 3 Draft B | December 2015 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2015. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| 960 | Specification upissued to implement changes per DCR. |

TABLE OF CONTENTS

1	General	6

1.1	SCOPE	6

1.2	APPLICABLE DOCUMENTS	6

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	6

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	6

1.4.1	The ESCC Component Number	6

1.4.2	Component Type Variants	7

1.4.3	Manufacturer Specific SAW Filter Identification	7

1.5	MAXIMUM RATINGS	8

1.6	HANDLING PRECAUTIONS	8

1.7	PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION	9

1.7.1	Type A : 4 Gullwing Stripline Terminals	9

1.7.2	Type B : 6 Gullwing Stripline Terminals	10

1.7.3	Type C-1 : 4 Gullwing Stripline Terminals	11

1.7.4	Type C-2 : 4 Gullwing Stripline Terminals	12

1.7.5	Type D : 10 Terminals	13

1.7.6	Type E : 12 Gullwing Stripline Terminals	14

1.7.7	Type F-1 : 8 Gullwing Stripline Terminals	15

1.7.8	Type F-2 : 8 Gullwing Stripline Terminals	16

1.7.9	Type G-1 : 16 Pre-formed Stripline Terminals	17

1.7.10	Type G-2 : 16 Formed Stripline Terminals	18

1.7.11	Type H : 26 Formed Stripline Terminals	19

1.7.12	Type I : 6 Stripline Terminals	20

1.7.13	Type J-1 : 6 Formed Stripline Terminals	21

1.7.14	Type J-2  : 6 Stripline Terminals	22

1.7.15	Type J-3 : 4 Stripline Terminals	23

1.8	FUNCTIONAL DIAGRAM	23

1.9	PIN ASSIGNMENT	23

2	REQUIREMENTS	24

2.1	GENERAL	24

2.1.1	Deviations from the Generic Specification	24

2.2	MARKING	24

2.3	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	24

2.3.1	Room Temperature Electrical Measurements	24

2.3.2	High and Low Temperatures Electrical Measurements	25

2.4	PARAMETER DRIFT VALUES	25

2.5	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	25

2.6	BURN-IN CONDITIONS	25

2.7	OPERATING LIFE CONDITIONS	25

APPENDIX A	26

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3502.

MIL-STD-883, Test Methods and Procedures for Microelectronics.

ESCC-Q-ST-70-38, High-reliability soldering for surface mount and mixed technology.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 350200201S#####

Detail Specification Reference: 3502002

Component Type Variant Number: 01 (as required)

Manufacturer Specific SAW Filter Identification: S##### (as applicable), where S##### is a unique code allocated by the Manufacturer to a specific SAW Filter design.

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Case Type Code Letter | Terminal Material and / or Finish (Note 2) | Weight max g |
| --- | --- | --- | --- |
| 01 | A | D2 | (Note 1) |
| 02 | B | D2 | (Note 1) |
| 03 | C-1 | D2 | (Note 1) |
| 04 | C-2 | D2 | (Note 1) |
| 05 | D | 2 | (Note 1) |
| 06 | E | D2 | (Note 1) |
| 07 | F-1 | D2 | (Note 1) |
| 08 | F-2 | D2 | (Note 1) |
| 09 | G-1 | D2 | (Note 1) |
| 10 | G-2 | D2 | (Note 1) |
| 11 | H | D2 | (Note 1) |
| 12 | I | D2 | (Note 1) |
| 13 | J-1 | D2 | (Note 1) |
| 14 | J-2 | D2 | (Note 1) |
| 15 | J-3 | D2 | (Note 1) |

NOTES:

The maximum weights of the individual SAW Filters are given in the applicable SAW Filter Data Sheets.

The terminal material and/or finish shall be in accordance with the requirements of ESCC Basic Specification No. 23500.

### Manufacturer Specific SAW Filter Identification

A SAW Filter Data Sheet shall be produced by the Manufacturer following negotiation with the Orderer that, as a minimum, specifies all the requirements unique to the specific SAW Filter design that are identified herein as being specified in a SAW Filter Data Sheet. The SAW Filter Data Sheet shall be held under configuration control by the Manufacturer.

For identification and traceability purposes the Manufacturer shall allocate a unique Manufacturer Specific SAW Filter Identification to the SAW Filter Data Sheet and the specific SAW Filter design, as specified in The ESCC Component Number herein.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| Device RF Input Power (Continuous) | PRF | See SAW Filter Data Sheet | dBm | Note 1 |
| Operating Temperature Range | Top | See SAW Filter Data Sheet | °C | Tcase |
| Storage Temperature Range | Tstg | See SAW Filter Data Sheet | °C | Tcase |
| Soldering Temperature | Tsol | +330 | °C | Note 2 |

NOTES:

Guaranteed maximum input level for an operating time ≤ 24 hours. Degradation of performance may occur if the specified Maximum Continuous RF Input Power is applied for periods longer than 24 hours. A derating of 6dBm from the specified Maximum Continuous RF Input Power must be applied under normal operating conditions.

Duration 10 seconds maximum at a distance of not less than 1.6mm from the device body and the same terminal shall not be resoldered until 3 minutes have elapsed.

## HANDLING PRECAUTIONS

These devices are susceptible to damage by electrostatic discharge. Therefore, suitable precautions shall be employed for protection during all phases of manufacture, testing, packaging, shipment and any handling.

These devices must not be subjected to any DC Voltage.

These components are categorised as Class 1 per ESCC Basic Specification No. 23800.

## PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION

For all package types, the base of the package is a ground terminal. The base of the package must be connected to GND in order to achieve the electrical performance specified in the applicable SAW Filter Data Sheet.

### Type A : 4 Gullwing Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.15mm.

Terminal identification:

1 - Input

2 - Input Return

3 - Output Return

4 - Output

### Type B : 6 Gullwing Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.15mm.

Terminal identification is not applicable (the numbering and assignment of the terminals is dependent on the specific SAW Filter design.)

### Type C-1 : 4 Gullwing Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.15mm.

Terminal identification:

1 - Input

2 - Input Return

3 - Output Return

4 - Output

### Type C-2 : 4 Gullwing Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.15mm.

Terminal identification:

1 - Input

2 - Input Return

3 - Output Return

4 - Output

### Type D : 10 Terminals

NOTES:

All dimensions are in millimetres.

The tolerance on all dimensions is ±0.15mm.

Terminal identification is dependent on the configuration of the SAW Filter.

          - Differential input and output:

1 - Input Return

2, 3, 4, 7, 8, 9 - GND

5 - Output

6 - Output Return

10 - Input

          - Differential input, single ended output:

1 - Input Return

2, 3, 4, 6, 7, 8, 9 - GND

5 - Output

10 - Input

          - Single-ended input, differential output:

1, 2, 3, 4, 7, 8, 9 - GND

5 - Output

6 - Output Return

10 - Input

- For all configurations, terminals 2, 3, 4 and 7, 8, 9 are internally connected.

### Type E : 12 Gullwing Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.15mm.

Terminal identification:

1, 12 - Input

2, 3, 4, 5, 8, 9, 10, 11 - GND

6, 7 - Output

### Type F-1 : 8 Gullwing Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.15mm.

Terminal identification is not applicable (the numbering and assignment of the terminals is dependent on the specific SAW Filter design.)

### Type F-2 : 8 Gullwing Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.15mm.

Terminal identification is not applicable (the numbering and assignment of the terminals is dependent on the specific SAW Filter design.)

### Type G-1 : 16 Pre-formed Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified, terminal dimensions/positions have a tolerance of ±0.2mm and package dimensions have a tolerance of ±0.13mm.

Terminal identification:

1 - Input

2, 4, 5, 6, 7, 8, 10, 12, 13, 14, 15, 16 - GND

3 - Input Return

9 - Output Return

11 - Output

- Terminal No. 1 may be identified by means of a coloured glass feed-through in addition to the top marking.

All 16 terminals shall be pre-formed prior to their assembly into the package body. The terminals on all four sides of the package shall be pre-formed in the same manner.

### Type G-2 : 16 Formed Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified, terminal dimensions/positions have a tolerance of ±0.2mm and package dimensions have a tolerance of ±0.13mm.

Terminal identification:

1 - Input

2, 4, 5, 6, 7, 8, 10, 12, 13, 14, 15, 16 - GND

3 - Input Return

9 - Output Return

11 - Output

- Terminal No. 1 may be identified by means of a coloured glass feed-through in addition to the top marking.

### Type H : 26 Formed Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified, terminal dimensions/positions have a tolerance of ±0.2mm and package dimensions have a tolerance of ±0.13mm.

Terminal identification:

1 - Input

2, 4, 5, 6, 7, 8, 10, 12, 13, 15, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26 - GND

3 - Input Return

14 - Output Return

16 - Output

- Terminal No. 1 may be identified by means of a coloured glass feed-through in addition to the top marking.

### Type I : 6 Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.13mm.

Terminal identification:

1 - Input

2, 3, 4, 5 - GND

6 - Output

- Terminal No. 1 may be identified by means of a coloured glass feed-through in addition to the top marking.

### Type J-1 : 6 Formed Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.13mm.

Terminal identification:

1 - Input

2, 5 - GND

3 - Input Return

4 - Output Return

6 - Output

- Terminal No. 1 may be identified by means of a coloured glass feed-through in addition to the top marking.

### Type J-2  : 6 Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.13mm.

Terminal identification:

1 - Input

2, 3, 4, 5 - GND

6 - Output

Terminal No. 1 may be identified by means of a coloured glass feed-through in addition to the top marking.

### Type J-3 : 4 Stripline Terminals

NOTES:

All dimensions are in millimetres.

Unless otherwise specified dimensions have a tolerance of ±0.13mm.

Terminal identification:

1 - Input

2 - Input Return

3 - Output Return

4 - Output

Terminal No. 1 may be identified by means of a coloured glass feed-through in addition to the top marking.

## FUNCTIONAL DIAGRAM

## PIN ASSIGNMENT

The pin assignment shall be defined in each individual SAW Filter Data Sheet.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

None.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component shall be:

Lead Identification.

The ESCC Qualified Component Symbol (for ESCC qualified components only).

The ESCC Component Number.

Traceability information.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures. The characteristics to be measured vary depending on the particular SAW Filter. Therefore, typical characteristics are given below and the specific characteristics to be measured shall be defined in each individual SAW Filter Data Sheet.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | MIL-STD-883 Test Method | Test Conditions Note 1 | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Centre Frequency | fC | - | See SAW Filter Data Sheet |  |  | MHz |
| In-band Ripple, fL to fU | - | - | See SAW Filter Data Sheet |  |  | dB(p-p) |
| Insertion Loss | IL | - | See SAW Filter Data Sheet |  |  | dB |
| Phase Linearity, fL to fU | - | - | See SAW Filter Data Sheet |  |  | °(p-p) |
| Out-of-band Rejection | - | - | See SAW Filter Data Sheet |  |  | dB |

NOTES:

All measurements shall be made with the use of external matching networks.

### High and Low Temperatures Electrical Measurements

The measurements shall be performed at the High and Low temperatures defined in each individual SAW Filter Data Sheet. Unless otherwise specified the characteristics, test methods, conditions and limits shall be the same as specified for Room Temperature Electrical Measurements.

## PARAMETER DRIFT VALUES

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value Δ | Absolute |  |  |
|  |  |  | Min | Max |  |
| Centre Frequency (at -6dB) | fC(-6dB) | See SAW Filter Data Sheet |  |  | MHz |
| Insertion Loss | IL | See SAW Filter Data Sheet |  |  | dB |

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C. Unless otherwise specified the characteristics, test methods, conditions and limits shall be the same as specified for Room Temperature Electrical Measurements.

## BURN-IN CONDITIONS

As specified in Burn-in in the Generic Specification.

## OPERATING LIFE CONDITIONS

As specified in Operating Life in the Generic Specification.

## APPENDIX A

AGREED DEVIATIONS FOR KONGSBERG NORSPACE AS (NORWAY)

| ITEMS AFFECTED | DESCRIPTION OF DEVIATIONS |
| --- | --- |
| Para. 2.1.1, Deviations from the Generic Specification | Chart F3 – Screening Tests: Radiographic Inspection shall not be performed. |
|  | Paragraph 5.1.1 – Rebonding: Wire re-bonding is allowed within the following limitations: All re-bonds shall be placed on undisturbed metal The total number of pin to substrate or substrate-to-substrate wire re-bondings shall be limited to a maximum of 10% of the total number of wires in the SAW device. |
|  | Paragraph 8.6 – Seal: MIL-STD-883, Test Method 1014, condition CH2 shall be employed for both Fine and Gross Leak. |