Page 1 of 18

TRANSISTORS, POWER, MOSFET, N-CHANNEL, RAD-HARD

BASED ON TYPEs BUY25CS12J-01 and BUY25CS04J-01

ESCC Detail Specification No. 5205/026

| Issue 3 Draft B | May 2016 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2016. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| tba | Specification upissued to incorporate changes per DCR. |

TABLE OF CONTENTS

1	General	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants	5

1.5	MAXIMUM RATINGS	6

1.6	HANDLING PRECAUTIONS	8

1.7	PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION	9

1.7.1	Leadless Chip Carrier Package (SMD0.5) - 3 Terminals	9

1.8	FUNCTIONAL DIAGRAM	10

1.9	MATERIALS AND FINISHES	10

2	REQUIREMENTS	10

2.1	GENERAL	10

2.1.1	Deviations from the Generic Specification	10

2.1.1.1	Deviations from Screening Tests - Chart F3	10

2.1.1.2	Deviations from Qualification and Periodic Tests - Chart F4	11

2.2	MARKING	11

2.3	WAFER LOT ACCEPTANCE	11

2.4	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	11

2.4.1	Room Temperature Electrical Measurements	11

2.4.2	High and Low Temperatures Electrical Measurements	13

2.4.3	Notes to Room, High and Low Temperatures Electrical Measurements	13

2.5	PARAMETER DRIFT VALUES	13

2.6	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	14

2.7	HIGH TEMPERATURE REVERSE BIAS BURN-IN CONDITIONS	15

2.8	HIGH TEMPERATURE STEADY-STATE GATE BIAS BURN-IN CONDITIONS	15

2.9	OPERATING LIFE CONDITIONS	15

2.10	TOTAL DOSE RADIATION TESTING	16

2.10.1	Bias Conditions and Total Dose Level for Total Dose Radiation Testing	16

2.10.2	Electrical Measurements for Total Dose Radiation Testing	16

APPENDIX ‘A’	17

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 5000

MIL-STD-750, Test Methods and Procedures for Semiconductor Devices

MIL-STD-883, Test Method Standard Microelectronics

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 520502601R

Detail Specification Reference: 5205026

Component Type Variant Number: 01 (as required)

Total Dose Radiation Level Letter: R (as required)

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Based on Type | Case (Note 1) | Weight max (g) | Total Dose Radiation Level Letter (Note 2) |
| --- | --- | --- | --- | --- |
| 01 | BUY25CS12J-01 | SMD0.5 | 1.1 | R [100kRAD(Si)] |
| 02 | BUY25CS04J-01 | SMD0.5 | 1.1 | R [100kRAD(Si)] |

NOTES:

See Physical Dimensions and Terminal Identification.

Total dose radiation level letters are defined in ESCC Basic Specification No. 22900. If an alternative radiation test level is specified in the Purchase Order the letter shall be changed accordingly.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| Drain-Source Voltage | VDS | 250 | V | Note 1 |
| Gate-Source Voltage | VGS | ±20 | V |  |
| Drain Current (Continuous) Variant 01 Variant 02 | IDS | 12.4 4.4 | A | At Tcase ≤ +25°C Notes 1, 2, 3 |
| Drain Current (Continuous) Variant 01 Variant 02 | IDS | 8 3 | A | At Tcase = +100°C Notes 2, 3 |
| Drain Current (Pulsed) Variant 01 Variant 02 | IDM | 50 18 | Apk | At Tcase ≤ +25°C Notes 1, 2 |
| Power Dissipation Variant 01 Variant 02 | Ptot | 75 20 | W | Note 4 |
| Avalanche Energy (Single Pulse) Variant 01 Variant 02 | EAS | 60 30 | mJ |  |
| Operating Temperature Range | Top | -55 to +150 | °C | Tamb |
| Storage Temperature Range | Tstg | -55 to +150 | °C |  |
| Junction Temperature | Tj | +150 | °C |  |
| Soldering Temperature | Tsol | +250 | °C | Note 5 |
| Thermal Resistance, Junction-to-Case Variant 01 Variant 02 | Rth(j-c) | 1.66 3 | °C/W |  |

NOTES:

Safe Operating Area applies as follows:

variant 01

variant 02

Tcase is measured on the PCB at the soldering point to the Drain terminal.

For Tcase > +25°C, derate as follows:

where (rDS(on) at Tjmax) = 490mΩ for Variant 01, 2.15Ω for Variant 02.

For Tcase > +25°C, derate linearly to 0W at Tcase = +150°C.

Duration 10 seconds maximum and the same terminal shall not be resoldered until 3 minutes have elapsed.

## HANDLING PRECAUTIONS

These devices are susceptible to damage by electrostatic discharge. Therefore, suitable precautions shall be employed for protection during all phases of manufacture, testing, packaging, shipment and any handling.

These components are categorised as Class 1 per ESCC Basic Specification No. 23800 with a Minimum Critical Path Failure Voltage of 1000V for Variant 01 and 900V for Variant 02.

## PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION

### Leadless Chip Carrier Package (SMD0.5) - 3 Terminals

| Symbols | Dimensions mm |  |
| --- | --- | --- |
|  | Min | Max |
| A | 7.35 | 7.69 |
| B | 9.97 | 10.41 |
| C | - | 3.3 |
| D | 7.14 | 7.39 |
| E | 5.59 | 5.84 |
| F | 0.76 | - |
| G | 2.92 | 3.18 |
| H | 0.76 | - |
| I | 2.29 | 2.54 |

NOTES:

Terminal identification: terminal 1 = Drain, terminal 2 = Gate, terminal 3 = Source.

## FUNCTIONAL DIAGRAM

NOTES:

The case is not connected to any terminal.

## MATERIALS AND FINISHES

Materials and finishes shall be as follows:

Case

- The case shall be hermetically sealed and have a ceramic/metal body.

Terminals

- The terminal material and finish shall be Q14 in accordance with the requirements of ESCC Basic Specification No. 23500.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests - Chart F3

Verification of Safe Operating Area: The Safe Operating Area shall be verified by performing the Thermal Impedance (Zth(j-s)) ΔVSD test specified in Room Temperature Electrical Measurements.

Particle Impact Noise Detection may be performed at any point after Temperature Cycling, prior to Seal.

Power Burn-in: A high temperature steady-state gate bias test (HTGB) shall be performed instead of Power Burn-in.

#### Deviations from Qualification and Periodic Tests - Chart F4

Terminal Strength is not applicable.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number.

Traceability information.

## WAFER LOT ACCEPTANCE

A SEM inspection shall be performed as specified in the ESCC Generic Specification.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures. Consolidated notes are given after the tables.

### Room Temperature Electrical Measurements

Unless otherwise specified, the measurements shall be performed at Tamb = +25 ±3°C.

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min. | Max. |  |
| Drain-to-Source Breakdown Voltage | V(BR)DSS | 3407 | VGS = 0V, ID = 0.25mA Bias condition C | 250 | - | V |
| Gate-to-Source Threshold Voltage | VGS(th) | 3403 | VDS ≥ VGS, ID = 1mA | 2 | 4 | V |
| Gate-to-Source Leakage Current | IGSS | 3411 | VGS = ±20V, VDS = 0V Bias condition C | -100 | +100 | nA |
| Drain Current | IDSS | 3413 | VGS = 0V, VDS = 200V Bias condition C | - | 25 | µA |
| Static Drain-to-Source On Resistance | rDS(on) | 3421 | VGS = 10V Note 1 Variant 01: ID = 8A Variant 02: ID = 3A | - - | 130 400 | mΩ |
| Source-to-Drain Diode Forward Voltage | VSD | 4011 | VGS = 0V, ISD = 12.4A Note 1 | - | 1.2 | V |
| Thermal Impedance | Zth(j-c) | 3161 | Note 2 Variant 01 Variant 02 | - - | 1.14 2.18 | °C/W |
| Turn-on Delay Time | td(on) | 3472 | VGS = 10V, RG = 4.7Ω VDS = 125V Note 3 Variant 01: ID = 8A Variant 02: ID = 3A | - - | 25 14 | ns |
| Rise Time | tr | 3472 | VGS = 10V, RG = 4.7Ω VDS = 125V Note 3 Variant 01: ID = 8A Variant 02: ID = 3A | - - | 25 14 | ns |
| Turn-off Delay Time | td(off) | 3472 | VGS = 10V, RG = 4.7Ω VDS = 125V Note 3 Variant 01: ID = 8A Variant 02: ID = 3A | - - | 35 20 | ns |
| Fall Time | tf | 3472 | VGS = 10V, RG = 4.7Ω VDS = 125V Note 3 Variant 01: ID = 8A Variant 02: ID = 3A | - - | 20 16 | ns |
| Reverse Recovery Time | trr | 3473 | VDD ≤ 50V, di/dt = 100A/µs Note 3 Variant 01: ISD = 12.4A Variant 02: ISD = 4.4A | - - | 400 250 | ns |
| Input Capacitance | Ciss | 3431 | VGS = 0V, VDS = 100V f = 1MHz Note 3 Variant 01 Variant 02 | 1.3 - | 1.9 0.7 | nF |
| Output Capacitance | Coss | 3453 | VGS = 0V, VDS = 100V f = 1MHz Note 3 Variant 01 Variant 02 | 90 - | 150 60 | pF |
| Reverse Transfer Capacitance | Crss | 3433 | VGS = 0V, VDS = 100V f = 1MHz Note 3 Variant 01 Variant 02 | 1 - | 6 3 | pF |
| Total Gate Charge | Qg | 3471 | VGS = 10V, VDS = 125V Note 3 Variant 01: ID = 12.4A Variant 02: ID = 4.4A | - - | 42 10 | nC |
| Gate-to-Source Charge | Qgs | 3471 | VGS = 10V, VDS = 125V, ID = 12.4A Note 3 Variant 01 Variant 02 | - - | 15 10 | nC |
| Gate-to-Drain Charge | Qgd | 3471 | VGS = 10V, VDS = 125V, ID = 12.4A Note 3 Variant 01 Variant 02 | - - | 15 10 | nC |

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions Note 4 | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Gate-to-Source Threshold Voltage | VGS(th) | 3403 | Tamb = +125 (+0 -5)°C VDS ≥ VGS, ID = 1mA | 1.5 | - | V |
|  |  |  | Tamb = -55 (+5 -0)°C VDS ≥ VGS, ID = 1mA | - | 5 | V |
| Gate-to-Source Leakage Current | IGSS | 3411 | Tamb = +125 (+0 -5)°C VGS = ±20V, VDS = 0V Bias condition C | -200 | +200 | nA |
| Drain Current | IDSS | 3413 | Tamb = +125 (+0 -5)°C VGS = 0V, VDS = 200V Bias condition C | - | 250 | µA |
| Static Drain-to-Source On Resistance | rDS(on) | 3421 | Tamb = +125 (+0 -5)°C VGS = 10V Note 1 Variant 01: ID = 8A Variant 02: ID = 3A | - - | 300 900 | mΩ |

### Notes to Room, High and Low Temperatures Electrical Measurements

Pulsed measurement: Pulse Width ≤300µs, Duty Cycle ≤2%.

The Zth(j-c) limit is guaranteed by performing a ΔVSD (go-no-go) test. The following test conditions and limits shall apply:

VDS = 20V

tM < 75µs

IM = 10mA

tH = 25ms

IH = 1.2A (Variant 01), 0.67A (Variant 02)

VSD = 40mV minimum, 60mV maximum

Read and record measurements shall be performed on a sample of 32 components with 0 failures allowed. Alternatively a 100% inspection may be performed.

Read and record measurements shall be performed on a sample of 5 components with 0 failures allowed. Alternatively a 100% inspection may be performed.

## PARAMETER DRIFT VALUES

Unless otherwise specified, the measurements shall be performed at Tamb = +25 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value Δ | Absolute |  |  |
|  |  |  | Min | Max |  |
| Gate-to-Source Threshold Voltage | VGS(th) | ±20% | 2 | 4 | V |
| Gate-to-Source Leakage Current | IGSS | ±20 or (1) ±100% | -100 | +100 | nA |
| Drain Current | IDSS | ±10 or (1) ±100% | - | 25 | µA |
| Static Drain-to-Source On Resistance (Note 2) Variant 01 Variant 02 | rDS(on) | ±20% (3) | - - | 130 400 | mΩ |

NOTES:

Whichever is the greater.

Measured only prior to HTRB Burn-in and after HTGB Burn-in.

Referred to the measurement prior to HTRB Burn-in.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +25 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value Δ | Absolute |  |  |
|  |  |  | Min | Max |  |
| Gate-to-Source Threshold Voltage | VGS(th) | ±20% | 2 | 4 | V |
| Gate-to-Source Leakage Current | IGSS | ±20 or (1) ±100% | -100 | +100 | nA |
| Drain Current | IDSS | ±10 or (1) ±100% | - | 25 | µA |
| Static Drain-to-Source On Resistance (Note 2) Variant 01 Variant 02 | rDS(on) | ±20% (3) | - - | 130 400 | mΩ |

NOTES:

Whichever is greater.

## HIGH TEMPERATURE REVERSE BIAS BURN-IN CONDITIONS

HTRB Burn-in shall be performed in accordance with MIL-STD-750, Test Method 1042, Test Condition A with the following conditions:

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | °C |
| Drain-to-Source Voltage | VDS | 200 (Note 1) | V |
| Gate-to-Source Voltage | VGS | 0 | V |
| Duration | t | 240 minimum | hours |

NOTES:

Voltage may be switched off during cool down.

## HIGH TEMPERATURE STEADY-STATE GATE BIAS BURN-IN CONDITIONS

HTGB Burn-in shall be performed in accordance with MIL-STD-750, Test Method 1042, Test Condition B with the following conditions:

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | °C |
| Drain-to-Source Voltage | VDS | 0 | V |
| Gate-to-Source Voltage | VGS | 16 | V |
| Duration | t | 48 minimum | hours |

## OPERATING LIFE CONDITIONS

Operating Life shall consist of High Temperature Reverse Bias in accordance with MIL-STD-750, Test Method 1042, Test Condition A, followed by High Temperature Steady-State Gate Bias in accordance with MIL-STD-750, Test Method 1042, Test Condition B. The test conditions are as follows:

High Temperature Reverse Bias Conditions

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | °C |
| Drain-to-Source Voltage | VDS | 200 (Note 1) | V |
| Gate-to-Source Voltage | VGS | 0 | V |
| Duration | t | 1000 minimum | Hours |

High Temperature Steady-State Gate Bias Conditions

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | °C |
| Drain-to-Source Voltage | VDS | 0 | V |
| Gate-to-Source Voltage | VGS | 16 | V |
| Duration | t | 1000 minimum | hours |

NOTES:

Voltage may be switched off during cool down.

## TOTAL DOSE RADIATION TESTING

### Bias Conditions and Total Dose Level for Total Dose Radiation Testing

The following bias condition shall be used during irradiation testing:

VGS = +15V

VDS = 0V

The total dose level applied shall be as specified in Component Type Variants herein or in the Purchase Order.

### Electrical Measurements for Total Dose Radiation Testing

Prior to irradiation testing the devices shall have successfully met Room Temperature Electrical Measurements specified herein.

Unless otherwise stated the measurements shall be performed at Tamb = +25 ±3°C.

Unless otherwise specified the test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The parameters to be measured during and on completion of irradiation testing are shown below.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value (Δ) | Absolute |  |  |
|  |  |  | Min | Max |  |
| Drain-to-Source Breakdown Voltage | V(BR)DSS | ±20% | 250 | - | V |
| Gate-to-Source Threshold Voltage | VGS(th) | +10%, -50% | 2 | 4 | V |
| Gate-to-Source Leakage Current | IGSS | ±20 | -100 | +100 | nA |
| Drain Current | IDSS | - | - | 25 | µA |
| Static Drain-to-Source On Resistance Variant 01 Variant 02 | rDS(on) | ±20% | - - | 130 400 | mΩ |
| Source-to-Drain Diode Forward Voltage | VSD | ±10% | - | 1.2 | V |

## APPENDIX ‘A’

AGREED DEVIATIONS FOR INFINEON TECHNOLOGIES (D)

| ITEMS AFFECTED | DESCRIPTION OF DEVIATIONS |
| --- | --- |
| Deviations from Production Control (Chart F2) | The 3 component sample Dimension Check need only be performed once on each component package production lot. |
| Deviations from Screening Tests (Chart F3) | Temperature Cycling shall be performed in accordance with MIL-STD-883, Test Method 1010, Test Condition C, 20 cycles at maximum storage temperature rating specified in the Detail Specification. |
|  | High and Low Temperatures Electrical Measurements may be performed at any point after High Temperature Steady-State Gate Bias Burn-in, prior to Seal, but shall still count towards Check for Lot Failure. |
|  | Radiographic Inspection is not applicable. |
|  | Seal, Fine Leak shall be performed in accordance with MIL-STD-883, Test Method 1014, Test Condition A1 or A2. |
|  | Solderability is not applicable unless otherwise stipulated in the Purchase Order. |
| Deviations from Qualification and Periodic Tests (Chart F4) | Temperature Cycling shall be performed in accordance with MIL-STD-883, Test Method 1010, Test Condition C, 100 cycles at maximum storage temperature rating specified in the Detail Specification. |
|  | Seal, Fine Leak shall be performed in accordance with MIL-STD-883, Test Method 1014, Test Condition A1 or A2. |
| Room Temperature Electrical Measurements | The read and record 32 component sample electrical measurements for characteristics td(on), tr, td(off), tf, trr, Ciss, Coss, Crss, Qg, Qgs and Qgd need only be performed once on each wafer lot used to supply components to this specification. Any failure shall result in rejection of the wafer lot. The sample measurement may be performed at any time during production. |
| Marking | For the purposes of marking of the ESCC Component Number on the body of the component, the Variant Number may be marked as a single digit (e.g. 1 for Variant 01). Otherwise the full ESCC Component Number shall be used. |

ADDITIONAL DATA - INFINEON TECHNOLOGIES (D)

Derating for Space Application

- These components are susceptible to Single Event Gate Rupture if operated in a space environment unless the following derating is applied: