Page 1 of 16

CAPACITORS, FIXED, TUBULAR, POROUS TANTALUM CATHODE AND ANODE, GELLED ELECTROLYTE, HERMETICALLY SEALED

BASED ON TYPE sT79

ESCC Detail Specification No. 3003/006

| Issue 3 Draft B | April 2016 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2016. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| 984 | Specification upissued to incorporate changes per DCR. |

TABLE OF CONTENTS

1	GENERAL	6

1.1	SCOPE	6

1.2	RANGE OF COMPONENTS	6

1.3	MAXIMUM RATINGS	6

1.4	PARAMETER DERATING INFORMATION	6

1.5	PHYSICAL DIMENSIONS	6

1.6	FUNCTIONAL DIAGRAM	6

2	APPLICABLE DOCUMENTS	9

3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	9

4	REQUIREMENTS	9

4.1	GENERAL	9

4.2	DEVIATIONS FROM GENERIC SPECIFICATION	9

4.2.1	Deviations from Special In-process Controls	9

4.2.2	Deviations from Final Production Tests (Chart II)	9

4.2.3	Deviations from Burn-in and Electrical Measurements (Chart III)	9

4.2.4	Deviations from Qualification Tests (Chart IV)	9

4.2.5	Deviations from Lot Acceptance Tests (Chart V)	9

4.3	MECHANICAL REQUIREMENTS	10

4.3.1	Dimension Check	10

4.3.2	Weight	10

4.3.3	Robustness of Terminations	10

4.4	MATERIALS AND FINISHES	10

4.4.1	Case	10

4.4.2	Lead Material and Finish	10

4.4.3	Insulation Sleeving	10

4.5	MARKING	10

4.5.1	General	10

4.5.2	Polarity	10

4.5.3	The ESCC Component Number	11

4.5.4	Characteristics and Ratings	11

4.5.5	Traceability Information	11

4.6	ELECTRICAL MEASUREMENTS	12

4.6.1	Electrical Measurements at Room Temperature	12

4.6.2	Electrical Measurements at High and Low Temperatures	12

4.7	Burn-In Tests	12

4.7.1	Parameter Drift Values	12

4.7.2	Conditions for Burn-in	12

4.8	ENVIRONMENTAL AND ENDURANCE TESTS (CHARTS IV AND V OF ESCC GENERIC SPECIFICATION No. 3003)	14

4.8.1	Measurements and Inspections on Completion of Environmental Tests	14

4.8.2	Measurements at Intermediate Points during Endurance Tests	14

4.8.3	Measurements and Inspections on Completion of Endurance Tests	14

4.8.4	Conditions for Operating Life	14

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, test and inspection data for Capacitors, Fixed, Tubular, Porous Tantalum Cathode and Anode, Gelled Electrolyte, Hermetically Sealed, Based on Type ST79. It shall be read in conjunction with ESCC Generic Specification No. 3003, the requirements of which are supplemented herein.

## RANGE OF COMPONENTS

The range of components covered by this specification is given in Table 1(a).

## MAXIMUM RATINGS

The maximum ratings, which shall not be exceeded at any time during use or storage, applicable to the components specified herein are scheduled in Table 1(b).

## PARAMETER DERATING INFORMATION

The parameter derating information applicable to the components specified herein is shown in Figure 1.

## PHYSICAL DIMENSIONS

The physical dimensions of the components specified herein are shown in Figure 2.

## FUNCTIONAL DIAGRAM

The functional diagram for the components specified herein is shown in Figure 3.

TABLE 1(a) - RANGE OF COMPONENTS

| UR (V) (Note 3) | Cn (µF) (Note 2) | IL max (µA) |  | DF max (%) at 100Hz | ΔC/C max (%) |  |  | Z max (Ω) at 100Hz | Case Size (Note 1) | Weight max (g) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | +22°C | +22°C | +85°C, +125°C | +22°C, +85°C, +125°C | +85°C | +125°C | -55°C | -55°C |  |  |
| 25 | 120 | 1 | 5 | 14 | +8 | +12 | -42 | 30 | A | 2.6 |
| 25 | 560 | 2 | 10 | 42 | +10 | +15 | -65 | 12 | B | 6.2 |
| 25 | 1200 | 5 | 20 | 70 | +12 | +18 | -70 | 7 | C | 11.6 |
| 25 | 1800 | 4 | 25 | 81 | +12 | +20 | -75 | 6 | D | 17.7 |
| 30 | 100 | 1 | 5 | 11 | +8 | +12 | -38 | 35 | A | 2.6 |
| 30 | 470 | 2 | 10 | 36 | +10 | +18 | -65 | 15 | B | 6.2 |
| 30 | 1000 | 7 | 25 | 63 | +10 | +18 | -70 | 7 | C | 11.6 |
| 30 | 1500 | 5 | 30 | 81 | +10 | +20 | -72 | 7 | D | 17.7 |
| 40 | 1000 | 8 | 65 | 40 | +10 | +20 | -60 | 8 | D | 17.7 |
| 50 | 68 | 1 | 5 | 9 | +8 | +15 | -25 | 40 | A | 2.6 |
| 50 | 220 | 2 | 10 | 18 | +8 | +15 | -50 | 20 | B | 6.2 |
| 50 | 470 | 3 | 25 | 31 | +8 | +15 | -50 | 10 | C | 11.6 |
| 50 | 680 | 5 | 40 | 43 | +10 | +20 | -60 | 8 | D | 17.7 |
| 50 | 750 | 6 | 55 | 40 | +10 | +20 | -60 | 8 | D | 17.7 |
| 60 | 47 | 1 | 5 | 9 | +8 | +12 | -25 | 60 | A | 2.6 |
| 60 | 150 | 2 | 10 | 15 | +8 | +15 | -40 | 25 | B | 6.2 |
| 60 | 390 | 3 | 25 | 31 | +8 | +15 | -50 | 13 | C | 11.6 |
| 60 | 560 | 5 | 40 | 40 | +8 | +15 | -60 | 8 | D | 17.7 |
| 60 | 700 | 7 | 60 | 42 | +10 | +20 | -60 | 8 | D | 17.7 |
| 63 | 500 | 5 | 40 | 40 | +8 | +15 | -60 | 8 | D | 17.7 |
| 75 | 33 | 1 | 5 | 8 | +5 | +9 | -25 | 85 | A | 2.6 |
| 75 | 330 | 3 | 30 | 29 | +10 | +20 | -50 | 12 | C | 11.6 |
| 75 | 470 | 5 | 50 | 38 | +10 | +20 | -60 | 10 | D | 17.7 |
| 100 | 15 | 1 | 5 | 5 | +3 | +10 | -18 | 165 | A | 2.6 |
| 100 | 68 | 2 | 10 | 12 | +4 | +12 | -30 | 45 | B | 6.2 |
| 100 | 150 | 3 | 25 | 21 | +6 | +12 | -38 | 22 | C | 11.6 |
| 100 | 220 | 5 | 50 | 23 | +6 | +12 | -50 | 18 | D | 17.7 |
| 125 | 10 | 1 | 5 | 5 | +3 | +10 | -15 | 240 | A | 2.6 |
| 125 | 47 | 2 | 10 | 10 | +5 | +12 | -25 | 60 | B | 6.2 |
| 125 | 100 | 3 | 25 | 16 | +5 | +10 | -25 | 35 | C | 11.6 |
| 125 | 150 | 5 | 50 | 21 | +6 | +12 | -35 | 22 | D | 17.7 |

Notes:

Refer to Figure 2 for Type Variant Number assigned to case size.

All capacitors are available with a tolerance of ±10% (K) and ±20% (M).

For Tamb ≤ +85°C (See Figure 1).

TABLE 1(b) - MAXIMUM RATINGS

| No. | Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Rated Voltage | UR | See Table 1(a) | V | For derating, see Figure 1 |
| 2 | Surge Voltage | US | 1.15UR | V |  |
| 3 | Operating Temperature Range | Top | -55 to +125 | °C | Tamb For derating, see Figure 1 |
| 4 | Storage Temperature Range | Tstg | -55 to +125 | °C | Tamb |
| 5 | Maximum Soldering Temperature | TL | +260 | °C | Soldering time: tL ≤ 5s (Note 1) |

NOTES:

            - ≥ 3mm from body on cathode side and ≥ 3mm from weld on anode side.

FIGURE 1 - PARAMETER DERATING INFORMATION

rated voltage versus ambient temperature (see table 1(b))

FIGURE 2 - PHYSICAL DIMENSIONS

| Variant | Case Size | Dimensions (mm) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A Max | B Max | C Min | D Max | E |  | F Max |
|  |  |  |  |  |  | Min | Max |  |
| 02 | A | 18 | 12.43 | 30 | 5.6 | 0.59 | 0.7 | 1.6 |
| 03 | B | 23 | 17.2 | 30 | 7.6 | 0.59 | 0.7 | 1.6 |
| 04 | C | 26 | 20.4 | 30 | 10 | 0.59 | 0.7 | 1.6 |
| 05 | D | 34 | 27.9 | 30 | 10 | 0.59 | 0.7 | 1.6 |

NOTES:

            - The anode lead-out stub must not be bent.

FIGURE 3 - FUNCTIONAL DIAGRAM

# APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

          - ESCC Generic Specification No. 3003 for Capacitors, Fixed, Tantalum, Non-solid Electrolyte.

# TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are stated in this specification and ESCC Generic Specification No. 3003. Deviations from the Generic Specification, applicable to this specification only, are listed in Para. 4.2.

Deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirements and do not affect the components' reliability, are listed in the appendices attached to this specification.

## DEVIATIONS FROM GENERIC SPECIFICATION

### Deviations from Special In-process Controls

Not applicable.

### Deviations from Final Production Tests (Chart II)

None.

### Deviations from Burn-in and Electrical Measurements (Chart III)

None.

### Deviations from Qualification Tests (Chart IV)

None.

### Deviations from Lot Acceptance Tests (Chart V)

None.

## MECHANICAL REQUIREMENTS

### Dimension Check

The dimensions of the components specified herein shall be verified in accordance with the requirements set out in Para. 9.4 of ESCC Generic Specification No. 3003 and shall conform to those shown in Figure 2 of this specification.

### Weight

The maximum weight of the components specified herein shall be as specified in Table 1(a).

### Robustness of Terminations

The requirements and test conditions for robustness of terminations are specified in Para. 9.9 of ESCC Generic Specification No. 3003.

## MATERIALS AND FINISHES

The materials and finishes shall be as specified herein. Where a definite material is not specified, a material which will enable the capacitors specified herein to meet the performance requirements of this specification shall be used. Acceptance or approval of any constituent material does not guarantee acceptance of the finished product.

### Case

Tantalum, hermetically sealed.

### Lead Material and Finish

Leads shall be Type E with Type 3 finish in accordance with the requirements of ESCC Basic Specification No. 23500. Leads shall be electrically welded to the case and shall be free from nonconductive and foreign materials.

### Insulation Sleeving

Insulation Sleeving shall be of a non-fungus-nutrient material (cardboard shall not be used). The material shall not soften, creep or shrink to the extent that it causes any part of the cylindrical case to become uncovered at any test temperature specified herein. At any cross-section, the maximum thickness of the sleeving shall not exceed twice its minimum thickness.

## MARKING

### General

The marking of all components delivered to this specification shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and the following paragraphs. When the component is too small to accommodate all the marking specified, as much as space permits shall be marked and the marking information, in full, shall accompany each component in its primary package.

The information to be marked and the order of precedence shall be as follows:

          - Polarity

          - The ESCC qualified components symbol (for ESCC qualified components only).

          - The ESCC Component Number (including Characteristics and Ratings).

          - Traceability Information.

### Polarity

The polarity shall be indicated by a + symbol on the anode end of the case.

### The ESCC Component Number

The ESCC Component Number shall be constituted and marked as follows:

Example: 300300605B567KM

Detail Specification Number: 3003006

Type Variant (see Figure 2): 05 (as required)

Testing Level: B (B or C, as applicable)

Characteristic code: Capacitance Value (560µF): 567 (as required)

Characteristic code: Capacitance Tolerance (±10%): K (as required)

Rating code: Rated Voltage (60V): M (as required)

### Characteristics and Ratings

The characteristics and ratings to be codified as part of the ESCC Component Number shall be as follows:

          - Rated Capacitance Value Cn expressed by means of the following codes in accordance with ESCC Basic Specification No. 21700. 	The unit quantity for marking shall be picofarads (pF).

| Capacitance Value Cn (pF) | Code |
| --- | --- |
| XX106 | XX6 |
| XX107 | XX7 |
| XX108 | XX8 |

          - Capacitance Tolerance expressed by the following codes in accordance with ESCC Basic Specification No. 21700:

| Tolerance (± %) | Code Letter |
| --- | --- |
| 10 | K |
| 20 | M |

          - Rated Voltage expressed by the following codes:

| Rated Voltage UR (V) | Code Letter |
| --- | --- |
| 25 | F |
| 30 | H |
| 40 | J |
| 50 | K |
| 60 | M |
| 63 | L |
| 75 | P |
| 100 | Q |
| 125 | R |

### Traceability Information

Traceability information shall be marked in accordance with the requirements of ESCC Basic Specification No. 21700.

## ELECTRICAL MEASUREMENTS

### Electrical Measurements at Room Temperature

The parameters to be measured at room temperature are scheduled in Table 2. The measurements shall be performed at Tamb = +22 ±3°C.

### Electrical Measurements at High and Low Temperatures

The parameters to be measured at high and low temperatures are scheduled in Table 3.

## Burn-In Tests

### Parameter Drift Values

The parameter drift values applicable to burn-in are specified in Table 4 of this specification. Unless otherwise stated, measurements shall be performed at Tamb = +22 ±3°C. The parameter drift values (Δ) applicable to the parameters scheduled shall not be exceeded. In addition to these drift value requirements for a given parameter, the appropriate limit value specified in Table 2 shall not be exceeded.

### Conditions for Burn-in

The requirements for burn-in are specified in Section 7 of ESCC Generic Specification No. 3003. The conditions for burn-in shall be as specified in Table 5 of this specification. On completion of burn-in, a recovery period of 24 ±2 hours is necessary before the end-measurements.

TABLE 2 - ELECTRICAL MEASUREMENTS AT ROOM TEMPERATURE (NOTE 1)

| No. | Characteristics | Symbols | ESCC 3003 Test Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| 1 | DC Leakage | IL | Para. 9.2.1.1 | See Table 1(a) |  | µA |
| 2 | Capacitance Value | C | Para. 9.2.1.2 | See Table 1(a) |  | µF |
| 3 | Dissipation Factor | DF | Para. 9.2.1.3 | See Table 1(a) |  | % |

NOTES:

            - Measurements shall be made in the order shown. On completion of electrical measurements of Chart III and LAT3, each capacitor shall be discharged through a 470Ω resistor for 5s followed by short circuit for 2 minutes.

TABLE 3 - ELECTRICAL MEASUREMENTS AT HIGH AND LOW TEMPERATURES

(NOTEs 1, 2)

| No. | Characteristics | Symbols | ESCC 3003 Test Conditions | Test Temperature (Tamb) | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Min | Max |  |
| 1 2 3 | DC Leakage Capacitance Dissipation Factor (Note 3) | IL C DF | Para. 9.2.1.1 Para. 9.2.1.2 Para. 9.2.1.3 | +22 ±3°C | See Table 1(a) |  | µA µF % |
| 4 5 | Impedance Capacitance | Z C | - Para. 9.2.1.2 | -55 (+0 -3)°C | See Table 1(a) |  | Ω µF |
| 6 7 8 | DC Leakage Capacitance Dissipation Factor (Note 3) | IL C DF | Para. 9.2.1.1 Para. 9.2.1.2 Para. 9.2.1.3 | +22 ±3°C | See Table 1(a) |  | µA µF % |
| 9 10 11 | DC Leakage Capacitance Dissipation Factor | IL C DF | Para. 9.2.1.1 Para. 9.2.1.2 Para. 9.2.1.3 | +85 (+3 -0)°C | See Table 1(a) |  | µA µF % |
| 12 13 14 | DC Leakage (Note 4) Capacitance Dissipation Factor | IL C DF | Para. 9.2.1.1 Para. 9.2.1.2 Para. 9.2.1.3 | +125 (+3 -0)°C | See Table 1(a) |  | µA µF % |
| 15 16 17 | DC Leakage Capacitance Dissipation Factor | IL C DF | Para. 9.2.1.1 Para. 9.2.1.2 Para. 9.2.1.3 | +22 ±3°C | See Table 1(a) |  | µA µF % |

NOTES:

            - Measurements shall be made in the order shown. Capacitors shall be brought to thermal stability before the measurements are made. Thermal stability will have been reached when no further change in capacitance is observed between 2 successive measurements made at 15 minute intervals.

            - Inspection Level II, single sampling, AQL = 2.5%, for each capacitance value. Each capacitance value shall be considered as constituting a complete lot.

            - These measurements shall not be performed during Chart III.

            - Derated voltage, 0.67UR, shall be applied for this measurement.

TABLE 4 - PARAMETER DRIFT VALUES

| No. | Characteristics | Symbols | ESCC 3003 Test Conditions | Change Limits (Δ) | Units |
| --- | --- | --- | --- | --- | --- |
| 1 | DC Leakage Change | ΔIL/IL | Para. 9.2.1.1 | (+200, -100) % of measured value or +(25% +0.05µA) of limit value (Notes 1, 2) | - |
| 2 | Capacitance Change | ΔC/C | Para. 9.2.1.2 | ±5 | % |

NOTES:

            - Whichever is smaller.

            - Leakage currents need not be recorded when less than, or equal to:

0.5µA for case sizes A and B.

0.8µA for case size C.

1.4µA for case size D.

TABLE 5 - CONDITIONS FOR BURN-IN AND OPERATING LIFE

| No. | Characteristics | Symbols | Conditions | Units |
| --- | --- | --- | --- | --- |
| 1 | Ambient Temperature | Tamb | +85 (+0 -3) | °C |
| 2 | Test Voltage | UR | UR (Note 1) | V |

NOTES:

            - See Table 1(a).

## ENVIRONMENTAL AND ENDURANCE TESTS (CHARTS IV AND V OF ESCC GENERIC SPECIFICATION No. 3003)

### Measurements and Inspections on Completion of Environmental Tests

The parameters to be measured and inspections to be performed on completion of environmental tests are scheduled in Table 6. Unless otherwise stated, the measurements shall be performed at Tamb = + 22 ±3°C.

### Measurements at Intermediate Points during Endurance Tests

The parameters to be measured at intermediate points during endurance tests are scheduled in Table 6. Unless otherwise stated, the measurements shall be performed at Tamb = + 22 ±3°C.

### Measurements and Inspections on Completion of Endurance Tests

The parameters to be measured and inspections to be performed on completion of endurance tests are scheduled in Table 6. Unless otherwise stated, the measurements shall be performed at Tamb = + 22 ±3°C.

### Conditions for Operating Life

The requirements for operating life test are specified in Section 9 of ESCC Generic Specification No. 3003. The conditions for operating life testing shall be as specified in Table 5 of this specification.

TABLE 6 - MEASUREMENTS AND INSPECTIONS ON COMPLETION OF ENVIRONMENTAL TESTS AND AT INTERMEDIATE POINTS AND ON COMPLETION OF ENDURANCE TESTING

| No. | ESCC Generic Spec. No. 3003 |  | Measurements and Inspections |  | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Environmental and Endurance Tests (1) | Test Methods and Conditions | Identification | Conditions |  | Min | Max |  |
| 01 | Rapid Change of Temperature | Para. 9.5 | DC Leakage | Table 2, Item 1 | IL | 2.5 x Limit Table 1(a) |  | µA |
|  |  |  | Capacitance Change | Table 2, Item 2 | ΔC/C | -15 | +15 | % |
|  |  |  | Dissipation Factor | Table 2, Item 3 | DF | 1.5 x Limit Table 1(a) |  | % |
|  |  |  | Visual Examination | No leakage of electrolyte | - | - |  | - |
| 02 | Vibration | Para. 9.6 | Visual Examination | No damage or leakage of electrolyte | - | - | - | - |
| 03 | Shock | Para. 9.7 | Visual Examination | No breakdown, arcing, damage or leakage of electrolyte | - | - | - | - |
| 04 | Climatic Sequence | Para. 9.17 | DC Leakage | Table 2, Item 1 | IL | 2.5 x Limit Table 1(a) |  | µA |
|  |  |  | Capacitance Change | Table 2, Item 2 | ΔC/C | -15 | +15 | % |
|  |  |  | Dissipation Factor | Table 2, Item 3 | DF | 1.5 x Limit Table 1(a) |  | % |
|  |  |  | Visual Examination | No damage or leakage of electrolyte |  |  |  |  |
| 05 | Seal Test | Para. 9.1.2 | - | - | - | - | - | - |
| 06 | External Visual Inspection | Para. 9.3 | - | - | - | - | - | - |
| 07 | Solderability | Para. 9.8 | - | - | - | - | - | - |
| 08 | Robustness of Terminations | Para. 9.9 | Visual Examination | No damage or leakage of electrolyte | - | - | - | - |
| 09 | Resistance to Soldering Heat | Para. 9.10 | Visual Examination | No damage or leakage of electrolyte | - | - | - | - |
| 10 | Damp Heat, Steady State | Para. 9.11 | DC Leakage | Table 2, Item 1 | IL | 2 x Limit Table 1(a) |  | µA |
|  |  |  | Capacitance Change | Table 2, Item 2 | ΔC/C | -10 | +10 | % |
|  |  |  | Dissipation Factor | Table 2, Item 3 | DF | 1.5 x Limit Table 1(a) |  | % |
|  |  |  | Insulation Resistance | Gen. 3003, Para. 9.2.1.4 | RI | 100 | - | MΩ |
|  |  |  | Visual Examination | No breakdown, arcing, corrosion, damage or leakage of electrolyte and marking legible | - | - | - | - |
| 11 | Cold Test | Para. 9.12 | DC Leakage | Table 2, Item 1 | IL | 2 x Limit Table 1(a) |  | µA |
|  |  |  | Capacitance Change | Table 2, Item 2 | ΔC/C | -5 | +5 | % |
|  |  |  | Dissipation Factor | Table 2, Item 3 | DF | 1.5 x Limit Table 1(a) |  | % |
|  |  |  | Visual Examination | No leakage of electrolyte | - | - | - | - |
| 12 | Low Air Pressure | Para. 9.13 | Visual Examination | No breakdown, arcing, damage or leakage of electrolyte | - | - | - | - |
| 13 | Surge Voltage | Para. 9.14 | Visual Examination | No damage or leakage of electrolyte | - | - | - | - |
| 14 | Reverse Voltage | Para. 9.15 | DC Leakage | Table 2, Item 1 | IL | 2 x Limit Table 1(a) |  | µA |
|  |  |  | Capacitance Change | Table 2, Item 2 | ΔC/C | -10 | +10 | % |
|  |  |  | Dissipation Factor | Table 2, Item 3 | DF | 1.5 x Limit Table 1(a) |  | % |
| 15 | Life Test | Para. 9.16 |  | At 500, 1000 & 2000 hours and after 24 hours recovery |  |  |  |  |
|  |  |  | DC Leakage | Table 2, Item 1 | IL | 2 x Limit Table 1(a) |  | µA |
|  |  |  | Capacitance Change | Table 2, Item 2 | ΔC/C | -10 | +10 | % |
|  |  |  | Dissipation Factor | Table 2, Item 3 | DF | 1.5 x Limit Table 1(a) |  | % |
|  |  |  | Insulation Resistance | Gen. 3003, Para. 9.2.1.4 | RI | 100 | - | MΩ |
| 16 | High and Low Temperature Measurements | Para. 9.2.3 | DC Leakage Capacitance Dissipation Factor (2) Impedance | Table 3, Items 1 to 17 | IL C DF Z | See Table 3 |  | µA µF % Ω |
| 17 | Electrical Measurements | Para. 9.2.4 | DC Leakage Capacitance Dissipation Factor | Table 2, Item 1 Table 2, Item 2 Table 2, Item 3 | IL C DF | See Table 2 See Table 2 See Table 2 |  | µA µF % |
| 18 | Permanence of Marking | Para. 9.18 | - | - | - | - |  | - |

NOTES:

            - The tests in this Table refer to either Chart IV or V and shall be used as applicable.

            - Not performed at low temperature.