Page 1 of 18

Power INDUCTORS, MOULDED, SMD

BASED ON SERIES SESI

ESCC Detail Specification No. 3201/009

| Issue 13 Draft B | July 2016 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2016. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| TBA | Specification upissued to incorporate changes per DCR. |
| 13/07/2016 S.Thacker (TW) | First draft revision similar to ESCC 3201/008 Draft 7B to go with ESCC 3201 Draft 6C. Content based on 3201/009 issue 12. |
| 27/07/2016 S.Thacker (TW) | New revision per Exxelia review of Draft 7B per emails dated 27/07/16 & 28/07/16. Changes in this revision are highlighted in yellow. All agreed comments have been removed. |

TABLE OF CONTENTS

1	General	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.1.1	Characteristics Codes	5

1.4.2	Component Type Variants and Range of Components	6

1.5	MAXIMUM RATINGS	12

1.6	PHYSICAL DIMENSIONS and terminal identification	13

1.7	FUNCTIONAL DIAGRAM	14

1.8	MATERIALS AND FINISHES	15

1.8.1	Case	15

1.8.2	Terminals	15

2	REQUIREMENTS	15

2.1	GENERAL	15

2.1.1	Deviations from the Generic Specification	15

2.1.1.1	Deviations from Qualification and Periodic Tests (Chart F4)	15

2.2	MARKING	15

2.3	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	16

2.3.1	Room Temperature Electrical Measurements	16

2.3.2	High and Low Temperatures Electrical Measurements	16

2.4	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	17

2.5	BURN-IN CONDITIONS	18

2.6	OPERATING LIFE CONDITIONS	18

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3201.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 3201009014L7M

Detail Specification Reference: 3201009

Component Type Variant Number: 01 (as required)

Characteristic code: Inductance Value (4.7µH): 4L7 (as required)

Characteristic code: Inductance Tolerance (±20%): M (as required)

#### Characteristics Codes

Characteristics to be codified as part of the ESCC Component Number shall be as follows:

Inductance Value expressed by means of the following codes in accordance with ESCC Basic Specification No. 21700. The unit quantity shall be microhenries (µH):

| Inductance Value L (µH) | Code |
| --- | --- |
| X.X | XLX |
| XX | XX0 |
| XX 101 | XX1 |
| XX 102 | XX2 |

Inductance Tolerance expressed by the following codes in accordance with ESCC Basic Specification No. 21700:

| Tolerance (± %) | Code Letter |
| --- | --- |
| 10 | K |
| 20 | M |
| 30 | N |

### Component Type Variants and Range of Components

The component type variants and range of components applicable to this specification are as follows:

| Variant Number | Type | Case Description (1) | Terminal Finish (2) | Weight Max (g) |
| --- | --- | --- | --- | --- |
| 01 | SESI 14SR | Size 14 with 2 SMD terminals | Sn60Pb40 | 4 |
| 02 | SESI 15SR | Size 15 with 2 SMD terminals | Sn60Pb40 | 5.5 |
| 03 | SESI 15WR | Size 15 with 8 Gullwing terminals | Sn60Pb40 | 6 |
| 04 | SESI 18WR | Size 18 with 8 Gullwing terminals | Sn60Pb40 | 11 |
| 05 | SESI 9.1WR | Size 9.1 with 4 Gullwing terminals | Sn60Pb40 | 2 |
| 06 | SESI 22WR | Size 22 with 8 Gullwing terminals | Sn60Pb40 | 28 |
| 07 | SESI 32WR | Size 32 with 16 Gullwing terminals | Sn60Pb40 | 58 |
| 08 | SESI 32PR | Size 32 with 16 Pin terminals | Sn60Pb40 | 58 |

RANGE OF COMPONENTS – VARIANT 01 (SESI 14SR)

| Inductance (3) L (μH) | Tolerance % | Rated DC Current IR (A) | Loaded Inductance at IR (3, 4) LR (μH) | Peak Current (5) IP (A) | Max. DC Resistance RDC (mΩ) |
| --- | --- | --- | --- | --- | --- |
| 3.3 | 20 | 5.8 | 2.3 | 8 | 15 |
| 4.7 | 20 | 5.4 | 3.3 | 6.9 | 17.5 |
| 6 | 20 | 4.3 | 4.2 | 5.7 | 26.5 |
| 8.2 | 20 | 3.7 | 5.7 | 5.2 | 42 |
| 10 | 20 | 3.3 | 7 | 4.6 | 47 |
| 15 | 20 | 2.7 | 10.5 | 3.8 | 90 |
| 22 | 20 | 2.2 | 15.4 | 3 | 110 |
| 33 | 20 | 1.8 | 23.1 | 2.5 | 170 |
| 47 | 10 | 1.6 | 32.9 | 2.1 | 200 |
| 56 | 10 | 1.5 | 39.2 | 1.9 | 240 |
| 68 | 10 | 1.3 | 47.6 | 1.7 | 290 |
| 82 | 10 | 1.2 | 57.4 | 1.5 | 315 |
| 100 | 10 | 1.1 | 70 | 1.4 | 440 |
| 120 | 10 | 1 | 84 | 1.3 | 500 |
| 150 | 10 | 0.9 | 105 | 1.1 | 645 |
| 180 | 10 | 0.83 | 126 | 1 | 740 |
| 220 | 10 | 0.72 | 154 | 1 | 980 |
| 330 | 10 | 0.57 | 231 | 0.8 | 1575 |

RANGE OF COMPONENTS – VARIANTS 02 and 03 (SESI 15SR, SESI 15WR)

| Inductance (3) L (μH) | Tolerance % | Rated DC Current IR (A) | Loaded Inductance at IR (3, 4) LR (μH) | Peak Current (5) IP (A) | Max. DC Resistance RDC (mΩ) |
| --- | --- | --- | --- | --- | --- |
| 1.5 | 30 | 14 | 0.9 | 19 | 5 |
| 1.8 | 30 | 10 | 1.05 | 14 | 5 |
| 2.7 | 20 | 8.2 | 1.9 | 11.5 | 6.5 |
| 4.9 | 20 | 6 | 3.4 | 8.5 | 11 |
| 6.4 | 20 | 5.3 | 4.5 | 7.5 | 12 |
| 8 | 20 | 4.8 | 5.6 | 6.5 | 16 |
| 12 | 20 | 4 | 8.4 | 5.5 | 23 |
| 16 | 20 | 3.4 | 11.2 | 4.5 | 27 |
| 18 | 20 | 3.1 | 12.6 | 4.2 | 29 |
| 21 | 20 | 2.9 | 14.7 | 4 | 36 |
| 27 | 20 | 2.6 | 18.9 | 3.5 | 44 |
| 33 | 20 | 2.3 | 23 | 3.2 | 59 |
| 48 | 10 | 1.9 | 33 | 2.7 | 72 |
| 56 | 10 | 1.8 | 39 | 2.5 | 82 |
| 68 | 10 | 1.6 | 47 | 2.2 | 110 |
| 82 | 10 | 1.5 | 57 | 2.1 | 120 |
| 100 | 10 | 1.35 | 70 | 1.9 | 155 |
| 120 | 10 | 1.2 | 84 | 1.7 | 180 |
| 150 | 10 | 1.1 | 105 | 1.5 | 230 |
| 220 | 10 | 0.9 | 154 | 1.3 | 355 |
| 330 | 10 | 0.74 | 231 | 1 | 630 |
| 1000 | 10 | 0.38 | 800 | 0.5 | 2130 |
| 2300 | 10 | 0.28 | 1900 | 0.36 | 4400 |

RANGE OF COMPONENTS – Variant 04 (SESI 18WR)

| Inductance (3) L (μH) | Tolerance % | Rated DC Current IR (A) | Loaded Inductance at IR (3, 4) LR (μH) | Peak Current (5) IP (A) | Max. DC Resistance RDC (mΩ) |
| --- | --- | --- | --- | --- | --- |
| 6.8 | 20 | 9.8 | 4.2 | 13.6 | 7.5 |
| 8.2 | 20 | 8.3 | 5.7 | 11.5 | 9 |
| 11 | 20 | 7.2 | 7.7 | 10 | 12 |
| 15 | 20 | 6.35 | 10.5 | 8.9 | 15 |
| 18 | 20 | 5.7 | 12.6 | 7.9 | 17 |
| 22 | 20 | 5.1 | 15.4 | 7.2 | 20 |
| 27 | 20 | 4.7 | 18.9 | 6.5 | 25 |
| 37 | 10 | 4 | 25.9 | 5.6 | 29 |
| 49 | 10 | 3.5 | 34.3 | 4.8 | 45 |
| 56 | 10 | 3.3 | 39 | 4.6 | 48 |
| 70 | 10 | 2.9 | 49 | 4.1 | 65 |
| 86 | 10 | 2.6 | 60 | 3.7 | 72 |
| 100 | 10 | 2.4 | 70 | 3.3 | 75 |
| 120 | 10 | 2.2 | 84 | 3.1 | 115 |
| 150 | 10 | 1.95 | 105 | 2.7 | 125 |
| 180 | 10 | 1.8 | 126 | 2.6 | 175 |
| 220 | 10 | 1.6 | 154 | 2.3 | 210 |
| 330 | 10 | 1.34 | 231 | 1.9 | 250 |
| 1000 | 10 | 0.38 | 800 | 0.5 | 2130 |
| 2300 | 10 | 0.28 | 1900 | 0.36 | 4400 |

RANGE OF COMPONENTS – VARIANT 05 (SESI 9.1WR)

| Inductance (3) L (μH) | Tolerance % | Rated DC Current IR (A) | Loaded Inductance at IR (3, 4) LR (μH) | Peak Current (5) IP (A) | Max. DC Resistance RDC (mΩ) |
| --- | --- | --- | --- | --- | --- |
| 1 | 30 | 6 | 0.6 | 11 | 8.5 |
| 1.5 | 30 | 5.4 | 0.9 | 9.5 | 11.5 |
| 2 | 30 | 4.3 | 1.4 | 8.2 | 17 |
| 2.6 | 20 | 3.6 | 1.8 | 7 | 23 |
| 3.4 | 20 | 3 | 2.4 | 6.2 | 35 |
| 4.3 | 20 | 2.8 | 3 | 5.5 | 40 |
| 6.2 | 20 | 2.3 | 4.3 | 4.3 | 59 |
| 8.5 | 20 | 1.9 | 6 | 3.7 | 87 |
| 10 | 20 | 1.85 | 7 | 3.4 | 93 |
| 15 | 20 | 1.5 | 10.5 | 2.8 | 140 |
| 18 | 20 | 1.27 | 12.6 | 2.5 | 192 |
| 22 | 20 | 1.21 | 15.4 | 2.3 | 215 |
| 26 | 20 | 1.03 | 18.2 | 2.14 | 290 |
| 33 | 10 | 0.92 | 23.1 | 1.9 | 350 |
| 47 | 10 | 0.8 | 32.9 | 1.6 | 470 |
| 66 | 10 | 0.73 | 46.2 | 1.3 | 565 |
| 81 | 10 | 0.63 | 56.7 | 1.21 | 745 |
| 100 | 10 | 0.6 | 70 | 1.1 | 795 |
| 150 | 10 | 0.53 | 105 | 0.8 | 750 |
| 220 | 10 | 0.43 | 154 | 0.7 | 1165 |
| 330 | 10 | 0.36 | 231 | 0.6 | 1475 |
| 470 | 10 | 0.3 | 329 | 0.5 | 2220 |
| 680 | 10 | 0.25 | 477 | 0.4 | 3255 |
| 1000 | 10 | 0.2 | 700 | 0.34 | 5865 |

RANGE OF COMPONENTS – Variant 06 (SESI 22WR)

| Inductance (3) L (μH) | Tolerance % | Rated DC Current IR (A) | Loaded Inductance at IR (3, 4) LR (μH) | Peak Current (5) IP (A) | Max. DC Resistance RDC (mΩ) |
| --- | --- | --- | --- | --- | --- |
| 7 | 20 | 18.9 | 3.8 | 24 | 5 |
| 7.7 | 20 | 16 | 5.4 | 20 | 4.5 |
| 10 | 20 | 13.8 | 7 | 17.7 | 5.5 |
| 13 | 20 | 12 | 9.1 | 15.6 | 7 |
| 19 | 20 | 10.9 | 11.5 | 14 | 11 |
| 24 | 20 | 8.4 | 16.8 | 11.5 | 13 |
| 33 | 20 | 7.7 | 23 | 9.8 | 20 |
| 47 | 10 | 5.7 | 37.6 | 8 | 16 |
| 64 | 10 | 5 | 51.2 | 7 | 21 |
| 82 | 10 | 4.3 | 65.6 | 6.1 | 24 |
| 100 | 10 | 3.9 | 80 | 5.5 | 30 |
| 150 | 10 | 3.2 | 120 | 4.7 | 44 |
| 210 | 10 | 2.7 | 168 | 3.8 | 70 |
| 340 | 10 | 2.1 | 272 | 3 | 120 |
| 470 | 10 | 1.8 | 376 | 2.5 | 180 |
| 680 | 10 | 1.5 | 544 | 2.1 | 220 |
| 820 | 10 | 1.4 | 656 | 2 | 300 |
| 1000 | 10 | 1.2 | 800 | 1.8 | 330 |
| 1500 | 10 | 1.1 | 1200 | 1.4 | 500 |
| 2200 | 10 | 0.8 | 1760 | 1.2 | 760 |

RANGE OF COMPONENTS – Variant 07 and 08 (SESI 32WR, SESI 32PR)

| Inductance (3) L (μH) | Tolerance % | Rated DC Current IR (A) | Loaded Inductance at IR (3, 4) LR (μH) | Peak Current (5) IP (A) | Max. DC Resistance RDC (mΩ) |
| --- | --- | --- | --- | --- | --- |
| 4.9 | 30 | 24 | 3 | 27.6 | 1.9 |
| 12 | 30 | 15.5 | 7.3 | 17.8 | 4.4 |
| 22 | 30 | 11.5 | 13.5 | 13.2 | 7.8 |
| 36 | 20 | 9.2 | 21.75 | 10.6 | 13 |
| 53 | 20 | 7.6 | 34.2 | 8.7 | 18 |
| 73 | 10 | 6.5 | 47.2 | 7.5 | 25 |
| 84 | 10 | 6 | 68 | 6.9 | 29 |
| 110 | 10 | 5.3 | 88 | 6.1 | 38.5 |
| 150 | 10 | 4.5 | 123 | 5.2 | 54.5 |
| 200 | 10 | 3.9 | 163 | 4.5 | 70 |
| 260 | 10 | 3.4 | 210 | 3.9 | 89.5 |
| 350 | 10 | 3 | 281 | 3.4 | 117.5 |
| 470 | 10 | 2.5 | 385 | 2.9 | 160 |
| 620 | 10 | 2.2 | 505 | 2.5 | 221 |
| 820 | 10 | 1.9 | 670 | 2.2 | 254 |
| 1000 | 10 | 1.7 | 826 | 2 | 353 |
| 2000 | 10 | 1.2 | 1650 | 1.4 | 665 |
| 4700 | 10 | 0.8 | 3760 | 0.92 | 1300 |

NOTES:

See Physical Dimensions and Terminal Identification.

See Materials and Finishes for details.

See Room Temperature Electrical Measurements and High and Low Temperature Electrical Measurements (as applicable) for test conditions.

Minimum value when the inductance is measured with Rated DC Current, IR, applied.

Peak current is the maximum current for a square pulse of duration < 10s.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| Rated DC Current | IR | Note 1 | A |  |
| Dielectric Withstanding Voltage | DWV | 500 | Vrms |  |
| Peak Current | IP | Note 1 |  | Square pulse < 10s |
| Operating Temperature Range | Top | -55 to +125 | °C | Tamb |
| Storage Temperature Range | Tstg | -55 to +140 | °C |  |
| Soldering Temperature | Tsol | +260 | °C | Note 2 |

NOTES:

See Component Type Variants and Range of Components for values.

Duration 5 seconds maximum, the same terminal shall not be resoldered until 3 minutes have elapsed.

## PHYSICAL DIMENSIONS and terminal identification

Variants 01, 02		                                                           Variants 03, 04, 06 (Note 1)

Variant 05 (Note 1)                                                     Variant 07 (Note 1)

variant 08 (Note 1)

| Symbols | Dimensions (mm) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Variant 01 |  | Variant 02 |  | Variant 03 |  | Variant 04 |  | Variant 05 |  | Variant 06 |  | Variant 07 |  | Variant 08 |  |
|  | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max |
| A | 15.7 | 16.3 | 15.7 | 16.3 | 16 | 16.5 | 21.7 | 22.3 | 10 | 10.6 | 30.4 | 31.2 | 42.1 | 42.9 | 42.1 | 42.9 |
| B | - | 16 | - | 16 | - | 16 | - | 19.8 | - | 10.7 | - | 23.5 | - | 33.5 | - | 33.5 |
| C | - | 5.4 | - | 7.4 | - | 7.9 | - | 8.9 | - | 5.8 | - | 12.2 | - | 13.7 | - | 13.7 |
| D | 2.3 | - | 2.3 | - |  |  |  |  |  |  |  |  |  |  |  |  |
| E | 1 | 1.5 | 1 | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |
| F |  |  |  |  | 21.1 | 22 | 25.1 | 26 | 12.8 | 13.4 | 34.4 | 35.3 | 52 | 53.5 | 48.1 | 48.5 |
| G |  |  |  |  | 3.7 | 3.9 | 3.7 | 3.9 | 5.6 | 5.8 | 3.7 | 3.9 | 3.7 | 3.9 | 3.7 | 3.9 |
| H |  |  |  |  | 1 | 1.2 | 1.9 | 2.1 | 1.3 | 1.5 | 1.9 | 2.1 | 0.8 | 1 | 0.8 | 1 |
| I |  |  |  |  | 1.3 | 1.6 | 1.3 | 1.6 | 0.9 | 1.3 | 1.3 | 1.6 | 2.15 | 2.35 | 4 | - |
| J |  |  |  |  | 0.2 | - | 0.2 | - | 0.2 | - | 0.2 | - | 0.4 | 0.7 | 0.4 | 0.7 |

NOTES:

Terminal identification: All terminals on each side of the body are connected together.

## FUNCTIONAL DIAGRAM

Variants 01, 02                                          Variants 03, 04, 06

Variant 05                                                 Variants 07, 08

## MATERIALS AND FINISHES

### Case

As a minimum, a resin moulding shall ensure the inductor’s protection.

### Terminals

The terminal material shall be brass, plated with 2 to 4µm of nickel or copper. The finish shall be Sn60Pb40.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Qualification and Periodic Tests (Chart F4)

Temperature Rise: Ambient test temperature shall be +90 ±5°C.

Immersion: shall not be performed.

Moisture Resistance, Polarisation: there shall be no polarising voltage applied during test.

## MARKING

The marking of all components delivered to this specification shall be in accordance with the requirements of ESCC Basic Specification No. 21700. When the component is too small to accommodate all of the marking specified, as much as space permits shall be marked and the marking information, in full, shall accompany each component in its primary package.

The information to be marked and the order of precedence, shall be as follows:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number.

Traceability information.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3 ºC.

| Characteristics | Symbols | Test Method and Conditions | Tolerance (± %) | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Inductance | L | ESCC No. 3201 f = 100kHz I = 0Adc V = 250mV | 10 | 0.9L (Note 1) | 1.1L (Note 1) | µH |
|  |  |  | 20 | 0.8L (Note 1) | 1.2L (Note 1) |  |
|  |  |  | 30 | 0.7L (Note 1) | 1.3L (Note 1) |  |
| Loaded Inductance | LR | ESCC No. 3201 f = 100kHz I = IR (Note 1) V = 250mV | All | Note 1 | - | µH |
| DC Resistance | RDC | ESCC No. 3201 | All | - | Note 1 | mΩ |
| Insulation Resistance | RI | ESCC No. 3201 V = 500Vdc | All | 1000 | - | MΩ |

NOTES:

See Component Type Variants and Range of Components for values.

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | Test Method and Conditions (Note 1) | Limits |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Loaded Inductance | LR | ESCC No. 3201 Tamb = +110(+0 -5)°C and -55 (+5 -0)°C f = 100kHz I = IR (Note 2) V = 250mV | Note 2 | - | µH |

NOTES:

The measurements shall be performed on a sample of 5 components from each lot with 0 failures allowed. In the event of any failure a 100% inspection may be performed.

See Component Type Variants and Range of Components for values.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3ºC.

Unless otherwise specified the test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

| Test Reference per ESCC No. 3201 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Thermal Shock | DC Resistance | RDC | Note 1 |  | mΩ |
| Barometric Pressure |  |  |  |  |  |
| Whilst at low pressure: | Dielectric Withstanding Voltage (2) | DWV | 500 | - | Vrms |
|  | DWV Leakage Current (3) | IL | - | 100 | µA |
| Temperature Rise | Test temperature: Tamb = +90 ±5°C |  |  |  |  |
| Initial Measurements | DC Resistance (at 0.1Ir) | RDC | Record Value |  | mΩ |
| Final Measurements (within 30s of the removal of power) | DC Resistance (at IR) | RDC | Record Value |  | mΩ |
|  | Temperature Rise | ΔT | - | 25 | °C |
| Overload | DC Resistance | RDC | Note 1 |  | mΩ |
|  | Insulation Resistance | RI | 1000 | - | MΩ |
|  | Dielectric Withstanding Voltage (2) | DWV | 500 | - | Vrms |
|  | DWV Leakage Current (3) | IL | - | 100 | µA |
| Resistance to Soldering Heat | Inductance | L | Note 1 |  | µH |
|  | DC Resistance | RDC | Note 1 |  | mΩ |
|  | Dielectric Withstanding Voltage (2) | DWV | 500 | - | Vrms |
|  | DWV Leakage Current (3) | IL | - | 100 | µA |
| Moisture Resistance |  |  |  |  |  |
| Within 30 min of removal from conditioning: | Dielectric Withstanding Voltage (2) | DWV | 500 | - | Vrms |
|  | DWV Leakage Current (3) | IL | - | 100 | µA |
|  | Insulation Resistance | RI | 1000 | - | MΩ |
| Within 1 hour of previous measurements: | Inductance | L | Note 1 |  | µH |
|  | DC Resistance | RDC | Note 1 |  | mΩ |
| Operating Life |  |  |  |  |  |
| Initial Measurements (0 hour) | Inductance | L | Note 1 |  | µH |
| Intermediate Measurements (1000 hours)(after 30 min recovery) | Dielectric Withstanding Voltage (2) | DWV | 500 | - | Vrms |
|  | DWV Leakage Current (3) | IL | - | 100 | µA |
|  | Inductance Change | ΔL/L | -10 | +10 | % |
| Final Measurements (2000 hours)(after 30 min recovery) | Dielectric Withstanding Voltage (2) | DWV | 500 | - | Vrms |
|  | DWV Leakage Current (3) | IL | - | 100 | µA |
|  | Inductance Change | ΔL/L | -10 | +10 | % |
|  | DC Resistance | RDC | Note 1 |  | mΩ |
|  | Insulation Resistance | RI | 1000 | - | MΩ |

NOTES:

See Component Type Variants and Range of Components for values.

Test method and conditions per ESCC No. 3201.

Measured during Dielectric Withstanding Voltage.

## BURN-IN CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +125 (+0 -3) | ºC |

NOTES:

After Burn-in, the components shall be removed from the chamber and allowed to cool under normal atmospheric conditions for recovery for 24 hours minimum.

## OPERATING LIFE CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +125 (+0 -3) | ºC |
| DC Current | I | IR (Note 1) | mA |

NOTES:

See Component Type Variants and Range of Components for values.