DCR to ESCC 3009/042 issue 1

Addition to paragraph 1.4.1.1:

(c) Rated Voltage expressed by the following codes

| Rated Voltage (V) | Code Letter |
| --- | --- |
| 10 | Y |
| 16 | X |
| 25 | A |
| 50 | C |

Addition to paragraph 1.4.2, Note 1.

            - Available rated voltages, capacitance values and tolerances are as follows:

| Rated Voltage UR (V) | Capacitance Range Cn (pF) |  | Tolerance (±) | Value Series |
| --- | --- | --- | --- | --- |
|  | Min | Max |  |  |
| 10 | 1 | 9.53 | 0.25pF | E48 |
|  |  | 9.1 | 0.5pF | E24 |
|  |  | 8.2 | 1pF | E12 |
|  | 10 | 330 | 1% | E96 |
|  |  |  | 2% | E48 |
|  |  |  | 5% | E24 |
|  |  |  | 10% | E12 |
| 16 | 1 | 9.53 | 0.25pF | E48 |
|  |  | 9.1 | 0.5pF | E24 |
|  |  | 8.2 | 1pF | E12 |
|  | 10 | 120 | 1% | E96 |
|  |  |  | 2% | E48 |
|  |  |  | 5% | E24 |
|  |  |  | 10% | E12 |
| 25 | 1 | 9.53 | 0.25pF | E48 |
|  |  | 9.1 | 0.5pF | E24 |
|  |  | 8.2 | 1pF | E12 |
|  | 10 | 100 | 1% | E96 |
|  |  |  | 2% | E48 |
|  |  |  | 5% | E24 |
|  |  |  | 10% | E12 |
| 50 | 1 | 9.53 | 0.25pF | E48 |
|  |  | 9.1 | 0.5pF | E24 |
|  |  | 8.2 | 1pF | E12 |
|  | 10 | 82 | 1% | E96 |
|  |  |  | 2% | E48 |
|  |  |  | 5% | E24 |
|  |  |  | 10% | E12 |

Any capacitance value in the capacitance range may be available on request.