DCR to ESCC 3009/023 issue 4

Addition to paragraph 1.4.1.1:

(c) Rated Voltage expressed by the following codes

| Rated Voltage (V) | Code Letter |
| --- | --- |
| 10 | Y |
| 16 | X |
| 25 | A |
| 50 | C |
| 100 | E |
| 200 | G |
| 400 | K |

Addition to paragraph 1.4.2, Note 1.

            - Available rated voltages, capacitance values and tolerances are as follows:

| Rated Voltage UR (V) | Capacitance Range Cn (pF) |  | Tolerance (± %) | Value Series |
| --- | --- | --- | --- | --- |
|  | Min | Max |  |  |
| 400 | 100 | 10000 | 5 | E24 |
|  |  |  | 10 | E12 |
|  |  |  | 20 | E6 |
| 200 | 100 | 47000 | 5 | E24 |
|  |  |  | 10 | E12 |
|  |  |  | 20 | E6 |
| 100 | 470 | 150000 | 5 | E24 |
|  |  |  | 10 | E12 |
|  |  |  | 20 | E6 |
| 50 | 470 | 220000 | 5 | E24 |
|  |  |  | 10 | E12 |
|  |  |  | 20 | E6 |
| 25 | 470 | 330000 | 5 | E24 |
|  |  |  | 10 | E12 |
|  |  |  | 20 | E6 |
| 16 | 470 | 1000000 | 5 | E24 |
|  |  |  | 10 | E12 |
|  |  |  | 20 | E6 |
| 10 | 470 | 1500000 | 5 | E24 |
|  |  |  | 10 | E12 |
|  |  |  | 20 | E6 |

Any capacitance value in the capacitance range may be available on request.