Changes applicable to ESCC5207/005 issue 4

Amend Para 1.4.1 & 1.4.2 as follows (in blue):

1.4.1	The ESCC Component Number

The ESCC Component Number shall be constituted as follows:
Example: 520700501R
•      Detail Specification Reference: 5207005

- •      Component Type Variant Number: 01 (as required)

- •      Total Dose Radiation Level Letter: R (as required)

1.4.2	Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Based on Type | Case | Lead/Terminal Material and/or Finish | Weight max g | Total Dose Radiation Level Letter |
| --- | --- | --- | --- | --- | --- |
| 01 |  |  |  |  | R [100kRAD(Si)] |
| 02 |  |  |  |  | R [100kRAD(Si)] |
| 05 |  |  |  |  | R [100kRAD(Si)] |
| 07 |  |  |  |  | R [100kRAD(Si)] |
| 09 |  |  |  |  | R [100kRAD(Si)] |
| 10 |  |  |  |  | R [100kRAD(Si)] |
| 11 |  |  |  |  | R [100kRAD(Si)] |

The lead/terminal material and/or finish shall be in accordance with the requirements of ESCC Basic Specification No. 23500.

Total dose radiation level letters are defined in ESCC Basic Specification No. 22900. If an alternative radiation test level is specified in the Purchase Order, the letter shall be changed accordingly.

Add new Para 2.10/2.10.1/2.10.2 as follows (in blue):

2.10 	TOTAL DOSE RADIATION TESTING

All lots shall be irradiated in accordance with ESCC Basic Specification No. 22900, low dose rate (window 2: 36 to 360 RAD/h).

2.10.1    Bias Conditions and Total Dose Level for Total dose Radiation Testing

The following bias conditions shall be used for Total Dose Radiation Testing:

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +20 ± 5 | °C |
| Bias Condition 1: Collector-Emitter Voltage | VCES | ≥ 80% V(BR)CEO | V |
| Bias Condition 2: Collector-Emitter Voltage | VCES | 0 | V |

The total dose level applied shall be as specified in the component type variant information herein or in the Purchase Order.

2.10.2    Electrical Measurements for Radiation Testing

Prior to irradiation testing the devices shall have successfully met Room Temperature Electrical Measurements specified herein.

Unless otherwise stated the measurements shall be performed at Tamb = +22±3°C.

Unless otherwise specified the test methods and test conditions shall be as per the corresponding test defined in Room Temperature Electrical Measurements.

The parameters to be measured during and on completion of irradiation testing are shown below.

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Collector-Base | V(BR)CBO | See Room Temperature Electrical Measurements | See Room Temperature Electrical Measurements | -60 | - | V |
| Breakdown |  |  |  |  |  |  |
| Voltage |  |  |  |  |  |  |
| Collector-Emitter Breakdown Voltage | V(BR)CEO | See Room Temperature Electrical Measurements | See Room Temperature Electrical Measurements | -60 | - | V |
| Emitter-Base | V(BR)EBO | See Room Temperature Electrical Measurements | See Room Temperature Electrical Measurements | -5 | - | V |
| Breakdown |  |  |  |  |  |  |
| Voltage |  |  |  |  |  |  |
| Collector-Base Cut-off Current | ICBO | See Room Temperature Electrical Measurements | See Room Temperature Electrical Measurements | - | -10 | nA |
| Emitter-Base Cut-off Current | IEBO | See Room Temperature Electrical Measurements | See Room Temperature Electrical Measurements | - | -20 | nA |
| Collector-Emitter Saturation Voltage | VCE(sat)1 | See Room Temperature Electrical Measurements | See Room Temperature Electrical Measurements | - | -200 | mV |
|  | VCE(sat)2 |  |  | - | -250 | mV |
| Base-Emitter Saturation Voltage | VBE(sat)1 | See Room Temperature Electrical Measurements | See Room Temperature Electrical Measurements | - | -700 | mV |
|  | VBE(sat)2 |  |  | - | -800 | mV |
| Forward-Current Transfer Ratio (post irradiation gain calculation) (Note 1) | [hFE1] | 3076 | VCE=-5V, IC = -10µA | [50] | - | - |
|  | [hFE2] |  | VCE=-5V, IC = -100µA | [75] | 450 | - |
|  | [hFE3] |  | VCE=-5V, IC = -500µA | [75] | 450 | - |
|  | [hFE4] |  | VCE=-5V, IC = -1mA | [75] | 450 | - |
|  | [hFE5] |  | VCE=-5V, IC = -10mA | [65] | - | - |

NOTES:

- The post-irradiation gain calculation of [hFE], made using hFE measurements from prior to and on completion of irradiation testing and after each annealing step if any, shall be as specified in MIL-STD-750 Method 1019.