| ITEMS AFFECTED | DESCRIPTION OF DEVIATIONS |
| --- | --- |
| Deviations from Total Dose Radiation Testing | Time intervals for measurement: Post irradiation electrical characterization starts more than 1 hour of completion of exposure. Flow chart for qualification and lot acceptance testing: At the end of the irradiation test, only a functional test is done. The duration of the room temperature anneal could be extended to an appropriate period of time to allow leakage-related parameters that may have exceeded their pre-irradiation specification to return to within specification. |