Modification for 4 ESCC specification of detail (in bleu the values to add in new revision)

Paragraph : 1.7.2 Metal Flange Mount Package (TO-257) - 3 lead page 11

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 10.41 | 10.67 |  |
| B | 10.41 | 10.67 |  |
| C | 16.51 | 16.76 |  |
| D | 4.70 | 5.33 | 4 |
|  | 4.98 | 5.23 | 5 |
| E | 0.89 | 1.14 |  |
| ØF | 3.56 | 3.81 |  |
| G | 13.39 | 13.64 |  |
| H | 5.13 | 5.38 |  |
| ØI | 0.64 | 0.89 | 2 |
| J | 2.92 | 3.16 3.18 |  |
| K | 2.41 | 2.67 |  |
| L | 15.24 | 16.51 |  |
| ØM | 2.29 |  | 2 |
| N | - | 0.71 |  |
| R | 1.65 |  | 3 |

NOTES:

1. Terminal identification is specified by the component’s geometry where Lead 1 = emitter, Lead 2 = base and Lead 3 = collector.

2. Applies to all leads.

3. Radius of body corner, 4 places.

4. Dimensions for package with Parallel seam welding process

5. Dimensions for package with Belt furnace process

Paragraph : 1.7.3 Surface Mount Package (SMD.5) - 3 terminal page 12

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 2.84 | 3.15 | 3 |
|  | 2.99 | 3.30 | 4 |
| A1 | 0.25 | 0.51 |  |
| b1 | 7.13 | 7.39 |  |
| b | 5.58 | 5.84 |  |
| b2 | 2.28 | 2.54 | 2 |
| b3 | 2.92 | 3.18 | 2 |
| D | 10.03 | 10.28 |  |
| D1 | 0.76 | - | 2 |
| E | 7.39 | 7.64 |  |
| e | 1.91 |  | 2 |

NOTES:

1. Terminal identification is specified by the components geometry where Terminal 1 = emitter (base for Variant 07), Terminal 2 = base (emitter for Variant 07) and Terminal 3 = collector.

2. 2 places.

3. Dimensions for package with Parallel seam welding process

4. Dimensions for package with Belt furnace process

Applied the same modification for the ESCC specification in table.

| 2N5154 | ESCC Detail Specification No. 5203/010 | Issue 7 | TO257 & SMD.5 | Page 11 & page 12 |
| --- | --- | --- | --- | --- |
| BUX77 | ESCC Detail Specification No 5203/016 | Issue 4 | TO257 only | Page 9 |
| 2N5153 | ESCC Detail Specification No. 5204/002 | Issue 7 | TO257 & SMD.5 | Page 10 & page 11 |
| BUX78 | ESCC Detail Specification No. 5204/006 | Issue 4 | TO 257 only | Page 9 |