Page 1 of 23

CONNECTOR SAVERS, ELECTRICAL, RECTANGULAR, MICROMINIATURE

BASED ON TYPE MDM

ESCC Detail Specification No. 3401/041

| Issue 6 Draft A | November 2018 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2018. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| tba | Specification upissued to incorporate changes per DCR. |

TABLE OF CONTENTS

1	General	6

1.1	SCOPE	6

1.2	TYPE VARIANTS	6

1.3	MAXIMUM RATINGS	6

1.4	PARAMETER DERATING INFORMATION	6

1.5	PHYSICAL DIMENSIONS	6

1.6	CONTACT ARRANGEMENTS	6

2	APPLICABLE DOCUMENTS	6

3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	7

4	REQUIREMENTS	13

4.1	GENERAL	13

4.2	DEVIATIONS FROM GENERIC SPECIFICATION	13

4.2.1	Deviations from Special In-Process Controls	13

4.2.2	Deviations from Final Production Tests (Chart II)	13

4.2.3	Deviations from Burn-in and Electrical Measurements (Chart III)	13

4.2.4	Deviations from Qualification Tests (Chart IV)	13

4.2.5	Deviations from Lot Acceptance Tests (Chart V)	13

4.3	MECHANICAL REQUIREMENTS	14

4.3.1	Dimension Check	14

4.3.2	Weight	14

4.3.3	Contact Capability	14

4.3.4	Contact Retention (in Insert)	14

4.3.5	Mating and Unmating Forces	14

4.3.6	Insert Retention (in Shell)	14

4.3.7	Jackscrew Retention	14

4.3.8	Contact Insertion and Withdrawal Forces	14

4.3.9	Engagement and Separation Forces (Male Contacts)	14

4.3.10	Oversize Pin Exclusion	15

4.3.11	Probe Damage	15

4.3.12	Solderability	15

4.4	MATERIALS AND FINISHES	15

4.4.1	Shells	15

4.4.2	Inserts	15

4.4.3	Contacts	15

4.4.3.1	Female Contacts	15

4.4.3.2	Male Contacts	15

4.4.4	Seals Interfacial	15

4.4.5	Uninsulated Solid Wire	15

4.4.6	Potting	15

4.5	MARKING	16

4.5.1	General	16

4.5.2	The ESCC Component Number	16

4.5.3	Characteristics	16

4.5.3.1	Shell Size	16

4.5.3.2	Type of Contact	16

4.5.4	Traceability Information	16

4.6	ELECTRICAL MEASUREMENTS	17

4.6.1	Electrical Measurements at Room Temperature	17

4.6.2	Electrical Measurements at High and Low Temperatures (Table 3)	17

4.6.3	Circuits for Electrical Measurements (Figure 4)	17

4.7	BURN-IN AND ELECTRICAL MEASUREMENTS (TABLES 4 AND 5)	17

4.8	ENVIRONMENTAL AND ENDURANCE TESTS (CHARTS IV AND V OF ESCC GENERIC SPECIFICATION NO. 3401)	19

4.8.1	Measurements and Inspections on Completion of Environmental Tests	19

4.8.2	Measurements and Inspections at Intermediate Points during Endurance Tests	19

4.8.3	Measurements and Inspections on Completion of Endurance Tests	19

4.8.4	Conditions for Operating Life Test (Part of Endurance Testing)	19

4.8.5	Electrical Circuit for Operating Life Test (Figure 5)	19

4.8.6	Conditions for High Temperature Storage Test (Part of Endurance Testing)	19

APPENDIX ‘A’	23

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics, test and inspection data for Connector Savers, Electrical, Rectangular, Microminiature with Non-removable Crimp-type Contacts, based on type MDM.

It shall be read in conjunction with:

ESCC Generic Specification No. 3401, Connectors, Electrical, Non-Filtered, Circular and Rectangular.

ESCC Detail Specification No. 3401/032, Accessories for Connectors, Microminiature, 3401/029, 3401/077 and Connector Savers 3401/041.

the requirements of which are supplemented herein.

## TYPE VARIANTS

Variants of the basic type connector savers specified herein, which are also covered by this specification, together with their mechanical characteristics, are given in Table 1(a).

## MAXIMUM RATINGS

The maximum ratings, which shall not be exceeded at any time during use or storage, applicable to the connector savers specified herein, are scheduled in Table 1(b).

## PARAMETER DERATING INFORMATION

The derating information applicable to the connector savers specified herein is shown in Figure 1.

## PHYSICAL DIMENSIONS

The physical dimensions of the connector savers specified herein are shown in Figure 2.

## CONTACT ARRANGEMENTS

Contact arrangements are shown in Figure 3.

# APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3401, Connectors, Electrical, Non-Filtered, Circular and Rectangular.

ESCC Detail Specification No. 3401/032, Accessories for Connectors, Microminiature, 3401/029, 3401/077 and Connector Savers 3401/041.

A-A-59551, Wire, Electrical, Copper, Uninsulated.

MIL-DTL-45204, Gold Plating, Electro-deposited.

MIL-DTL-14550, Copper Plating, Electro-deposited.

MIL-DTL-83513, Connectors, Electrical, Rectangular, Microminiature, Polarised Shell, Generic Specification for.

# TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

TABLE 1(a) - TYPE VARIANTS

| Variant | Shell Size Notes 1, 2 | Max. Weight (g) Note 3 | Max. Mating Force (N) | Unmating Force (N) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Max. | Min. |
| 01 | 9 | 4 | 25 | 25 | 1.3 |
| 02 | 15 | 5.5 | 42 | 42 | 2 |
| 03 | 21 | 7 | 58 | 58 | 2.9 |
| 04 | 25 | 8 | 70 | 70 | 3.5 |
| 05 | 31 | 9.5 | 86 | 86 | 4.3 |
| 06 | 37 | 10 | 103 | 103 | 5.1 |
| 07 | 51 | 13.5 | 142 | 142 | 7.1 |

NOTES:

Contacts are fixed in the connector saver.

Shell Size 51 is not applicable to MDMA connectors.

Connector saver with contacts and without screw-locks.

TABLE 1(b) - MAXIMUM RATINGS

| No. | Characteristics | Symbol | Maximum Rating | Unit | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Working Voltage (Sea Level) | UR | 150 | Vrms | Note 1 |
| 2 | Rated Current: (uninsulated solid wire) | IR | 2.5 | A | Note 2 |
| 3 | Operating Temperature Range | Top | -55 to +125 | ºC |  |
| 4 | Storage Temperature Range | Tstg | -55 to +125 | ºC |  |

NOTES:

            - Between contacts, and contact and shell.

IR requires derating if the number of current-carrying contacts in the connector saver is 2 or greater. See Figure 1(b).

FIGURE 1 - PARAMETER DERATING INFORMATION

FIGURE 1(a) - Working Voltage Versus Altitude

FIGURE 1(b) - Maximum Current Versus Number Of Contacts

| Number of Current-Carrying Contacts per Connector Saver | Maximum Current per Contact (A) |
| --- | --- |
|  | Uninsulated Solid Wire |
| 2 - 4 | 2 |
| 5 - 14 | 1.8 |
| 15 and over | 1.4 |

FIGURE 2 - PHYSICAL DIMENSIONS

Figure 2.0 – male and female interface dimensions

| FEMALE RECEPTACLE |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| MALE PLUG |  |  |  |  |  |  |
| Shell Size |  | Dint Min. | Eint Min. | Rint Min. | Dext Max. | Eext Max. | Rext Max. |
| 9 |  | 8.49 | 4.7 | 1.704 | 8.48 | 4.69 | 1.699 |
| 15 |  | 12.3 | 4.7 | 1.704 | 12.29 | 4.69 | 1.699 |
| 21 |  | 16.11 | 4.7 | 1.704 | 16.1 | 4.69 | 1.699 |
| 25 |  | 18.65 | 4.7 | 1.704 | 18.64 | 4.69 | 1.699 |
| 31 |  | 22.46 | 4.7 | 1.704 | 22.45 | 4.69 | 1.699 |
| 37 |  | 26.27 | 4.7 | 1.704 | 26.26 | 4.69 | 1.699 |
| 51 |  | 25 | 5.8 | 1.704 | 24.99 | 5.78 | 1.699 |

FIGURE 2.1 - CONNECTOR SAVER DIMENSIONS

| Variant | Size | A Max. | E Max. | F Max. | G Typical | H Typical | ØS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Min. | Max. |
| 01 | 09 | 19.94 | 7.75 | 14.35 | 2.29 | 22.86 | 2.23 | 2.44 |
| 02 | 15 | 23.75 | 7.75 | 18.16 | 2.29 | 22.86 | 2.23 | 2.44 |
| 03 | 21 | 27.56 | 7.75 | 21.97 | 2.29 | 22.86 | 2.23 | 2.44 |
| 04 | 25 | 30.1 | 7.75 | 24.51 | 2.29 | 22.86 | 2.23 | 2.44 |
| 05 | 31 | 33.91 | 7.75 | 28.32 | 2.29 | 22.86 | 2.23 | 2.44 |
| 06 | 37 | 37.72 | 7.75 | 32.13 | 2.29 | 22.86 | 2.23 | 2.44 |
| 07 | 51 | 36.45 | 7.75 | 30.86 | 2.29 | 22.86 | 2.23 | 2.44 |

NOTES:

            - All dimensions are in millimetres.

FIGURE 2.2 - CONTACT POSITION

Figure 2.2.1 - Mounting Condition

FIGURE 2.2.2 - Plug Male Contact

FIGURE 2.2.3 - Receptacle Female Contact

| F |  | S |  | Z |
| --- | --- | --- | --- | --- |
| Min. | Max. | Min. | Max. | Max. |
| 0.25 | 0.91 | 3.3 | 3.66 | 5.49 |

NOTES:

            - All dimensions are in millimetres.

FIGURE 3 - CONTACT ARRANGEMENTS

FRONT VIEW OF MALE INSERT - USE MIRROR VIEW FOR FEMALE INSERT

NOTES:

            - Only the outside contact cavities on each row are identified in the drawing, the remainder follow sequentially. Contact numbers are shown outside the insert for readability.

Contact Centres

NOTES:

            - a = Distance between contact centres: 1.27mm typical.

            - b = Distance between rows: 1.09mm typical.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the connector savers specified herein are stated in this specification and ESCC Generic Specification No. 3401. Deviations from the Generic Specification, applicable to this specification only, are listed in Para. 4.2.

Deviations from the applicable Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirements and do not affect the components’ reliability, are listed in the appendices attached to this specification.

## DEVIATIONS FROM GENERIC SPECIFICATION

### Deviations from Special In-Process Controls

Para. 9.15, Joint Strength: The contacts shall be crimped to uninsulated solid wire AWG25. The value of failure shall be recorded together with the information to whether the failure was “pull-out”, “break in crimp” or “break in wire’. The minimum tensile strength shall be as follows:

| Wire | Male and Female Contacts |
| --- | --- |
|  | AWG25 - Solid Uninsulated |
| Tensile Strength (N) | 22 |

### Deviations from Final Production Tests (Chart II)

Para. 9.3, Contact Retainer Test: Not applicable.

Para. 9.4, Contact Capability: This test shall be performed on the male contacts. For details see Para. 4.3.3 of this specification.

Para. 9.5, Magnetism Level: Not applicable.

### Deviations from Burn-in and Electrical Measurements (Chart III)

Chart III is not applicable.

### Deviations from Qualification Tests (Chart IV)

Para. 9.9, Seal Test: Not applicable.

Para. 9.15, Joint Strength: Not applicable.

Para. 9.17, Contact Retention (in insert): Not applicable with male contact.

Para. 9.27, Maintenance Ageing: Not applicable.

Para. 9.29, Oversize Pin Exclusion: Not applicable.

Para. 9.30, Probe Damage: Not applicable.

### Deviations from Lot Acceptance Tests (Chart V)

Para. 9.9, Seal Test: Not applicable.

Para. 9.15, Joint Strength: Not applicable.

Para. 9.17, Contact Retention (in insert): Not applicable with male contact.

Para. 9.27, Maintenance Ageing: Not applicable.

Para. 9.29, Oversize Pin Exclusion: Not applicable.

Para. 9.30, Probe Damage: Not applicable.

## MECHANICAL REQUIREMENTS

### Dimension Check

The dimensions of the connector savers specified herein shall be verified in accordance with the requirements set out in Para. 9.6 of ESCC Generic Specification No. 3401 and shall conform to those shown in Figure 2 of this specification. Only the underlined dimensions shall be checked during procurement.

### Weight

The maximum weight of the connector savers specified herein shall be as specified in Table 1(a).

### Contact Capability

For the purpose of this test, the pick-up and drop weights shall be as follows.

| Measurements | Pick-up Weight | Drop Weight |
| --- | --- | --- |
| Weight (g) | 14 | 170 |
| Inner Gauge Diameter (mm) (1) | 0.582 - 0.587 | 0.559 - 0.564 |
| Insertion Depth (mm) | 1.5 | 1.5 |

NOTES:

See Figure 4 for ØA.

### Contact Retention (in Insert)

Contact retention within the insert shall be 22.25 Newtons. There shall be no displacement of the contact. Not applicable to male contacts.

### Mating and Unmating Forces

The forces applied for the mating and unmating of the connector savers shall conform to the values specified in Table 1(a).

### Insert Retention (in Shell)

Connector saver inserts shall withstand a pressure of 34.4N/cm2 applied from the mating side to the rear side.

### Jackscrew Retention

Not applicable.

### Contact Insertion and Withdrawal Forces

Not applicable.

### Engagement and Separation Forces (Male Contacts)

The contact engagement and separation forces of the male contacts shall be tested to a depth of 1.5mm with the applicable test gauge fixtures specified in Figure 4 of this specification, and shall not exceed the values of the table hereunder.

| Measurements | Inner Diameter (mm) |  | Separation Force Min. (N) | Engagement Force Max. (N) |
| --- | --- | --- | --- | --- |
|  | Min. | Max. |  |  |
| Minimum Diameter Test Sleeve | 0.559 | 0.564 | - | 1.667 |
| Maximum Diameter Test Sleeve | 0.582 | 0.587 | 0.137 | - |

### Oversize Pin Exclusion

Not applicable.

### Probe Damage

Not applicable.

### Solderability

Not applicable.

## MATERIALS AND FINISHES

The materials and finishes shall be as specified herein. Where a definite material is not specified, a material which will enable the components specified herein to meet the performance requirements of this specification shall be used. Acceptance or approval of any constituent material does not guarantee acceptance of the finished product.

### Shells

Shells shall be made of aluminium alloy plated with a minimum thickness of 25.4μm of electroless nickel.

### Inserts

Inserts shall be made of glass-fibre filled diallylphthalate resin or a suitable thermoplastic material.

### Contacts

#### Female Contacts

The contact body shall be made of copper alloy with an underplate of 1μm minimum of copper to MIL-C-14550, gold plated with 1.27μm minimum of gold, Type 2, Grade C of MIL-DTL-45204. Measurement of thickness shall be performed at a distance of 1.5mm from the engagement end.

#### Male Contacts

The contact body and the bundle shall be made of copper alloy with an underplate of 1μm minimum of copper to MIL-C-14550, gold plated with 1.27μm minimum of gold, Type 2 Grade C of MIL-DTL-45204. Measurement of thickness shall be performed at a distance of 1.5mm from the engagement end.

### Seals Interfacial

Interfacial seals shall be made of silicon base rubber.

### Uninsulated Solid Wire

Uninsulated solid wires shall be made of copper alloy in accordance with Type S as specified in A-A-59551. They shall be gold-plated in accordance with Class 00, Grade C or D, as specified in MIL-DTL-45204.

### Potting

Potting shall be made of epoxy resin.

## MARKING

### General

The marking of components delivered to this specification shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and the following paragraphs. When the component is too small to accommodate all of the marking specified, as much as space permits shall be marked and the marking information, in full, shall accompany the component in its primary package.

The information to be marked and the order of precedence shall be as follows:

The ESCC Component Number.

Characteristics.

Traceability Information.

### The ESCC Component Number

Each component shall bear the ESCC Component Number which shall be constituted and marked as follows:

Example: 340104101B

Detail Specification Number: 3401041

Type Variant (See Table 1(a)): 01

Testing Level: B

### Characteristics

The characteristics to be marked in the following order of precedence are:

Shell Size.

Type of Contact.

The information shall be constituted and marked as follows (example): 9PS

Shell size: 9

Type of Contact: PS

#### Shell Size

Shell size shall be designated by the number of contacts. Specified numbers are: 9, 15, 21, 25, 31, 37 and 51.

#### Type of Contact

The contact types shall be indicated by the following code letters.

| Code Letter | Contact Type |
| --- | --- |
| PS | Male/Female |

### Traceability Information

Traceability information shall be marked in accordance with the requirements of ESCC Basic Specification No. 21700.

## ELECTRICAL MEASUREMENTS

### Electrical Measurements at Room Temperature

The parameters to be measured in respect of electrical characteristics are scheduled in Table 2. Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3 ºC.

### Electrical Measurements at High and Low Temperatures (Table 3)

Not applicable.

### Circuits for Electrical Measurements (Figure 4)

Not applicable.

## BURN-IN AND ELECTRICAL MEASUREMENTS (TABLES 4 AND 5)

Not applicable.

Table 2 - ELECTRICAL MEASUREMENTS AT ROOM TEMPERATURE

| No. | Characteristics | Symbol | ESCC 3401 Test Method | Test Condition | Limits |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Min. | Max. |  |
| 1 | Insulation Resistance | Ri | Para. 9.1.1.1 | Para. 9.1.1.1 | 5000 | - | MΩ |
| 2 | Voltage Proof Leakage Current | IL | Para. 9.1.1.2 | 600Vrms |  | 2 | mA |
| 3 | Mated Shell Conductivity (Voltage Drop) Note 1 | Vd | Para. 9.1.1.4 | Para. 9.1.1.4 | Not applicable |  | mV |
| 4 | Contact Resistance Low Level Current | Rcl | Para. 9.1.1.3 | Para. 9.1.1.3 | - | 12 | mΩ |
| 5 | Contact Resistance Rated Current | Rcr | Para. 9.1.1.3 | Table 1(b) | - | 10 | mΩ |

NOTES:

Applicable to mated connectors with grounding option.

TABLES 3, 4 AND 5

Not applicable.

FIGURE 4 - GAUGE FIXTURE

MINIMUM DIAMETER TEST SLEEVE

| Weight (g) 170 |  |  | Remarks |
| --- | --- | --- | --- |
|  | Min. | Max. |  |
| ØA | 0.559 | 0.564 | Note 2 |
| ØB | 0.749 | 0.775 | - |
| ØC | 0.813 | 0.825 | - |
| J | 4 | - | - |
| J1 | 3.13 | 3.23 | - |
| R | 0.381 | 0.483 | Note 1 |

MAXIMUM DIAMETER TEST SLEEVE

| Weight (g) 14 |  |  | Remarks |
| --- | --- | --- | --- |
|  | Min. | Max. |  |
| ØA | 0.582 | 0.587 | Note 2 |
| ØB | 0.749 | 0.775 | - |
| ØC | 0.813 | 0.825 | - |
| J | 4 | - | - |
| J1 | 3.13 | 3.23 | - |
| R | 0.381 | 0.483 | Note 1 |

NOTES:

            - Radius R, must be tangent to entry chamfer and ØA.

            - ØA and entry chamfer shall have a surface roughness of 3.2μm (roughness grade N8).

## ENVIRONMENTAL AND ENDURANCE TESTS (CHARTS IV AND V OF ESCC GENERIC SPECIFICATION NO. 3401)

### Measurements and Inspections on Completion of Environmental Tests

The parameters to be measured and inspections to be performed on completion of environmental testing shall be those specified in Table 6. Unless otherwise specified, these measurements shall be performed at Tamb = +22 ±3 ºC.

### Measurements and Inspections at Intermediate Points during Endurance Tests

Not applicable.

### Measurements and Inspections on Completion of Endurance Tests

The parameters to be measured and inspections to be performed on completion of endurance testing shall be those specified in Table 6. Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3 ºC.

### Conditions for Operating Life Test (Part of Endurance Testing)

Not applicable.

### Electrical Circuit for Operating Life Test (Figure 5)

Not applicable.

### Conditions for High Temperature Storage Test (Part of Endurance Testing)

The requirements for the high temperature storage test are specified in Section 9 of ESCC Generic Specification No. 3401. The temperature to be applied shall be the maximum storage temperature specified in Table 1(b) of this specification.

Table 6 - MEASUREMENTS AND INSPECTIONS ON COMPLETION OF ENVIRONMENTAL AND ENDURANCE TESTING

| No. | ESCC Generic Spec. No. 3401 |  | Measurements and Inspections |  | Symbol | Limits |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Environmental and Endurance Tests (Note 1) | Test Method and Conditions | Identification | Conditions |  | Min. | Max. |  |
| 01 | Seal Test | Para. 9.9 | ESCC 3401 Para. 9.9 | - | - | Not applicable |  | - |
| 02 | Wiring | Para. 9.10 | Low Level Contact Resistance | - | Rcl | Not applicable |  | - |
| 03 | Vibration | Para. 9.11 | Initial Measurements |  |  |  |  |  |
|  |  |  | Coupling Screw(s) Unlocking Torque | - | - | Record Values |  | - |
|  |  |  | Final Measurements |  |  |  |  |  |
|  |  |  | Full Engagement |  |  |  |  |  |
|  |  |  | Coupling Screw(s) Unlocking Torque Drift | - | ΔTqe/Tqe | -25 | +25 | % |
|  |  |  | Visual Examination | - | - | - | - | - |
| 04 | Shock or Bump | Para. 9.12 | Full Engagement Visual Examination | - | - | - | - | - |
| 05 | Climatic Sequence | Para. 9.13 | Dry Heat | At High Temperature |  |  |  |  |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | 10 | - | MΩ |
|  |  |  | Low Air Pressure |  |  |  |  |  |
|  |  |  | Voltage Proof Leakage Current | Figure 1 | IL | ESCC 3401 Para. 9.13.5 |  | mA |
|  |  |  | Damp Heat | Immediately after test |  |  |  |  |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | 100 | - | MΩ |
|  |  |  | Final Measurements | After 1-24 hrs Recovery |  |  |  |  |
|  |  |  | External Visual Inspection | ESCC 3401 Para. 9.7 |  | ESCC 3401 Para. 9.7 |  |  |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2 Item 1 |  | MΩ |
|  |  |  | Voltage Proof Leakage Current | Table 2 Item 2 | IL | Table 2 Item 2 |  | mA |
| 06 | Plating Thickness | Para. 9.14 | Thickness | - | - | Para. 4.4.3 of this spec. |  | - |
| 07 | Joint Strength | Para. 9.15 | ESCC 3401 Para. 9.15 | - | - | Not applicable |  | - |
| 08 | Rapid Change of Temperature | Para. 9.16 | Visual Examination | - | - | - | - | - |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2 Item 1 |  | MΩ |
|  |  |  | Voltage Proof Leakage Current | Table 2 Item 2 | IL | Table 2 Item 2 |  | mA |
| 09 | Contact Retention (In Insert) | Para. 9.17 & Para. 4.3.4 of this spec. | Contact Displacement | Not applicable for male contacts | - | ESCC 3401 Para. 9.17 |  | - |
| 10 | Endurance | Para. 9.18 | Initial Measurements |  |  |  |  |  |
|  |  |  | Mating/Unmating Forces | - | F | Para. 4.3.5 of this spec. |  | N |
|  |  |  | Low Level Contact Resistance | Table 2 Item 4 | Rcl | Record Values |  | mΩ |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | Vd | Not applicable |  | mV |
|  |  |  | Final Measurements |  |  |  |  |  |
|  |  |  | Visual Examination | - | - | - | - | - |
|  |  |  | Mating/Unmating Forces | - | F | Para. 4.3.5 of this spec. |  | N |
|  |  |  | Low Level Contact Resistance Drift | Table 2 Item 4 | ΔRcl | - | 3 | mΩ |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | Vd | Not applicable |  | mV |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2 Item 1 |  | MΩ |
|  |  |  | Voltage Proof Leakage Current | Table 2 Item 2 | IL | Table 2 Item 2 |  | mA |
| 11 | Permanence of Marking | Para. 9.19 | - | - | - | - | - | - |
| 12 | Mating/Unmating Forces | Para. 9.20 | Force | - | F | Para. 4.3.5 of this spec. |  | N |
| 13 | High Temperature Storage | Para. 9.21 | Initial Measurements |  |  |  |  |  |
|  |  |  | Low Level Contact Resistance | Table 2 Item 4 | Rcl | Record Values |  | mΩ |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | Vd | Not applicable |  | mV |
|  |  |  | Final Measurements |  |  |  |  |  |
|  |  |  | Visual Examination | - | - | - | - | - |
|  |  |  | Mating/Unmating Forces | - | F | Para. 4.3.5 of this spec. |  | N |
|  |  |  | Low Level Contact Resistance Drift | Table 2 Item 4 | ΔRcl | - | 3 | mΩ |
|  |  |  | Rated Current Contact Resistance | Table 2 Item 5 | Rcr | Table 2 Item 5 |  | mV |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | Vd | Not applicable |  | mV |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2 Item 1 |  | MΩ |
|  |  |  | Voltage Proof Leakage Current | Table 2 Item 2 | IL | Table 2 Item 2 |  | mA |
|  |  |  | Contact Retention (In Insert) | Para. 4.3.4 of this spec. | - | ESCC 3401 Para. 9.17 |  | N |
| 14 | Corrosion | Para. 9.22 | Visual Examination | - | - | - | - | - |
| 15 | Insert Retention (In Shell) | Para. 9.23 & Para. 4.3.6 of this spec. | Visual Examination | - | - | Para. 4.3.6 of this spec. |  | - |
| 16 | Jackscrew Retention | Para. 9.24 & Para. 4.3.7 of this spec. | Visual Examination | - | - | Not applicable |  | - |
| 17 | High Temperature Measurements | Para. 9.25 | Insulation Resistance | Table 2 Item 1 | Ri | 5000 | - | MΩ |
| 18 | Overload Test | Para. 9.26 | Internal Temperature | - | T | - | +100 | °C |
|  |  |  | Rated Current Contact Resistance | Table 2 Item 5 | Rcr | Table 2 Item 5 |  | mΩ |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | Vd | Not applicable |  | mV |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2 Item 1 |  | MΩ |
|  |  |  | Voltage Proof Leakage Current | Table 2 Item 2 | IL | Table 2 Item 2 |  | mA |
| 19 | Maintenance Aging | Para. 9.27 | - | - | - | Not applicable |  | - |
| 20 | Engage/Separation Forces | Para. 9.28 & Para. 4.3.9 of this spec. | Force | - | F | Para. 4.3.9 of this spec. |  | N |
| 21 | Oversize Pin Exclusion | Para. 9.29 & Para. 4.3.10 of this spec. | - | - | - | Not applicable |  | - |
| 22 | Probe Damage | Para. 9.30 & Para. 4.3.11 of this spec. | Contact Separation Force | - | - | Not applicable |  | - |
| 23 | Solderability | Para. 9.31 & Para. 4.3.12 of this spec. | - | - | - | Not applicable |  | - |

NOTES:

The tests in this Table refer to either Chart IV or V and shall be used as applicable.

## APPENDIX ‘A’

AGREED DEVIATIONS FOR C & K COMPONENTS (F)

| ITEMS AFFECTED | DESCRIPTION OF DEVIATIONS |
| --- | --- |
| Para. 4.2.2, Deviations from Final Production Tests (Chart II) | Para. 9.4, Contact Capability: 100% Contact Capability Test may be omitted provided that a 100% visual inspection of the contacts is performed on each batch submitted to tests defined in the C&K PID requirements. |