Page 1 of 25

CONNECTORS, ELECTRICAL, RECTANGULAR, MICROMINIATURE, REMOVABLE CRIMP CONTACTS

BASED ON TYPE MDMA

ESCC Detail Specification No. 3401/077

| Issue 8 Draft A | November 2018 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2018. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| tba | Specification upissued to incorporate changes per DCR. |

TABLE OF CONTENTS

1	GENERAL	6

1.1	SCOPE	6

1.2	COMPONENT TYPE VARIANTS AND RANGE OF COMPONENTS	6

1.3	MAXIMUM RATINGS	6

1.4	PARAMETER DERATING INFORMATION	6

1.5	PHYSICAL DIMENSIONS	6

1.6	Contact Arrangements	6

2	APPLICABLE DOCUMENTS	6

3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	6

4	REQUIREMENTS	16

4.1	GENERAL	16

4.2	DEVIATIONS FROM GENERIC SPECIFICATION	16

4.2.1	Deviations from Special In-process Controls	16

4.2.2	Deviations from Final Production Tests (Chart II)	16

4.2.3	Deviations from Burn-in and Electrical Measurements (Chart III)	17

4.2.4	Deviations from Qualification Tests (Chart IV)	17

4.2.5	Deviations from Lot Acceptance Tests (Chart V)	17

4.3	MECHANICAL REQUIREMENTS	17

4.3.1	Dimension Check	17

4.3.2	Weight	17

4.3.3	Contact Capability	17

4.3.4	Contact Retention (In insert)	17

4.3.5	Mating and Unmating Forces	17

4.3.6	Insert Retention (In shell)	17

4.3.7	Jackscrew Retention	17

4.3.8	Contact Insertion and Withdrawal Forces	17

4.3.9	Engagement and Separation Forces (Male Contacts)	17

4.3.10	Oversize Pin Exclusion	17

4.3.11	Probe Damage	18

4.3.12	Solderability	18

4.4	MATERIALS AND FINISHES	18

4.4.1	Shells	18

4.4.2	Inserts	18

4.4.3	Contacts	18

4.4.4	Contact Retaining Clip	18

4.5	MARKING	18

4.5.1	General	18

4.5.2	The ESCC Component Number	18

4.5.3	Series	18

4.5.4	Characteristics	19

4.5.4.1	Fixing Option	19

4.5.4.2	Shell Size	19

4.5.4.3	Contact Type	19

4.5.4.4	Contact Information	19

4.5.4.5	Traceability Information	19

4.6	ELECTRICAL MEASUREMENTS	19

4.6.1	Electrical Measurements at Room Temperature	19

4.6.2	Electrical Measurements at High and Low Temperatures	19

4.6.3	Circuits for Electrical Measurements	20

4.7	BURN-IN AND ELECTRICAL MEASUREMENTS (TABLES 4 AND 5)	20

4.8	ENVIRONMENTAL AND ENDURANCE TESTS (CHARTS IV AND V OF ESCC GENERIC SPECIFICATION No. 3401)	21

4.8.1	Measurements and Inspections on Completion of Environmental Tests	21

4.8.2	Measurements and Inspections at Intermediate Points during Endurance Tests	21

4.8.3	Measurements and Inspections on Completion of Endurance Tests	21

4.8.4	Conditions for Operating Life Test (Part of Endurance Testing)	21

4.8.5	Electrical Circuit for Operating Life Test	21

4.8.6	Conditions for High Temperature Storage Test (Part of Endurance Testing)	21

Appendix A	25

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, test and inspection data for Electrical, Rectangular, Microminiature Connectors with Removable Crimp Contacts, based on type MDMA. It shall be read in conjunction with:

ESCC Generic Specification No. 3401, Connectors, Electrical, Rectangular and Circular.

ESCC Detail Specification No. 3401/032, Accessories for Connectors, Microminiature, 3401/029, 3401/077 and Connector Savers 3401/041.

ESCC Detail Specification No. 3401/078, Contacts, Electrical, Crimp, for 3401/077 Microminiature Connectors based on type MDMA.

ESCC Detail Specification No. 3401/087, Lightweight Accessories for Rectangular, Microminiature Connectors, 3401/029 and 3401/077.

the requirements of which are supplemented herein.

## COMPONENT TYPE VARIANTS AND RANGE OF COMPONENTS

The different sizes of the basic type connectors specified herein, together with their mechanical characteristics, are scheduled in Table 1(a).

## MAXIMUM RATINGS

The maximum ratings, which shall not be exceeded at any time during use or storage, applicable to the connectors specified herein, are given in Table 1(b).

## PARAMETER DERATING INFORMATION

The derating information applicable to the connectors specified herein is shown in Figure 1.

## PHYSICAL DIMENSIONS

The physical characteristics of the connectors specified herein are shown in Figure 2.

## Contact Arrangements

Contact arrangements are shown in Figure 3.

# APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3401, Connectors, Electrical, Circular and Rectangular.

ESCC Detail Specification No. 3401/032, Accessories for Connectors, Microminiature, 3401/029, 3401/077 and Connector Savers 3401/041.

ESCC Detail Specification No. 3401/078, Contacts, Electrical, Crimp, for 3401/077 Microminiature Connectors based on type MDMA.

ESCC Detail Specification No. 3401/087, Lightweight Accessories for Rectangular, Microminiature Connectors, 3401/029 and 3401/077.

MIL-DTL-45204, Gold Plating, Electro-deposited.

# TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

TABLE 1(a) – COMPONENT TYPE VARIANTS AND range of components

| Variants |  | Shell Size Note 1 | Weight Max. g Note 2 | Mating Force N Max. | Unmating Force |  |
| --- | --- | --- | --- | --- | --- | --- |
| Shell Finish |  |  |  |  |  |  |
| Nickel | Gold |  |  |  | N Max. | N Min. |
| 01 | 02 | 9 | 2.3 | 25 | 25 | 1.3 |
| 01 | 02 | 15 | 3.1 | 42 | 42 | 2 |
| 01 | 02 | 21 | 3.9 | 58 | 58 | 2.9 |
| 01 | 02 | 25 | 4.4 | 70 | 70 | 3.5 |
| 01 | 02 | 31 | 5.2 | 86 | 86 | 4.3 |
| 01 | 02 | 37 | 6 | 103 | 103 | 5.1 |
| 01 | 02 | 51 | 7.3 | 147 | 147 | 7.1 |

Notes:

See Figures 2.1(a) and 2.1(b).

Weight without cables, backshell, floating eyelets, captive nut and contacts (see Para. 4.5.4.4). Add 0.4 grammes for connectors with floating mounts and 1 gramme for connectors with captive nuts. See Figure 2.3 and ESCC Detail Specification Nos. 3901/002 and 3901/013 for the weight of cable and ESCC Detail Specification No. 3401/078 for contact weights.

TABLE 1(b) - MAXIMUM RATINGS

| No. | Characteristic | Symbol | Maximum Rating | Unit | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Working Voltage | UR | 150 | Vrms | Note 1 |
| 2 | Rated Current with | IR |  | A | Note 2 |
|  | AWG24 wire |  | 3.5 |  |  |
|  | AWG26 and uninsulated solid wire |  | 2.5 |  |  |
|  | AWG28 wire |  | 1.5 |  |  |
| 3 | Operating Temperature Range | Top | -55 to +125 | °C | Tamb |
| 4 | Storage Temperature Range | Tstg | -65 to +125 | °C | - |

NOTES:

            - At Sea Level, between contacts, and contact and shell. UR requires derating at altitudes above sea level. See Figure 1(a).

            - IR requires derating if the number of carrying contacts in the connector is 2 or greater. See Figure 1(b).

FIGURE 1 - PARAMETER DERATING INFORMATION

FIGURE 1(a) - WORKING VOLTAGE VERSUS ALTITUDE

FIGURE 1(b) - MAXIMUM CURRENT VERSUS NUMBER OF CONTACTS

| Number of Current-Carrying Contacts per Connector | Maximum Current per Contact (A) |  |  |
| --- | --- | --- | --- |
|  | Wire Size |  |  |
|  | AWG24 | AWG26 and Uninsulated Solid Wire | AWG28 |
| 2 - 4 | 2.6 | 2 | 1.4 |
| 5 - 14 | 2.4 | 1.8 | 1.2 |
| 15 and over | 1.9 | 1.4 | 0.9 |

FIGURE 2 - PHYSICAL DIMENSIONS

Figure 2.0 – male and female interface dimensions

FEMALE RECEPTACLE

MALE PLUG

NOTE: These dimensions are specified in the Plug (Male Contacts) Figure on the following page.

| Shell Size | Dint Min. | Eint Min. | Rint Min. | Rext Max. |
| --- | --- | --- | --- | --- |
| 9 | 8.49 | 4.7 | 1.704 | 1.699 |
| 15 | 12.3 | 4.7 | 1.704 | 1.699 |
| 21 | 16.11 | 4.7 | 1.704 | 1.699 |
| 25 | 18.65 | 4.7 | 1.704 | 1.699 |
| 31 | 22.46 | 4.7 | 1.704 | 1.699 |
| 37 | 26.27 | 4.7 | 1.704 | 1.699 |
| 51 | 25 | 5.8 | 1.704 | 1.699 |

Figure 2.1(a) – Connector Shells - Plug (Male Contacts)

| Shell Size | A Max. | B Max. | C Max. | D Max. | E Max. | F |  | G Max. | H Max. | J Max. | M Max. | N |  | ØS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Min. | Max. |  |  |  |  | Min. | Max. | Min. | Max. |
| 9 | 19.94 | 8.48 | 10.16 | 6.86 | 7.82 | 14.22 | 14.48 | 4.69 | 10.57 | 4.72 | 7.26 | 2.23 | 2.49 | 2.23 | 2.44 |
| 15 | 23.75 | 12.29 | 13.97 | 6.86 | 7.82 | 18.03 | 18.29 | 4.69 | 10.57 | 4.72 | 7.26 | 2.23 | 2.49 | 2.23 | 2.44 |
| 21 | 27.56 | 16.1 | 17.78 | 6.86 | 7.82 | 21.84 | 22.1 | 4.69 | 10.57 | 4.72 | 7.26 | 2.23 | 2.49 | 2.23 | 2.44 |
| 25 | 30.1 | 18.64 | 20.32 | 6.86 | 7.82 | 24.38 | 24.64 | 4.69 | 10.57 | 4.72 | 7.26 | 2.23 | 2.49 | 2.23 | 2.44 |
| 31 | 33.91 | 22.45 | 24.13 | 6.86 | 7.82 | 28.19 | 28.45 | 4.69 | 10.57 | 4.72 | 7.26 | 2.23 | 2.49 | 2.23 | 2.44 |
| 37 | 37.72 | 26.26 | 27.94 | 6.86 | 7.82 | 32 | 32.26 | 4.69 | 10.57 | 4.72 | 7.26 | 2.23 | 2.49 | 2.23 | 2.44 |
| 51 | 36.45 | 24.99 | 26.67 | 7.87 | 8.92 | 30.73 | 30.99 | 5.78 | 10.57 | 4.72 | 7.26 | 2.23 | 2.49 | 2.23 | 2.44 |
| Shell Size | P Max. | Q Max. | R Max. |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | 4.4 | 7.8 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 4.4 | 11.6 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 | 4.4 | 15.4 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 | 4.4 | 17.95 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | 4.4 | 21.75 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 | 4.4 | 25.6 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 51 | 5.3 | 24.3 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |

NOTES:

            - All dimensions are in millimetres.

Figure 2.1(b) – Connector Shells - Receptacle (Female Contacts)

| Shell Size | A Max. | B Max. | C Max. | D Max. | E Max. | F |  | G Max. | H Max. | J Max. | M Max. | N |  | ØS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Min. | Max. |  |  |  |  | Min. | Max. | Min. | Max. |
| 9 | 19.94 | 10.16 | 10.16 | 6.86 | 7.82 | 14.22 | 14.48 | 6.38 | 10.9 | 5.05 | 7.59 | 2.23 | 2.49 | 2.23 | 2.44 |
| 15 | 23.75 | 13.97 | 13.97 | 6.86 | 7.82 | 18.03 | 18.29 | 6.38 | 10.9 | 5.05 | 7.59 | 2.23 | 2.49 | 2.23 | 2.44 |
| 21 | 27.56 | 17.78 | 17.78 | 6.86 | 7.82 | 21.84 | 22.1 | 6.38 | 10.9 | 5.05 | 7.59 | 2.23 | 2.49 | 2.23 | 2.44 |
| 25 | 30.1 | 20.32 | 20.32 | 6.86 | 7.82 | 24.38 | 24.64 | 6.38 | 10.9 | 5.05 | 7.59 | 2.23 | 2.49 | 2.23 | 2.44 |
| 31 | 33.91 | 24.13 | 24.13 | 6.86 | 7.82 | 28.19 | 28.45 | 6.38 | 10.9 | 5.05 | 7.59 | 2.23 | 2.49 | 2.23 | 2.44 |
| 37 | 37.72 | 27.94 | 27.94 | 6.86 | 7.82 | 32 | 32.26 | 6.38 | 10.9 | 5.05 | 7.59 | 2.23 | 2.49 | 2.23 | 2.44 |
| 51 | 36.45 | 26.67 | 26.67 | 7.87 | 8.92 | 30.73 | 30.99 | 7.47 | 10.9 | 5.05 | 7.59 | 2.23 | 2.49 | 2.23 | 2.44 |
| Shell Size | P Max. | Q Max. | R Max. |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 | 4.4 | 7.8 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 15 | 4.4 | 11.6 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 | 4.4 | 15.4 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 | 4.4 | 17.95 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | 4.4 | 21.75 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 | 4.4 | 25.6 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |
| 51 | 5.3 | 24.3 | 5.7 |  |  |  |  |  |  |  |  |  |  |  |  |

NOTES:

            - All dimensions are in millimetres.

FIGURE 2.2 – contact positionS

Figure 2.2.1 – Mounting Condition

Figure 2.2.2 – Plug Male Contact

Figure 2.2.3 – Receptacle Female Contact

| F |  | S |  | Z |
| --- | --- | --- | --- | --- |
| Min. | Max. | Min. | Max. | Max. |
| 0.25 | 0.91 | 3.3 | 3.66 | 5.49 |

NOTES:

            - All dimensions in millimetres.

            - The Interfacial Seal is optional and is specified in ESCC Detail Specification No. 3401/032.

figure 2.3 – uninsulated solid wires accepted

| Wire Size AWG | 25 |
| --- | --- |
| Max Diameter D mm | 0.51 |
| Min Diameter D mm | 0.4 |
| Min Gold Plating Thickness µm | 0.5 |
| Max. Weight g/m | 1.6 |

figure 2.4 – insulated solid wires accepted

|  | ESCC 3901/002 |  |  | ESCC 3901/013 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Wire Size AWG |  |  |  |  |  |
|  | 24 | 26 | 28 | 24 (1) | 26 | 28 |
| Maximum Conductor Diameter mm | 0.64 | 0.53 | 0.43 | 0.62 | 0.5 | 0.42 |
| Maximum Wire Diameter mm | 0.88 | 0.78 | 0.68 | 1.04 | 0.89 | 0.82 |

Notes:

            - Not applicable for shell size 51 (3 rows of contacts).

FIGURE 2.5 – floating mount (Notes 2 and 3)

| d | d1 | ØM Max. | ØN Min. | P Max. |
| --- | --- | --- | --- | --- |
| 1 | 0.8 | 4 | 2.26 | 4.7 |

NOTES:

            - All dimensions are in millimetres.

            - Total Lateral Float 0.4 (typical).

            - Total Axial Float 0.4 (typical).

Figure 2.6 - Captive Nut

| Øg | J Max. | ØM Max. |
| --- | --- | --- |
| Note 2 | 2.6 | 5.1 |

NOTES:

            - All dimensions are in millimetres.

            - Øg: 2-56 UNC 28, Maximum Torque 0.44Nm.

FIGURE 3 – contact arrangements

front view of male insert – use mirror view for female insert

NOTES:

            - Only the outside contact cavities on each row are identified in the drawing, the remainder follow sequentially. Contact numbers are shown outside the insert for readability.

Contact Centres

NOTES:

            - a = Distance between contact centres: 1.27mm (typical).

            - b = Distance between rows: 1.09mm (typical).

Figure 4 – Contact Removal Tool

| Shell Size | Dimensions (mm) |  |  |  |  |  |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A |  | B |  | C |  | ØD |  | E |  |
|  | Min | Max | Min | Max | Min | Max | Min | Max | Max. |  |
| 9 to 37 | 0.13 | 0.17 | 5.7 | 6 | 0.1 | 0.11 | 0.83 | 0.87 | 0.75 | 2 rows of contacts |
| 51 | 0.07 | 0.09 | 5.7 | 6 | 0.06 | 0.07 | 0.83 | 0.87 | 0.75 | 3 rows of contacts |

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the connectors specified herein are stated in this specification and ESCC Generic Specification No. 3401. Deviations from the Generic Specification, applicable to this specification only, are listed in Para. 4.2.

Deviations from the applicable Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirements and do not affect the components’ reliability, are listed in the appendices attached to this specification.

## DEVIATIONS FROM GENERIC SPECIFICATION

### Deviations from Special In-process Controls

None.

### Deviations from Final Production Tests (Chart II)

Para. 9.5, Magnetism Level: Not applicable.

Para. 9.2, Mating Verification: Not applicable.

Para. 9.3, Contact Retainer Test: Replaced by a 100% visual inspection with dedicated equipment.

### Deviations from Burn-in and Electrical Measurements (Chart III)

Chart III is not applicable.

### Deviations from Qualification Tests (Chart IV)

Para. 9.9, Seal Test: Not applicable.

Para. 9.29, Oversize Pin Exclusion: Not applicable.

Para. 9.30, Probe Damage: Not applicable.

Para. 9.31, Solderability: Not applicable.

### Deviations from Lot Acceptance Tests (Chart V)

Para. 9.9, Seal Test: Not applicable.

Para. 9.29, Oversize Pin Exclusion: Not applicable.

Para. 9.30, Probe Damage: Not applicable.

## MECHANICAL REQUIREMENTS

### Dimension Check

The dimensions of the connectors specified herein shall be verified in accordance with the requirements set out in Para. 9.6 of ESCC Generic Specification No. 3401 and shall conform to those shown in Figure 2 of this specification. Only the underlined dimensions shall be checked during procurement.

### Weight

The maximum weight of the connectors specified herein, without contacts and interfacial seals, shall be in accordance with the values given in Table 1(a) of this specification.

### Contact Capability

As specified in ESCC Detail Specification No. 3401/078.

### Contact Retention (In insert)

As specified in ESCC Detail Specification No. 3401/078.

### Mating and Unmating Forces

The forces applied for the mating and unmating of the connectors shall conform to the values specified in Table 1(a).

### Insert Retention (In shell)

Connector inserts shall withstand a pressure of 34.4N/cm2 applied from the mating side to the rear side.

### Jackscrew Retention

Not applicable.

### Contact Insertion and Withdrawal Forces

As specified in ESCC Detail Specification No. 3401/078 with removal tool defined in Figure 4.

### Engagement and Separation Forces (Male Contacts)

As specified in ESCC Detail Specification No. 3401/078.

### Oversize Pin Exclusion

Not applicable.

### Probe Damage

Not applicable.

### Solderability

Not applicable.

## MATERIALS AND FINISHES

The materials and finishes shall be as specified herein. Where a definite material is not specified, a material which will enable the components specified herein to meet the performance requirements of this specification shall be used. Acceptance or approval of any constituent material does not guarantee acceptance of the finished product.

### Shells

Shells shall be made of aluminium alloy. Variant 01 shall have a minimum plating thickness of 25.4µm of electroless nickel. The plating for Variant 02 shall be 0.7µm minimum of gold with 25.4µm minimum electroless nickel underplating.

### Inserts

Inserts shall be made of a suitable thermoplastic material.

### Contacts

As specified in ESCC Detail Specification No. 3401/078.

### Contact Retaining Clip

The retaining clip shall be made of beryllium copper or stainless steel.

## MARKING

### General

The marking of components delivered to this specification shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and the following paragraphs.

Each component shall be marked in respect of:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number.

Series.

Characteristics.

Traceability Information.

### The ESCC Component Number

Each component shall bear the ESCC Component Number which shall be constituted and marked as follows:

Example: 340107701B

Detail Specification Number: 3401077

Type Variant (See Table 1(a))	: 01

Testing Level: B

### Series

The series of the connector, MDMA, shall be marked on the connector if space permits.

### Characteristics

The characteristics to be marked in the following order of precedence are:

Mounting.

Shell Size.

Contact Type.

Contact Information.

The information shall be constituted and marked as follows (example): Y37P-FO

Mounting: Y

Shell size: 37

Contact Type: P

Contact Information: -FO

#### Fixing Option

The letter "Y" shall indicate a floating mount. The letter "E" shall indicate captive nuts. If the shell has standard mounting holes, the letter shall be omitted.

#### Shell Size

Shell size shall be designated by the number of contacts.

Specified numbers are: 9, 15, 21, 25, 31 and 37 and 51.

#### Contact Type

Contact types shall be indicated by the following code letters.

| Code Letter | Contact Type |
| --- | --- |
| P | Male |
| S | Female |

#### Contact Information

-FO = Connector ordered without contacts. This information shall not be marked on the connector; it is only included on the packaging and all relevant data documentation.

Contacts must be from the same Manufacturer as the connector in which they are fitted and this shall be verified prior to assembly.

#### Traceability Information

Traceability information shall be marked in accordance with the requirements of ESCC Basic Specification No. 21700.

## ELECTRICAL MEASUREMENTS

### Electrical Measurements at Room Temperature

The parameters to be measured in respect of electrical characteristics are scheduled in Table 2. Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

### Electrical Measurements at High and Low Temperatures

Not applicable.

### Circuits for Electrical Measurements

Not applicable.

## BURN-IN AND ELECTRICAL MEASUREMENTS (TABLES 4 AND 5)

Not applicable.

TABLE 2 - ELECTRICAL MEASUREMENTS AT ROOM TEMPERATURE

| No. | Characteristic | Symbol | ESCC 3401 Test Method | Test Condition | Limits |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Min. | Max. |  |
| 1 | Insulation Resistance | Ri | Para. 9.1.1.1 | Para. 9.1.1.1 | 5000 (Note 2) | - | MΩ |
|  |  |  |  |  | 2500 (Note 3) | - |  |
| 2 | Voltage Proof Leakage Current | IL | Para. 9.1.1.2 | 600 Vrms Note 2 | - | 2 | mA |
|  |  |  |  | 450 Vrms Note 3 | - | 2 |  |
| 3 | Mated Shell Conductivity (Voltage Drop) Note 1 | VD | Para. 9.1.1.4 | Para. 9.1.1.4 | Not applicable |  | mV |

NOTES:

Applicable to mated connectors with grounding option.

Applicable for shell sizes 9 to 37 (2 rows of contacts).

Applicable for shell size 51 (3 rows of contacts).

TABLES 3, 4 AND 5

Not applicable.

## ENVIRONMENTAL AND ENDURANCE TESTS (CHARTS IV AND V OF ESCC GENERIC SPECIFICATION No. 3401)

### Measurements and Inspections on Completion of Environmental Tests

The parameters to be measured and inspections to be performed on completion of environmental testing shall be those specified in Table 6. Unless otherwise specified, these measurements shall be performed at Tamb = +22 ±3°C.

### Measurements and Inspections at Intermediate Points during Endurance Tests

Not applicable.

### Measurements and Inspections on Completion of Endurance Tests

The parameters to be measured and inspections to be performed on completion of endurance testing shall be those specified in Table 6. Unless otherwise specified, these measurements shall be performed at Tamb = +22 ±3°C.

### Conditions for Operating Life Test (Part of Endurance Testing)

Not applicable.

### Electrical Circuit for Operating Life Test

Not applicable.

### Conditions for High Temperature Storage Test (Part of Endurance Testing)

The requirements for the high temperature storage test are specified in Section 9 of ESCC Generic Specification No. 3401. The temperature to be applied shall be the maximum storage temperature specified in Table 1(b) of this specification.

TABLE 6 – MEASUREMENTS AND INSPECTIONS ON COMPLETION OF ENVIRONMENTAL AND ENDURANCE TESTING

| No. | ESCC Generic Spec. No. 3401 |  | Measurements and Inspections |  | Symbol | Limits |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Environmental and Endurance Tests Note 1 | Test Method and Conditions | Identification | Conditions |  | Min. | Max. |  |
| 01 | Seal Test | Para. 9.9 | Not applicable | - | - | - | - |  |
| 02 | Wiring | Para. 9.10 | ESCC 3401/078 | - | - | - | - |  |
| 03 | Vibration | Para. 9.11 | Initial Measurements Coupling Screw(s) Unlocking Torque |  | Tqe | Record Values |  |  |
|  |  |  | Final Measurements Full Engagement |  |  |  |  |  |
|  |  |  | Coupling Screw(s) Unlocking Torque Drift | - | ΔTqe/Tqe | -25 | +25 | % |
|  |  |  | Visual Examination | - | - | - | - |  |
| 04 | Shock or Bump | Para. 9.12 | Full Engagement Visual Examination | - | - | - | - |  |
| 05 | Climatic Sequence | Para. 9.13 | Dry Heat Insulation Resistance | At High Temperature Table 2, Item 1 | Ri | 10 | - | MΩ |
|  |  |  | Low Air Pressure Voltage Proof Leakage Current | Figure 1 | IL | ESCC 3401 Para. 9.13.5 |  | mA |
|  |  |  | Damp Heat | Immediately after test |  |  |  |  |
|  |  |  | Insulation Resistance | Table 2, Item 1 | Ri | 100 | - | MΩ |
|  |  |  | Final Measurements | After 1-24 hrs Recovery |  |  |  |  |
|  |  |  | External Visual Inspection | ESCC 3401 Para. 9.7 | - | ESCC 3401 Para. 9.7 |  |  |
|  |  |  | Insulation Resistance | Table 2, Item 1 | Ri | Table 2, Item 1 |  | MΩ |
|  |  |  | Voltage Proof Leakage Current | Table 2, Item 2 | IL | Table 2, Item 2 |  | mA |
| 06 | Plating Thickness | Para. 9.14 | Thickness | - | - | - |  |  |
| 07 | Joint Strength | Para. 9.15 | ESCC 3401/078 | - | - | - |  |  |
| 08 | Rapid Change of Temperature | Para. 9.16 | Visual Examination | - | - | - | - |  |
|  |  |  | Insulation Resistance | Table 2, Item 1 | Ri | Table 2, Item 1 |  | MΩ |
|  |  |  | Voltage Proof Leakage Current | Table 2, Item 2 | IL | Table 2, Item 2 |  | mA |
| 09 | Contact Retention (in Insert) | Para. 9.17 & Para. 4.3.4 of this spec. | Contact Displacement | ESCC 3401/078 | - | ESCC 3401/078 |  |  |
| 10 | Endurance | Para. 9.18 | Initial Measurements |  |  |  |  |  |
|  |  |  | Mating/Unmating Forces | - | F | Para. 4.3.5 of this spec. |  | N |
|  |  |  | Low Level Contact Resistance | ESCC 3401/078 | Rcl | ESCC 3401/078 |  | mΩ |
|  |  |  | Mated Shell Conductivity | Table 2, Item 3 | VD | Not applicable |  | mV |
|  |  |  | Final Measurements |  |  |  |  |  |
|  |  |  | Visual Examination | - | - | - | - |  |
|  |  |  | Mating/Unmating Forces | - | F | Para. 4.3.5 of this spec. |  | N |
|  |  |  | Low Level Contact Resistance Drift | ESCC 3401/078 | ΔRcl | ESCC 3401/078 |  | mΩ |
|  |  |  | Mated Shell Conductivity | Table 2, Item 3 | VD | Not applicable |  | mV |
|  |  |  | Insulation Resistance | Table 2, Item 1 | Ri | Table 2, Item 1 |  | MΩ |
|  |  |  | Voltage Proof Leakage Current | Table 2, Item 2 | IL | Table 2, Item 2 |  | mA |
| 11 | Permanence of Marking | Para. 9.19 | As applicable | - | - | - |  |  |
| 12 | Mating / Unmating Forces | Para. 9.20 | Force | - | F | Para. 4.3.5 of this spec. |  | N |
| 13 | High Temperature Storage | Para. 9.21 | Initial Measurements |  |  |  |  |  |
|  |  |  | Low Level Contact Resistance | ESCC 3401/078 | Rcl | ESCC 3401/078 |  | mΩ |
|  |  |  | Mated Shell Conductivity | Table 2, Item 3 | VD | Not applicable |  | mV |
|  |  |  | Final Measurements |  |  |  |  |  |
|  |  |  | Visual Examination | - | - | - | - |  |
|  |  |  | Mating/Unmating Forces | - | F | Para. 4.3.5 of this spec. |  | N |
|  |  |  | Low Level Contact Resistance Drift | ESCC 3401/078 | ΔRcl | ESCC 3401/078 |  | mΩ |
|  |  |  | Rated Current Contact Resistance | ESCC 3401/078 | Rcr | ESCC 3401/078 |  | mΩ |
|  |  |  | Mated Shell Conductivity | Table 2, Item 3 | VD | Not applicable |  | mV |
|  |  |  | Insulation Resistance | Table 2, Item 1 | Ri | Table 2, Item 1 |  | MΩ |
|  |  |  | Voltage Proof Leakage Current | Table 2, Item 2 | IL | Table 2, Item 2 |  | mA |
|  |  |  | Contact Retention (In Insert) | ESCC 3401/078 |  | ESCC 3401/078 |  | N |
| 14 | Corrosion | Para. 9.22 | Visual Examination | - | - | - | - |  |
| 15 | Insert Retention (In Shell) | Para. 9.23 & Para. 4.3.6 of this spec. | Visual Examination | - | - | Para. 4.3.6 of this spec. |  |  |
| 16 | Jackscrew Retention | Para. 9.24 & Para. 4.3.7 of this spec. | Not applicable | - | - | - | - |  |
| 17 | High Temperature Measurements | Para. 9.25 | Insulation Resistance | Table 2 Item 1 | Ri | 10 | - | MΩ |
| 18 | Overload Test | Para. 9.26 | Internal Temperature | - | T | - | +100 | ºC |
|  |  |  | Rated Current Contact Resistance | ESCC 3401/078 | Rcr | ESCC 3401/078 |  | mΩ |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | VD | Not applicable |  | mV |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2, Item 1 |  | MΩ |
|  |  |  | Voltage Proof Leakage Current | Table 2 Item 2 | IL | Table 2, Item 2 |  | mA |
| 19 | Maintenance Ageing | Para. 9.27 | Visual Examination | - | - | - | - |  |
|  |  |  | Contact Retention (In insert) | ESCC 3401/078 | - | ESCC 3401/078 |  | N |
|  |  |  | Contact insertion and Withdrawal Forces | ESCC 3401/078 |  | ESCC 3401/078 |  | N |
| 20 | Engage/Separation Forces | Para. 9.28 & Para. 4.3.9 of this spec. | Force | ESCC 3401/078 | F | ESCC 3401/078 |  | N |
| 21 | Oversize Pin Exclusion | Para. 9.29 & Para. 4.3.10 of this spec. | Not applicable | - | - | - | - |  |
| 22 | Probe Damage | Para. 9.30 & Para. 4.3.11 of this spec. | Not applicable | - | - | - | - |  |
| 23 | Solderability | Para. 9.31 & Para. 4.3.12 of this spec. | Not applicable | - | - | - | - |  |

NOTES:

The tests in this Table refer to either Chart IV or V and shall be used as applicable.

## Appendix A

agreed deviations for c&K Components (F)

| ITEMS AFFECTED | DESCRIPTION OF DEVIATIONS |
| --- | --- |
| Para. 4.2.2, Deviations from Final Production Tests (Chart II) | Para. 9.4, Contact Capability: 100% Contact Capability Test may be omitted provided that a 100% visual inspection of the contacts is performed on each batch submitted to tests defined in the C&K PID requirements. |