Page 1 of 13

CAPACITOR FILTERS, PI-TYPE, FEEDTHROUGH, ELECTROMAGNETIC INTERFERENCE SUPPRESSION

BASED ON TYPE SFP040

ESCC Detail Specification No. 3008/014

| Issue 4 Draft C | January 2019 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2019. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| tbc | Specification upissued to incorporate changes per DCR. |

TABLE OF CONTENTS

1	General	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants	5

1.5	MAXIMUM RATINGS	7

1.6	PHYSICAL DIMENSIONS	7

1.6.1	Straight Terminals – Type Variants 01 to 10 and 21 to 30	7

1.6.2	Button Body-End Terminal – Type Variants 11 to 20 and 31 to 40	8

1.6.3	Consolidated Dimensions and Notes	8

1.7	functional diagram	9

1.8	MATERIALS AND FINISHES	9

1.8.1	Case	9

1.8.2	Lead Material and Finish	9

1.8.3	Accessories	9

2	REQUIREMENTS	9

2.1	GENERAL	9

2.1.1	Deviations from the Generic Specification	9

2.1.1.1	Deviations from Screening Tests - Chart F3	9

2.1.1.2	Deviations from Qualification and Periodic Tests – Chart F4	9

2.2	MARKING	10

2.3	robustness of terminations	10

2.4	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	10

2.4.1	Room Temperature Electrical Measurements	10

2.4.2	High and Low Temperatures Electrical Measurements	11

2.5	PARAMETER DRIFT VALUES	11

2.6	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	12

2.7	BURN-IN CONDITIONS	13

2.8	OPERATING LIFE CONDITIONS	13

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3008.

MIL-STD-202, Test Methods for Electronic and Electrical Component Parts.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 300801401

Detail Specification Reference: 3008014

Component Type Variant Number: 01 (as required)

### Component Type Variants

The Component Type Variants applicable to this specification are as follows:

| Type Variants (Note 1) | Case Description and Terminal Configuration (Note 2) |
| --- | --- |
| 01 to 10 | Non-hermetically sealed feedthrough case with M4×0.70 thread and straight terminals |
| 11 to 20 | Non-hermetically sealed feedthrough case with M4×0.70 thread, button body-end terminal and straight thread-end terminal |
| 21 to 30 | Non-hermetically sealed feedthrough case with #8-32 UNC thread and straight terminals |
| 31 to 40 | Non-hermetically sealed feedthrough case with #8-32 UNC thread, button body-end terminal and straight thread-end terminal |

notes:

Characteristics applicable to each set of Component Type Variants are as follows. See Para. 1.6 for Case Sizes:

| Type Variants | Case Size | Rated DC Voltage UR (V) (at Tamb ≤ +85°C) | Insulation Resistance Ri (GΩ) |  | DC Resistance Rs (mΩ) | Voltage Drop Vdr (V) | Voltage Proof VP (V) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | -55 / +25°C | +125°C |  |  |  |
| 01, 11, 21, 31 | 1 | 200 | 10 | 1 | 15 | 0.15 | 500 |
| 02, 12, 22, 32 | 2 | 200 | 10 | 1 | 5 | 0.05 | 500 |
| 03, 13, 23, 33 | 2 | 250 | 10 | 1 | 4 | 0.04 | 625 |
| 04, 14, 24, 34 | 3 | 100 | 3 | 0.3 | 15 | 0.15 | 250 |
| 05, 15, 25, 35 | 1 | 200 | 10 | 1 | 15 | 0.15 | 500 |
| 06, 16, 26, 36 | 1 | 200 | 10 | 1 | 10 | 0.1 | 500 |
| 07, 17, 27, 37 | 4 | 200 | 10 | 1 | 10 | 0.1 | 500 |
| 08, 18, 28, 38 | 4 | 250 | 10 | 1 | 10 | 0.1 | 625 |
| 09, 19, 29, 39 | 1 | 200 | 10 | 1 | 15 | 0.15 | 500 |
| 10, 20, 30, 40 | 1 | 200 | 10 | 1 | 15 | 0.15 | 500 |

| Type Variants | Capacitance C (pF) | Insertion Loss IL (dB) With No Load / Rated Current Applied |  |  |  |  | Weight Max. (g) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10MHz | 50MHz | 100MHz | 500MHz | 1GHz |  |
| 01, 11, 21, 31 | 2400 | 10 / 10 | 36 / 20 | 47 / 25 | 70 / 70 | 75 / 75 | 2.7 |
| 02, 12, 22, 32 | 2400 | 5 / 5 | 30 / 25 | 40 / 30 | 54 / 54 | 60 / 60 | 2 |
| 03, 13, 23, 33 | 750 | - | - | 15 / 15 | 50 / 50 | 55 / 55 | 2 |
| 04, 14, 24, 34 | 44800 | 39 / 30 | 54 / 50 | 75 / 75 | 75 / 75 | 75 / 75 | 3.5 |
| 05, 15, 25, 35 | 1600 | 10 / 5 | 38 / 20 | 50 / 25 | 75 / 70 | 75 / 75 | 2.7 |
| 06, 16, 26, 36 | 4320 | 13 / 10 | 45 / 22 | 60 / 30 | 75 / 70 | 75 / 75 | 2.7 |
| 07, 17, 27, 37 | 2400 | 5 / 5 | 30 / 25 | 40 / 30 | 60 / 55 | 60 / 60 | 2 |
| 08, 18, 28, 38 | 750 | - | - | 15 / 15 | 50 / 45 | 55 / 55 | 2 |
| 09, 19, 29, 39 | 750 | - | - | 15 / 15 | 50 / 45 | 55 / 55 | 2.7 |
| 10, 20, 30, 40 | 160 | - | - | 4 / 4 | 40 / 35 | 54 / 54 | 2.7 |

See Para. 1.6 for physical dimensions.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| Rated DC Voltage | UR | See Note 1 of Para. 1.4.2 | V | Note 1 |
| Torque | Tqe | 0.4 | Nm |  |
| Voltage Drop | Vdr | See Note 1 of Para. 1.4.2 | V | DC |
| DC and Low Frequency Current | ILF | 10 | A |  |
| Operating Temperature Range | Top | -55 to +125 | °C | Tamb |
| Storage Temperature Range | Tstg | -55 to +125 | °C |  |
| Soldering Temperature | Tsol | +260 | °C | Note 2 |

NOTES:

At Tamb ≤ +85°C. For Tamb > +85°C, the following derating shall apply:

For Variants with UR = 250V at Tamb ≤ +85°C, derate linearly to 150V at Tamb = +125°C.

For Variants with UR = 200V at Tamb ≤ +85°C, derate linearly to 100V at Tamb = +125°C.

For Variants with UR = 100V at Tamb ≤ +85°C, derate linearly to 70V at Tamb = +125°C.

Duration 10 seconds maximum at a distance of not less than 2mm from the body and the same lead shall not be resoldered until 3 minutes have elapsed.

## PHYSICAL DIMENSIONS

Consolidated Dimensions and Notes follow the case drawings.

### Straight Terminals – Type Variants 01 to 10 and 21 to 30

### Button Body-End Terminal – Type Variants 11 to 20 and 31 to 40

### Consolidated Dimensions and Notes

| Symbols | Dimensions (mm) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Case Size 1 |  | Case Size 2 |  | Case Size 3 |  | Case Size 4 |  |
|  | Min | Max | Min | Max | Min | Max | Min | Max |
| A | 6.9 | 7.1 | 3.4 | 3.6 | 7.8 | 8.4 | 3.4 | 3.6 |
| B | 2.4 | 2.6 | 1.5 | 1.7 | 3.5 | 4.1 | 1.5 | 1.7 |
| C | 7 | 12 | 3.5 | 4.5 | 8 | 9 | 8 | 13 |
| D | 17 | 19 | 5.5 | 6.5 | 13 | 14 | 8 | 13 |
| E (Note 1) | Note 2 |  | Note 2 |  | Note 2 |  | Note 2 |  |
| F (Notes 1, 3) | 0.72 | 0.88 | 0.72 | 0.88 | 0.72 | 0.88 | 0.72 | 0.88 |
| G | 1 | 1.2 | 1 | 1.2 | 1 | 1.2 | 1 | 1.2 |
| J | - | 5 | - | 5 | - | 5 | - | 5 |
| K | - | 6 | - | 6 | - | 6 | - | 6 |
| L | - | 2.5 | - | 1.6 | - | 2.5 | - | 1.6 |
| M | - | 6.4 | - | 6.4 | - | 6.4 | - | 6.4 |
| N | - | 0.4 | - | 0.4 | - | 0.4 | - | 0.4 |

NOTES:

Lead finish shall commence not more than 1.5mm from encapsulant.

This dimension shall be that of the applicable thread, which is either M4×0.70 or #8-32 UNC, depending on the Component Type Variant. See Para. 1.4.2.

Applies to both leads.

## functional diagram

## MATERIALS AND FINISHES

### Case

The case shall be silver plated brass with potting encapsulant sealing the filter element.

### Lead Material and Finish

The lead material shall be Type B with Type 10 finish in accordance with the requirements of ESCC Basic Specification No. 23500.

### Accessories

The materials and finishes of the accessories shown in Paras. 1.6.1 and 1.6.2 shall be as follows:

Nut: silver plated brass.

Lock-washer: silver plated bronze.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests - Chart F3

Room Temperature Electrical Measurements: Voltage Drop shall be replaced by a DC resistance measurement performed in accordance with MIL-STD-202, Test Method 303.

External Visual Inspection: Any discolouration of the silver plating of the case is acceptable.

#### Deviations from Qualification and Periodic Tests – Chart F4

Intermediate and End-Point Electrical Measurements: Voltage Drop shall be replaced by a DC resistance measurement performed in accordance with MIL-STD-202, Test Method 303.

Accelerated Damp Heat: Not applicable.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component or its primary package shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## robustness of terminations

The leads of the components are rigid.

The test conditions for Robustness of Terminations shall be as specified in the ESCC Generic Specification and as follows:

Test Ua1, tensile, with an applied force of 10N and a duration of 10 ±1 seconds.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

The measurements shall be performed at room, high and low temperatures.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| DC Resistance | Rs | MIL-STD-202, Test Method 303 | - | Note 1 | mΩ |
| Voltage Proof | VP | ESCC No. 3008 2.5 × UR | Note 1 | - | V |
| Insulation Resistance | Ri | ESCC No. 3008 | Note 1 | - | GΩ |
| Insertion Loss |  | ESCC No. 3008 |  |  |  |
|  | IL1 | f = 10MHz (Notes 2, 3) | Note 1 | - | dB |
|  | IL2 | f = 50MHz (Notes 2, 4) | Note 1 | - | dB |
|  | IL3 | f = 100MHz (Notes 2, 4) | Note 1 | - | dB |
|  | IL4 | f = 500MHz (Notes 2, 3) | Note 1 | - | dB |
|  | IL5 | f = 1GHz (Notes 2, 4) | Note 1 | - | dB |
| Capacitance | C | ESCC No. 3008 f = 1000 ±100Hz V = 0.1 to 1.2Vrms | Note 1 | - | pF |

NOTES:

See Note 1 of Para. 1.4.2.

For Qualification Testing and Periodic Testing for renewal of qualification after lapse, measurements shall be made with a load current of zero and, during either Subgroup 1B or Subgroup 1C of Chart F4 in the Generic Specification, measurements shall also be made at the rated current specified in Para. 1.5.

For Screening and Periodic Testing for extension of qualification, IL1 and IL4 are guaranteed but not tested.

For Screening and Periodic Testing for extension of qualification, IL2, IL3 and IL5 shall be measured with no load current applied.

### High and Low Temperatures Electrical Measurements

Unless otherwise specified, the measurements shall be performed at Tamb = +125 (+0 -5)°C and Tamb = -55 (+5 -0)°C.

| Characteristics | Symbols | Test Method and Conditions (Note 1) | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance | Ri | ESCC No. 3008 Tamb = +125 (+0 -5)°C | Note 2 | - | GΩ |
| Insertion Loss |  | ESCC No. 3008 |  |  |  |
|  | IL2 | f = 50MHz (Note 3) | Note 2 | - | dB |
|  | IL3 | f = 100MHz (Note 3) | Note 2 | - | dB |
|  | IL5 | f = 1GHz (Note 3) | Note 2 | - | dB |

NOTES:

Measurements shall be performed on a sample of 5 components. In the event of any failure a 100% inspection shall be performed.

See Note 1 of Para. 1.4.2.

Measurements shall be made with no load current applied.

## PARAMETER DRIFT VALUES

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.4.1, Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic where specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Drift Value Δ | Units |
| --- | --- | --- | --- |
| Change in Capacitance | ΔC/C | ±10 | % |

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified, test methods and test conditions shall be as per the corresponding test defined in Para. 2.4.1, Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic where specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Test Reference per ESCC No. 3008 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Low Air Pressure | Voltage Proof | VP | 125% UR | - | V |
| Overload | DC Resistance | Rs | - | Note 1 | mΩ |
|  | Insulation Resistance | Ri | Note 1 | - | GΩ |
| Damp Heat | Insulation Resistance | Ri | Note 2 | - | GΩ |
| Vibration | Insertion Loss | IL | Note 1 | - | dB |
| Resistance to Soldering Heat | Insulation Resistance | Ri | Note 1 | - | GΩ |
|  | Insertion Loss | IL | Note 1 | - | dB |
| Shock | Insertion Loss | IL | Note 1 | - | dB |
| Operating Life |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 1 | - | pF |
| Intermediate Measurements (at 500 hours (Note 4)) / (at 1000 hours (Note 5)) | Insulation Resistance at +125 (+0 -3)°C | Ri | Note 1 | - | GΩ |
|  | After recovery |  |  |  |  |
|  | Capacitance | C | Note 1 | - | pF |
|  | Change in Capacitance | ΔC/C | - | ±10 | % |
|  | Voltage Proof | VP | 90% UR | - | V |
|  | Insulation Resistance | Ri | Note 3 | - | GΩ |
|  | Insertion Loss | IL | Note 1 | - | dB |
| Final Measurements (at 1000 or 2000 hours (Note 6)) | Insulation Resistance at +125 (+0 -3)°C | Ri | Note 1 | - | GΩ |
|  | After recovery |  |  |  |  |
|  | Capacitance | C | Note 1 | - | pF |
|  | Change in Capacitance | ΔC/C | - | ±10 | % |
|  | Voltage Proof | VP | 90% UR | - | V |
|  | Insulation Resistance | Ri | Note 3 | - | GΩ |
|  | Insertion Loss | IL | Note 1 | - | dB |
| Robustness of Terminations | DC Resistance | Rs | - | Note 1 | mΩ |

NOTES:

See Note 1 of Para. 1.4.2.

> 10% of the value given in Note 1 of Para. 1.4.2.

> 50% of the value given in Note 1 of Para. 1.4.2.

500 hours is applicable to Qualification Testing, Periodic Testing for extension of qualification and to Periodic Testing for renewal of qualification after lapse.

1000 hours is applicable to Qualification Testing, and to Periodic Testing for renewal of qualification after lapse.

1000 hours is applicable to Periodic Testing for extension of qualification. 2000 hours is applicable to Qualification Testing, and to Periodic Testing for renewal of qualification after lapse.

## BURN-IN CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +125 (+0 -3) | °C |
| Applied Voltage | UA |  | V |
| Variants 03, 08, 13, 18, 23, 28, 33, 38 |  | 300 |  |
| Variants 01, 02, 05, 06, 07, 09, 10, 11, 12, 15, 16, 17, 19, 20, 21, 22, 25, 26, 27, 29, 30, 31, 32, 35, 26, 37, 39, 40 |  | 200 |  |
| Variants 04, 14, 24, 34 |  | 140 |  |
|  |  | (Note 1) |  |

notes:

Between terminal and case. The voltage polarity shall be positive on the case during the first 24 to 72 hours and then negative on the case for the remaining burn-in period.

## OPERATING LIFE CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +125 (+0 -3) | °C |
| Applied Voltage | UA |  | V |
| Variants 03, 08, 13, 18, 23, 28, 33, 38 |  | 300 |  |
| Variants 01, 02, 05, 06, 07, 09, 10, 11, 12, 15, 16, 17, 19, 20, 21, 22, 25, 26, 27, 29, 30, 31, 32, 35, 26, 37, 39, 40 |  | 200 |  |
| Variants 04, 14, 24, 34 |  | 140 |  |
|  |  | (Note 1) |  |
| Current | ILF | 10 (Note 2) | A |

NOTES:

Between terminal and case. The voltage polarity shall be positive on the case during the first 24 to 72 hours and then negative on the case for the remaining operating life period.

To flow between the terminals.