Page 1 of 13

CAPACITORS, LEADLESS SURFACE MOUNTED, ORGANIC POLYMER TANTALUM, SOLID ELECTROLYTE, ENCLOSED ANODE CONNECTION

BASED ON TYPE T583

ESCC Detail Specification No. 3012/005

| Issue 2 Draft B | January 2019 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2019. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| 1186 | Specification upissued to incorporate changes per DCR. |
| Draft 2A S.Thacker (TW) 07/03/2017 | Draft 2A created per converted format to go with converted Generic 3012 Draft 4A. |
| Draft 2B S.Thacker (TW) 09/01/2019 | Draft 2B created per converted format to go with converted Generic 3012 Draft 4A. Note: Specific technical changes per DCR1186 are highlighted blue. Changes resulting from conversion of the Generic 3012 are highlighted yellow |

TABLE OF CONTENTS

1	GENERAL	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND Component Type Variants	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants and Range of Components	6

1.5	MAXIMUM RATINGS	7

1.6	PHYSICAL DIMENSIONS and terminal identification	8

1.7	FUNCTIONAL DIAGRAM	8

1.8	MATERIALS AND FINISHES	8

1.8.1	Case	8

1.8.2	Terminal Material and Finish	8

2	REQUIREMENTS	9

2.1	GENERAL	9

2.1.1	Deviations from the Generic Specification	9

2.2	MARKING	9

2.3	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	9

2.3.1	Room Temperature Electrical Measurements	9

2.3.2	High and Low Temperatures Electrical Measurements	10

2.4	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	10

APPENDIX A	13

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, and test and inspection data for the component type variants and/or the Component Type Variants and Range of Components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3012.

IPC/JEDEC J-STD-020, Moisture/Reflow Sensitivity Classification for Nonhermetic Solid State Surface Mount Devices.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND Component Type Variants

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 301200501336MCE060

Detail Specification Reference: 3012005

Component Type Variant Number: 01

Characteristic code: Rated Capacitance (33μF): 336 (as required)

Characteristic code: Capacitance Tolerance (±20%): M

Rating code: DC Rated Voltage (16V): C (as required)

Characteristic code: Maximum Equivalent Series Resistance (60mΩ): E060 (as required)

#### Characteristics and Ratings Codes

Characteristics and ratings to be codified as part of the ESCC Component Number shall be as follows:

Rated Capacitance, Cn, expressed by the following codes in accordance with ESCC Basic Specification No. 21700. The unit quantity shall be picofarad (pF).

| Capacitance Value C (pF) | Code |
| --- | --- |
| XX 106 | XX6 |
| XX 107 | XX7 |

Capacitance Tolerance expressed by the following codes in accordance with ESCC Basic Specification No. 21700:

| Tolerance (± %) | Code |
| --- | --- |
| 20 | M |

DC Rated Voltage, UR, expressed by the following codes:

| DC Rated Voltage UR (V) | Code |
| --- | --- |
| 6.3 | J |
| 10 | A |
| 16 | C |

Equivalent Series Resistance maximum value, ESR, expressed by the following codes. The unit quantity shall be milliohm (mΩ):

| Equivalent Series Resistance ESR (mΩ) | Code |
| --- | --- |
| XX | E0XX |
| XXX | EXXX |

### Component Type Variants and Range of Components

The Component Type Variants and Range of Components applicable to this specification are as follows:

| Variant Number | Case Code (Style) (Note 1) | Capacitance Range Cn (µF) (Notes 2, 3) | DC Rated Voltage UR (V) (Note 2) | Maximum Equivalent Series Resistance ESR (mΩ) (Note 2) | Weight Max (g) |
| --- | --- | --- | --- | --- | --- |
| 01 | D (7343-31) | 33 to 150 | 6.3, 10, 16 | 45 to 100 | 0.3 |

NOTES:

See Para. 1.6.

The following rated Capacitance (Cn), maximum DC Rated Voltage (UR) and maximum Equivalent Series Resistance values (ESR) are available for Variant 01 (Case Code D) (numbers indicate maximum ESR in mΩ):

| Rated Capacitance Cn (F) | DC Rated Voltage UR |  |  |
| --- | --- | --- | --- |
|  | 6.3V | 10V | 16V |
| 33 |  |  | 60, 70 |
| 47 |  |  | 70 |
| 68 |  | 45, 60, 100 |  |
| 100 | 45 | 55, 80 |  |
| 150 | 45, 55 |  |  |

The following Capacitance Tolerance is available:

±20% (M)

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| DC Rated Voltage | UR | See Para. 1.4.2 | V | Note 1 |
| DC Surge Voltage | US | 1.3 x UR | V | Tamb ≤ +85°C |
| DC Category Voltage For UR = 6.3V, 10V: For UR = 16V: | UC | 0.9 x UR 0.8 x UR | V |  |
| Ripple Current | Iripple | See Note 2 | A | Tamb ≤ +45°C, f = 100kHz, Note 3 |
| Operating Temperature Range | Top | -55 to +105 | °C | Tamb |
| Rated Temperature | TR | +85 | °C |  |
| Upper Category Temperature | TC | +105 | °C |  |
| Storage Temperature Range | Tstg | -55 to +105 | °C | 4 |
| Soldering Temperature | Tsol | +235 | °C | Note 4, 5 |

Notes:

            - At Tamb ≤ +85°C. For Tamb > +85°C, derate linearly to UC at Tamb = +105°C.

            - Maximum Iripple, which depends on Cn, UR, and ESR, shall be as follows at Tamb ≤ +45°C and f = 100kHz:

| Capacitance Cn (µF) | DC Rated Voltage UR (V) | Maximum Equivalent Series Resistance ESR (mΩ) | Maximum Ripple Current Iripple (A) |
| --- | --- | --- | --- |
| 100 | 6.3 | 45 | 2.2 |
| 150 | 6.3 | 45 | 2.2 |
| 150 | 6.3 | 55 | 2 |
| 68 | 10 | 45 | 2.2 |
| 68 | 10 | 60 | 1.9 |
| 68 | 10 | 100 | 1.5 |
| 100 | 10 | 55 | 2 |
| 100 | 10 | 80 | 1.7 |
| 33 | 16 | 60 | 1.9 |
| 33 | 16 | 70 | 1.8 |
| 47 | 16 | 70 | 1.8 |

            - For +45°C < Tamb ≤ +85°C, derate linearly to 0.7Iripple at Tamb = +85°C. For Tamb > +85°C, derate linearly to 0.47Iripple at Tamb = +105°C.

            - These components are classified as Moisture Sensitivity Level 3 in accordance with J-STD-020. Components shall be delivered in moisture barrier bags with a desiccant and moisture indicator card. Components should be stored still contained within the moisture barrier bags in a non-condensating atmospheric environment of Tamb ≤ +40°C and relative humidity RH ≤ 90%.

- These components have a floor life of 168 hours at Tamb ≤ +30°C and RH ≤ 60%.

            - Duration 20 seconds maximum for reflow soldering.

## PHYSICAL DIMENSIONS and terminal identification

| Variant Number | Case Code | Dimensions (mm) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | L |  | H |  | W1 |  | W2 |  | Z |  |
|  |  | Min | Max | Min | Max | Min | Max | Min | Max | Min | Max |
| 01 | D | 7 | 7.6 | 2.5 | 3.1 | 4 | 4.6 | 2.3 | 2.5 | 1 | 1.6 |

NOTES:

Terminal identification: The anode terminal shall be indicated by a polarity stripe marked on the top surface of the component.

## FUNCTIONAL DIAGRAM

## MATERIALS AND FINISHES

### Case

The case shall be moulded epoxy.

### Terminal Material and Finish

Terminal material shall be nickel plated copper alloy, and finish shall be as type 15 in accordance with the requirements of ESCC Basic Specification No. 23500.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests – Chart F3

          - Para. 8.3, Surge Current: shall be performed both at high and room temperatures in the sequence as follows:

- Surge Current in accordance with Para. 8.3 with a test temperature: Tamb = +105 (+0 -3)°C

- Whilst still at the high test temperature, DC Leakage Current shall be measured in accordance with Para. 2.3.2 High and Low Temperatures Electrical Measurements herein at Tamb = +105°C only.

- Surge Current in accordance with Para. 8.3 with a test temperature: Tamb = +25 ±5°C.

#### Deviations from Qualification, Periodic Testing and Lot Validation Testing – Chart F4

Para. 8.16, Operating Life: for the Intermediate Data Points during testing, DC Leakage Current shall be measured at Tamb = +25 ±5°C as specified in Para.2.4 herein.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700. The information to be marked shall be:

Terminal Identification (see Para. 1.6).

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number.

Traceability information.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +25 ±5°C.

| Characteristics | Symbols | Test Method and Conditions | Tolerance | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Capacitance | C | ESCC No. 3012 | ±20% | 0.8Cn | 1.2Cn | µF |
| DC Leakage Current | IL | ESCC No. 3012 | All | - | 0.1Cn x UR | µA |
| Dissipation Factor | DF | ESCC No. 3012 f = 120Hz | All | - | 10 | % |
| Equivalent Series Resistance | ESR | ESCC No. 3012 | All | - | Note 1 | mΩ |

NOTES:

See Para. 1.4.2, Note 2.

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | Test Method and Conditions (Note 1) | Tolerance | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Temperature Characteristic | ΔC/C | ESCC No. 3012 | All |  |  |  |
|  |  | Tamb = +25 ±5°C (ref)(2): |  | See Para. 2.3.1 |  |  |
|  |  | Tamb = -55 (+3 -0)°C: |  | -20% | +20% | % (2) |
|  |  | Tamb = +85 ±3°C: |  | -20% | +20% | % (2) |
|  |  | Tamb = +105 (+0 -3)°C: |  | -30% | +30% | % (2) |
| DC Leakage Current | IL | ESCC No. 3012 | All |  |  | µA |
|  |  | Tamb = +85 ±3°C, V = UR ±2%: |  | - | Cn x UR |  |
|  |  | Tamb = +105 (+0 -3)°C, V = UC ±2%: |  | - | Cn x UR |  |
| Dissipation Factor | DF | ESCC No. 3012, f = 120Hz | All |  |  | % |
|  |  | Tamb = -55 (+3 -0)°C: |  | - | 10 |  |
|  |  | Tamb = +85 ±3°C: |  | - | 12 |  |
|  |  | Tamb = +105 (+0 -3)°C: |  | - | 15 |  |

NOTES:

            - Measurements shall be performed on a sample of 5 components from each manufacturing lot and each capacitance value with 0 failures allowed. In the event of any failure a 100% inspection may be performed.

            - Referenced to the measurement made at Tamb = +25 ±5°C.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +25 ±5°C.

Unless otherwise specified the test methods and test conditions shall be as per the corresponding test defined in Para. 2.3.1.

| Test Reference per ESCC No. 3012 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Mounting |  |  |  |  |  |
| Final Measurements | Capacitance | C | Note 1 |  |  |
|  | DC Leakage Current | IL | Note 1 |  |  |
|  | Dissipation Factor | DF | Note 1 |  |  |
|  | Equivalent Series Resistance | ESR | Note 1 |  |  |
| Robustness of Terminations |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 2 |  |  |
| Final Measurements | Capacitance | C | Record Value |  |  |
|  | Capacitance Change | ΔC/C | -10 | +10 | % (3) |
| Rapid Change of Temperature |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 2 |  |  |
| Final Measurements | Capacitance | C | Record Value |  |  |
|  | Capacitance Change | ΔC/C | -10 | +10 | % (3) |
|  | DC Leakage Current | IL | Note 1 |  |  |
|  | Dissipation Factor | DF | Note 1 |  |  |
|  | Equivalent Series Resistance | ESR | Note 1 |  |  |
| Vibration |  |  |  |  |  |
| During Last Sweep Cycle | No intermittent contact ≥ 0.5ms, arcing, or open/short circuits | - | - | - |  |
| Climatic Sequence |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 2 |  |  |
| Dry Heat | DC Leakage Current (at +105°C; UC) | IL | Note 4 |  |  |
| Final Measurements | Capacitance | C | Record Value |  |  |
|  | Capacitance Change | ΔC/C | -5 | +5 | % (3) |
|  | DC Leakage Current | IL | Note 1 |  |  |
|  | Dissipation Factor | DF | Note 5 |  |  |
|  | Equivalent Series Resistance | ESR | Note 5 |  |  |
| High and Low Temperature Stability |  |  |  |  |  |
| Step 1 (at +25°C) | Note 1 (All Characteristics) | Note 1 | Note 1 |  |  |
| Step 2 (at -55°C) | Note 4 (All Characteristics) | Note 4 | Note 4 |  |  |
| Step 3 (at +25°C) | Note 1 (All Characteristics) | Note 1 | Note 1 |  |  |
| Step 4 (at +85°C) | Note 4 (All Characteristics) | Note 4 | Note 4 |  |  |
| Step 5 (at +105°C) | Note 4 (All Characteristics) | Note 4 | Note 4 |  |  |
| Step 6 (at +25°C) | Note 1 (All Characteristics) | Note 1 | Note 1 |  |  |
| Surge Voltage |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 1 (6) |  |  |
| Final Measurements | Capacitance | C | Note 1 |  |  |
|  | Capacitance Change | ΔC/C | -20 | +5 | % (3) |
|  | DC Leakage Current | IL | Note 1 |  |  |
|  | Dissipation Factor | DF | Note 1 |  |  |
|  | Equivalent Series Resistance | ESR | Note 1 |  |  |
| Damp Heat, Steady State |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 2 |  |  |
| Final Measurements | Capacitance | C | Record Value |  |  |
|  | Capacitance Change | ΔC/C | -5 | +35 | % (3) |
|  | DC Leakage Current | IL | Note 7 |  |  |
|  | Dissipation Factor | DF | Note 5 |  |  |
|  | Equivalent Series Resistance | ESR | Note 5 |  |  |
| Operating Life |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 2 |  |  |
| Intermediate Measurements (250h and 1000h)(Note 9) | DC Leakage Current (at Tamb = +25°C) | IL | Note 8 |  |  |
| Intermediate Measurements (250h)(Note 10) | DC Leakage Current (at Tamb = +25°C) | IL | Note 8 |  |  |
| Final Measurements (1000h or 2000h)(Note 11) | Capacitance | C | Record Value |  |  |
|  | Capacitance Change | ΔC/C | -20 | +10 | % (3) |
|  | DC Leakage Current | IL | Note 8 |  |  |
|  | Dissipation Factor | DF | Note 5 |  |  |
|  | Equivalent Series Resistance | ESR | Note 5 |  |  |

NOTES:

As specified in Para. 2.3.1.

Capacitance measured during the final measurements during Mounting shall be used as the initial measurement for this test.

Capacitance Change shall be referred to the initial measurement.

As specified in Para. 2.3.2.

2× the limit specified in Para. 2.3.1.

Capacitance measured during the previous test may be used as the initial measurement for this test.

5× the limit specified in Para. 2.3.1.

1.25× the limit specified in Para. 2.3.1.

Applicable to Qualification Testing, and to Periodic Testing for renewal of qualification after lapse.

Applicable to Periodic Testing for extension of qualification.

1000 hours is applicable to Periodic Testing for extension of qualification. 2000 hours is applicable to Qualification Testing, and to Periodic Testing for renewal of qualification after lapse.

## Burn-in

The requirements for Burn-in are specified in the ESCC Generic Specification. The following conditions shall also apply:

After exposure, the components shall be allowed to cool from elevated temperature to Tamb = +25 ±5°C, under normal atmospheric conditions for 1 to 2 hours whilst maintaining the applied voltage. After cooling, the components shall be discharged for a minimum of 5 minutes.

## Operating Life

The requirements for Operating Life are specified in the ESCC Generic Specification. The following conditions shall also apply:

After each exposure, the components shall be allowed to cool from elevated temperature to Tamb = +25 ±5°C, under normal atmospheric conditions for 1 to 2 hours whilst maintaining the applied voltage. After cooling, the components shall be discharged for a minimum of 5 minutes.

## APPENDIX A

AGREED DEVIATIONS FOR KEMET (P)

| Items Affected | Description of Deviations |
| --- | --- |
| Para. 1.4.1.1(c), Characteristics and Ratings Codes: DC Rated Voltage | DC Rated Voltage may be marked with the following alternate codes for case code D: DC Rated Voltage UR (V); Original Code; Alternate Marking Code. 6.3; J; 6V. 10; A; 10V. 16; C; 16V. |
| Para. 2.1.1.2 Deviations from Qualification, Periodic Testing and Lot Validation Testing – Chart F4 | Para. 9.18, Permanence of Marking: Not applicable. |