Figure 2(b)-Variants (Continued)

VARIANT (R126.410.061) – SQUARE FLANGE RECEPTACLE (Low VSWR & frequency band extended)

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 22 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05+0.0027 f (GHz) |  |
| Maximum reflection coefficient | 0.052 |  |
| Maximum insertion loss (1) | 0.03 +0.02f (GHz) | dB |
| RF leakage (1) | -120+f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| Corona level | NA | Vrms |

Notes :

- For information only.

- Contact engagement and separation forces shall be measured on rear contact (see Para. 4.3.8)

| MECANICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Minimum center contact retention force (axial) | 27 | N |
| Minimum center contact retention torque | <0.2 | N.cm |
| Minimum cable retention force | NA | N |
| Minimum cable retention torque | NA | N.cm |
| Maximum weight | 1.8 | g |

| OTHER CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Rapid change of temperature – peak value | +180 | °C |
| Operating Temperature Range | -65 +165°C | °C |
| Maximum leakage (panel sealed connectors) | NA |  |
| Maximum leakage (hermetic sealed connectors) | NA |  |
| Solderability | NA |  |
| Soldering Proof | NA |  |
| Cables Used | NA |  |