Page 1 of 21

Page 1 of 22

Crystal Oscillators, Class 2, 4MHz to 100MHz, AHCMOS COMPATIBLE OUTPUT, RAD-HARD

BASED ON TYPE RK135

ESCC Detail Specification No. 3503/001

| Issue 2 | January 2020 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2019. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| 1254 | Specification upissued to incorporate changes per DCR |

TABLE OF CONTENTS

1	General	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Characteristics Codes	5

1.4.3	Component Type Variants	6

1.5	MAXIMUM RATINGS	7

1.6	PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION	8

1.6.1	Flat Package (FP1) – 14 leads	8

1.6.2	Flat Package (FP2) – 20 leads	9

1.6.3	Flat Package (FP3) – 12 leads	10

1.6.4	Flat Package (FP4) – 16 leads	11

1.6.5	Dual-in–Line Package (DIL1) – 14 leads	12

1.6.6	J-Lead Package (JL2) – 4 leads	13

1.7	FUNCTIONAL DIAGRAM	14

1.8	MATERIALS AND FINISHES	14

2	REQUIREMENTS	15

2.1	GENERAL	15

2.1.1	Oscillator Class	15

2.1.2	Deviations from the Generic Specification	15

2.2	MARKING	15

2.3	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	16

2.3.1	Room Temperature Electrical Measurements	16

2.3.2	High and Low Temperatures Electrical Measurements	17

2.3.3	Notes to Paras. 2.3.1 and 2.3.2 Room, High and Low Electrical Measurements	17

2.4	PARAMETER DRIFT VALUES	18

2.5	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	19

2.6	BURN-IN CONDITIONS	19

2.7	Frequency Ageing Conditions	20

2.8	OPERATING LIFE CONDITIONS	20

2.9	TOTAL DOSE RADIATION TESTING	20

2.9.1	Bias Conditions and Total Dose Level for Total Dose Radiation Testing	20

2.9.2	Electrical Measurements for Total Dose Radiation Testing	21

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3503.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 350300101R100M000000

Detail Specification Reference: 3503001

Component Type Variant Number: 01 (as required)

Total Dose Radiation Level Letter (100krad(Si)): R (as required)

Characteristic code: Nominal Frequency (100MHz): 100M000000 (as required)

### Characteristics Codes

Characteristics to be codified as part of the ESCC Component Number shall be as follows:

Nominal Frequency expressed by means of the following codes. The unit quantity shall be MHz:

| Nominal Frequency fNom (MHz) | Code |
| --- | --- |
| X.XXXXXX | XMXXXXXX |
| XX.XXXXXX | XXMXXXXXX |
| XXX.XXXXXX | XXXMXXXXXX |

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Nominal Output Frequency fNom (MHz) | Case | Nominal Supply Voltage VCCNom (V) | Terminal Material and Finish | Weight max g | Total Dose Radiation Level Letter |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | 4 to 100 | FP1 | 3.3 | D2 | 5 | R [100krad(Si)] |
| 02 | 4 to 100 | FP2 | 3.3 | D2 | 5 | R [100krad(Si)] |
| 03 | 4 to 100 | FP3 | 3.3 | D2 | 5 | R [100krad(Si)] |
| 04 | 4 to 100 | FP4 | 3.3 | D2 | 5 | R [100krad(Si)] |
| 05 | 4 to 100 | DIL1 | 3.3 | G2 | 5 | R [100krad(Si)] |
| 06 | 4 to 100 | JL2 | 3.3 | D2 | 2 | R [100krad(Si)] |
| 07 | 4 to 100 | FP1 | 5 | D2 | 5 | R [100krad(Si)] |
| 08 | 4 to 100 | FP2 | 5 | D2 | 5 | R [100krad(Si)] |
| 09 | 4 to 100 | FP3 | 5 | D2 | 5 | R [100krad(Si)] |
| 10 | 4 to 100 | FP4 | 5 | D2 | 5 | R [100krad(Si)] |
| 11 | 4 to 100 | DIL1 | 5 | G2 | 5 | R [100krad(Si)] |
| 12 | 4 to 100 | JL2 | 5 | D2 | 2 | R [100krad(Si)] |

The terminal material and finish shall be in accordance with the requirements of ESCC Basic Specification No. 23500.

Total dose radiation level letters are defined in ESCC Basic Specification No. 22900. If an alternative radiation test level is specified in the Purchase Order the letter shall be changed accordingly.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Supply Voltage | Vcc | -0.5 to +7 | V | Note 1 |
| Load Impedance | CL | 50 | pF | Note 2 |
| Operating Temperature Range | Top | -55 to +110 | °C | Tamb |
| Storage Temperature Range | Tstg | -55 to +110 | °C |  |
| Soldering Temperature | Tsol | +260 | °C | Note 3 |

Notes:

Device is functional as follows:

Variants 01 to 06: +2.97V ≤ VCC ≤ +3.63V (where nominal VCC, VCCNom = +3.3V)

Variants 07 to 12: +4.5V ≤ VCC ≤ +5.5V (where nominal VCC, VCCNom = +5V)

Device is functional as follows:

Variants 01 to 06:

- For 4MHz ≤ fNom < 80MHz: CLMin = 13pF ≤ CL ≤ CLMax = 18pF (load in parallel with RL = 1kΩ) (where nominal CL, CLNom = 15pF)

- For 80MHz ≤ fNom ≤ 100MHz: CLMin = 8.2pF ≤ CL ≤ CLMax = 11pF (load in parallel with RL = 1kΩ) (where nominal CL, CLNom = 10pF)

Variants 07 to 12: CLMin = 13pF ≤ CL ≤ CLMax = 18pF (load in parallel with RL = 1kΩ) ) (where nominal CL, CLNom = 15pF)

Hand soldering: duration 10 seconds maximum at a distance of not less than 1.5mm from the device body and the same lead shall not be resoldered until 3 minutes have elapsed.

## PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION

### Flat Package (FP1) – 14 leads

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 14.86 | 15.12 |  |
| B | 14 | 15 |  |
| C | 19.94 | 20.2 |  |
| D | - | 3.71 |  |
| E | 15.24 BSC |  | 2 places |
| F | 2.54 BSC |  | All leads |
| G | 0.28 | 0.48 | All leads |
| H | 0.15 | 0.35 | All leads |

NOTES:

The terminal identification is specified by marking of the terminal number on the lid as shown. See Para. 1.7 for the terminal connections.

### Flat Package (FP2) – 20 leads

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 15.75 | 16.01 |  |
| B | 14 | 15 |  |
| C | 15.75 | 16.01 |  |
| D | - | 3.83 |  |
| E | 11.43 BSC |  | 2 places |
| F | 1.27 BSC |  | All leads |
| G | 0.28 | 0.48 | All leads |
| H | 0.15 | 0.35 | All leads |

NOTES:

The terminal identification is specified by marking of the terminal number on the lid as shown. See Para. 1.7 for the terminal connections.

### Flat Package (FP3) – 12 leads

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 15.75 | 16.01 |  |
| B | 14 | 15 |  |
| C | 15.75 | 16.01 |  |
| D | - | 3.58 |  |
| E | 12.7 BSC |  | 2 places |
| F | 2.54 BSC |  | All leads |
| G | 0.28 | 0.48 | All leads |
| H | 0.15 | 0.35 | All leads |

NOTES:

The terminal identification is specified by marking of the terminal number on the lid as shown. See Para. 1.7 for the terminal connections.

### Flat Package (FP4) – 16 leads

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | - | 9.66 |  |
| B | 13.9 | 15.1 |  |
| C | - | 12.83 |  |
| D | - | 3.1 |  |
| E | 8.89 BSC |  | 2 places |
| F | 1.27 BSC |  | All leads |
| G | 0.28 | 0.48 | All leads |
| H | 0.15 | 0.35 | All leads |

NOTES:

The terminal identification is specified by marking of the terminal number on the lid as shown. See Para. 1.7 for the terminal connections.

### Dual-in–Line Package (DIL1) – 14 leads

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | - | 20.7 |  |
| B | - | 13.1 |  |
| C | - | 5.45 |  |
| D | 15.24 BSC 5.18 |  | 2 places |
| E | 7.62 BSC |  | All leads |
| F | 2.54 BSC |  | All leads |
| G | 6.22 | 6.48 | All leads |
| ØH | 0.25 | 0.65 | All leads |
| J | - | 6 | 4 places |

NOTES:

The terminal identification is specified by reference to the index corner as shown. See Para. 1.7 for the terminal connections.

### J-Lead Package (JL2) – 4 leads

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 13.72 | 14.22 |  |
| B | 8.8 | 9.8 | 2 places |
| C | - | 4.2 | All leads |
| D | 4.93 | 5.23 | 2 places |
| E | 0.46 | 0.56 | All leads |
| F | 7.42 | 7.82 | All leads |
| G | 1.58 | 1.78 | All leads |
| H | 0.89 | 1.15 | All leads |

NOTES:

The terminal identification is specified by reference to the index corner as shown. See Para. 1.7 for the terminal connections.

## FUNCTIONAL DIAGRAM

| Variant Number | Case | Terminal Number |  |  |  | Notes |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Output | VCC | Ground | Not Connected |  |
| 01, 07 | FP1 | 8 | 14 | 1, 2, 3, 4, 7, 10, 11, 12, 13 | 5, 6, 9 | 1, 2 |
| 02, 08 | FP2 | 11 | 13 | 1, 2, 3, 4, 5, 6, 7, 10, 14, 15, 16, 17, 18, 19, 20 | 8, 9, 12 | 1, 2 |
| 03, 09 | FP3 | 7 | 12 | 1, 2, 3, 6, 9, 10, 11 | 4, 5, 8 | 1, 2 |
| 04, 10 | FP4 | 10 | 8 | 1, 2, 3, 4, 5, 9, 12, 13, 14, 15, 16 | 6, 7, 11 | 1, 2 |
| 05, 11 | DIL1 | 8 | 14 | 7 | 1, 2, 3, 4, 5, 6, 9, 10, 11, 12, 13 | 1, 2 |
| 06, 12 | JL2 | 3 | 4 | 2 | 1 | 1, 2 |

NOTES:

The case is connected to Ground.

Not connected pins must be connected to a potential (e.g. Ground)

## MATERIALS AND FINISHES

Materials and finishes shall be as follows:

Case

- The FP1, FP2, FP3, FP4 and DIL1 cases shall be hermetically sealed, and have a metal body with hard glass seals and a seam sealed metal lid.

- The JL2 case shall be hermetically sealed, and have a ceramic body with brazed leads and a seam sealed lid.

Terminals

- As specified in Para. 1.4.3 Component Type Variants.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Oscillator Class

The components specified herein shall satisfy the requirements Class 2 Oscillators in accordance with the Generic Specification.

### Deviations from the Generic Specification

None.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component shall be:

Terminal identification (see Para. 1.6).

The ESCC Qualified Component symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures. Consolidated notes are given in Para. 2.3.3.

### Room Temperature Electrical Measurements

| Characteristics | Symbols | Test Method | Test Conditions Note 1 | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Input Current | IIN | ESCC No. 24200 | For 4MHz ≤ fNom < 20MHz: For 20MHz ≤ fNom < 50MHz: For 50MHz ≤ fNom ≤ 100MHz: | - - - | 20 25 30 | mA |
| Output Waveform | - | ESCC No. 24200 |  | Symmetrical Square Wave |  | - |
| Output Voltage High Level Variants 01 to 06: Variants 07 to 12: | VOH | ESCC No. 24200 |  | 2.4 4.5 | - - | V |
| Output Voltage Low Level Variants 01 to 06: Variants 07 to 12: | VOL | ESCC No. 24200 |  | - - | 0.4 0.5 | V |
| Frequency Accuracy | Δf/fNom | ESCC No. 24200 | At Tamb = +25 ±1°C Referred to fNom | - | ±25 | ppm |
| Frequency-Voltage Tolerance Variants 01 to 06: Variants 07 to 12: | Δf/f(V) | ESCC No. 24200 | At Tamb = +25 ±1°C Referred to f at VCCNom For 2.97V, 3.3V & 3.63V: For 4.5V, 5V & 5.5V: | - - | ±3 ±3 | ppm |
| Frequency-Load Tolerance | Δf/f(L) | ESCC No. 24200 | At Tamb = +25 ±1°C For CL = CLMin, CLNom & CLMax, RL = 1kΩ, Referred to f at CLNom | - | ±5 | ppm |
| Startup Time | tsu | ESCC No. 24200 |  | - | 10 | ms |
| Rise Time | tr | ESCC No. 24200 | For 4MHz ≤ fNom < 16MHz: For 16MHz ≤ fNom ≤ 100MHz: | - - | 10 5 | ns |
| Fall Time | tf | ESCC No. 24200 | For 4MHz ≤ fNom < 16MHz: For 16MHz ≤ fNom ≤ 100MHz: | - - | 10 5 | ns |
| Duty Cycle | DC | ESCC No. 24200 |  | 45 | 55 | % |
| Ageing Analysis | Δf/f | ESCC No. 3503 | Ageing Period = 1 year Note 3 | - | ±5 | ppm |

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | Test Method | Test Conditions Note 1 | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Input Current | IIN | ESCC No. 24200 | At Tamb = -55 (+5 -0)°C and +110 (+0 -5)°C For 4MHz ≤ fNom < 20MHz: For 20MHz ≤ fNom < 50MHz: For 50MHz ≤ fNom ≤ 100MHz: | - - - | 20 25 30 | mA |
| Output Waveform | - | ESCC No. 24200 | At Tamb = -55 (+5 -0)°C and +110 (+0 -5)°C | Symmetrical Square Wave |  | - |
| Output Voltage High Level Variants 01 to 06: Variants 07 to 12: | VOH | ESCC No. 24200 | At Tamb = -55 (+5 -0)°C and +110 (+0 -5)°C | 2.4 4.5 | - - | V |
| Output Voltage Low Level Variants 01 to 06: Variants 07 to 12: | VOL | ESCC No. 24200 | At Tamb = -55 (+5 -0)°C and +110 (+0 -5)°C | - - | 0.4 0.5 | V |
| Frequency-Temperature Stability | Δf/f(T) | ESCC No. 24200 | At Tamb = -55 (+1 -0)°C to +110 (+0 -1)°C. Note 2 Referred to f at Tamb = +25 ±1°C | - | 50 | ppm |
| Frequency-Voltage Tolerance Variants 01 to 06: Variants 07 to 12: | Δf/f(V) | ESCC No. 24200 | At Tamb = -55 (+1 -0)°C and +110 (+0 -1)°C Referred to f at VCCNom For 2.97V, 3.3V & 3.63V: For 4.5V, 5V & 5.5V: | - - | ±3 ±3 | ppm |
| Startup Time | tsu | ESCC No. 24200 | At Tamb = -55 (+5 -0)°C and +110 (+0 -5)°C |  | 10 | ms |
| Rise Time | tr | ESCC No. 24200 | At Tamb = -55 (+5 -0)°C and +105 (+0 -5)°C For 4MHz ≤ fNom < 16MHz: For 16MHz ≤ fNom ≤ 100MHz: At Tamb = +110 (+0 -5)°C For 4MHz ≤ fNom < 16MHz: For 16MHz ≤ fNom ≤ 100MHz: | - - - - | 10 5 10 7 | ns |
| Fall Time | tf | ESCC No. 24200 | At Tamb = -55 (+5 -0)°C and +105 (+0 -5)°C For 4MHz ≤ fNom < 16MHz: For 16MHz ≤ fNom ≤ 100MHz: At Tamb = +110 (+0 -5)°C For 4MHz ≤ fNom < 16MHz: For 16MHz ≤ fNom ≤ 100MHz: | - - - - | 10 5 10 7 | ns |
| Duty Cycle | DC | ESCC No. 24200 | At Tamb = -55 (+5 -0)°C and +110 (+0 -5)°C | 45 | 55 | % |

### Notes to Paras. 2.3.1 and 2.3.2 Room, High and Low Electrical Measurements

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C and the component under test shall be operated at VCCNom with an output load of CLNom in parallel with RL = 1kΩ. VCCNom and CLNom are specified in Para. 1.5 Maximum Ratings.

Frequency-Temperature Stability shall be measured at a minimum of 10 equally spaced increments over the specified temperature range.

All measurements aquired during Frequency Ageing shall be taken into account when performing Ageing Analysis.

## PARAMETER DRIFT VALUES

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.3.1 Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value Δ | Absolute |  |  |
|  |  |  | Min | Max |  |
| Input Current For 4MHz ≤ fNom < 20MHz: For 20MHz ≤ fNom < 50MHz: For 50MHz ≤ fNom ≤ 100MHz: | IIN | ±5% ±5% ±5% | - - - | 20 25 30 | mA |
| Frequency Accuracy Initial measurement: Final measurement: | Δf/fNom | ±10 | - - | ±15 ±25 | ppm |

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3 °C.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.3.1 Room Temperature Electrical Measurements or Para. 2.3.2 High and Low Temperatures Electrical Measurements, as follows.

The drift values (Δ) shall not be exceeded for each characteristic specified. Unless otherwise specified, the corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Test Conditions | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Drift Value Δ | Absolute |  |  |
|  |  |  |  | Min | Max |  |
| Input Current For 4MHz ≤ fNom < 20MHz: For 20MHz ≤ fNom < 50MHz: For 50MHz ≤ fNom ≤ 100MHz: | IIN | As per Para. 2.3.1 | - - - | - - - | 20 25 30 | mA |
| Output Waveform | - | As per Para. 2.3.1 | - | Symmetrical Square Wave |  | - |
| Output Voltage High Level Variants 01 to 06: Variants 07 to 12: | VOH | As per Para. 2.3.1 | - - | 2.4 4.5 | - - | V |
| Output Voltage Low Level Variants 01 to 06: Variants 07 to 12: | VOL | As per Para. 2.3.1 | - - | - - | 0.4 0.5 | V |
| Frequency Accuracy | Δf/fNom | As per Para. 2.3.1 | ±8.5 (1) | - | ±25 (2) | ppm |
| Frequency-Temperature Stability | Δf/f(T) | As per Para. 2.3.1 |  | - | 50 | ppm |
| Rise Time For 4MHz ≤ fNom < 16MHz: For 16MHz ≤ fNom ≤ 100MHz: | tr | As per Para. 2.3.1 | - - | - - | 10 5 | ns |
| Fall Time For 4MHz ≤ fNom < 16MHz: For 16MHz ≤ fNom ≤ 100MHz: | tr | As per Para. 2.3.1 | - - | - - | 10 5 | ns |
| Duty Cycle | DC | As per Para. 2.3.1 | - | 45 | 55 | % |

NOTES:

Drift value (Δ) is only applicable to testing during the Endurance Subgroup.

Absolute limit is only applicable to testing during the Environmental/Mechanical Subgroup.

## BURN-IN CONDITIONS

The test conditions for Burn-in, tested as specified in the ESCC Generic Specification, shall be as follows:

Ouptut Load: CLNom in parallel with RL = 1kΩ. CLNom is specified in Para. 1.5 Maximum Ratings.

## Frequency Ageing Conditions

The test conditions for Frequency Ageing, tested as specified in the ESCC Generic Specification, shall be as follows:

Ouptut Load: CLNom in parallel with RL = 1kΩ. CLNom is specified in Para. 1.5 Maximum Ratings.

## OPERATING LIFE CONDITIONS

The test conditions for Operating Life, tested as specified in the ESCC Generic Specification, shall be as follows:

Ouptut Load: CLNom in parallel with RL = 1kΩ. CLNom is specified in Para. 1.5 Maximum Ratings.

## TOTAL DOSE RADIATION TESTING

All lots shall be irradiated in accordance with ESCC Basic Specification No. 22900, low dose rate (window 2: 36rad(Si) to 360rad(Si) per hour).

### Bias Conditions and Total Dose Level for Total Dose Radiation Testing

The following bias condition (worst-case) shall be used for Total Dose Radiation Testing at Tamb = +22 ±3°C:

With Supply Voltage:

Variants 01 to 06: VCC = 3.63V during irradiation.

Variants 07 to 12: VCC = 5.5V during irradiation.

The total dose level applied shall be as specified in Para. 1.4.3 or in the Purchase Order.

### Electrical Measurements for Total Dose Radiation Testing

Prior to irradiation testing the devices shall have successfully met Para. 2.3.1 Room Temperature Electrical Measurements specified herein.

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified, the test methods and test conditions shall be as per the corresponding test defined in Para. 2.3.1 Room Temperature Electrical Measurements.

The parameters to be measured during irradiation testing, on completion of irradiation testing, after 24 hours anneal at Room Temperature and after 168 hours anneal at Tamb = +100 ±3°C are shown below.

| Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- |
|  |  | Absolute |  |  |
|  |  | Min | Max |  |
| Input Current For 4MHz ≤ fNom < 20MHz: For 20MHz ≤ fNom < 50MHz: For 50MHz ≤ fNom ≤ 100MHz: | IIN | - - - | 20 25 30 | mA |
| Output Waveform | - | Symmetrical Square Wave |  | - |  |
| Output Voltage High Level Variants 01 to 06: Variants 07 to 12: | VOH | 2.4 4.5 | - - | V |
| Output Voltage Low Level Variants 01 to 06: Variants 07 to 12: | VOL | - - | 0.4 0.5 | V |
| Frequency Accuracy | Δf/fNom | - | ±25 | ppm |
| Rise Time For 4MHz ≤ fNom < 16MHz: For 16MHz ≤ fNom ≤ 100MHz: | tr | - - | 10 5 | ns |
| Fall Time For 4MHz ≤ fNom < 16MHz: For 16MHz ≤ fNom ≤ 100MHz: | tf | - - | 10 5 | ns |
| Duty Cycle | DC | 45 | 55 | % |