Page 1 of 40

CONNECTORS AND SAVERS, ELECTRICAL, RECTANGULAR, NON-REMOVABLE PCB CONTACTS

BASED ON TYPE MHD

ESCC Detail Specification No. 3401/065

| Issue 6 Draft A | May 2020 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2020. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without prior permission of the European Space Agency and provided it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| 1338 | Specification updated to incorporate changes per DCR. |

TABLE OF CONTENTS

1	GENERAL	6

1.1	SCOPE	6

1.2	RANGE OF COMPONENTS	6

1.3	MAXIMUM RATINGS	6

1.4	PARAMETER DERATING INFORMATION	6

1.5	PHYSICAL DIMENSIONS	6

2	APPLICABLE DOCUMENTS	6

3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	6

4	REQUIREMENTS	32

4.1	GENERAL	32

4.2	DEVIATIONS FROM GENERIC SPECIFICATION	32

4.2.1	Deviations from Special In-process Controls	32

4.2.2	Deviations from Final Production Tests (Chart II)	32

4.2.3	Deviations from Burn-in and Electrical Measurements (Chart III)	32

4.2.4	Deviations from Qualification Tests (Chart IV)	32

4.2.5	Deviations from Lot Acceptance Tests (Chart V)	32

4.3	MECHANICAL REQUIREMENTS	33

4.3.1	Dimension Check	33

4.3.2	Weight	33

4.3.3	Contact Capability	33

4.3.4	Contact Retention (In Insert)	33

4.3.5	Mating and Unmating Forces	33

4.3.6	Insert Retention (In Shell)	33

4.3.7	Jackscrew Retention	33

4.3.8	Contact Insertion and Withdrawal Forces	33

4.3.9	Engagement and Separation Forces	34

4.3.10	Oversize Pin Exclusion	34

4.3.11	Probe Damage	34

4.3.12	Solderability	34

4.4	MATERIALS AND FINISHES	34

4.4.1	Shells	34

4.4.2	Inserts	34

4.4.3	Contacts	34

4.4.4	Contact Retaining Clip	35

4.4.5	Guiding and Locking Devices	35

4.4.6	Magnetism Level	35

4.5	MARKING	35

4.5.1	General	35

4.5.2	Contact Position	35

4.5.3	The ESCC Component Number	35

4.5.4	Characteristics	35

4.5.5	Traceability Information	36

4.6	ELECTRICAL MEASUREMENTS	36

4.6.1	Electrical Measurements at Room Temperature	36

4.6.2	Electrical Measurements at High and Low Temperatures (Table 3)	36

4.6.3	Circuits for Electrical Measurements (Figure 4)	36

4.7	BURN-IN AND ELECTRICAL MEASUREMENTS (TABLES 4 AND 5)	36

4.8	ENVIRONMENTAL AND ENDURANCE TESTS (CHARTS IV AND V OF ESCC GENERIC SPECIFICATION NO. 3401)	36

4.8.1	Measurements and Inspections on Completion of Environmental Tests	36

4.8.2	Measurements and Inspections at Intermediate Points during Endurance Tests	36

4.8.3	Measurements and Inspections on Completion of Endurance Tests	37

4.8.4	Conditions for Operating Life Test (Part of Endurance Testing)	37

4.8.5	Electrical Circuits for Operating Life Test	37

4.8.6	Conditions for High Temperature Storage Test (Part of Endurance Testing)	37

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, test and inspection data for Connectors and Savers, Electrical, Rectangular, with Non-Removable PCB Contacts, Based on Type MHD. It shall be read in conjunction with:

ESCC Generic Specification No. 3401, Connectors, Electrical, Non-Filtered, Circular and       Rectangular,

the requirements of which are supplemented herein.

## RANGE OF COMPONENTS

The different configurations of the connectors specified herein and guiding and locking devices, which are also covered by this specification, are given in Table 1(a).

## MAXIMUM RATINGS

The maximum ratings applicable to the connectors specified herein, which shall not be exceeded at any time during use or storage, are scheduled in Table 1(b).

## PARAMETER DERATING INFORMATION

The applicable derating information for the connectors specified herein is shown in Figure 1.

## PHYSICAL DIMENSIONS

The physical dimensions of the connectors, plugs and receptacles, guiding and locking devices specified herein, and the contact mounting configurations, are shown in Figures 2(a), 2(b) and 2(c).

# APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3401, Connectors, Electrical, Non-Filtered, Circular and Rectangular.

# TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

Table 1(a) – RANGe OF COMPONEnts

NUMBER OF CONTACTS

| No. of Contacts (Note 1) | Max. Weight (2) (g) |  | Max. Engagement Force | Separation Force (N) |  |
| --- | --- | --- | --- | --- | --- |
|  | Plug | Receptacle | (N Max.) | N Min. | N Max. |
| 52 | 14 | 8 | 36.4 | 2.6 | 36.4 |
| 100 | 20 | 15 | 70 | 5 | 70 |
| 152 | 32 | 22 | 106.4 | 7.6 | 106.4 |
| 200 | 38 | 28 | 140 | 10 | 140 |
| 252 | 50 | 36 | 176.4 | 12.6 | 176.4 |
| 300 | 56 | 42 | 210 | 15 | 210 |
| 352 | 65 | 47 | 246.4 | 17.6 | 246.4 |
| 400 | 75 | 55 | 280 | 20 | 280 |

NOTES:

See Figure 2(a).

Weights with contacts and without accessories.

CONTACT TYPES

| Contact Codes | Contact Types |
| --- | --- |
| 10 | Through board solder – 90° for PCB thickness: 1.44 - 1.76 mm |
| 11 | Through board solder – 90° for PCB thickness: 1.98 - 2.42 mm |
| 12 | Through board solder – 90° for PCB thickness: 2.88 - 3.52 mm |
| 30 | Through board solder – straight for PCB thickness: 2.16 - 2.64 mm |
| 31 | Through board solder – straight for PCB thickness: 2.88 - 3.52 mm |
| 43 | Surface mount for PCB thickness: 2.16 - 2.64 mm (Centred PCB) |
| 45 | Surface mount for PCB thickness: 1.44 - 2.4 mm (Off-centre PCB) |
| 47 | Surface mount for PCB thickness: 2.16 - 2.64 mm (Off-centre PCB) |
| 91 | Contact for connector saver male - female |

NOTES:

            - See Figure 2(c).

GUIDING AND LOCKING DEVICES

| Guiding and Locking Devices Code | Mounting On | Max. Weight (g) |
| --- | --- | --- |
| 110 | Plug | 0.55 |
| 111 | Plug | 1.35 |
| 121 | Receptacle | 0.65 |
| 124 | Receptacle | 1.5 |
| 134 | Receptacle | 1.3 |
| 201 | Saver Receptacle | - |

NOTES:

            - See Figure 2(b).

Table 1(b) – MAXIMUM Ratings

| No. | Characteristic | Symbol | Maximum Rating | Unit | Remarks |
| --- | --- | --- | --- | --- | --- |
| 1 | Working Voltage Sea Level | V | 200 | Vrms | Note 1 |
| 2 | Rated Current | IR | 2 | A |  |
| 3 | Operating Temperature Range | Top | -55 to +125 | °C | Tamb |
| 4 | Storage Temperature Range | Tstg | -55 to +125 | °C |  |
| 5 | Soldering Temperature | Tsol | +260 | °C | Note 2 |

NOTES:

            - Between contacts and shell.

            - Duration 10 seconds minimum and the same contact shall not be resoldered until 3 minutes have elapsed.

Figure 1 – PARAMETER Derating Information

Figure 2 – Physical Dimensions

figure 2(a) – INSERTS: PLUGS AND rECepTACleS

receptacle, 52 CONTACTS

| Symbol / Dim. (1) | A | A2 | B | C | C1 | D | E | E1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 35.1 | 22.66 | 7.75 | 30.65 | 3.77 | 2.34 | 9.9 | 4.9 |
| Max. | 35.4 | 23.06 | 8.05 | 30.95 | 4.17 | 2.74 | 10.1 | 5.1 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

receptacle, 100 CONTACTS

| Symbol / Dim. (1) | A | A3 | B | C | C1 | D | E | E1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 57.9 | 45.52 | 7.75 | 53.51 | 3.77 | 2.34 | 9.9 | 4.9 |
| Max. | 58.2 | 45.92 | 8.05 | 53.81 | 4.17 | 2.74 | 10.1 | 5.1 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

RECEPTACLE, 152 CONTACTS

| Symbol / Dim. (1) | A | A2 | A3 | A4 | B | C | C1 | C2 | C3 | D | E | E1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 88.8 | 22.66 | 45.52 | 7.74 | 7.75 | 84.3 | 3.77 | 30.65 | 53.51 | 2.34 | 9.9 | 4.9 |
| Max. | 89.1 | 23.06 | 45.92 | 8.14 | 8.05 | 84.6 | 4.17 | 30.95 | 53.81 | 2.74 | 10.1 | 5.1 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

RECEPTACLE, 200 CONTACTS

| Symbol / Dim. (1) | A | A3 | A4 | B | C | C1 | C3 | D | E | E1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 111.6 | 45.52 | 7.74 | 7.75 | 107.16 | 3.77 | 53.51 | 2.34 | 9.9 | 4.9 |
| Max. | 111.9 | 45.92 | 8.14 | 8.05 | 107.46 | 4.17 | 53.81 | 2.74 | 10.1 | 5.1 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

RECEPTACLE, 252 CONTACTS

| Symbol / Dim. (1) | A | A2 | A3 | A4 | B | C | C1 | C2 | C3 | D | E | E1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 142.5 | 22.66 | 45.52 | 7.74 | 7.75 | 137.96 | 3.77 | 30.65 | 53.51 | 2.34 | 9.9 | 4.9 |
| Max. | 142.8 | 23.06 | 45.92 | 8.14 | 8.05 | 138.26 | 4.17 | 30.95 | 53.81 | 2.74 | 10.1 | 5.1 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

RECEPTACLE, 300 CONTACTS

| Symbol / Dim. (1) | A | A3 | A4 | B | C | C1 | C3 | D | E | E1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 165.3 | 45.52 | 7.74 | 7.75 | 160.82 | 3.77 | 53.51 | 2.34 | 9.9 | 4.9 |
| Max. | 165.6 | 45.92 | 8.14 | 8.05 | 161.12 | 4.17 | 53.81 | 2.74 | 10.1 | 5.1 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

RECEPTACLE, 352 CONTACTS

| Symbol / Dim. (1) | A | A2 | A3 | A4 | B | C | C1 | C2 | C3 | D | E | E1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 196.2 | 22.66 | 45.52 | 7.74 | 7.75 | 191.62 | 3.77 | 30.65 | 53.51 | 2.34 | 9.9 | 4.9 |
| Max. | 196.8 | 23.06 | 45.92 | 8.14 | 8.05 | 191.92 | 4.17 | 30.95 | 53.81 | 2.74 | 10.1 | 5.1 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

RECEPTACLE, 400 CONTACTS

| Symbol / Dim. (1) | A | A3 | A4 | B | C | C1 | C3 | D | E | E1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 219 | 45.52 | 7.74 | 7.75 | 214.48 | 3.77 | 53.51 | 2.34 | 9.9 | 4.9 |
| Max. | 219.3 | 45.92 | 8.14 | 8.05 | 214.78 | 4.17 | 53.81 | 2.74 | 10.1 | 5.1 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

PLUG, 52 CONTACTS

| Symbol / Dim. (1) | A | A2 | B1 | B2 | B3 | B4 | C | C1 | D | E | E1 | E2 | F1 | F2 | ØG | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 35.1 | 22.66 | 7 | - | 3.5 | 14.3 | 30.65 | 3.77 | 2.34 | - | 5.05 | 10.2 | 3.29 | 23.61 | 0.42 | 2 |
| Max. | 35.4 | 23.06 | 7.2 | 7.4 | 3.6 | 14.6 | 30.95 | 4.17 | 2.74 | 12 | 5.25 | 10.45 | 3.69 | 24.01 | 0.44 | 3 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

PLUG, 100 CONTACTS

| Symbol / Dim. (1) | A | A3 | B1 | B2 | B3 | B4 | C | C1 | D | E | E1 | E2 | F1 | F3 | ØG | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 57.9 | 45.52 | 7 | - | 3.5 | 14.3 | 53.51 | 3.77 | 2.34 | - | 5.05 | 10.2 | 3.29 | 46.47 | 0.42 | 2 |
| Max. | 58.2 | 45.92 | 7.2 | 7.4 | 3.6 | 14.6 | 53.81 | 4.17 | 2.74 | 12 | 5.25 | 10.45 | 3.69 | 46.87 | 0.44 | 3 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

PLUG, 152 CONTACTS

| Symbol / Dim. (1) | A | A2 | A3 | A4 | B1 | B2 | B3 | B4 | C | C1 | C2 | C3 | D | E | E1 | E2 | F1 | F2 | F3 | F4 | ØG | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 88.8 | 22.66 | 45.52 | 7.74 | 7 | - | 3.5 | 14.3 | 84.305 | 3.77 | 30.65 | 53.51 | 2.34 | - | 5.05 | 10.2 | 3.29 | 23.61 | 46.47 | 6.79 | 0.42 | 2 |
| Max. | 89.1 | 23.06 | 45.92 | 8.14 | 7.2 | 7.4 | 3.6 | 14.6 | 84.605 | 4.17 | 30.95 | 53.81 | 2.74 | 12 | 5.25 | 10.45 | 3.69 | 24.01 | 46.87 | 7.19 | 0.44 | 3 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

PLUG, 200 CONTACTS

| Symbol / Dim. (1) | A | A3 | A4 | B1 | B2 | B3 | B4 | C | C1 | C3 | D | E | E1 | E2 | F1 | F3 | F4 | ØG | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 111.6 | 45.52 | 7.74 | 7 | - | 3.5 | 14.3 | 107.17 | 3.77 | 53.51 | 2.34 | - | 5.05 | 10.2 | 3.29 | 46.47 | 6.79 | 0.42 | 2 |
| Max. | 111.9 | 45.92 | 8.14 | 7.2 | 7.4 | 3.6 | 14.6 | 107.47 | 4.17 | 53.81 | 2.74 | 12 | 5.25 | 10.45 | 3.69 | 46.87 | 7.19 | 0.44 | 3 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

PLUG, 252 CONTACTS

| Symbol / Dim. (1) | A | A2 | A3 | A4 | B1 | B2 | B3 | B4 | C | C1 | C2 | C3 | D | E | E1 | E2 | F1 | F2 | F3 | F4 | ØG | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 142.5 | 22.66 | 45.52 | 7.74 | 7 | - | 3.5 | 14.3 | 137.96 | 3.77 | 30.65 | 53.51 | 2.34 | - | 5.05 | 10.2 | 3.29 | 23.61 | 46.47 | 6.79 | 0.42 | 2 |
| Max. | 142.8 | 23.06 | 45.92 | 8.14 | 7.2 | 7.4 | 3.6 | 14.6 | 138.26 | 4.17 | 30.95 | 53.81 | 2.74 | 12 | 5.25 | 10.45 | 3.69 | 24.01 | 46.87 | 7.19 | 0.44 | 3 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

PLUG, 300 CONTACTS

| Symbol / Dim. (1) | A | A3 | A4 | B1 | B2 | B3 | B4 | C | C1 | C3 | D | E | E1 | E2 | F1 | F3 | F4 | ØG | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 165.3 | 45.52 | 7.74 | 7 | - | 3.5 | 14.3 | 160.82 | 3.77 | 53.51 | 2.34 | - | 5.05 | 10.2 | 3.29 | 46.47 | 6.79 | 0.42 | 2 |
| Max. | 165.6 | 45.92 | 8.14 | 7.2 | 7.4 | 3.6 | 14.6 | 161.12 | 4.17 | 53.81 | 2.74 | 12 | 5.25 | 10.45 | 3.69 | 46.87 | 7.19 | 0.44 | 3 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

PLUG, 352 CONTACTS

| Symbol / Dim. (1) | A | A2 | A3 | A4 | B1 | B2 | B3 | B4 | C | C1 | C2 | C3 | D | E | E1 | E2 | F1 | F2 | F3 | F4 | ØG | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 196.2 | 22.66 | 45.52 | 7.74 | 7 | - | 3.5 | 14.3 | 191.62 | 3.77 | 30.65 | 53.51 | 2.34 | - | 5.05 | 10.2 | 3.29 | 23.61 | 46.47 | 6.79 | 0.42 | 2 |
| Max. | 196.5 | 23.06 | 45.92 | 8.14 | 7.2 | 7.4 | 3.6 | 14.6 | 191.92 | 4.17 | 30.95 | 53.81 | 2.74 | 12 | 5.25 | 10.45 | 3.69 | 24.01 | 46.87 | 7.19 | 0.44 | 3 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

PLUG, 400 CONTACTS

| Symbol / Dim. (1) | A | A3 | A4 | B1 | B2 | B3 | B4 | C | C1 | C3 | D | E | E1 | E2 | F1 | F3 | F4 | ØG | H |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Min. | 219 | 45.52 | 7.74 | 7 | - | 3.5 | 14.3 | 214.48 | 3.77 | 53.51 | 2.34 | - | 5.05 | 10.2 | 3.29 | 46.47 | 6.79 | 0.42 | 2 |
| Max. | 219.3 | 45.92 | 8.14 | 7.2 | 7.4 | 3.6 | 14.6 | 214.78 | 4.17 | 53.81 | 2.74 | 12 | 5.25 | 10.45 | 3.69 | 46.87 | 7.19 | 0.44 | 3 |

notes:

            - All dimensions are in millimetres.

            - Torque: 10N.cm.

            - Underlined dimensions, in table, are critical to ensure intermateability.

            - The front of the insert shall be marked with the minimum marking shown.

            - Orientation of labelling of contacts and guiding devices is not a true representation.

            - Pitch: 1.905mm.

figure 2(b) – gUIdiNG AnD lOCKInG DEVICEs

CODE 110

| Symbol | Millimetres |  |
| --- | --- | --- |
|  | Min. | Max. |
| A | 7 | 7.35 |

NOTES:

            - Torque: 15N.cm.

code 111

| Symbol | Millimetres |  |
| --- | --- | --- |
|  | Min. | Max. |
| A B C D | 7 0.45 1.9 - | 7.35 0.55 2.1 5.5 |

NOTES:

            - Torque: 25N.cm.

M2.5.

CODE 121

| Symbol | Millimetres |  |
| --- | --- | --- |
|  | Min. | Max. |
| B C D | 0.45 1.9 - | 0.55 2.1 5.5 |

NOTES:

            - Torque: 25N.cm.

            - M2.5.

CODES 124 AND 134

| Symbol | Millimetres |  | Notes |
| --- | --- | --- | --- |
|  | Min. | Max. |  |
| D | - | 7 |  |
| ØE | 2.45 | 2.55 |  |
| F | - | 6 | Code 124 |
| F | - | 6.6 | Code 134 |
| G | 2.6 | 2.7 |  |

NOTES:

            - Torque: 15N.cm.

            - Torque: 25N.cm.

            - M2.

            - M2.5.

code 201

| Symbol | Millimetres |  |
| --- | --- | --- |
|  | Min. | Max. |
| B C H | 0.45 1.9 - | 0.55 2.1 15 |

NOTES:

            - Torque: 15N.cm.

            - Torque: 25N.cm.

            - M2.5.

FIGURE 2(c) – CONTACT MOUNTING CONFIGURATIONS – VIEW OF REAR PART OF CONNECTOR

CODEs 10, 11 AND 12

right-angle solder contacts

RECEPTACLE SIDE							                                                                        PLUG SIDE

|  |  | Code 10 |  | Code 11 |  | Code 12 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Plug and Receptacle |  | Receptacle Only |  | Plug Only |  |
|  |  | Millimetres |  |  |  |  |  |
| Symbol |  | Min. | Max. | Min. | Max. | Min. | Max. |
| A |  | 2.6 | 3.8 | 3.1 | 4.3 | 4.1 | 5.3 |
| B | Plug | 3.4 | 3.6 | - | - | 3.4 | 3.6 |
|  | Recept. | 3.2 | 3.8 | 3.2 | 3.8 | - | - |

| Symbol | Millimetres |  |
| --- | --- | --- |
|  | Minimum | Maximum |
| Øa | 0.36 | 0.44 |
| D | 2.6 | 2.7 |

NOTES:

            - Pitch: 1.905mm.

CODEs 30 and 31

straight solder contacts

|  | Code 30 |  | Code 31 |  | Code 31 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Plug and Receptacle |  | Receptacle |  | Plug |  |
|  | Millimetres |  |  |  |  |  |
| Symbol | Min. | Max. | Min. | Max. | Min. | Max. |
| A | 3.5 | 4 | 6.1 | 6.6 | 4.6 | 5.1 |

| Symbol | Millimetres |  |
| --- | --- | --- |
|  | Minimum | Maximum |
| Øa | 0.36 | 0.44 |

NOTES:

            - Pitch: 1.905mm.

            - Pitch: 2.54mm.

Code 91 – saver

| Symbol | Millimetres |  |
| --- | --- | --- |
|  | Minimum | Maximum |
| A | 4.5 | 5.5 |
| Øa | 0.42 | 0.44 |

NOTES:

            - Pitch: 1.905mm.

Pitch: 2.54mm.

Code 43

surface mount – applicable to plugs only

| Symbol | Millimetres |  |
| --- | --- | --- |
|  | Min. | Max. |
| a | 0.25 | 0.45 |
| A | - | 5.5 |
| b | 0.2 | 0.4 |
| B | 3 | 4.1 |
| D | - | 4.8 |

NOTES:

            - See section. Pitch: 0.9525mm.

            - Contact point.

CODES 45 and 47

surface mount off-centre – applicable to plugs only

|  | Code 45 |  | Code 47 |  |
| --- | --- | --- | --- | --- |
|  | Millimetres |  |  |  |
| Symbol | Min. | Max. | Min. | Max. |
| A | - | 5 | - | 5 |

| Symbol | Millimetres |  |
| --- | --- | --- |
|  | Minimum | Maximum |
| a | 0.25 | 0.45 |
| b | 0.2 | 0.4 |
| B | 3 | 4.1 |
| D | 0.9 | 1.2 |
| D1 | - | 7 |
| E1 | 0.7 | - |
| E2 | 0 | 0.18 |

NOTES:

            - See section. Pitch: 0.9525mm.

            - Contact point. Details as follows:

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the connectors specified herein are stated in this specification and ESCC Generic Specification No. 3401. Deviations from the Generic Specification, applicable to this specification only, are listed in Para. 4.2.

Deviations from the applicable Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirements and do not affect the components' reliability, are listed in the appendices attached to this specification.

## DEVIATIONS FROM GENERIC SPECIFICATION

### Deviations from Special In-process Controls

None.

### Deviations from Final Production Tests (Chart II)

Para. 9.1.1.4, Mated Shell Conductivity: Not applicable.

Para. 9.4, Contact Capability: Sampling in accordance with Para. 9.6 of ESCC No. 3401.

Para. 9.5, Magnetism Level: Not applicable.

Para. 9.9, Seal Test: Not applicable.

### Deviations from Burn-in and Electrical Measurements (Chart III)

Chart III is not applicable.

### Deviations from Qualification Tests (Chart IV)

Para. 9.1.1.4, Mated Shell Conductivity: Not applicable.

Para. 9.9, Seal Test: Not applicable.

Para. 9.15, Joint Strength: Not applicable.

Para. 9.24, Jackscrew Retention: Not applicable.

Para. 9.27, Maintenance Aging: Not applicable

Para. 9.30, Probe Damage: Not applicable.

### Deviations from Lot Acceptance Tests (Chart V)

Para. 9.1.1.4, Mated Shell Conductivity: Not applicable.

Para. 9.9, Seal Test: Not applicable.

Para. 9.15, Joint Strength: Not applicable.

Para. 9.27, Maintenance Aging: Not applicable.

Para. 9.30, Probe Damage: Not applicable.

## MECHANICAL REQUIREMENTS

### Dimension Check

The dimensions of the connectors specified herein shall be verified in accordance with the requirements set out in Para. 9.6 of ESCC Generic Specification No. 3401 and shall conform to those shown in Figure 2 of this specification. Only the following dimensions shall be checked during procurement:

Figure 2(a):

Between centres of guiding or locking device. (Dimensions C, C2 and C3).

Dimension E1 (where applicable).

Figure 2(b):

Protrusion of guiding/locking devices.

Overall dimensions of guiding/locking devices.

Figure 2(c):

All dimensions, except pitch.

### Weight

The maximum weight of the connectors, with contacts and guiding and locking devices specified herein, shall be as specified in Table 1(a).

### Contact Capability

For the purpose of this test, the pick-up and drop weights shall be as follows.

|  | Pick-up Weight | Drop Weight |
| --- | --- | --- |
| Weight (g) | 5 | 70 |
| Pin Diameter (mm) | 0.415 - 0.42 | 0.44 - 0.445 |
| Insertion Depth (mm) | 5 | 5 |

### Contact Retention (In Insert)

The contact retention force (compression) within the insert shall be 40N.

### Mating and Unmating Forces

The forces applied for mating and unmating of the connectors shall not exceed the values specified in Table 1(a).

### Insert Retention (In Shell)

Connector inserts shall withstand a pressure of 25N/cm2 without being dislodged from the shell.

### Jackscrew Retention

Not applicable.

### Contact Insertion and Withdrawal Forces

Not applicable.

### Engagement and Separation Forces

The diameter of the test pin and the engagement and separation forces of the female contacts shall be as specified hereunder.

|  | Diameter (mm) |  | Engagement | Separation (N) |  |
| --- | --- | --- | --- | --- | --- |
|  | Min. | Max. | Max. (N) | Min. | Max. |
| Minimum Diameter Test Pin | 0.415 | 0.42 | - | 0.05 | - |
| Maximum Diameter Test Pin | 0.44 | 0.445 | 0.7 | - | 0.7 |

### Oversize Pin Exclusion

The diameter of the test pin shall be 0.598mm minimum and 0.602mm maximum and the force applied to it shall be 0.7N.

### Probe Damage

Not applicable.

### Solderability

No special conditions.

## MATERIALS AND FINISHES

The materials and finishes shall be as specified herein. Where a definite material is not specified, a material which will enable the connectors specified herein to meet the performance requirements of this specification shall be used. Acceptance or approval of any constituent material does not guarantee acceptance of the finished product.

### Shells

Shells shall be made of aluminium alloy with an appropriate surface treatment against corrosion.

### Inserts

The inserts shall be made of glass-fibre-filled diallylphthalate resin.

### Contacts

#### Body

The contact body shall be made of copper alloy.

Male Contact and Saver: The plating shall be 1.27µm minimum gold over 1.27µm minimum nickel.

Female Contact: The plating shall be 0.25µm minimum gold over 1.27µm minimum nickel.

#### Female Contact Wire

The wire shall be made of copper alloy.

The plating shall be 1.27µm minimum gold over 0.2µm minimum nickel.

#### Female Contact Sleeve

The sleeve shall be made of copper alloy. The plating shall be 0.25µm minimum gold over 0.8µm minimum nickel.

#### Tinned Terminations (Insert Codes 46 and 57)

Solder dipped, tin/lead 63/37, minimum thickness 1µm, minimum length 3.5mm.

### Contact Retaining Clip

Not applicable.

### Guiding and Locking Devices

Guiding and locking devices shall be made of brass (nickel-plated), stainless steel or arcap alloy.

### Magnetism Level

Not applicable.

## MARKING

### General

The marking of all components delivered to this specification shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and the following paragraphs. When the component is too small to accommodate all of the marking as specified, as much as space permits shall be marked and the marking information, in full, shall accompany the component in its primary package.

The information to be marked, and the order of precedence, shall be as follows:

Contact Position.

The ESCC Component Number.

Characteristics.

Traceability information.

### Contact Position

Contact position shall be marked on the inserts in accordance with Figure 2(a).

### The ESCC Component Number

Each component shall be marked with the ESCC Component Number which shall be constituted and marked as follows:

Example: 340106501B

Detail Specification Number: 3401065

Type Variant (Note 1): 01

Testing Level: B

notes:

Marking of the Type Variant is mandatory. No further reference to type variants is made in this specification.

### Characteristics

The characteristics to be marked in the following order of precedence are (example):

052 44 10 110

Number of contacts (52): 052

Insert type: 44

Type of contacts: 10

Guiding and locking devices: 110

#### Number of Contacts

052 – 100 – 152 – 200 – 252 – 300 – 352 – 400.

#### Insert

Inserts shall be designated by the following code numbers.

| Code No. | Description |
| --- | --- |
| 44 | Receptacle (female contacts) |
| 46 | Receptacle (female contact pre-tinned termination) |
| 55 | Plug (male contacts) |
| 57 | Plug (male contact pre-tinned termination) |

#### Contacts

Contact rear configurations shall be designated by the codes given in Table 1(a) and Figure 2(c).

#### Guiding and Locking Devices

Guiding and locking devices shall be designated by the codes given in Figure 2(b).

N.B.

If the purchase order does not specify any guiding or locking devices, receptacles shall be delivered without.

### Traceability Information

Traceability information shall be marked in accordance with the requirements of ESCC Basic Specification No. 21700.

## ELECTRICAL MEASUREMENTS

### Electrical Measurements at Room Temperature

The parameters to be measured in respect of electrical characteristics are scheduled in Table 2. Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

### Electrical Measurements at High and Low Temperatures (Table 3)

Not applicable.

### Circuits for Electrical Measurements (Figure 4)

Not applicable.

## BURN-IN AND ELECTRICAL MEASUREMENTS (TABLES 4 AND 5)

Not applicable.

## ENVIRONMENTAL AND ENDURANCE TESTS (CHARTS IV AND V OF ESCC GENERIC SPECIFICATION NO. 3401)

### Measurements and Inspections on Completion of Environmental Tests

The parameters to be measured and inspections to be performed on completion of environmental testing shall be those specified in Table 6. Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

### Measurements and Inspections at Intermediate Points during Endurance Tests

Not applicable.

### Measurements and Inspections on Completion of Endurance Tests

The parameters to be measured and inspections to be performed on completion of endurance testing are scheduled in Table 6. Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

### Conditions for Operating Life Test (Part of Endurance Testing)

Not applicable.

### Electrical Circuits for Operating Life Test

Not applicable.

### Conditions for High Temperature Storage Test (Part of Endurance Testing)

The requirements for the high temperature storage test are specified in Section 9 of ESCC Generic Specification No. 3401. The conditions for high temperature storage testing shall be the maximum storage temperature specified in Table 1(b) of this specification.

TABLE 2 – ELECTRICAL MEASUREMENTS AT ROOM TEMPERATURE

| No. | Characteristics | Symbol | Spec. and/or | Test Condition | Limits |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Test Method |  | Min. | Max. |  |
| 1 | Insulation Resistance | Ri | ESCC No. 3401 Para. 9.1.1.1 | Para. 9.1.1.1 | 10000 | - | MΩ |
| 2 | Voltage Proof Leakage Current (Sea Level) | IL | ESCC No. 3401 Para. 9.1.1.2 | 800Vrms |  | 1 | mA |
| 3 | Mated Shell Conductivity (Voltage Drop) | Vd | ESCC No. 3401 Para. 9.1.1.4 | Para. 9.1.1.4 | Not applicable |  | mV |
| 4 | Contact Resistance (Low Level Current) | Rcl | ESCC No. 3401 Para. 9.1.1.3 | All | - | 12 | mΩ |
| 5 | Contact Resistance (Rated Current) (1) | Rcr | ESCC No. 3401 Para. 9.1.1.3 | 2A | - | 12 | mΩ |

NOTES:

            - Contact Resistance at Rated Current is guaranteed but not tested during Final Production Tests (Chart II).

TABLes 3, 4 and 5

Not applicable.

Table 6 – Measurements and Inspections on Completion of Environmental and Endurance TestS

| No. | ESCC Generic No. 3401 |  | Measurements and Inspections |  | Symbol | Limits |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Environmental and Endurance Tests (1) | Test Method and Conditions | Identification | Conditions |  | Min. | Max. |  |
| 01 | Seal Test | Para. 9.9 | Not applicable |  |  | Not applicable |  |  |
| 02 | Wiring | Para. 9.10 |  |  |  | - | - |  |
| 03 | Vibration | Para. 9.11 | Initial Measurements |  |  |  |  |  |
|  |  |  | Coupling Screw(s) Unlocking Torque | - | - | Record Values |  |  |
|  |  |  | Final Measurements |  |  |  |  |  |
|  |  |  | Full Engagement | - |  |  |  |  |
|  |  |  | Coupling Screw(s) Unlocking Torque Drift | - | Δ | -25 | +25 | % |
|  |  |  | Visual Examination | - | - | - | - |  |
| 04 | Shock or Bump | Para. 9.12 | Full Engagement Visual Examination | - | - | - | - |  |
| 05 | Climatic Sequence | Para. 9.13 | Dry Heat |  |  |  |  |  |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | 1000 | - | MΩ |
|  |  |  | Low Air Pressure |  |  |  |  |  |
|  |  |  | Voltage Proof Leakage Curr. | Figure 1 | IL | ESCC 3401 Para. 9.13.5 |  |  |
|  |  |  | Damp Heat | Immediately after test |  |  |  |  |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | 100 | - | MΩ |
|  |  |  | Final Measurements | After 1-24 hrs Recovery |  |  |  |  |
|  |  |  | External Visual Inspection | ESCC 3401 Para. 9.7 | - | ESCC 3401 Para. 9.7 |  |  |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2 Item 1 |  |  |
|  |  |  | Voltage Proof Leakage Curr. | Table 2 Item 2 | IL | Table 2 Item 2 |  |  |
| 06 | Plating Thickness | Para. 9.14 | Thickness |  |  | Para. 4.4.3 of this spec. |  |  |
| 07 | Joint Strength | Para. 9.15 | ESCC 3401 Para. 9.15 |  |  | Not applicable |  |  |
| 08 | Rapid Change of Temperature | Para. 9.16 | Visual Examination | - | - | - | - |  |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2 Item 1 |  |  |
|  |  |  | Voltage Proof Leakage Curr. | Table 2 Item 2 | IL | Table 2 Item 2 |  |  |
| 09 | Contact Retention (In Insert) | Para. 9.17 & Para. 4.3.4 of spec. | Contact Displacement | - | - | ESCC 3401 Para. 9.17 |  |  |
| 10 | Endurance | Para. 9.18 | Initial Measurements |  |  |  |  |  |
|  |  |  | Mating/Unmating Forces | - | F | Para. 4.3.5 of this spec. |  |  |
|  |  |  | Low Level Contact Resist. | Table 2 Item 4 | Rcl | Record Values |  |  |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | Vd | Not applicable |  |  |
|  |  |  | Final Measurements |  |  |  |  |  |
|  |  |  | Visual Examination | - | - | - | - |  |
|  |  |  | Mating/Unmating Forces | - | F | Para. 4.3.5 of this spec. |  |  |
|  |  |  | Low Level Contact Resistance Drift | Table 2 Item 4 | ΔRcl | - | 4 | mΩ |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | Vd | Table 2 Item 3 |  |  |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2 Item 1 |  |  |
|  |  |  | Voltage Proof Leakage Curr. | Table 2 Item 2 | IL | Table 2 Item 2 |  |  |
| 11 | Permanence of Marking | Para. 9.19 | As applicable | - | - | - | - |  |
| 12 | Mating/Unmating Forces | Para. 9.20 | Force | - | F | Para. 4.3.5 of this spec. |  |  |
| 13 | High Temperature Storage | Para. 9.21 | Initial Measurements |  |  |  |  |  |
|  |  |  | Low Level Contact Resist. | Table 2 Item 4 | Rcl | Record Values |  |  |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | Vd | Not applicable |  |  |
|  |  |  | Final Measurements |  |  |  |  |  |
|  |  |  | Visual Examination | - | - | - | - |  |
|  |  |  | Mating/Unmating Forces | - | F | Para. 4.3.5 of this spec. |  |  |
|  |  |  | Low Level Contact Resistance Drift | Table 2 Item 4 | ΔRcl | - | 4 | mΩ |
|  |  |  | Rated Current Contact Resistance | Table 2 Item 5 | Rcr | Table 2 Item 5 |  |  |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | Vd | Not applicable |  |  |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2 Item 1 |  |  |
|  |  |  | Voltage Proof Leakage Curr. | Table 2 Item 2 | IL | Table 2 Item 2 |  |  |
|  |  |  | Contact Retention (In Insert) | Para. 4.3.4 of this spec. | - | ESCC 3401 Para. 9.17 |  |  |
| 14 | Corrosion | Para. 9.22 | Visual Examination | - | - | Not applicable |  |  |
| 15 | Insert Retention (In Shell) | Para. 9.23 & Para. 4.3.6 of this spec. | Visual Examination | - | - | Para. 4.3.6 of this spec. |  |  |
| 16 | Jackscrew Retention | Para. 9.24 & Para. 4.3.7 of this spec. | Visual Examination | - | - | Not applicable |  |  |
| 17 | High Temperature Measurements | Para. 9.25 | Insulation Resistance | Table 2 Item 1 | Ri | 500 | - | MΩ |
| 18 | Overload Test | Para. 9.26 | Internal Temperature |  | T | - | +100 | °C |
|  |  |  | Rated Current Contact Resistance | Table 2 Item 5 | Rcr | Table 2 Item 5 |  |  |
|  |  |  | Mated Shell Conductivity | Table 2 Item 3 | Vd | Not applicable |  |  |
|  |  |  | Insulation Resistance | Table 2 Item 1 | Ri | Table 2 Item 1 |  |  |
|  |  |  | Voltage Proof Leakage Curr. | Table 2 Item 2 | IL | Table 2 Item 2 |  |  |
| 19 | Maintenance Aging | Para. 9.27 | Visual Examination | - | - | - | - |  |
|  |  |  | Contact Retention | Para. 4.3.4 of this spec. |  | Not applicable |  |  |
|  |  |  | Contact Insertion and Withdrawal Forces | Para. 4.3.8 of this spec. |  | Not applicable |  |  |
| 20 | Engage/Separation Forces | Para. 9.28 & Para. 4.3.9 of this spec. | Force | - | F | Para. 4.3.9 of this spec. |  |  |
| 21 | Oversize Pin Exclusion | Para. 9.29 & Para. 4.3.10 of this spec. | - | - | - | ESCC 3401 Para. 9.29 |  |  |
| 22 | Probe Damage | Para. 9.30 & Para. 4.3.11 of this spec. | Contact Separation Force | Para. 4.3.9 of this spec. | - | Not applicable |  |  |
| 23 | Solderability | Para. 9.31 & Para. 4.3.12 of this spec. | - | - | - | ESCC 3401 Para. 9.31 |  |  |

notes:

            - The tests in this Table refer to either Chart IV or V and shall be used as applicable.