Page 1 of 14

TRANSISTORS, LOW POWER, NPN

BASED ON TYPE 2ST15300

ESCC Detail Specification No. 5201/020

| Issue 2 | October 2020 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2020. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| 1378 | Specification upissued to incorporate changes per DCR. |

TABLE OF CONTENTS

1	General	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants	5

1.5	MAXIMUM RATINGS	6

1.6	PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION	7

1.6.1	Surface Mount Package (SMD.5) - 3 terminal	7

1.7	FUNCTIONAL DIAGRAM	7

1.8	MATERIALS AND FINISHES	8

2	REQUIREMENTS	8

2.1	GENERAL	8

2.1.1	Deviations from the Generic Specification	8

2.2	MARKING	8

2.3	TERMINAL STRENGTH	8

2.4	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	8

2.4.1	Room Temperature Electrical Measurements	9

2.4.2	High and Low Temperatures Electrical Measurements	11

2.5	PARAMETER DRIFT VALUES	11

2.6	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	11

2.7	HIGH TEMPERATURE REVERSE BIAS BURN-IN CONDITIONS	12

2.8	POWER BURN-IN CONDITIONS	12

2.9	OPERATING LIFE CONDITIONS	12

Appendix A	14

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 5000

MIL-STD-750, Test Methods and Procedures for Semiconductor Devices

MIL-STD-883, Test Methods and Procedures for Micro-electronics.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 520102001

Detail Specification Reference: 5201020

Component Type Variant Number: 01 (as required)

Total Dose Radiation Level Letter: R (as required)

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Based on Type | Case | Terminal Material and Finish | Weight max g | Total Dose Radiation Level Letter |
| --- | --- | --- | --- | --- | --- |
| 01 | 2ST15300 | SMD.5 | Q14 | 2 | R [100krad(Si)] |

The terminal material and finish shall be in accordance with the requirements of ESCC Basic Specification No. 23500.

Total dose radiation level letters are defined in ESCC Basic Specification No. 22900. If an alternative radiation test level is specified in the Purchase Order, the letter shall be changed accordingly.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| Collector-Base Voltage | VCBO | 300 | V | Over entire operating temperature range |
| Collector-Emitter Voltage | VCEO | 100 | V |  |
| Emitter-Base Voltage | VEBO | 6 | V |  |
| Collector Current | IC | 5 | A | Continuous |
| Power Dissipation | Ptot1 | 2.2 | W | At Tamb ≤ +25°C Note 1 |
|  | Ptot2 | 40 | W | At Tcase ≤ +25°C Note 1 |
| Thermal Resistance, Junction-to-Ambient | Rth(j-a) | 80 | °C/W |  |
| Thermal Resistance, Junction-to-Case | Rth(j-c) | 4.38 | °C/W |  |
| Junction Temperature | Tj | +200 | °C |  |
| Operating Temperature Range | Top | -65 to +200 | °C |  |
| Storage Temperature Range | Tstg | -65 to +200 | °C |  |
| Soldering Temperature | Tsol | +245 | °C | Note 2 |

NOTES:

For Tamb or Tcase > +25ºC, derate linearly to 0W at +200ºC.

Duration 5 seconds maximum and the same terminal shall not be resoldered until 3 minutes have elapsed.

## PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION

### Surface Mount Package (SMD.5) - 3 terminal

| Symbols | Dimensions mm |  | Remarks |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 2.84 | 3.15 |  |
| A1 | 0.25 | 0.51 |  |
| b | 7.13 | 7.39 |  |
| b1 | 5.58 | 5.84 |  |
| b2 | 2.28 | 2.54 | 2 places |
| b3 | 2.92 | 3.18 | 2 places |
| D | 10.03 | 10.28 |  |
| D1 | 0.76 | - | 2 places |
| E | 7.39 | 7.64 |  |
| e | 1.91 BSC |  | 2 places |

NOTES:

Terminal identification is specified by the components geometry where Terminal 1 = base, Terminal 2 = emitter and Terminal 3 = collector.

## FUNCTIONAL DIAGRAM

|  | Base Emitter Collector |
| --- | --- |

NOTES:

            - The lid is not connected to any terminal.

## MATERIALS AND FINISHES

Materials and finishes shall be as follows:

Case

- The case shall be hermetically sealed and have a ceramic body with a Kovar lid.

Terminals

- As specified in Para. 1.4.2 Component Type Variants.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

None.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component or its primary package shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## TERMINAL STRENGTH

The test conditions for terminal strength, tested as specified in the ESCC Generic Specification, shall be in accordance with MIL-STD-883, Test Method 2004, Test Condition D.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Collector-Emitter Breakdown Voltage | V(BR)CEO | 3011 | IC = 10mA Bias condition D Note 1 | 100 | - | V |
| Collector-Base Breakdown Voltage | V(BR)CBO | 3001 | IC = 10mA Bias condition D | 300 | - | V |
| Emitter-Base Breakdown Voltage | V(BR)EBO | 3026 | IE = 10µA Bias condition D | 6 | - | V |
| Collector-Base Cut-off Current | ICBO | 3036 | VCE = 300V Bias condition D | - | 10 | µA |
| Emitter-Base Cut-off Current | IEBO | 3061 | VEB = 6V Bias condition D | - | 50 | µA |
| Forward-Current Transfer Ratio | hFE1 | 3076 | VCE = 0.6V; IC = 50mA | 50 | - | - |
|  | hFE2 |  | VCE = 0.6V; IC = 250mA | 55 | - | - |
|  | hFE3 |  | VCE = 5V; IC = 1A | 55 | - | - |
|  | hFE4 |  | VCE = 5V; IC = 5A | 35 | - | - |
| Collector-Emitter Saturation Voltage | VCE(sat) | 3071 | IC = 5A, IB = 1A Note 1 | - | 0.7 | V |
| Base-Emitter Saturation Voltage | VBE(sat) | 3066 | IC = 5A, IB = 1A Test condition A Note 1 | - | 1.4 | V |
| Output Capacitance | Cobo | 3236 | VCB = 10V, IE = 0A f = 1MHz Note 2 | - | 120 | pF |
| Turn-on Time | ton | - | VCC = 30V, VBB = -8V Ic = 3A IB1 = IB2 = 300mA Notes 2, 3 | - | 0.4 | µs |
| Turn-off Time | toff | - |  | - | 3.5 | µs |

NOTES:

Pulse measurement: Pulse Width ≤ 300µs, Duty Cycle ≤ 2%.

For AC characteristics, read and record measurements shall be performed on a sample of 32 components with 0 failures allowed. Alternatively a 100% inspection may be performed.

ton and toff shall be measured as follows:

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions Note 1 | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Collector-Base Cut-off Current | ICBO | 3036 | Tamb = +150 (+0 -5)°C VCB = 300V Bias condition D | - | 100 | µA |

NOTES:

Read and record measurements shall be performed on a sample of 5 components with 0 failures allowed. Alternatively a 100% inspection may be performed.

## PARAMETER DRIFT VALUES

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.4.1 Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value Δ | Absolute |  |  |
|  |  |  | Min | Max |  |
| Collector-Base Cut-off Current | ICBO | ±100 or (1) ±100% | - | 10 | µA |
| Collector-Emitter Saturation Voltage | VCE(sat) | ±30mV or (1) ±15% | - | 0.7 | V |
| Forward-Current Transfer Ratio 2 | hFE2 | ±15% | 55 | - | - |

NOTES:

Whichever is the greater referred to initial value.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.4.1 Room Temperature Electrical Measurements.

The limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- |
|  |  | Min | Max |  |
| Collector-Base Cut-off Current | ICBO | - | 10 | µA |
| Collector-Emitter Saturation Voltage | VCE(sat) | - | 0.7 | V |
| Forward-Current Transfer Ratio 2 | hFE2 | 55 | - | - |

## HIGH TEMPERATURE REVERSE BIAS BURN-IN CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | °C |
| Collector-Base Voltage | VCB | 100 | V |
| Duration | t | 48 minimum | Hours |

## POWER BURN-IN CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +20 to +50 | °C |
| Power Dissipation | Ptot | As per Para. 1.5 Maximum Ratings. Derate Ptot1 at the chosen Tamb using the specified Rth(j-a). | W |
| Collector-Base Voltage | VCB | 20 to 50 | V |

## OPERATING LIFE CONDITIONS

The conditions shall be as specified in Para. 2.8 Power Burn-in Conditions.

  - TOTAL DOSE RADIATION TESTING

All lots shall be irradiated in accordance with ESCC Basic Specification No. 22900, low dose rate (window 2: 36 to 360 rad(Si)/hour).

    - Bias Conditions and Total Dose Level for Total dose Radiation Testing

The following bias conditions shall be used for Total Dose Radiation Testing:

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +20 ±5 | °C |
| Bias Condition 1: Collector-Emitter Voltage | VCES | ≥ 80% V(BR)CEO | V |
| Bias Condition 2: Collector-Emitter Voltage | VCES | 0 | V |

The total dose level applied shall be as specified in Para. 1.4.2 or in the Purchase Order.

    - Electrical Measurements for Radiation Testing

Prior to irradiation testing, the devices shall have successfully met Para. 2.4.1 Room Temperature Electrical Measurements.

Unless otherwise stated, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified, the test methods and test conditions shall be as per the corresponding test defined in Para. 2.4.1 Room Temperature Electrical Measurements.

The parameters to be measured during and on completion of irradiation testing are shown below.

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Collector-Emitter Breakdown Voltage | V(BR)CEO | See Para. 2.4.1 | See Para. 2.4.1 | 100 | - | V |
| Collector-Base Breakdown Voltage | V(BR)CBO | See Para. 2.4.1 | See Para. 2.4.1 | 240 | - | V |
| Emitter-Base Breakdown Voltage | V(BR)EBO | See Para. 2.4.1 | See Para. 2.4.1 | 6 | - | V |
| Collector-Base Cut-off Current | ICBO | See Para. 2.4.1 | See Para. 2.4.1 | - | 10 | µA |
| Emitter-Base Cut-off Current | IEBO | See Para. 2.4.1 | See Para. 2.4.1 | - | 50 | µA |
| Forward-Current Transfer Ratio (post irradiation gain calculation) (Note 1) | [hFE1] | 3076 | VCE = 0.6V; IC = 50mA | [25] | - | - |
|  | [hFE2] |  | VCE = 0.6V; IC = 250mA | [27.5] | - | - |
|  | [hFE3] |  | VCE = 5V; IC = 1A | [27.5] | - | - |
|  | [hFE4] |  | VCE = 5V; IC = 5A | [17.5] | - | - |
| Collector-Emitter Saturation Voltage | VCE(sat) | See Para. 2.4.1 | See Para. 2.4.1 | - | 0.7 | V |
| Base-Emitter Saturation Voltage | VBE(sat) | See Para. 2.4.1 | See Para. 2.4.1 | - | 1.4 | V |

NOTES:

            - The post-irradiation gain calculation of [hFE], made using hFE measurements from prior to and on completion of irradiation testing and after each annealing step if any, shall be as specified in MIL-STD-750 Method 1019.

## Appendix A

Agreed deviations for STMICROELECTRONICS (F)

| ITEMS AFFECTED | DESCRIPTION OF DEVIATIONS |
| --- | --- |
| Para. 2.1.1 Deviations from the Generic Specification: Deviations from Screening Tests - Chart F3 | Solderability is not applicable unless specifically stipulated in the Purchase Order. |
| Para. 2.4.1 Room Temperature Electrical Measurements | All AC characteristics (see Para. 2.4.1 Note 2) may be considered guaranteed but not tested if successful pilot lot testing has been performed on the wafer lot which includes AC characteristic measurements per the Detail Specification. A summary of the pilot lot testing shall be provided if required by the Purchase Order. |
| Para. 2.4.2 High and Low Temperatures Electrical Measurements | All characteristics specified may be considered guaranteed but not tested if successful pilot lot testing has been performed on the wafer lot which includes characteristic measurements at high and low temperatures per the Detail Specification. A summary of the pilot lot testing shall be provided if required by the Purchase Order. |