Page 1 of 19

CAPACITORS, FIXED, DC SELF-HEALING, METALLISED FILM DIELECTRIC

BASED ON TYPE PM90S

ESCC Detail Specification No. 3006/020

| Issue 7 Draft A | November 2020 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2020. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| tba | Specification updated to incorporate changes per DCR. |

TABLE OF CONTENTS

1	GENERAL	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND Component Type Variants	5

1.4.1	The ESCC Component Number	5

1.4.1.1	Characteristics and Ratings Codes	5

1.4.2	Component Type Variants and Range of Components	6

1.4.2.1	Component Type Variants	6

1.4.2.2	Range of Components	6

1.5	MAXIMUM RATINGS	10

1.6	PHYSICAL DIMENSIONS and terminal identification	11

1.6.1	Component Type Variants 01 to 08	11

1.6.2	Component Type Variants 09 to 16	12

1.6.3	Component Type Variants 17 to 24	13

1.7	FUNCTIONAL DIAGRAM	13

1.8	MATERIALS AND FINISHES	14

1.8.1	Lead/Terminal Material	14

1.8.2	Lead/Terminal Finish	14

2	REQUIREMENTS	14

2.1	GENERAL	14

2.1.1	Deviations from the Generic Specification	14

2.1.1.1	Deviations from Qualification and Periodic Tests – Chart F4	14

2.2	MARKING	14

2.3	Robustness of terminations	15

2.4	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	15

2.4.1	Room Temperature Electrical Measurements	15

2.4.2	High and Low Temperatures Electrical Measurements	16

2.5	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	16

2.6	BURN-IN conditions	18

APPENDIX ‘A’	19

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, and test and inspection data for the component type variants and/or the Component Type Variants and Range of Components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3006.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND Component Type Variants

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 300602001155KH

Detail Specification Reference: 3006020

Component Type Variant Number: 01 (01 to 21, as required or as applicable)

Characteristic code: Capacitance Value (1.5µF): 155 (as required)

Characteristic code: Capacitance Tolerance (±10%): K (as required)

Rating code: Rated Voltage (250V): H (as required)

#### Characteristics and Ratings Codes

Characteristics and ratings to be codified as part of the ESCC Component Number shall be as follows:

Capacitance Value, C, expressed by means of the following codes in accordance with ESCC Basic Specification No. 21700. The unit quantity shall be picofarads (pF).

| Capacitance Value C (pF) | Code |
| --- | --- |
| XX 103 | XX3 |
| XX 104 | XX4 |
| XX 105 | XX5 |
| XX 106 | XX6 |
| XX 107 | XX7 |

Capacitance Tolerance expressed by the following codes in accordance with ESCC Basic Specification No. 21700:

| Tolerance (±) | Code Letter |
| --- | --- |
| 10% | K |
| 20% | M |

DC Rated Voltage, UR, expressed by the following codes:

| DC Rated Voltage UR (V) | Code Letter |
| --- | --- |
| 50 | C |
| 100 | E |
| 250 | H |
| 400 | K |
| 630 | Z |

### Component Type Variants and Range of Components

#### Component Type Variants

24 Component Type Variants are defined in this specification and three different package types are available, as follows:

Radial lead package: Component Type Variants 01 to 08

Surface mount package with bar-type terminations: Component Type Variants 09 to 16

Surface mount package with tab-type terminations: Component Type Variants 17 to 24

Eight different sizes are therefore available for each package type. See Para. 1.6 for details.

#### Range of Components

The range of components applicable to this specification are as follows:

| DC Rated Voltage (UR) Max (V) | Capacitance Value C (µF) (Note 1) | AC Rated Current (IRA) Max (Arms) | Available Type Variants (See Para 1.6) | Weight Max (g) |
| --- | --- | --- | --- | --- |
| 50 | 8.2 | 3.1 | 01, 09, 17 | 4.9 |
| 50 | 10 | 3.8 | 01, 09, 17 | 4.9 |
| 50 | 12 | 4 | 01, 09, 17 | 4.9 |
| 50 | 15 | 5 | 02, 10, 18 | 6 |
| 50 | 18 | 6.9 | 03, 11, 19 | 9.5 |
| 50 | 22 | 8.4 | 03, 11, 19 | 9.5 |
| 50 | 27 | 10.4 | 04, 12, 20 | 13.6 |
| 50 | 33 | 12.5 | 04, 12, 20 | 13.6 |
| 50 | 33 | 7.2 | 06, 14, 22 | 21.2 |
| 50 | 39 | 12.5 | 04, 12, 20 | 13.6 |
| 50 | 39 | 8.5 | 06, 14, 22 | 21.2 |
| 50 | 47 | 12.5 | 05, 13, 21 | 20.4 |
| 50 | 47 | 10.3 | 06, 14, 22 | 21.2 |
| 50 | 56 | 12.5 | 05, 13, 21 | 20.4 |
| 50 | 68 | 15 | 07, 15, 23 | 37.3 |
| 50 | 82 | 15 | 07, 15, 23 | 37.3 |
| 50 | 100 | 15 | 07, 15, 23 | 37.3 |
| 50 | 120 | 15 | 08, 16, 24 | 54.2 |
| 50 | 150 | 15 | 08, 16, 24 | 54.2 |
| 100 | 3.3 | 1.6 | 01, 09, 17 | 4.9 |
| 100 | 3.9 | 2 | 01, 09, 17 | 4.9 |
| 100 | 4.7 | 2.4 | 01, 09, 17 | 4.9 |
| 100 | 5.6 | 2.8 | 01, 09, 17 | 4.9 |
| 100 | 6.8 | 3.1 | 01, 09, 17 | 4.9 |
| 100 | 8.2 | 3.7 | 02, 10, 18 | 6 |
| 100 | 10 | 5.1 | 03, 11, 19 | 9.5 |
| 100 | 12 | 6.1 | 03, 11, 19 | 9.5 |
| 100 | 15 | 7.7 | 04, 12, 20 | 13.6 |
| 100 | 18 | 9.2 | 04, 12, 20 | 13.6 |
| 100 | 18 | 6.3 | 06, 14, 22 | 21.2 |
| 100 | 22 | 10.1 | 04, 12, 20 | 13.6 |
| 100 | 22 | 7.7 | 06, 14, 22 | 21.2 |
| 100 | 27 | 12.5 | 05, 13, 21 | 20.4 |
| 100 | 33 | 12.5 | 05, 13, 21 | 20.4 |
| 100 | 33 | 11.4 | 06, 14, 22 | 21.2 |
| 100 | 39 | 13.5 | 07, 15, 23 | 37.3 |
| 100 | 47 | 15 | 07, 15, 23 | 37.3 |
| 100 | 56 | 15 | 07, 15, 23 | 37.3 |
| 100 | 68 | 15 | 08, 16, 24 | 54.2 |
| 100 | 82 | 15 | 08, 16, 24 | 54.2 |
| 100 | 100 | 15 | 08, 16, 24 | 54.2 |
| 250 | 1 | 1.2 | 01, 09, 17 | 4.9 |
| 250 | 1.2 | 1.3 | 01, 09, 17 | 4.9 |
| 250 | 1.5 | 1.5 | 01, 09, 17 | 4.9 |
| 250 | 1.8 | 1.8 | 01, 09, 17 | 4.9 |
| 250 | 2.2 | 2.2 | 01, 09, 17 | 4.9 |
| 250 | 2.7 | 2.8 | 01, 09, 17 | 4.9 |
| 250 | 3.3 | 3.4 | 02, 10, 18 | 6 |
| 250 | 3.9 | 4 | 02, 10, 18 | 6 |
| 250 | 4.7 | 4.8 | 03, 11, 19 | 9.5 |
| 250 | 5.6 | 5.8 | 03, 11, 19 | 9.5 |
| 250 | 6.8 | 7.5 | 04, 12, 20 | 13.6 |
| 250 | 6.8 | 4.6 | 06, 14, 22 | 21.2 |
| 250 | 8.2 | 8.5 | 04, 12, 20 | 13.6 |
| 250 | 10 | 10.3 | 04, 12, 20 | 13.6 |
| 250 | 10 | 6.7 | 06, 14, 22 | 21.2 |
| 250 | 12 | 12.4 | 05, 13, 21 | 20.4 |
| 250 | 12 | 8 | 06, 14, 22 | 21.2 |
| 250 | 15 | 12.5 | 05, 13, 21 | 20.4 |
| 250 | 18 | 12 | 07, 15, 23 | 37.3 |
| 250 | 22 | 15 | 07, 15, 23 | 37.3 |
| 250 | 27 | 15 | 07, 15, 23 | 37.3 |
| 250 | 33 | 15 | 08, 16, 24 | 54.2 |
| 250 | 39 | 15 | 08, 16, 24 | 54.2 |
| 400 | 0.39 | 1.1 | 01, 09, 17 | 4.9 |
| 400 | 0.47 | 1.3 | 01, 09, 17 | 4.9 |
| 400 | 0.56 | 1.3 | 01, 09, 17 | 4.9 |
| 400 | 0.68 | 1.6 | 01, 09, 17 | 4.9 |
| 400 | 0.82 | 1.9 | 01, 09, 17 | 4.9 |
| 400 | 1 | 2.4 | 02, 10, 18 | 6 |
| 400 | 1.2 | 2.9 | 02, 10, 18 | 6 |
| 400 | 1.5 | 3.6 | 03, 11, 19 | 9.5 |
| 400 | 1.8 | 4.3 | 03, 11, 19 | 9.5 |
| 400 | 2.2 | 5.3 | 04, 12, 20 | 13.6 |
| 400 | 2.2 | 3 | 06, 14, 22 | 21.2 |
| 400 | 2.7 | 6 | 04, 12, 20 | 13.6 |
| 400 | 3.3 | 7.9 | 04, 12, 20 | 13.6 |
| 400 | 3.3 | 4.5 | 06, 14, 22 | 21.2 |
| 400 | 3.9 | 9.4 | 05, 13, 21 | 20.4 |
| 400 | 4.7 | 11.3 | 05, 13, 21 | 20.4 |
| 400 | 4.7 | 6.4 | 06, 14, 22 | 21.2 |
| 400 | 5.6 | 7.6 | 07, 15, 23 | 37.3 |
| 400 | 6.8 | 9.3 | 07, 15, 23 | 37.3 |
| 400 | 8.2 | 11.5 | 07, 15, 23 | 37.3 |
| 400 | 10 | 14 | 07, 15, 23 | 37.3 |
| 400 | 12 | 15 | 08, 16, 24 | 54.2 |
| 400 | 15 | 15 | 08, 16, 24 | 54.2 |
| 630 | 0.22 | 0.9 | 01, 09, 17 | 4.9 |
| 630 | 0.27 | 1.1 | 01, 09, 17 | 4.9 |
| 630 | 0.33 | 1.3 | 02, 10, 18 | 6 |
| 630 | 0.39 | 1.6 | 02, 10, 18 | 6 |
| 630 | 0.47 | 1.9 | 03, 11, 19 | 9.5 |
| 630 | 0.56 | 2.3 | 03, 11, 19 | 9.5 |
| 630 | 0.68 | 2.8 | 03, 11, 19 | 9.5 |
| 630 | 0.82 | 3.3 | 04, 12, 20 | 13.6 |
| 630 | 1 | 4.1 | 04, 12, 20 | 13.6 |
| 630 | 1 | 2.2 | 06, 14, 22 | 21.2 |
| 630 | 1.2 | 5 | 04, 12, 20 | 13.6 |
| 630 | 1.5 | 6.1 | 05, 13, 21 | 20.4 |
| 630 | 1.5 | 3.3 | 06, 14, 22 | 21.2 |
| 630 | 1.8 | 7.3 | 05, 13, 21 | 20.4 |
| 630 | 1.8 | 4 | 06, 14, 22 | 21.2 |
| 630 | 2.2 | 4.9 | 07, 15, 23 | 37.3 |
| 630 | 2.7 | 6 | 07, 15, 23 | 37.3 |
| 630 | 3.3 | 7.3 | 07, 15, 23 | 37.3 |
| 630 | 3.9 | 8.7 | 07, 15, 23 | 37.3 |
| 630 | 4.7 | 10.3 | 08, 16, 24 | 54.2 |
| 630 | 5.6 | 12.5 | 08, 16, 24 | 54.2 |

NOTES:

All Capacitance Values are available with tolerances of ±10% and ±20%.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| DC Rated Voltage | UR | 50, 100, 250, 400, 630 | V | Note 1 |
| AC Rated Voltage | UA | 35%UR | Vrms | 50/60Hz |
| AC Rated Current | IRA | See Para. 1.4.2.2 | Arms | 50/60Hz |
| Operating Temperature Range | Top | -55 to +100 | °C | Tamb |
| Storage Temperature Range | Tstg | -55 to +100 | °C |  |
| Soldering Temperature | Tsol | +260 | °C | Note 2 |

NOTES:

As required; See Para. 1.4.2.2.

Duration 5 seconds maximum at a distance of not less than 6mm from the case and the same lead shall not be resoldered until 3 minutes have elapsed.

## PHYSICAL DIMENSIONS and terminal identification

### Component Type Variants 01 to 08

| Type Variant | Dimensions (mm) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A |  | B Max. | B1 Min. | D |  | Ød |  | F |  | G |  | L Min. |
|  | Min. | Max. |  |  | Min. | Max. | Min. | Max. | Min. | Max. | Min. | Max. |  |
| 01 | 19.5 | 20.5 | 6.5 | 0.25 | 19.5 | 20.5 | 0.95 | 1.1 | 9.66 | 10.66 | 17.3 | 18.3 | 20 |
| 02 | 19.5 | 20.5 | 8 | 0.25 | 19.5 | 20.5 | 0.95 | 1.1 | 9.66 | 10.66 | 17.3 | 18.3 | 20 |
| 03 | 19.5 | 20.5 | 12.5 | 0.25 | 19.5 | 20.5 | 0.95 | 1.1 | 9.66 | 10.66 | 17.3 | 18.3 | 20 |
| 04 | 19.5 | 20.5 | 20 | 0.25 | 19.5 | 20.5 | 0.95 | 1.1 | 9.66 | 10.66 | 17.3 | 18.3 | 20 |
| 05 | 19.5 | 20.5 | 30 | 0.25 | 19.5 | 20.5 | 0.95 | 1.1 | 9.66 | 10.66 | 17.3 | 18.3 | 20 |
| 06 | 30.5 | 31.5 | 12.5 | 0.4 | 31.5 | 32.5 | 0.95 | 1.1 | 14.74 | 15.74 | 27.44 | 28.44 | 20 |
| 07 | 30.5 | 31.5 | 22 | 0.4 | 31.5 | 32.5 | 0.95 | 1.1 | 14.74 | 15.74 | 27.44 | 28.44 | 20 |
| 08 | 30.5 | 31.5 | 32 | 0.4 | 31.5 | 32.5 | 0.95 | 1.1 | 14.74 | 15.74 | 27.44 | 28.44 | 20 |

NOTES:

Terminal identification: A voltage polarity symbol shall be marked on the top of the body, as shown, to indicate the row of terminals which should be connected to the highest potential.

### Component Type Variants 09 to 16

| Type Variant | Dimensions (mm) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A |  | B |  | D |  | d Max. | d1 |  | F |  | L |  |
|  | Min. | Max. | Min. | Max. | Min. | Max. |  | Min. | Max. | Min. | Max. | Min. | Max. |
| 09 | 19.5 | 20.5 | 6.5 | 7.5 | 19.5 | 20.5 | 0.3 | 14.5 | 15.5 | 9.5 | 10.5 | 2.75 | 3.25 |
| 10 | 19.5 | 20.5 | 8 | 9 | 19.5 | 20.5 | 0.3 | 14.5 | 15.5 | 9.5 | 10.5 | 2.75 | 3.25 |
| 11 | 19.5 | 20.5 | 12.5 | 13.5 | 19.5 | 20.5 | 0.3 | 14.5 | 15.5 | 9.5 | 10.5 | 2.75 | 3.25 |
| 12 | 19.5 | 20.5 | 20 | 21 | 19.5 | 20.5 | 0.3 | 14.5 | 15.5 | 9.5 | 10.5 | 2.75 | 3.25 |
| 13 | 19.5 | 20.5 | 30 | 31 | 19.5 | 20.5 | 0.3 | 14.5 | 15.5 | 9.5 | 10.5 | 2.75 | 3.25 |
| 14 | 30.5 | 31.5 | 12.5 | 13.5 | 31.5 | 32.5 | 0.3 | 23.5 | 24.5 | 14.5 | 15.5 | 2.75 | 3.25 |
| 15 | 30.5 | 31.5 | 22 | 23 | 31.5 | 32.5 | 0.3 | 23.5 | 24.5 | 14.5 | 15.5 | 2.75 | 3.25 |
| 16 | 30.5 | 31.5 | 32 | 33 | 31.5 | 32.5 | 0.3 | 23.5 | 24.5 | 14.5 | 15.5 | 2.75 | 3.25 |

notes:

Terminal identification: A voltage polarity symbol shall be marked on the top of the body, as shown, to indicate the row of terminals which should be connected to the highest potential.

### Component Type Variants 17 to 24

| Type Variant | Dimensions (mm) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A |  | B |  | D |  | d Max. | d1 |  | F |  | L |  |
|  | Min. | Max. | Min. | Max. | Min. | Max. |  | Min. | Max. | Min. | Max. | Min. | Max. |
| 17 | 19.5 | 20.5 | 6.5 | 7.5 | 19.5 | 20.5 | 0.3 | 2 | 3 | 9.5 | 10.5 | 2.75 | 3.25 |
| 18 | 19.5 | 20.5 | 8 | 9 | 19.5 | 20.5 | 0.3 | 2 | 3 | 9.5 | 10.5 | 2.75 | 3.25 |
| 19 | 19.5 | 20.5 | 12.5 | 13.5 | 19.5 | 20.5 | 0.3 | 2 | 3 | 9.5 | 10.5 | 2.75 | 3.25 |
| 20 | 19.5 | 20.5 | 20 | 21 | 19.5 | 20.5 | 0.3 | 2 | 3 | 9.5 | 10.5 | 2.75 | 3.25 |
| 21 | 19.5 | 20.5 | 30 | 31 | 19.5 | 20.5 | 0.3 | 2 | 3 | 9.5 | 10.5 | 2.75 | 3.25 |
| 22 | 30.5 | 31.5 | 12.5 | 13.5 | 31.5 | 32.5 | 0.3 | 2 | 3 | 9.5 | 10.5 | 2.75 | 3.25 |
| 23 | 30.5 | 31.5 | 22 | 23 | 31.5 | 32.5 | 0.3 | 2 | 3 | 9.5 | 10.5 | 2.75 | 3.25 |
| 24 | 30.5 | 31.5 | 32 | 33 | 31.5 | 32.5 | 0.3 | 2 | 3 | 9.5 | 10.5 | 2.75 | 3.25 |

NOTES:

Terminal identification: A voltage polarity symbol shall be marked on the top of the body, as shown, to indicate the row of terminals which should be connected to the highest potential.

## FUNCTIONAL DIAGRAM

notes:

These capacitors are not polarised; however, marking includes the voltage polarity symbol (see Para. 1.6) which should be respected in use.

For Component Type Variants 01 to 08 and 17 to 24, each row of terminals is connected internally to a single electrode.

## MATERIALS AND FINISHES

### Lead/Terminal Material

The lead/terminal material shall be Type A in accordance with the requirements of ESCC Basic Specification No. 23500.

### Lead/Terminal Finish

The lead/terminal finish shall be 95% tin, remainder lead.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Qualification and Periodic Tests – Chart F4

Operating Life: Test Condition (c), Applied voltage: 1.25×UR

Robustness of Terminations: for Component Type Variants 09 to 24 Robustness of Terminations shall be performed as follows:

- The components shall be mounted on a suitable substrate. After mounting, examination shall be made for good tinning as evidenced by flowing of the solder with wetting of the terminations. A force of 10N shall be applied normal to the line joining the terminals and in a plane parallel to the substrate, for a duration of 10 ±1s. The components shall then be examined and there shall be no evidence of damage or loosening of the components from the substrate.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component shall be:

Terminal Identification (see Para. 1.6).

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## Robustness of terminations

The terminals of the components are rigid.

For Component Type Variants 01 to 08 the test conditions for Robustness of Terminations shall be as specified in the ESCC Generic Specification and as follows:

Test Ua, tensile, with an applied force of 20N and a duration of 7.5 ±2.5s

All leads of the component shall be tested.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Capacitance | C | ESCC No. 3006 Test Frequency = 1kHz | Note 1 | Note 2 | µF |
| Tangent of Loss Angle | tgδ | ESCC No. 3006 Test Frequency = 1kHz | - | 10×10-3 | - |
| Insulation Resistance, Dielectric | RI | ESCC No. 3006 |  |  |  |
|  |  | C ≤ 0.33µF; UR ≤ 100V UR > 100V | 3750 7500 | - - | MΩ MΩ |
|  |  | C > 0.33µF; UR ≤ 100V UR > 100V (Note 3) | 1250 2500 | - - | MΩ.µF MΩ.µF |
| Insulation Resistance, Body Insulation | RIB | ESCC No. 3006 | 50 | - | GΩ |
| Voltage Proof, Terminal-to-Terminal | VP | ESCC No. 3006 | 1.6×UR (Note 3) | - | V |
| Voltage Proof, Terminal-to-Case | VPB | ESCC No. 3006 |  |  |  |
|  |  | UR = 50V UR ≥ 100V (Note 3) | 200 2×UR (Note 3) | - - | V V |

NOTES:

Capacitance Value of the component minus the applicable Tolerance (see Para. 1.4.2.2).

Capacitance Value of the component plus the applicable Tolerance (see Para. 1.4.2.2).

For the applicable Rated Voltage (UR) see Para. 1.4.2.2.

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | Test Method and Conditions (Note 1) | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Temperature Coefficient | ΔC/C | ESCC No. 3006 Test Frequency = 1kHz |  |  |  |
|  |  | Tamb = -55 ±2°C | - | -10 (Note 2) | % |
|  |  | Tamb = +100 ±2°C | - | +8 (Note 2) | % |

NOTES:

The measurements shall be performed on a sample of 6 components from each manufacturing lot with 0 failures allowed. In the event of any failure a 100% inspection may be performed.

The Temperature Coefficient limits are with respect to the capacitance at +22 ±2°C (reference point temperature).

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified the test methods and test conditions shall be as per the corresponding test defined in Para. 2.4.1, Room Temperature Electrical Measurements.

| Test Reference per ESCC No. 3006 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Resistance to Soldering Heat |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 1 |  | µF |
| Final Measurements | Capacitance | C | Note 1 |  | µF |
|  | Change in Capacitance | ΔC/C | -3 | +3 | % |
|  | Insulation Resistance (Terminal-to-Terminal) | RI |  |  |  |
|  | C ≤ 0.33µF; UR ≤ 100V UR > 100V |  | Note 1 Note 1 | - - | MΩ MΩ |
|  | C > 0.33µF; UR ≤ 100V UR > 100V |  | Note 1 Note 1 | - - | MΩ.µF MΩ.µF |
|  | Tangent of Loss Angle | tgδ | - | Note 1 | - |
| Temperature Coefficient | Temperature Coefficient (Note 2) | ΔC/C | Note 3 |  | % |
| Rapid Change of Temperature |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 1 |  | µF |
| Final Measurements | Capacitance | C | Note 1 |  | µF |
|  | Change in Capacitance | ΔC/C | -3 | +3 | % |
|  | Tangent of Loss Angle | tgδ | - | Note 1 | - |
| Climatic Sequence |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 1 |  | µF |
| Final Measurements | Capacitance | C | Note 1 |  | µF |
|  | Change in Capacitance | ΔC/C | -3 | +3 | % |
|  | Tangent of Loss Angle | tgδ | - | Note 4 | - |
|  | Voltage Proof, Terminal-to-Terminal | VP | Note 1 | - | V |
|  | Voltage Proof, Terminal-to-Case UR = 50V UR ≥ 100V | VPB | Note 1 Note 1 | - - | V |
|  | Insulation Resistance, Dielectric | RI |  |  |  |
|  | C ≤ 0.33µF; UR ≤ 100V UR > 100V |  | Note 4 Note 4 | - - | MΩ MΩ |
|  | C > 0.33µF; UR ≤ 100V UR > 100V |  | Note 4 Note 4 | - - | MΩ.µF MΩ.µF |
|  | Insulation Resistance, Body Insulation | RIB | Note 4 | - | GΩ |
| Operating Life |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 1 |  | µF |
| Intermediate Measurements (1000 hours) | Capacitance | C | Note 1 |  | µF |
|  | Change in Capacitance | ΔC/C | -5 | +5 | % |
| Final Measurements (1000 or 2000 hours) (Note 5) | Capacitance | C | Note 1 |  | µF |
|  | Change in Capacitance | ΔC/C | -5 | +5 | % |
|  | Tangent of Loss Angle | tgδ | - | Note 4 | - |
|  | Insulation Resistance, Dielectric | RI |  |  |  |
|  | C ≤ 0.33µF; UR ≤ 100V UR > 100V |  | Note 4 Note 4 | - - | MΩ MΩ |
|  | C > 0.33µF; UR ≤ 100V UR > 100V |  | Note 4 Note 4 | - - | MΩ.µF MΩ.µF |
|  | Insulation Resistance, Body Insulation | RIB | 5 | - | GΩ |

NOTES:

As specified in Para. 2.4.1.

The test method and test conditions shall be as specified in Para. 2.4.2.

As specified in Para. 2.4.2.

50% of the limit specified in Para. 2.4.1.

1000 hours is applicable to Periodic Testing for extension of qualification. 2000 hours is applicable to Qualification Testing, and to Periodic Testing for renewal of qualification after lapse.

## BURN-IN conditions

| Characteristics | Symbols | Conditions (Note 1) | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +100 (+0 -5) | °C |
| Test Voltage | VT | 1.25×UR (Note 2) | V |

NOTES:

On completion of Burn-in the components shall be removed from the burn-in chamber and allowed to cool, under normal atmospheric conditions, for recovery for 24 ±2 hours.

For the applicable Rated Voltage (UR) see Para. 1.4.2.2.

## APPENDIX ‘A’

AGREED DEVIATIONS FOR EXXELIA TECHNOLOGIES (F)

| Items Affected | Description of Deviations |
| --- | --- |
| Para. 2.1.1, Deviations from the Generic Specification: Deviations from Screening Tests – Chart F3 | All procurement lots shall have 100% Serialisation carried out at any point during Chart F3 testing prior to the performance of High and Low Temperatures Electrical Measurements. |
|  | If a particular lot is required to undergo testing in accordance with Chart F4, then the Temperature Coefficient measurements, made during High and Low Temperatures Electrical Measurements on a sample of 6 randomly-selected serialised capacitors, shall be Read-and-Record measurements. See “Deviations from Qualification and Periodic Tests – Chart F4” below. |
| Para. 2.1.1.1, Deviations from Qualification and Periodic Tests – Chart F4 | The Temperature Coefficient testing specified in Subgroup 2B of Chart F4 may not be performed. In instances when Temperature Coefficient testing per Subgroup 2B of Chart F4 is not performed, the Read-and-Record measurements data obtained during High and Low Temperature Electrical Measurements (part of Chart F3, Screening Tests) shall be submitted instead. |