Page 1 of 59

RF 	COAXIAL CONNECTOR WITH MALE CONTACT

BASED ON TYPE SMA

ESCC Detail Specification No. 3402/001

| Issue 4 Draft A | June 2021 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2021. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| TBA | Specification updated to incorporate changes per DCR. |
| Draft 4A S.Thacker ESCC-TW 07/06/2021 | Initial draft converted Detail spec to make it consistent with the newly published, converted Generic spec: ESCC No. 3402 issue 5. Changes in this specification from the published issue 3 are mainly editorial in nature so have not been highlighted. Any actual technical changes, including those resulting from the new Generic spec, are highlighted yellow for information. |

TABLE OF CONTENTS

1	GENERAL	6

1.1	SCOPE	6

1.2	APPLICABLE DOCUMENTS	6

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	6

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	6

1.4.1	The ESCC Component Number	6

1.4.2	Component Type Variants	8

1.5	MAXIMUM RATINGS	9

1.6	PHYSICAL DIMENSIONS (SEE ALSO PARA. 3)	11

1.6.1	Connector Interface Dimensions and Connector Interface Gauge Dimensions	11

1.7	MATERIALS AND FINISHES	11

1.7.2	Type 1	11

1.7.3	Type 2	12

1.7.4	Type 3	12

1.7.5	Type 4	13

2	REQUIREMENTS	13

2.1	GENERAL	13

2.1.1	Deviations from the Generic Specification	13

2.2	MARKING	13

2.3	ENVIRONMENTAL AND MECHANICAL TESTS	14

2.4	Room Temperature Electrical Measurements (NOTE 1)	14

2.5	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	15

3	COMPONENT TYPE VARIANTS – DETAIL REQUIREMENTS	16

3.1	variant 01 – Straight Plug, Solder-Type, for Semi-Rigid Cable Ø2.2mm	16

3.2	variant 02 - STRAIGHT PLUG, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø3.58mm	17

3.3	variant 03 - STRAIGHT PLUG with cable clamp, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø6.35mm	18

3.4	variant 04 - STRAIGHT PLUG with cable clamp, SOLDER-TYPE, FOR SEMI-RIGID microporous CABLE, Ø6.35mm	19

3.5	variant 05 - STRAIGHT PLUG, Crimp-type	20

3.6	variant 06 - STRAIGHT PLUG, Crimp-type, for 50 CIS Cable	21

3.7	variant 07 - STRAIGHT PLUG, Crimp- or SOLDER-TYPE, cable Ø5mm/50Ω, single braid	22

3.8	variant 08 - STRAIGHT PLUG, Crimp- or SOLDER-TYPE, CABLE Ø5mm/50Ω, DOUBLE BRAID	23

3.9	variant 09 - Right angle PLUG, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø2.2mm	24

3.10	variant 10 - right angle PLUG, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø3.58mm	25

3.11	variant 12 - RIGHT ANGLE PLUG, CRIMP-TYPE	26

3.12	variant 13 - RIGHT ANGLE PLUG, CRIMP-TYPE	27

3.13	variant 14 - RIGHT ANGLE PLUG, CRIMP-TYPE, FOR CABLE Ø5mm/50Ω, DOUBLE BRAID	28

3.14	variant 15 - SQUARE FLANGE MALE RECEPTACLE	29

3.15	variant 16 - 2-HOLE FLANGE MALE RECEPTACLE	30

3.16	variant 17 - square flange male receptacle	31

3.17	variant 18 - male flange receptacle, triplate launcher	32

3.18	variant 20 - MALE FLANGE RECEPTACLE, TRIPLATE LAUNCHER	33

3.19	variant 21 - MALE FLANGE RECEPTACLE, TRIPLATE LAUNCHER	34

3.20	variant 22 - SQUARE FLANGE MALE RECEPTACLE, LOW RF LEAKAGE	35

3.21	variant 22 - SQUARE FLANGE MALE RECEPTACLE, LOW RF LEAKAGE	36

3.22	variant 24 - SQUARE FLANGE MALE RECEPTACLE, LOW RF LEAKAGE	37

3.23	variant 25 - SQUARE FLANGE MALE RECEPTACLE	38

3.24	variant 26 - STRAIGHT PLUG, CRIMP-TYPE	39

3.25	variant 27 - SQUARE FLANGE MALE RECEPTACLE	40

3.26	variant 28 - 2-HOLE FLANGE MALE RECEPTACLE	41

3.27	variant 29 - STRAIght PLUg, SOLDER-TYPE FOR SHF 5 CABLE	42

3.28	variant 30 - 2-HOLE FLANGE MALE RECEPTACLE	43

3.29	variant 32 - 2-Hole Flange Male Receptacle, with Hermetic Glass Seal Contact Ø0.3mm and EMI Gasket	44

3.30	variant 33 - 2-Hole Flange Male Receptacle, with Hermetic Glass Seal Contact Ø0.46mm and EMI Gasket	45

3.31	variant 34 - BULKHEAD RECEPTACLE, with Hermetic Glass Seal Contact Ø0.3mm	46

3.32	variant 35 - BULKHEAD RECEPTACLE, with Hermetic Glass Seal Contact Ø0.46mm	47

3.33	variant 36 - HERMETIC BULKHEAD RECEPTACLE, with COMPRESSION GASKET	48

3.34	variant 37 - STRAIght PLUg, SOLDER-TYPE FOR SHF 3 CABLE	49

3.35	variant 38 - STRAIght PLUg, SOLDER-TYPE FOR SHF 8 CABLE	50

3.36	variant 39 - RIGHT ANGLE PLUG, SOLDER-TYPE, FOR SHF 3 CABLE	51

3.37	variant 40 - RIGHT ANGLE PLUG, SOLDER-TYPE, FOR SHF 5 CABLE	52

3.38	variant 41 - ELBOW PLUG, SOLDER-TYPE, FOR SHF 3 CABLE	53

3.39	variant 42 - ELBOW PLUG, SOLDER-TYPE, FOR SHF 5 CABLE	54

3.40	variant 43 - ELBOW PLUG, SOLDER-TYPE, FOR SHF 8 CABLE	55

3.41	variant 44 - RIGHT ANGLE PLUG, CRIMP-TYPE, FOR 50 CIS CABLE	56

3.42	variant 45 - SQUARE FLANGE MALE RECEPTACLE	57

3.43	variant 46 - 2-HOLE FLANGE MALE RECEPTACLE	58

3.44	variant 47 - RIGHT ANGLE PLUG, CRIMP-TYPE, FOR 50 CIS CABLE	59

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3402.

MIL-STD-348, Department of Defence Interface Standard: Radio Frequency Connector Interfaces.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

For components with a fixed configuration (see Paras. 1.4.2 and 3): Variants 01 to 10, 12 to 18, 20 to 23, 25, 26, 39, 30, 32 to 44, 47:

- Example: 340200101201

Detail Specification Reference: 3402001

Component Type Variant Number: 01 (as required)

Characteristic code: Material/Plating (Type 2): 2 (as required)

Characteristic code: Fixed configuration identifier: 01 (see Note 1)

NOTES:

- For components with a fixed configuration, this code shall always be 01.

For components with a variable configuration where the rear contact and insert dimensions A and B shall be selected by the Orderer (see Paras. 1.4.2 and 3): Variants 24, 27, 28, 45, 46:

- Example: 340200124213D00W02D50C

Detail Specification Reference: 3402001

Component Type Variant Number: 24 (as required)

Characteristic code: Material/Plating (Type 2): 2 (as required)

Characteristic code: Dimension A: Rear contact length (13mm): 13D00 (as required)

Characteristic code: Dimension A tolerance (±0.05mm): W (as required)

Characteristic code: Dimension B: Rear insert length (2.5mm): 02D50 (as required)

Characteristic code: Dimension B tolerance (±0.25mm): C (as required)

#### Characteristics Codes

Characteristics to be codified as part of the ESCC Component Number (as applicable) shall be as follows:

Material/Plating: Connector material and plating (see Para. 1.7) expressed by means of the following codes:

| Material/Plating | Code |
| --- | --- |
| Beryllium Copper, Gold plated, Copper underplate (amagnetic version) (Type 1) | 1 |
| Beryllium Copper, Gold plated, Nickel underplate (standard version) (Type 2) | 2 |
| Stainless Steel, Electro-passivated (Type 3) | 3 |
| Stainless Steel, Gold plated (Type 4) | 4 |

Dimension A and B: Rear contact and insert lengths (see Para. 3) expressed by the following codes. The unit quantity shall be mm:

| Dimension A or B Length (mm) | Code |
| --- | --- |
| XX.XX | XXDXX |

NOTES:

Dimension A shall always be greater than B. Both values shall always be positive.

When dimension B (insert) is flush with the flange (B = 0), the insert length shall be marked 00D00 with the appropriate tolerance.

Dimension A and B tolerance: Tolerances on contact and insert lengths (see Para. 3) expressed by the following codes. The unit quantity shall be mm:

| Dimension A or B Tolerance (mm) | Code |
| --- | --- |
| ±0.05 | W |
| ±0.1 | B |
| ±0.25 | C |

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Description (Notes 1, 2) | Configuration Type (Note 3) |
| --- | --- | --- |
| 01 | Straight Plug, Solder-Type, for Semi-Rigid Cable Ø2.2mm | Fixed |
| 02 | Straight Plug, Solder-Type, for Semi-Rigid Cable Ø3.58mm | Fixed |
| 03 | Straight Plug with Cable Clamp, Solder-Type, for Semi-Rigid Cable Ø6.35mm | Variable |
| 04 | Straight Plug with Cable Clamp, Solder-Type, for Semi-Rigid Microporous Cable Ø6.35mm | Fixed |
| 05 | Straight Plug, Crimp-Type | Fixed |
| 06 | Straight Plug, Crimp-Type, for 50 CIS Cable | Fixed |
| 07 | Straight Plug, Crimp- or Solder-Type, Cable Ø5mm/50Ω, Single Braid | Fixed |
| 08 | Straight Plug, Crimp- or Solder-Type, Cable Ø5mm/50Ω, Double Braid | Fixed |
| 09 | Right Angle Plug, Solder-Type for Semi-Rigid Cable Ø2.2mm | Fixed |
| 10 | Right Angle Plug, Solder-Type for Semi-Rigid Cable Ø3.58mm | Fixed |
| 12 | Right Angle Plug, Crimp-Type | Fixed |
| 13 | Right Angle Plug, Crimp-Type | Fixed |
| 14 | Right Angle Plug, Crimp-Type, for Cable Ø5mm/50Ω, Double Braid | Fixed |
| 15 | Square Flange Male Receptacle | Fixed |
| 16 | 2-Hole Flange Male Receptacle | Fixed |
| 17 | Square Flange Male Receptacle | Fixed |
| 18 | Male Flange Receptacle, Triplate Launcher | Fixed |
| 20 | Male Flange Receptacle, Triplate Launcher | Fixed |
| 21 | Male Flange Receptacle, Triplate Launcher | Fixed |
| 22 | Square Flange Male Receptacle, Low RF Leakage | Fixed |
| 23 | Square Flange Male Receptacle, Low RF Leakage | Fixed |
| 24 | Square Flange Male Receptacle, Low RF Leakage | Variable |
| 25 | Square Flange Male Receptacle | Fixed |
| 26 | Straight Plug, Crimp-Type | Fixed |
| 27 | Square Flange Male Receptacle (Note 4) | Variable |
| 28 | 2-Hole Flange Male Receptacle (Note 4) | Variable |
| 29 | Straight Plug, Solder-Type for SHF 5 Cable | Fixed |
| 30 | 2-Hole Flange Male Receptacle | Fixed |
| 32 | 2-Hole Flange Male Receptacle, with Hermetic Glass Seal Contact Ø0.3mm and EMI Gasket | Fixed |
| 33 | 2-Hole Flange Male Receptacle, with Hermetic Glass Seal Contact Ø0.46mm and EMI Gasket | Fixed |
| 34 | Bulkhead Receptacle, with Hermetic Glass Seal Contact Ø0.3mm | Fixed |
| 35 | Bulkhead Receptacle, with Hermetic Glass Seal Contact Ø0.46mm | Fixed |
| 36 | Hermetic Bulkhead Receptacle, with Compression Gasket | Fixed |
| 37 | Straight Plug, Solder-Type, for SHF 3 Cable | Fixed |
| 38 | Straight Plug, Solder-Type, for SHF 8 Cable | Fixed |
| 39 | Right Angle Plug, Solder-Type, for SHF 3 Cable | Fixed |
| 40 | Right Angle Plug, Solder-Type, for SHF 5 Cable | Fixed |
| 41 | Elbow Plug, Solder-Type, for SHF 3 Cable | Fixed |
| 42 | Elbow Plug, Solder-Type, for SHF 5 Cable | Fixed |
| 43 | Elbow Plug, Solder-Type, for SHF 8 Cable | Fixed |
| 44 | Right Angle Plug, Crimp-Type, for 50 CIS Cable (Note 4) | Fixed |
| 45 | Square Flange Male Receptacle, Maximum Operating Temperature = +165°C (Note 4) | Variable |
| 46 | 2-Hole Flange Male Receptacle, Maximum Operating Temperature = +165°C (Note 4) | Variable |
| 47 | Right Angle Plug, Crimp-Type, for 50 CIS Cable, Maximum Operating Temperature = +165°C (Note 4) | Fixed |

NOTES:

See Para. 3 for details.

For available connector materials and finishes, see Paras. 1.4.1.2(a) and 1.7.

See Paras. 1.4.1, 1.4.1.2(b), 1.4.1.2(c) and 3.

Variants 45, 46 and 47 are high temperature capability versions of Variants 27, 28 and 44 respectively

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbol | Maximum Ratings | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Peak Power at +25°C | Pmax | 20 | kW | For duration 1µs max. |
| Power | P | 2 | kW | Notes 1, 2 |
| Nominal Impedance | Z | 50 | Ω | - |
| Operating Frequency Range | f | See Para. 3 | GHz | Note 2 |
| Operating Voltage | Vop | 335 | Vrms | - |
| Operating Temperature Range | Top | See Para. 3 | °C | - |
| Storage Temperature Range | Tstg | As per Top | °C | - |
| Coupling Nut Torque | Tq | 170 | N.cm | Coupling Proof Torque Note 3 |

NOTES:

Derate Power with respect to Operating Temperature as follows:

Derate Power with respect to Operating Frequency as follows:

Whenever a test is performed on mated pairs of connectors, the pairs shall be torqued at 100 ±20N.cm.

## PHYSICAL DIMENSIONS (SEE ALSO PARA. 3)

### Connector Interface Dimensions and Connector Interface Gauge Dimensions

SMA Male Connector Interface: compatible with series SMA pin contact interface as specified in MIL-STD-348.

SMA Female Gauge Interface: compatible with series SMA socket contact test connector interface as specified in MIL-STD-348.

## MATERIALS AND FINISHES

Materials and finishes shall be as follows (as applicable, see Para. 3):

| Type | Description |
| --- | --- |
| 1 | Beryllium Copper, Gold plated, Copper underplate - Amagnetic Version (Note 1) (see Para. 3) |
| 2 | Beryllium Copper, Gold plated, Nickel underplate - Standard Version (Note 1) (see Para. 1.7.3) |
| 3 | Stainless Steel, Electro-passivated (see Para. 1.7.4) |
| 4 | Stainless Steel, Gold plated (see Para. 1.7.5) |

NOTES:

For information purposes, the residual magnetism of types 1 and 2 components, which is guaranteed but not tested, is as follows:

Type 1 ≤ 20 gamma.

Type 2 ≤ 2000 gamma.

### Type 1

Shell, Coupling Nut: beryllium copper, plated gold 2.5μm minimum over copper 2.5μm minimum.

Centre Contact: beryllium copper (or brass for male contact only), plated gold 2.5μm minimum over copper 2.5μm minimum.

Insulator: PTFE.

Gaskets:

Front: Silicone.

EMI: TBA.

Compression: TBA.

Crimping and soldering elements: brass or copper, plated gold 2.5μm minimum over copper 2.5μm minimum.

Rear Nut: brass or copper, plated gold 2.5μm minimum over copper 2.5μm minimum.

Washers: TBA.

Hermetic Glass Seal:

Shell: iron/nickel alloy, plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact: steel, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: glass.

### Type 2

Shell, Coupling Nut: beryllium copper, plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact: Beryllium copper (or brass for male contact only), plated gold 1.27μm minimum over nickel 2μm.

Insulator: PTFE.

Gaskets:

Front: Silicone.

EMI: TBA.

Compression: TBA.

Crimping and soldering elements: brass or copper, plated gold 0.5μm minimum over nickel 2μm.

Rear Nut: brass or copper, plated gold 0.5μm minimum over nickel 2μm.

Washers: TBA.

Hermetic Glass Seal:

Shell: iron/nickel alloy, plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact: steel, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: glass.

### Type 3

Shell, Coupling Nut: stainless steel, electro-passivated.

- NOTE: For solder-type connectors, the rear part of the shell shall be plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact: beryllium copper (or brass for male contact only), plated gold 1.27μm minimum over nickel 2μm.

Insulator: PTFE.

Gaskets:

Front: Silicone.

EMI: TBA.

Compression: TBA.

Crimping elements: brass or copper, plated nickel 2μm.

Soldering elements: brass or copper, plated gold 0.5μm minimum over nickel 2μm.

Rear Nut: stainless steel, electro-passivated.

Washers: beryllium copper, plated nickel 2μm.

Hermetic Glass Seal:

Shell: iron/nickel alloy, plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact: steel, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: glass

### Type 4

Shell, Coupling Nut: stainless steel, plated gold 1.27μm minimum over nickel 2μm.

Centre Contact: beryllium copper (or brass for male contact only), plated gold 1.27μm minimum over nickel 2μm.

Insulator: PTFE.

Gaskets:

Front: Silicone.

EMI: TBA.

Compression: TBA.

Crimping and soldering elements: brass or copper, plated gold 0.5μm minimum over nickel 2μm.

Rear Nut: stainless steel, plated gold 0.5μm minimum over nickel 2μm.

Washers: beryllium copper, plated gold 0.5μm minimum over nickel 2μm.

Hermetic Glass Seal:

Shell: iron/nickel alloy, plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact: steel, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: glass

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

None.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component or the primary package shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## ENVIRONMENTAL AND MECHANICAL TESTS

The following requirements apply to tests performed on the connector (and contact) lot as specified in the ESCC Generic Specification:

Para. 8.1, Contact Engagement and Separation Forces:

|  | - Oversize Test Pin Test | - Minimum Diameter Test Pin Test |
| --- | --- | --- |
| - Engagement Force (N) | - 138 maximum | - - |
| - Separation Force (N) | - - | - 2.9 minimum |
| - Oversize Test Pin Diameter (mm) (1) | - 0.9525 to 0.955 | - - |
| - Maximum Diameter Test Pin Diameter (mm) (1) | - 0.94 to 0.942 | - - |
| - Minimum Diameter Test Pin Diameter (mm) (1) | - - | - 0.902 to 0.904 |
| - Engagement Depth (mm) (1) | - 1.27 to 1.91 | - 1.27 to 1.91 |

- NOTES:

Test Pin details:

Para. 8.10, Coupling Proof Torque: See Para. 1.5.

Para. 8.11, Mating and Unmating Forces: Maximum torque: 24N.cm.

Para. 8.12, Centre Contact Retention: See Para. 3.

Para. 8.13, Seal: See Para. 3.

Para. 8.19, Endurance:

During Qualification Testing: 500 cycles.

During Periodic Testing: 100 cycles

## Room Temperature Electrical Measurements (NOTE 1)

The measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance | RI | ESCC No. 3402 | 5 | - | GΩ |
| Voltage Proof Leakage Current | IL | ESCC No. 3402 See Para. 3 Note 2 | - | 2 | mA |

NOTES:

For Variants 01, 02, 03, 04, 29, 37, 38, 39, 40, 41, 42, 43 measurements shall not be performed during Chart F3; see Para. 3.

Between centre contact and the body.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified, the test methods and test conditions shall be as per the corresponding test defined in Para. 2.4 Room Temperature Electrical Measurements.

| Test Reference per ESCC No. 3402 | Characteristics and Test Conditions Ref. ESCC No. 3402 | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Random Vibration | Contact Resistance: IT = 10mA, VT = 6V Centre contact: | Rctc | - | 3 | mΩ |
| Mechanical Shock | Contact Resistance: IT = 10mA, VT = 6V Centre contact: | Rctc |  | 3 | mΩ |
| Temperature Cycling | Contact Resistance: IT = 10mA, VT = 6V Centre contact: | Rctc |  | 3 | mΩ |
|  | Voltage Proof Leakage Current: | IL | Note 1 |  |  |
| Electrical Measurements at Room Temperature | Insulation Resistance: | RI | Note 1 |  |  |
|  | Voltage Proof Leakage Current: | IL | Note 1 |  |  |
|  | Contact Resistance: IT = 10mA, VT = 6mV Centre contact: Shell: Hermetic Glass Seal Centre Contact: | Rctc Rcts Rhgs | - - - | 3 2 10 | mΩ mΩ mΩ |
|  | VSWR: | VSWR | Note 2 |  |  |
|  | Insertion Loss: | LI | Note 2 |  |  |
| Endurance | Contact Resistance: IT = 10mA, VT = 6mV Centre contact: Shell: Hermetic Glass Seal Centre Contact: | Rctc Rcts Rhgs | - - - | 4 3 12 | mΩ mΩ mΩ |

NOTES:

As specified in Para. 2.4.

As specified in Para. 3.

# COMPONENT TYPE VARIANTS – DETAIL REQUIREMENTS

## variant 01 – Straight Plug, Solder-Type, for Semi-Rigid Cable Ø2.2mm

|  | Symbol; Dimensions mm. Min; Max. a; 8.4; 8.7. ØN; 2.25; 2.35. |
| --- | --- |

NOTES:

Removable coupling nut.

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.07 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.02 f (GHz) | dB |
| RF leakage (Note 3) | - [100 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 3) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 3) | 200 | N |
| Mini cable retention torque value (Note 3) | 11.5 | N.cm |
| Maximum weight | 2.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +200 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 1, RG 405/U, (Ø2.2mm) |  |

NOTES:

For information purposes only.

## variant 02 - STRAIGHT PLUG, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø3.58mm

|  | Symbol; Dimensions mm. Min; Max. a; 8.4; 8.7. ØN; 3.65; 3.75. |
| --- | --- |

NOTES:

Removable coupling nut.

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.004 f (GHz) |  |
| Maximum insertion loss | 0.02 f (GHz) | dB |
| RF leakage (Note 3) | - [100 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 3) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 3) | 500 | N |
| Mini cable retention torque value (Note 3) | 39.6 | N.cm |
| Maximum weight | 2.4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +200 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 2, RG 402/U, (Ø3.58mm) |  |

NOTES:

For information purposes only.

## variant 03 - STRAIGHT PLUG with cable clamp, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø6.35mm

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 21.5; 22.5. ØD; 10.9; 11. K1; -; 10; 2 flats. K2; -; 8; 2 flats. ØN; 6.45; 6.7. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.004 f (GHz) |  |
| Maximum insertion loss | 0.02 f (GHz) | dB |
| RF leakage (Note 2) | - [100 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | 500 | N |
| Mini cable retention torque value (Note 2) | 39.6 | N.cm |
| Maximum weight | 2.4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +200 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 2, RG 402/U, (Ø3.58mm) |  |

NOTES:

For information purposes only.

## variant 04 - STRAIGHT PLUG with cable clamp, SOLDER-TYPE, FOR SEMI-RIGID microporous CABLE, Ø6.35mm

|  | Symbol; Dimensions mm; Notes. Min; Max. a; -; 22.5. ØD; 10.9; 11.1. K1; -; 10; 2 flats. K2; -; 8; 2 flats. ØN; 6.45; 6.7. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.02 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | 300 | N |
| Mini cable retention torque value (Note 2) | 39.6 | N.cm |
| Maximum weight | 8.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +200 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | Microporous Ø6.35mm |  |

NOTES:

For information purposes only.

## variant 05 - STRAIGHT PLUG, Crimp-type

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 23.1; 23.95. K; -; 7; 2 flats. ØN; 3.15; 3.35. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.03 √f (GHz) | dB |
| RF leakage (Note 1) | - [95 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 1) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 0.5 |  |
| Mini cable retention force (Note 1) | 90 | N |
| Mini cable retention torque value (Note 1) | 2 x 180° @ point 50 x ØN | N.cm |
| Maximum weight | 5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +200 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 3B, KX 22A, RG 174/U, RG 316/U |  |

NOTES:

For information purposes only.

## variant 06 - STRAIGHT PLUG, Crimp-type, for 50 CIS Cable

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 23.1; 23.95. K; -; 7; 2 flats. ØN; 2; 2.2. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.03 √f (GHz) | dB |
| RF leakage (Note 1) | - [95 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 1) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 0.5 |  |
| Mini cable retention force (Note 1) | 80 | N |
| Mini cable retention torque value (Note 1) | 2 x 180° @ point 50 x ØN | N.cm |
| Maximum weight | 4.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +200 (see cables used) | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 50 CIS |  |

NOTES:

For information purposes only.

## variant 07 - STRAIGHT PLUG, Crimp- or SOLDER-TYPE, cable Ø5mm/50Ω, single braid

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 25.1; 25.95. K; -; 7; 2 flats. ØN; 5.55; 5.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.03 √f (GHz) | dB |
| RF leakage (Note 1) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 0.5 |  |
| Mini cable retention force (Note 1) | 180 | N |
| Mini cable retention torque value (Note 1) | 2 x 180° @ point 50 x ØN | N.cm |
| Maximum weight | 6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +200 (see cables used) | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 15, RG 58 C/U, RG 141 A/U |  |

NOTES:

For information purposes only.

## variant 08 - STRAIGHT PLUG, Crimp- or SOLDER-TYPE, CABLE Ø5mm/50Ω, DOUBLE BRAID

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 25.1; 25.95. K; -; 7; 2 flats. ØN; 5.55; 5.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.03 √f (GHz) | dB |
| RF leakage (Note 1) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 0.5 |  |
| Mini cable retention force (Note 1) | 180 | N |
| Mini cable retention torque value (Note 1) | 2 x 180° @ point 50 x ØN | N.cm |
| Maximum weight | 6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +200 (see cables used) | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 23, RG 142 B/U, RG 223/U |  |

NOTES:

For information purposes only.

## variant 09 - Right angle PLUG, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø2.2mm

|  | Symbol; Dimensions mm. Min; Max. J1; 10; 10.4. J2; 7.25; 7.7. ØN; 2.25; 2.35. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.02 f (GHz) | dB |
| RF leakage (Note 3) | - [95 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 3) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 3) | 200 | N |
| Mini cable retention torque value (Note 3) | 11.5 | N.cm |
| Maximum weight | 5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 1, RG 405/U (Ø2.2mm) |  |

NOTES:

For information purposes only.

## variant 10 - right angle PLUG, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø3.58mm

|  | Symbol; Dimensions mm. Min; Max. J1; 10; 10.4. J2; 7.25; 7.7. ØN; 3.65; 3.75. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.15 f (GHz) | dB |
| RF leakage (Note 3) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 3) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 3) | 500 | N |
| Mini cable retention torque value (Note 3) | 39.6 | N.cm |
| Maximum weight | 4.1 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 2, RG 402/U (Ø3.58mm) |  |

NOTES:

For information purposes only.

## variant 12 - RIGHT ANGLE PLUG, CRIMP-TYPE

|  | Symbol; Dimensions mm. Min; Max. J1; 10; 10.4. J2; -; 16.8. ØN; 3.25; 3.3. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.025 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| RF leakage (Note 3) | - [95 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 3) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 3) | 90 | N |
| Mini cable retention torque value (Note 3) | 2 x 180o @ point 50 x ØN | N.cm |
| Maximum weight | 4.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 (see cables used) | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 3B, KX 22A, RG 174/U, RG 316/U |  |

NOTES:

For information purposes only.

## variant 13 - RIGHT ANGLE PLUG, CRIMP-TYPE

|  | Symbol; Dimensions mm. Min; Max. J1; 10; 10.4. J2; -; 18. ØN; 5.55; 5.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.02 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| RF leakage (Note 3) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 3) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 3) | 180 | N |
| Mini cable retention torque value (Note 3) | 2 x 180o @ point 50 x ØN | N.cm |
| Maximum weight | 4.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 (see cables used) | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 15, RG 58C/U, RG 141A/U |  |

NOTES:

For information purposes only.

## variant 14 - RIGHT ANGLE PLUG, CRIMP-TYPE, FOR CABLE Ø5mm/50Ω, DOUBLE BRAID

|  | Symbol; Dimensions mm. Min; Max. J1; 10; 10.4. J2; -; 18. ØN; 5.55; 5.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.02 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| RF leakage (Note 3) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 3) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 3) | 180 | N |
| Mini cable retention torque value (Note 3) | 2 x 180o @ point 50 x ØN | N.cm |
| Maximum weight | 4.1 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 (see cables used) | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 23, RG 142 B/U, RG 223/U |  |

NOTES:

For information purposes only.

## variant 15 - SQUARE FLANGE MALE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. Ød; 1.24; 1.3. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. k; 2.4; -. L; 4.75; 5.25. m; 1.4; 1.8. ØN; 0.7; 1. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| RF leakage (Note 1) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 4.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 16 - 2-HOLE FLANGE MALE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. Ød; 1.24; 1.3. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. k; 2.4; -. L; 4.75; 5.25. m; 1.4; 1.8. ØN; 0.7; 1. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| RF leakage (Note 1) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 3.4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 17 - square flange male receptacle

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. e; 0.18; 0.41. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(d)).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.06 + 0.007 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | Not applicable | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 4.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 18 - male flange receptacle, triplate launcher

|  | Symbol; Dimensions mm. Min; Max. a; 9.9; 10.1. b1; 13.9; 14.1. b2; 5.6; 5.8. d; 0.1; 0.15. Ød; 1.25; 1.3. Øe; 0.4; 0.6. G; 9.4; 9.6. H; 9.4; 9.6. I; 2.7; 2.9. T; 0.9; 1.1. w; 1.55; 1.6. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.2 |  |
| Maximum insertion loss | Not applicable | dB |
| RF leakage (Note 1) | Not applicable | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 16 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 20 - MALE FLANGE RECEPTACLE, TRIPLATE LAUNCHER

|  | Symbol; Dimensions mm. Min; Max. a; 9.9; 10.1. b1; 13.9; 14.1. b2; 5.6; 5.8. d; 0.1; 0.15. Ød; 1.25; 1.3. G; 9.4; 9.6. H; 9.4; 9.6. I; 2.7; 2.9. T; 2.4; 2.6. w; 3.15; 3.2. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.2 |  |
| Maximum insertion loss | Not applicable | dB |
| RF leakage (Note 1) | Not applicable | dB |
| Voltage proof | 150 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 16 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 21 - MALE FLANGE RECEPTACLE, TRIPLATE LAUNCHER

|  | Symbol; Dimensions mm. Min; Max. a; 9.9; 10.1. b1; 13.9; 14.1. b2; 5.6; 5.8. d; 0.1; 0.15. Ød; 1.25; 1.3. G; 9.4; 9.6. H; 9.4; 9.6. I; 2.7; 2.9. T; 2.4; 2.6. w; 6.35; 6.4. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 2 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.2 |  |
| Maximum insertion loss | Not applicable | dB |
| RF leakage (Note 1) | Not applicable | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 16 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 22 - SQUARE FLANGE MALE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. e; 0.18; 0.41. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(d)).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.06 + 0.007 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| RF leakage (Note 2) | -120 at 10GHz | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | Not applicable | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 4.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 22 - SQUARE FLANGE MALE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. ØD; 5.9; 6.1. e; 0.18; 0.41. f; 0.3; 0.5. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(d)).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.06 + 0.007 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| RF leakage (Note 2) | -120 at 10GHz | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | Not applicable | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 4.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 24 - SQUARE FLANGE MALE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. A; -; 40.1; Note 1. B; -; 20; Note 1. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. ØN; 1.25; 1.3. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f (GHz) | dB |
| RF leakage (Note 2) | -120 at 10GHz | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 2) | Not applicable | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 5.1 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 25 - SQUARE FLANGE MALE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. Øc; 1.24; 1.3. d; 0.1; 0.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. T; 2.1; 2.55. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| RF leakage (Note 1) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 4.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 26 - STRAIGHT PLUG, CRIMP-TYPE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 22.9; 24.1. K; -; 7; 2 flats. ØN; 2.55; 2.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.03 √f (GHz) | dB |
| RF leakage (Note 1) | - [95 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 1) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 0.5 |  |
| Mini cable retention force (Note 1) | 80 | N |
| Mini cable retention torque value (Note 1) | 2 x 180° @ point 50 x ØN | N.cm |
| Maximum weight | 4.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +200 (see cables used) | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | RG 178/U, KX 21A |  |

NOTES:

For information purposes only.

## variant 27 - SQUARE FLANGE MALE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. A; -; 40.1; Note 1. B; -; 20; Note 1. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. ØN; 1.25; 1.3. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f (GHz) | dB |
| RF leakage (Note 2) | -120 at 10GHz | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 2) | Not applicable | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 4.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 28 - 2-HOLE FLANGE MALE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. A; -; 40.1; Note 1. B; -; 20; Note 1. E; 5.5; 5.8. Øf; 4; 4.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. ØN; 1.25; 1.3. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 2) | Not applicable | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 3.4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 29 - STRAIght PLUg, SOLDER-TYPE FOR SHF 5 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 20.8; 21.15. K; -; 8.5; 2 flats. K1; -; 8; 2 flats. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | 10 | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 5.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +200 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 1703-145 |  |

NOTES:

For information purposes only.

## variant 30 - 2-HOLE FLANGE MALE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. e; 0.18; 0.41. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. a; 9.45; 9.55. |
| --- | --- |

NOTES:

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(d)).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.06 + 0.007 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | Not applicable | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 4.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 32 - 2-Hole Flange Male Receptacle, with Hermetic Glass Seal Contact Ø0.3mm and EMI Gasket

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.4; 9.55. Øb; 0.25; 0.35. ØD; 2.47; 2.57. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. L; 7.8; 8.2. L1; 4.45; 4.7. m; 1.4; 1.8. Y; 1.55; 1.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.3 | dB |
| RF leakage (Note 1) | -70 | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 3.7 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 (hermetic glass seal only) | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 33 - 2-Hole Flange Male Receptacle, with Hermetic Glass Seal Contact Ø0.46mm and EMI Gasket

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.4; 9.55. Øb; 0.41; 0.51. ØD; 2.47; 2.57. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. L; 7.8; 8.2. L1; 4.45; 4.7. m; 1.4; 1.8. Y; 1.55; 1.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.3 | dB |
| RF leakage (Note 1) | -70 | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 3.7 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 (hermetic glass seal only) | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 34 - BULKHEAD RECEPTACLE, with Hermetic Glass Seal Contact Ø0.3mm

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 12.95; 13.95. c; 4.3; 4.5. Øb; 0.25; 0.35. ØD; 2.47; 2.57. K; -; 7; 2 flats. L; 7.8; 8.2. L1; 4.45; 4.7. Y; 1.55; 1.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.3 | dB |
| RF leakage (Note 1) | -70 | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 4.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 (hermetic glass seal only) | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 35 - BULKHEAD RECEPTACLE, with Hermetic Glass Seal Contact Ø0.46mm

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 12.95; 13.95. c; 4.3; 4.5. Øb; 0.41; 0.51. ØD; 2.8; 2.9. K; -; 7; 2 flats. L; 7.8; 8.2. L1; 4.45; 4.7. Y; 1.55; 1.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.3 | dB |
| RF leakage (Note 1) | -70 | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 4.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 (hermetic glass seal only) | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 36 - HERMETIC BULKHEAD RECEPTACLE, with COMPRESSION GASKET

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 14.65; 15.65. Øb1; 0.45; 0.55. Øb2; 1.62; 1.72. c; 5.95; 6.25. ØD; 5.3; 5.4. K; -; 7; 2 flats. L; 1.3; 1.5. L1; 3.1; 3.35. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.3 | dB |
| RF leakage (Note 1) | -70 | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 1) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 1) | Not applicable | N |
| Mini cable retention torque value (Note 1) | Not applicable | N.cm |
| Maximum weight | 4.4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 37 - STRAIght PLUg, SOLDER-TYPE FOR SHF 3 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 18.7; 19.1. K; -; 6.5; 2 flats. K1; -; 5; 2 flats. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 17 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 2) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | 30 | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 3.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +165 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 1703-143 |  |

NOTES:

For information purposes only.

## variant 38 - STRAIght PLUg, SOLDER-TYPE FOR SHF 8 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 34.8; 35.4. K; -; 13; 2 flats. K1; -; 12; 2 flats. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | 50 | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 24 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +165 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 1703-148 |  |

NOTES:

For information purposes only.

## variant 39 - RIGHT ANGLE PLUG, SOLDER-TYPE, FOR SHF 3 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 8.65; 10. J1; 23.3; 23.65. K; -; 5; 2 flats. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 17 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.3 from 0 to 14.5 GHz 1.4 from 14.5 to 17 GHz |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 2) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | 30 | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 7.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +165 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 1703-143 |  |

NOTES:

For information purposes only.

## variant 40 - RIGHT ANGLE PLUG, SOLDER-TYPE, FOR SHF 5 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 8.65; 10. J1; 25.25; 25.7. K; -; 8; 2 flats. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.3 from 0 to 14.5 GHz 1.4 from 14.5 to 17 GHz |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | 40 | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 10.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +165 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 1703-145 |  |

NOTES:

For information purposes only.

## variant 41 - ELBOW PLUG, SOLDER-TYPE, FOR SHF 3 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 21.2; 23.5. J1; 19.75; 22. K; -; 6.5; 2 flats. K1; -; 5; 2 flats. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 17 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 2) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | 30 | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 4.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +165 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 1703-143 |  |

NOTES:

For information purposes only.

## variant 42 - ELBOW PLUG, SOLDER-TYPE, FOR SHF 5 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 21.8; 27.1. J1; 23.45; 25.75. K; -; 8.5; 2 flats. K1; -; 8; 2 flats. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | 40 | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 7.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +165 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 1703-145 |  |

NOTES:

For information purposes only.

## variant 43 - ELBOW PLUG, SOLDER-TYPE, FOR SHF 8 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 38.35; 40.6. J1; 35; 37. K; -; 13; 2 flats. K1; -; 12; 2 flats. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | 250 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | Not applicable |  |
| Mini centre contact retention torque | Not applicable |  |
| Mini cable retention force (Note 2) | 50 | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 30.4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +165 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 1703-148 |  |

NOTES:

For information purposes only.

## variant 44 - RIGHT ANGLE PLUG, CRIMP-TYPE, FOR 50 CIS CABLE

|  | Symbol; Dimensions mm. Min; Max. J1; 10; 10.4. J2; -; 16.8. ØN; 2; 2.2. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.02 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| RF leakage (Note 3) | - [95 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 3) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 3) | 80 | N |
| Mini cable retention torque value (Note 3) | 2 x 180o @ point 50 x ØN | N.cm |
| Maximum weight | 4.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +115 (see cables used) | °C |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 50 CIS |  |

NOTES:

For information purposes only.

## variant 45 - SQUARE FLANGE MALE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. A; -; 40.1; Note 1. B; -; 20; Note 1. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. ØN; 1.25; 1.3. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f (GHz) | dB |
| RF leakage (Note 2) | -120 at 10GHz | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 2) | Not applicable | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 5.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +180 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 46 - 2-HOLE FLANGE MALE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.45; 9.55. A; -; 40.1; Note 1. B; -; 20; Note 1. E; 5.5; 5.8. Øf; 4; 4.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. ØN; 1.25; 1.3. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f (GHz) | dB |
| RF leakage (Note 2) | - [95 - f (GHz)] | dB |
| Voltage proof | 1000 | Vrms |
| Corona level (Note 2) | Not applicable | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 2) | Not applicable | N |
| Mini cable retention torque value (Note 2) | Not applicable | N.cm |
| Maximum weight | 4.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +180 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## variant 47 - RIGHT ANGLE PLUG, CRIMP-TYPE, FOR 50 CIS CABLE

|  | Symbol; Dimensions mm. Min; Max. J1; 10; 10.4. J2; -; 16.8. ØN; 2; 2.2. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.02 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| RF leakage (Note 3) | - [95 - f (GHz)] | dB |
| Voltage proof | 750 | Vrms |
| Corona level (Note 3) | 190 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Mini centre contact retention force (axial) | 27 |  |
| Mini centre contact retention torque | 2.8 |  |
| Mini cable retention force (Note 3) | 80 | N |
| Mini cable retention torque value (Note 3) | 2 x 180o @ point 50 x ØN | N.cm |
| Maximum weight | 4.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Rapid change of temperature - peak value | +185 | °C |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 50 CIS |  |

NOTES:

For information purposes only.