Page 1 of 103

RF 	COAXIAL CONNECTORS WITH FEMALE CONTACT

BASED ON TYPE SMA

ESCC Detail Specification No. 3402/002

| Issue 9 Draft A | July 2021 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2021. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| TBA | Specification updated to incorporate changes per DCR. |
| Draft 9A S.Thacker ESCC-TW 05/07/2021 | Initial draft converted Detail spec to make it consistent with the newly published, converted Generic spec: ESCC No. 3402 issue 5. Changes in this specification from the published issue 8 are mainly editorial in nature so have not been highlighted. Any actual technical changes, including those resulting from the new Generic spec, are highlighted yellow for information. |

TABLE OF CONTENTS

1	GENERAL	8

1.1	SCOPE	8

1.2	APPLICABLE DOCUMENTS	8

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	8

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	8

1.4.1	The ESCC Component Number	8

1.4.2	Component Type Variants	9

1.5	MAXIMUM RATINGS	12

1.6	PHYSICAL DIMENSIONS (SEE ALSO PARA. 3)	14

1.6.1	Connector Interface Dimensions and Connector Interface Gauge Dimensions	14

1.7	MATERIALS AND FINISHES	14

1.7.1	Type 1	14

1.7.2	Type 2	14

1.7.3	Type 3	15

1.7.4	Type 4	15

2	REQUIREMENTS	15

2.1	GENERAL	15

2.1.1	Deviations from the Generic Specification	15

2.2	MARKING	16

2.3	ENVIRONMENTAL AND MECHANICAL TESTS	16

2.4	ROOM TEMPERATURE ELECTRICAL MEASUREMENTS (NOTE 1)	17

2.5	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	17

3	COMPONENT TYPE VARIANTS – DETAIL REQUIREMENTS	18

3.1	VARIANT 01 – STRAIGHT JACK, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø2.2MM	18

3.2	VARIANT 02 - STRAIGHT JACK, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø3.58MM	19

3.3	VARIANT 03 - STRAIGHT JACK WITH CABLE CLAMP, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø6.35MM	20

3.4	VARIANT 04 - STRAIGHT JACK WITH CABLE CLAMP, SOLDER-TYPE, FOR SEMI-RIGID MICROPOROUS CABLE Ø6.35MM	21

3.5	VARIANT 05 - STRAIGHT JACK, CRIMP-TYPE, SQUARE FLANGE	22

3.6	VARIANT 06 - STRAIGHT JACK, CRIMP-TYPE	23

3.7	VARIANT 07 - STRAIGHT JACK, CRIMP-TYPE, FOR CABLE Ø5MM, SINGLE BRAID	24

3.8	VARIANT 08 - STRAIGHT JACK, CRIMP-TYPE, FOR CABLE Ø5MM, DOUBLE BRAID	25

3.9	VARIANT 09 - STRAIGHT JACK, SOLDER-TYPE, BACK MOUNTING, 2-HOLE, FLANGE-MOUNTED, FOR SEMI-RIGID CABLE Ø2.2MM	26

3.10	VARIANT 10 - STRAIGHT JACK, SOLDER-TYPE, BACK MOUNTING, 2-HOLE, FLANGE-MOUNTED, FOR SEMI-RIGID CABLE Ø3.58MM	27

3.11	VARIANT 11 - STRAIGHT JACK, CRIMP-TYPE, SQUARE FLANGE	28

3.12	VARIANT 12 - STRAIGHT JACK, CRIMP-TYPE, SQUARE FLANGE, FOR 50 CIS CABLE	29

3.13	VARIANT 13 - STRAIGHT JACK, CRIMP- OR SOLDER-TYPE, SQUARE FLANGE	30

3.14	VARIANT 14 - STRAIGHT JACK, CRIMP- OR SOLDER-TYPE, SQUARE FLANGE, DOUBLE BRAID CABLE	31

3.15	VARIANT 15 - SQUARE FLANGE RECEPTACLE, FRONT MOUNTING	32

3.16	VARIANT 16 – 2-HOLE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING	33

3.17	VARIANT 17 - 2-HOLE FLANGE RECEPTACLE, FRONT MOUNTING	34

3.18	VARIANT 18 - 2-HOLE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING	35

3.19	VARIANT 19 - SQUARE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING, OFF-SET TAB	36

3.20	VARIANT 20 - 2-HOLE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING	37

3.21	VARIANT 21 - SQUARE FLANGE RECEPTACLE FOR STRIP-LINE, FRONT MOUNTING	38

3.22	VARIANT 22 - SQUARE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING	39

3.23	VARIANT 23 - BULKHEAD RECEPTACLE	40

3.24	VARIANT 24 - BULKHEAD RECEPTACLE WITH COMPRESSION GASKET	41

3.25	VARIANT 25 - HERMETIC BULKHEAD RECEPTACLE	42

3.26	VARIANT 26 - HERMETIC RECEPTACLE, SOLDER-TYPE	43

3.27	VARIANT 27 - ELBOW RECEPTACLE, SQUARE FLANGE	44

3.28	VARIANT 28 - ELBOW RECEPTACLE, SQUARE FLANGE	45

3.29	VARIANT 29 - SQUARE FLANGE RECEPTACLE	46

3.30	VARIANT 30 - 2-HOLE FLANGE RECEPTACLE	47

3.31	VARIANT 31 - SQUARE FLANGE RECEPTACLE	48

3.32	VARIANT 32 - 2-HOLE FLANGE RECEPTACLE	49

3.33	VARIANT 34 - FLANGE RECEPTACLE, TRIPLATE LAUNCHER	50

3.34	VARIANT 36 - SQUARE FLANGE RECEPTACLE	51

3.35	VARIANT 37 - FLANGE RECEPTACLE, TRIPLATE LAUNCHER	52

3.36	VARIANT 38 - FLANGE RECEPTACLE, TRIPLATE LAUNCHER	53

3.37	VARIANT 39 - SQUARE FLANGE RECEPTACLE	54

3.38	VARIANT 40 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE	55

3.39	VARIANT 41 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE	56

3.40	VARIANT 42 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE	57

3.41	VARIANT 43 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE	58

3.42	VARIANT 44 - BULKHEAD RECEPTACLE	59

3.43	VARIANT 45 - SQUARE FLANGE RECEPTACLE FOR MICRO-STRIP	60

3.44	VARIANT 46 - 2-HOLE FLANGE RECEPTACLE FOR STRIP-LINE	61

3.45	VARIANT 47 - 2-HOLE FLANGE RECEPTACLE FOR STRIP-LINE	62

3.46	VARIANT 48 - SQUARE FLANGE MALE RECEPTACLE FOR MICRO-STRIP	63

3.47	VARIANT 49 - 2-HOLE FLANGE RECEPTACLE	64

3.48	VARIANT 50 - SQUARE FLANGE RECEPTACLE	65

3.49	VARIANT 51 - SQUARE FLANGE RECEPTACLE	66

3.50	VARIANT 53 - STRAIGHT JACK, SOLDER TYPE, FOR SHF 5 CABLE	67

3.51	VARIANT 54 - 2-HOLE FLANGE RECEPTACLE, LOW RF LEAKAGE	68

3.52	VARIANT 55 - ELBOW RECEPTACLE, ROUND FLANGE, TRIPLATE LAUNCHER	69

3.53	VARIANT 56 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE	70

3.54	VARIANT 57 - ROUND FLANGE RECEPTACLE, TRIPLATE LAUNCHER	71

3.55	VARIANT 58 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE	72

3.56	VARIANT 59 - 2-HOLE FLANGE MALE RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.46MM AND EMI GASKET	73

3.57	VARIANT 60 - BULKHEAD RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.3MM	74

3.58	VARIANT 61 - BULKHEAD RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.46MM	75

3.59	VARIANT 62 - HERMETIC BULKHEAD RECEPTACLE, WITH COMPRESSION GASKET	76

3.60	VARIANT 63 - HERMETIC BULKHEAD RECEPTACLE, WITH COMPRESSION GASKET	77

3.61	VARIANT 64 - HERMETIC BULKHEAD RECEPTACLE, WITH COMPRESSION GASKET	78

3.62	VARIANT 65 - 2-HOLE FLANGE MALE RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.3MM AND EMI GASKET	79

3.63	VARIANT 66 - BULKHEAD JACK, SOLDER-TYPE, FOR SHF 3 CABLE	80

3.64	VARIANT 67 - BULKHEAD JACK, SOLDER-TYPE, FOR SHF 8 CABLE	81

3.65	VARIANT 68 - STRAIGHT JACK, SOLDER-TYPE, BACK MOUNTING, FLANGE-MOUNTED, FOR SEMI-RIGID CABLE Ø2.2MM	82

3.66	VARIANT 69 - STRAIGHT JACK, SOLDER-TYPE, BACK MOUNTING, FLANGE-MOUNTED, FOR SEMI-RIGID CABLE Ø3.58MM	83

3.67	VARIANT 70 - STRAIGHT JACK, CRIMP-TYPE, FOR 50 CIS CABLE	84

3.68	VARIANT 71 - ELBOW RECEPTACLE, SQUARE FLANGE (SOLID CONTACT)	85

3.69	VARIANT 72 - SQUARE FLANGE RECEPTACLE TAB CONTACT, LOW RF LEAKAGE	86

3.70	VARIANT 73 - SQUARE FLANGE RECEPTACLE, FRONT MOUNTING	87

3.71	VARIANT 74 - 2-HOLE FLANGE RECEPTACLE, FRONT MOUNTING	88

3.72	VARIANT 75 - 2-HOLE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING	89

3.73	VARIANT 76 - SQUARE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING	90

3.74	VARIANT 77 - SQUARE FLANGE RECEPTACLE	91

3.75	VARIANT 78 - 2-HOLE FLANGE RECEPTACLE	92

3.76	VARIANT 79 - SQUARE FLANGE RECEPTACLE	93

3.77	VARIANT 80 - 2-HOLE FLANGE RECEPTACLE	94

3.78	VARIANT 81 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE	95

3.79	VARIANT 82 - 2-HOLE FLANGE RECEPTACLE, LOW RF LEAKAGE	96

3.80	VARIANT 83 - BULKHEAD RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.3MM	97

3.81	VARIANT 84 - BULKHEAD RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.46MM	98

3.82	VARIANT 85 - ELBOW RECEPTACLE, SQUARE FLANGE (SOLID CONTACT)	99

3.83	VARIANT 86 - SQUARE FLANGE RECEPTACLE, LOW VSWR, EXTENDED FREQUENCY RANGE	100

3.84	VARIANT 87 - SQUARE FLANGE RECEPTACLE, LOW VSWR, EXTENDED FREQUENCY RANGE	101

3.85	VARIANT 88 - SQUARE FLANGE RECEPTACLE, LOW VSWR, EXTENDED FREQUENCY RANGE	102

3.86	VARIANT 89 - SQUARE FLANGE RECEPTACLE, LOW VSWR, EXTENDED FREQUENCY RANGE	103

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3402.

MIL-STD-348, Department of Defence Interface Standard: Radio Frequency Connector Interfaces.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

For components with a fixed configuration (see Paras. 1.4.2 and 3):

- Example: 3402002012

Detail Specification Reference: 3402002

Component Type Variant Number: 01 (as required)

Characteristic code: Material/Plating (Type 2): 2 (as required)

For components with a variable configuration where the rear contact and insert dimensions A and B shall be selected by the Orderer (see Paras. 1.4.2 and 3):

- Example: 340200215213D00W02D50C

Detail Specification Reference: 3402002

Component Type Variant Number: 15 (as required)

Characteristic code: Material/Plating (Type 2): 2 (as required)

Characteristic code: Dimension A: Rear contact length (13mm): 13D00 (as required)

Characteristic code: Dimension A tolerance (±0.05mm): W (as required)

Characteristic code: Dimension B: Rear insert length (2.5mm): 02D50 (as required)

Characteristic code: Dimension B tolerance (±0.25mm): C (as required)

#### Characteristics Codes

Characteristics to be codified as part of the ESCC Component Number (as applicable) shall be as follows:

Material/Plating: Connector material and plating (see Para. 1.7) expressed by means of the following codes:

| Material/Plating | Code |
| --- | --- |
| Beryllium Copper, Gold plated, Copper underplate (Type 1) | 1 |
| Beryllium Copper, Gold plated, Nickel underplate (Type 2) | 2 |
| Amagnetic Stainless Steel, Electro-passivated (Type 3) | 3 |
| Amagnetic Stainless Steel, Gold plated (Type 4) | 4 |

Dimension A and B: Rear contact and insert lengths (see Para. 3) expressed by the following codes. The unit quantity shall be mm:

| Dimension A or B Length (mm) | Code |
| --- | --- |
| XX.XX | XXDXX |

NOTES:

Dimension A shall always be greater than B. Both values shall always be positive.

When dimension B (insert) is flush with the flange (B = 0), the insert length shall be marked 00D00 with the appropriate tolerance.

Dimension A and B tolerance: Tolerances on contact and insert lengths (see Para. 3) expressed by the following codes. The unit quantity shall be mm:

| Dimension A or B Tolerance (mm) | Code |
| --- | --- |
| ±0.05 | W |
| ±0.1 | B |
| ±0.25 | C |

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Description (Notes 1, 2) | Configuration Type (Note 3) |
| --- | --- | --- |
| 01 | Straight Jack, Solder-Type, for Semi-Rigid Cable Ø2.2mm | Fixed |
| 02 | Straight Jack, Solder-Type, for Semi-Rigid Cable Ø3.58mm | Fixed |
| 03 | Straight Jack with Cable Clamp, Solder-Type, for Semi-Rigid Cable Ø6.35mm | Fixed |
| 04 | Straight Jack with Cable Clamp, Solder-Type, for Semi-Rigid Microporous Cable Ø6.35mm | Fixed |
| 05 | Straight Jack, Crimp-Type, Square Flange | Fixed |
| 06 | Straight Jack, Crimp-Type | Fixed |
| 07 | Straight Jack, Crimp-Type, Cable Ø5mm, Single Braid | Fixed |
| 08 | Straight Jack, Crimp-Type, Cable Ø5mm, Double Braid | Fixed |
| 09 | Straight Jack, Solder-Type, Back Mounting, 2-Hole, Flange-Mounted, for Semi-Rigid Cable Ø2.2mm | Fixed |
| 10 | Straight Jack, Solder-Type, Back Mounting, 2-Hole, Flange-Mounted, for Semi-Rigid Cable Ø3.58mm | Fixed |
| 11 | Straight Jack, Crimp-Type, Square Flange | Fixed |
| 12 | Straight Jack, Crimp-Type, Square Flange, for 50 CIS Cable | Fixed |
| 13 | Straight Jack, Crimp- or Solder-Type, Square Flange | Fixed |
| 14 | Straight Jack, Crimp- or Solder-Type, Square Flange, Double Braid Cable | Fixed |
| 15 | Square Flange Receptacle, Front Mounting (Note 4) | Variable |
| 16 | 2-Hole Flange Receptacle for Micro-Strip, Front Mounting | Fixed |
| 17 | 2-Hole Flange Receptacle, Front Mounting (Note 4) | Variable |
| 18 | 2-Hole Flange Receptacle for Micro-Strip, Front Mounting (Note 4) | Fixed |
| 19 | Square Flange Receptacle for Micro-Strip, Front Mounting, Off-Set Tab | Fixed |
| 20 | 2-Hole Flange Receptacle for Strip-Line, Front Mounting | Fixed |
| 21 | Square Flange Receptacle for Strip-Line, Front Mounting | Fixed |
| 22 | Square Flange Receptacle for Micro-Strip, Front Mounting (Note 4) | Fixed |
| 23 | Bulkhead Receptacle | Fixed |
| 24 | Bulkhead Receptacle, with Compression Gasket | Fixed |
| 25 | Hermetic Bulkhead Receptacle | Fixed |
| 26 | Hermetic Receptacle, Solder-Type | Fixed |
| 27 | Elbow Receptacle, Square Flange | Fixed |
| 28 | Elbow Receptacle, Square Flange | Variable |
| 29 | Square Flange Receptacle (Note 4) | Variable |
| 30 | 2-Hole Flange Receptacle (Note 4) | Variable |
| 31 | Square Flange Receptacle (Note 4) | Fixed |
| 32 | 2-Hole Flange Receptacle (Note 4) | Fixed |
| 34 | Flange Receptacle, Triplate Launcher | Fixed |
| 36 | Square Flange Receptacle | Fixed |
| 37 | Flange Receptacle, Triplate Launcher | Fixed |
| 38 | Flange Receptacle, Triplate Launcher | Fixed |
| 39 | Square Flange Receptacle | Fixed |
| 40 | Square Flange Receptacle, Low RF Leakage (Note 4) | Variable |
| 41 | Square Flange Receptacle, Low RF Leakage | Fixed |
| 42 | Square Flange Receptacle, Low RF Leakage | Fixed |
| 43 | Square Flange Receptacle, Low RF Leakage | Fixed |
| 44 | Bulkhead Receptacle | Variable |
| 45 | Square Flange Receptacle for Micro-Strip | Variable |
| 46 | 2-Hole Flange Receptacle for Strip-Line | Fixed |
| 47 | 2-Hole Flange Receptacle for Strip-Line | Fixed |
| 48 | Square Flange Male Receptacle for Micro-Strip | Fixed |
| 49 | 2-Hole Flange Receptacle | Fixed |
| 50 | Square Flange Receptacle | Fixed |
| 51 | Square Flange Receptacle | Variable |
| 53 | Straight Jack, Solder-Type, for SHF 5 Cable | Fixed |
| 54 | 2-Hole Flange Receptacle, Low RF Leakage (Note 4) | Variable |
| 55 | Elbow Receptacle, Round Flange, Triplate Launcher | Fixed |
| 56 | Square Flange Receptacle, Low RF Leakage | Variable |
| 57 | Round Flange Receptacle, Triplate Launcher | Fixed |
| 58 | Square Flange Receptacle, Low RF Leakage | Variable |
| 59 | 2-Hole Flange Male Receptacle, with Hermetic Glass Seal Ø0.46mm and EMI Gasket | Fixed |
| 60 | Bulkhead Receptacle, with Hermetic Glass Seal Ø0.3mm (Note 4) | Fixed |
| 61 | Bulkhead Receptacle, with Hermetic Glass Seal Ø0.46mm (Note 4) | Fixed |
| 62 | Hermetic Bulkhead Receptacle, with Compression Gasket | Fixed |
| 63 | Hermetic Bulkhead Receptacle, with Compression Gasket | Fixed |
| 64 | Hermetic Bulkhead Receptacle, with Compression Gasket | Fixed |
| 65 | 2-Hole Flange Male Receptacle, with Hermetic Glass Seal Ø0.3mm and EMI Gasket | Fixed |
| 66 | Bulkhead Jack, Solder-Type, for SHF 3 Cable | Fixed |
| 67 | Bulkhead Jack, Solder-Type, for SHF 8 Cable | Fixed |
| 68 | Straight Jack, Solder-Type, Back Mounting, Flange-Mounted, For Semi-Rigid Cable Ø2.2mm | Fixed |
| 69 | Straight Jack, Solder-Type, Back Mounting, Flange-Mounted, For Semi-Rigid Cable Ø3.58mm | Fixed |
| 70 | Straight Jack, Crimp-Type, for 50 CIS Cable | Fixed |
| 71 | Elbow Receptacle, Square Flange (Solid Contact) (Note 4) | Variable |
| 72 | Square Flange Receptacle, Tab Contact, Low RF Leakage | Fixed |
| 73 | Square Flange Receptacle, Front Mounting, Maximum Operating Temperature = +165ºC (Note 4) | Variable |
| 74 | 2-Hole Flange receptacle, Front Mounting, Maximum Operating Temperature = +165ºC (Note 4) | Variable |
| 75 | 2-Hole Flange Receptacle for Micro-Strip, Front Mounting, Maximum Operating Temperature = +165ºC (Note 4) | Fixed |
| 76 | Square Flange Receptacle for Micro-Strip, Front Mounting, Maximum Operating Temperature = +165ºC (Note 4) | Fixed |
| 77 | Square Flange Receptacle, Maximum Operating Temperature = +165ºC (Note 4) | Variable |
| 78 | 2-Hole Flange Receptacle, Maximum Operating Temperature = +165ºC (Note 4) | Variable |
| 79 | Square Flange Receptacle, Maximum Operating Temperature = +165ºC (Note 4) | Fixed |
| 80 | 2-Hole Flange Receptacle, Maximum Operating Temperature = +165ºC (Note 4) | Fixed |
| 81 | Square Flange Receptacle, Low RF Leakage, Maximum Operating Temperature = +165ºC (Note 4) | Variable |
| 82 | 2-Hole Flange Receptacle, Low RF Leakage, Maximum Operating Temperature = +165ºC (Note 4) | Variable |
| 83 | Bulkhead Receptacle, with Hermetic Glass Seal Ø0.3mm, Maximum Operating Temperature = +165ºC (Note 4) | Fixed |
| 84 | Bulkhead Receptacle, with Hermetic Glass Seal Ø0.46mm, Maximum Operating Temperature = +165ºC (Note 4) | Fixed |
| 85 | Elbow Receptacle, Square Flange (Solid Contact), Maximum Operating Temperature = +165ºC (Note 4) | Variable |
| 86 | Square Flange Receptacle, Low VSWR, Extended Frequency Range | Fixed |
| 87 | Square Flange Receptacle, Low VSWR, Extended Frequency Range | Fixed |
| 88 | Square Flange Receptacle, Low VSWR, Extended Frequency Range | Fixed |
| 89 | Square Flange Receptacle, Low VSWR, Extended Frequency Range | Fixed |

NOTES:

See Para. 3 for details.

For available connector materials and finishes, see Paras. 1.4.1.1(a) and 1.7.

See Paras. 1.4.1, 1.4.1.1(b), 1.4.1.1(c) and 3.

Variants 73 to 85 inclusive are high temperature capability versions of Variants 15, 17, 18, 22, 29, 30, 31, 32, 40, 54, 60, 61, 71 respectively.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbol | Maximum Ratings | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Peak Power at +25°C | Pmax | 20 | kW | For duration 1µs max. |
| Power | P | 2 | kW | Notes 1, 2 |
| Nominal Impedance | Z | 50 | Ω | - |
| Operating Frequency Range | f | See Para. 3 | GHz | Note 2 |
| Operating Voltage | Vop | 335 | Vrms | - |
| Operating Temperature Range | Top | See Para. 3 | °C | - |
| Storage Temperature Range | Tstg | As per Top | °C | - |
| Coupling Nut Torque | Tq | 170 | N.cm | Coupling Proof Torque Note 3 |

NOTES:

Derate Power with respect to Operating Temperature as follows:

Derate Power with respect to Operating Frequency as follows:

Whenever a test is performed on mated pairs of connectors, the pairs shall be torqued at 100 ±20N.cm.

## PHYSICAL DIMENSIONS (SEE ALSO PARA. 3)

### Connector Interface Dimensions and Connector Interface Gauge Dimensions

SMA Female Connector Interface: compatible with series SMA socket contact interface as specified in MIL-STD-348.

SMA Male Gauge Interface: compatible with series SMA pin contact test connector interface as specified in MIL-STD-348.

## MATERIALS AND FINISHES

Materials and finishes shall be as follows (as applicable, see Para. 3):

| Type | Description |
| --- | --- |
| 1 | Beryllium Copper, Gold plated, Copper underplate (see Para. 1.7.1) |
| 2 | Beryllium Copper, Gold plated, Nickel underplate (see Para. 1.7.2) |
| 3 | Amagnetic Stainless Steel, Electro-passivated (see Para. 1.7.3) |
| 4 | Amagnetic Stainless Steel, Gold plated (see Para. 1.7.4) |

### Type 1

Shell: beryllium copper, plated gold 2.5μm minimum over copper 2.5μm minimum.

Centre Contact: beryllium copper, plated gold 2.5μm minimum over copper 2.5μm minimum.

Insulator: PTFE.

Gaskets:

EMI: TBA.

Compression: TBA.

Crimping and soldering elements: brass or copper, plated gold 2.5μm minimum over copper 2.5μm minimum.

Rear Nut: brass or copper, plated gold 2.5μm minimum over copper 2.5μm minimum.

Washers: copper alloy, plated gold 0.5μm minimum over nickel 2μm minimum.

Hermetic Glass Seal:

Shell, Centre Contact: iron-nickel alloy, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: glass.

### Type 2

Shell: beryllium copper, plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact: beryllium copper, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: PTFE.

Gaskets:

EMI: TBA.

Compression: TBA.

Crimping and soldering elements: brass or copper, plated gold 0.5μm minimum over nickel 2μm minimum.

Rear Nut: brass or copper, plated gold 0.5μm minimum over nickel 2μm minimum.

Washers: copper alloy, plated gold 0.5μm minimum over nickel 2μm minimum.

Hermetic Glass Seal:

Shell, Centre Contact: iron-nickel alloy, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: glass.

### Type 3

Shell: Amagnetic stainless steel, electro-passivated.

- NOTE: For solder-type connectors, the rear part of the shell shall be plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact: beryllium copper, plated gold 1.27μm minimum over nickel 2μm.

Insulator: PTFE.

Gaskets:

EMI: TBA.

Compression: TBA.

Crimping elements: brass or copper, plated nickel 2μm minimum.

Soldering elements: brass or copper, plated gold 0.5μm minimum over nickel 2μm minimum.

Rear Nut: Amagnetic stainless steel, electro-passivated.

Washers: beryllium copper, plated nickel 2μm minimum.

Hermetic Glass Seal:

Shell, Centre Contact: iron-nickel alloy, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: glass

### Type 4

Shell: Amagnetic stainless steel, plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact: beryllium copper, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: PTFE.

Gaskets:

EMI: TBA.

Compression: TBA.

Crimping and soldering elements: brass or copper, plated gold 0.5μm minimum over nickel 2μm minimum.

Rear Nut: Amagnetic stainless steel, plated gold 0.5μm minimum over nickel 2μm minimum.

Washers: copper alloy, plated gold 0.5μm minimum over nickel 2μm minimum.

Hermetic Glass Seal:

Shell, Centre Contact: iron-nickel alloy, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: glass

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests – Chart F3

Coupling Proof Torque: shall not be performed.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component or the primary package shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## ENVIRONMENTAL AND MECHANICAL TESTS

The following requirements apply to tests performed on the connector (and contact) lot as specified in the ESCC Generic Specification:

Para. 8.1, Contact Engagement and Separation Forces:

|  | - Oversize Test Pin Test | - Maximum Diameter Test Pin Test | - Minimum Diameter Test Pin Test |
| --- | --- | --- | --- |
| - Test Pin Diameter (mm) (1) | - 0.9525 to 0.955 | - 0.94 to 0.942 | - 0.902 to 0.904 |
| - Engagement Depth (mm) (1) | - 0.76 to 1.14 | - 1.27 to 1.91 | - 1.27 to 1.91 |
| - Engagement Force (N) | - - | - 13.34 maximum | - - |
| - Separation Force (N) | - - | - - | - 0.28 minimum |

- NOTES:

Test Pins details:

Para. 8.10, Coupling Proof Torque: See Para. 1.5.

Para. 8.11, Mating and Unmating Forces: Maximum torque: 24N.cm.

Para. 8.12, Centre Contact Retention: See Para. 3.

Para. 8.13, Seal: See Para. 3.

Para. 8.19, Endurance:

During Qualification Testing: 500 cycles.

During Periodic Testing: 100 cycles

## ROOM TEMPERATURE ELECTRICAL MEASUREMENTS (NOTE 1)

The measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance | RI | ESCC No. 3402 | 5 | - | GΩ |
| Voltage Proof Leakage Current | IL | ESCC No. 3402 See Para. 3 Note 2 | - | 2 | mA |

NOTES:

For Variants 01, 02, 03, 04, 09, 10, 20, 46, 47, 48, 53, 55, 57, 66, 67 measurements shall not be performed during Chart F3; see Para. 3.

Between centre contact and shell.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified, the test methods and test conditions shall be as per the corresponding test defined in Para. 2.4 Room Temperature Electrical Measurements.

| Test Reference per ESCC No. 3402 | Characteristics and Test Conditions Ref. ESCC No. 3402 | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Random Vibration | Contact Resistance: IT = 10mA, VT = 6V Centre Contact: | Rctc | - | 3 | mΩ |
| Mechanical Shock | Contact Resistance: IT = 10mA, VT = 6V Centre Contact: | Rctc | - | 3 | mΩ |
| Temperature Cycling | Contact Resistance: IT = 10mA, VT = 6V Centre Contact: | Rctc | - | 3 | mΩ |
|  | Voltage Proof Leakage Current: | IL | Note 1 |  |  |
| Electrical Measurements at Room Temperature | Insulation Resistance: | RI | Note 1 |  |  |
|  | Voltage Proof Leakage Current: | IL | Note 1 |  |  |
|  | Contact Resistance: IT = 10mA, VT = 6V Centre Contact: Shell: Hermetic Glass Seal (Centre Contact): | Rctc Rcts Rhgs | - - - | 3 2 10 | mΩ mΩ mΩ |
|  | VSWR: | VSWR | Note 2 |  |  |
|  | Insertion Loss: | LI | Note 2 |  |  |
| Endurance | Contact Resistance: IT = 10mA, VT = 6V Centre Contact: Shell: Hermetic Glass Seal (Centre Contact): | Rctc Rcts Rhgs | - - - | 4 3 12 | mΩ mΩ mΩ |

NOTES:

As specified in Para. 2.4.

As specified in Para. 3.

# COMPONENT TYPE VARIANTS – DETAIL REQUIREMENTS

## VARIANT 01 – STRAIGHT JACK, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø2.2MM

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 10.5; 11. ØD; 6.6; 6.8. K; -; 6; 2 flats. ØN; 2.25; 2.35. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.02f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 1.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 1, RG 405/U, (Ø2.2mm) |  |

## VARIANT 02 - STRAIGHT JACK, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø3.58MM

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 10.5; 11. ØD; 6.6; 6.8. K; -; 6; 2 flats. ØN; 3.65; 3.75. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.004 f (GHz) |  |
| Maximum insertion loss | 0.02 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 1.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 2, RG 402/U, (Ø3.58mm) |  |

## VARIANT 03 - STRAIGHT JACK WITH CABLE CLAMP, SOLDER-TYPE, FOR SEMI-RIGID CABLE Ø6.35MM

|  | Symbol; Dimensions mm; Notes. Min; Max. A; -; 8; 2 flats. ØB; 10.9; 11. C; -; 10; 2 flats. ØD; 6.45; 6.7. E; -; 22.5. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.02 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 7.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | KS 3, RG 401/U, (Ø6.35mm) |  |

## VARIANT 04 - STRAIGHT JACK WITH CABLE CLAMP, SOLDER-TYPE, FOR SEMI-RIGID MICROPOROUS CABLE Ø6.35MM

|  | Symbol; Dimensions mm; Notes. Min; Max. A; -; 8; 2 flats. ØB; 10.9; 11. C; -; 10; 2 flats. ØD; 6.45; 6.7. E; -; 22.5. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.02 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 7.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | Microporous Ø6.35mm |  |

## VARIANT 05 - STRAIGHT JACK, CRIMP-TYPE, SQUARE FLANGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.4; 7.75. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. J; -; 17.3. K; -; 7; 2 flats. m; 1.4; 1.8. ØN; 2.55; 2.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.03 √f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 0.5 | N.cm |
| Maximum weight | 4.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 21A, RG 178/U |  |

## VARIANT 06 - STRAIGHT JACK, CRIMP-TYPE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 22.7; 24.15. ØD; 7.5; 7.9. K; -; 7; 2 flats. ØN; 3.15; 3.35. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.03 √f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 0.5 | N.cm |
| Maximum weight | 4.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 3B, KX 22A, RG 174/U, RG 316/U |  |

## VARIANT 07 - STRAIGHT JACK, CRIMP-TYPE, FOR CABLE Ø5MM, SINGLE BRAID

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 24.9; 26.15. ØD; 7.5; 7.9. K; -; 7; 2 flats. ØN; 5.55; 5.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 0.5 | N.cm |
| Maximum weight | 5.1 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 15, RG 58 C/U, RG 141 A/U |  |

## VARIANT 08 - STRAIGHT JACK, CRIMP-TYPE, FOR CABLE Ø5MM, DOUBLE BRAID

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 24.9; 26.15. ØD; 7.5; 7.9. K; -; 7; 2 flats. ØN; 5.55; 5.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 0.5 | N.cm |
| Maximum weight | 5.1 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 23, RG 142 B/U, RG 223/U |  |

## VARIANT 09 - STRAIGHT JACK, SOLDER-TYPE, BACK MOUNTING, 2-HOLE, FLANGE-MOUNTED, FOR SEMI-RIGID CABLE Ø2.2MM

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 5.87; 6.12. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. J; 4.6; 5. m; 1.4; 1.8. ØN; 2.25; 2.35. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

Maximum panel thickness: 2.3mm.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.02 f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 1, RG 405/U (Ø2.2mm) |  |

## VARIANT 10 - STRAIGHT JACK, SOLDER-TYPE, BACK MOUNTING, 2-HOLE, FLANGE-MOUNTED, FOR SEMI-RIGID CABLE Ø3.58MM

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 5.87; 6.12. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. J; 4.6; 5. m; 1.4; 1.8. ØN; 3.65; 3.75. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

Maximum panel thickness: 2.3mm

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.04 f (GHz) |  |
| Maximum insertion loss | 0.02 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 2, RG 402/U (Ø3.58mm) |  |

## VARIANT 11 - STRAIGHT JACK, CRIMP-TYPE, SQUARE FLANGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.4; 7.75. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. J; 15.1; 17.6. K; -; 7; 2 flats. m; 1.4; 1.8. ØN; 3.25; 3.35. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 0.5 | N.cm |
| Maximum weight | 5.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 3B, KX 22A, RG 174/U, RG 316/U |  |

## VARIANT 12 - STRAIGHT JACK, CRIMP-TYPE, SQUARE FLANGE, FOR 50 CIS CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.4; 7.75. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. J; 15.1; 17.6. K; -; 7; 2 flats. m; 1.4; 1.8. ØN; 2; 2.2. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 0.5 | N.cm |
| Maximum weight | 4.4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 50 CIS |  |

## VARIANT 13 - STRAIGHT JACK, CRIMP- OR SOLDER-TYPE, SQUARE FLANGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.4; 7.75. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. J; 17.1; 20.4. K; -; 7; 2 flats. m; 1.4; 1.8. ØN; 5.55; 5.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.012 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 0.5 | N.cm |
| Maximum weight | 5.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX15, RG 58C/U, RG 141A/U |  |

## VARIANT 14 - STRAIGHT JACK, CRIMP- OR SOLDER-TYPE, SQUARE FLANGE, DOUBLE BRAID CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.4; 7.75. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. J; 17.1; 20.4. K; -; 7; 2 flats. m; 1.4; 1.8. ØN; 5.55; 5.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.012 f (GHz) |  |
| Maximum insertion loss | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 0.5 | N.cm |
| Maximum weight | 5.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KX 23, RG 142B/U, RG 223/U |  |

## VARIANT 15 - SQUARE FLANGE RECEPTACLE, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; 35; Note 1. B; -; 20; Note 1. Ød; 1.24; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. k; 2.4; -. m; 1.4; 1.8. ØN; 0.7; 1. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 16 – 2-HOLE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.6. A; 3.1; 3.3. B; 0.15; 0.35. d; 0.2; 0.3. Ød; 0.7; 0.9. d1; 0.4; 0.5. Øf; 1.75; 1.8. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. T; 0.70; 0.95. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.05 + 0.005 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 1.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 17 - 2-HOLE FLANGE RECEPTACLE, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 35; Note 1. B; -; 20; Note 1. Ød; 1.24; 1.3. E; 5.5; 5.8. Øf; 4; 4.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. k; 2.4; -. m; 1.4; 1.8. ØN; 0.7; 1. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 18 - 2-HOLE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.6. A; 4.5; 5. B; 3.05; 3.3. Ød; 0.2; 0.3. E; 5.5; 5.8. Øf; 2.1; 2.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.04 + 0.018 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 19 - SQUARE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING, OFF-SET TAB

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; 3.1; 3.3. B; 0.15; 0.35. d; 0.2; 0.3. Ød; 0.7; 0.9. d1; 0.4; 0.5. Øf; 1.75; 1.8. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. T; 0.7; 0.95. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.05 + 0.05 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 2.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 20 - 2-HOLE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. d; 0.1; 0.2; Tab thickness. Ød; 1.24; 1.3. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. T; 2.35; 2.7. |
| --- | --- |

NOTES:

Insert may stand proud of, or be recessed from, the flange by 0.05mm.

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 3) | 1.05 + 0.005 f (GHz) |  |
| Maximum insertion loss (Note 3) | 0.03f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 21 - SQUARE FLANGE RECEPTACLE FOR STRIP-LINE, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. d; 0.1; 0.2; Tab thickness. Ød; 1.24; 1.3. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. T; 2.35; 2.7. |
| --- | --- |

NOTES:

Insert may stand proud of, or be recessed from, the flange by 0.05mm.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.005 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 22 - SQUARE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.6. A; 4.6; 4.9. B; 3.05; 3.3. Ød; 0.2; 0.3. Øf; 2.1; 2.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.04 + 0.018 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 23 - BULKHEAD RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 9.4; 9.85. A; 5.4; 5.7. B; 2.6; 3.15. Ød; 1.24; 1.3. ØD; -; 9.1. Øf; 4; 4.2. Øg; 0.75; -. k; 2.4; -. K1; -; 6; 2 flats. K2; -; 8. |
| --- | --- |

NOTES:

Maximum panel thickness: 3.4mm.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.06 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 2.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 24 - BULKHEAD RECEPTACLE WITH COMPRESSION GASKET

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 10.9; 11.3. A; 6.1; 6.4. B; 2.8; 3. Ød; 1.24; 1.3. ØD; -; 12.5. Øf; 4; 4.2. Øg; 0.8; -. k; 2.4; -. K1; -; 6; 2 flats. K2; -; 9. K3; -; 11. |
| --- | --- |

NOTES:

Maximum panel thickness: 3.4mm.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.06 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage (panel-sealed) | Applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 25 - HERMETIC BULKHEAD RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 10.5; 10.85. A; 2.95; 4. b; 12.9; 13.25. ØD; -; 13.5. D1; 1.9; 2.1. Øg; 1; -. K1; -; 6; 1 flat. K2; -; 9; Hex.. K3; -; 12; Hex.. |
| --- | --- |

NOTES:

Maximum panel thickness: 4mm.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | Not applicable |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 4.4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -40 to +100 | °C |
| Seal: Maximum leakage | 10-8 | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

## VARIANT 26 - HERMETIC RECEPTACLE, SOLDER-TYPE

|  | Symbol; Dimensions mm. Min; Max. a; 8.15; 8.45. A; 2.8; 3.2. Ød; 0.45; 0.55. ØD; 5.2; 5.4. Øg; 4; 4.2. Y; 6.25; 6.3. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.05 + 0.015 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 4.4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | 10-8 | atm.cm3/s |
| Solderability | Applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 27 - ELBOW RECEPTACLE, SQUARE FLANGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a1; 7.1; 9.4. a2; 9.2; 9.4. A; 4.25; 4.75. B; 1.4; 1.7. Ød; 1.25; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. k; 2.3; -. m; 1.4; 1.8. ØN; 0.75; -. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.05 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 5.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 28 - ELBOW RECEPTACLE, SQUARE FLANGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a1; 7.9; 8.1. a2; 9.2; 9.4. A; -; 15; Note 1. B; -; 10; Note 1. Ød; 1.25; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. k; 2.3; -. m; 1.4; 1.8. ØN; 0.75; -. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 5.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 29 - SQUARE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 40.1; Note 1. B; -; 30; Note 1. Ød; 1.25; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f(GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 4.1 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 30 - 2-HOLE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.6. A; -; 40.1; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.3. E; 5.5; 5.8. Øf; 4; 4.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 31 - SQUARE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. e; 0.18; 0.41. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.007 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 32 - 2-HOLE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.6. e; 0.18; 0.41. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.006 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 2.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 34 - FLANGE RECEPTACLE, TRIPLATE LAUNCHER

|  | Symbol; Dimensions mm. Min; Max. a; 7.8; 8.1. b1; 13.8; 14.1. b2; 5.60; 5.85. d; 0.1; 0.15. Ød; 1.25; 1.3. G; 7.45; -. H; 9.35; 9.85. I; 2.6; 2.85. T; 1.8; 2.6. w; -; 3.17. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.2 |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 15 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

## VARIANT 36 - SQUARE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. B; 3.1; 3.3. d; 0.1; 0.35. Ød; 1.25; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. T; 2.1; 2.3. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 37 - FLANGE RECEPTACLE, TRIPLATE LAUNCHER

|  | Symbol; Dimensions mm. Min; Max. a; 7.8; 8.1. b1; 13.9; 14.1. b2; 5.6; 5.85. d; 0.1; 0.15. Ød; 1.25; 1.3. Øe; 0.4; 0.6. G; 7.45; -. H; 9.35; 9.85. I; 2.6; 2.9. T; 1.8; 2.6. w; 1.5; 1.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 8 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.2 |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 150 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 15 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

## VARIANT 38 - FLANGE RECEPTACLE, TRIPLATE LAUNCHER

|  | Symbol; Dimensions mm. Min; Max. a; 7.8; 8.1. b1; 13.9; 14.1. b2; 5.6; 5.85. d; 0.1; 0.15. Ød; -; 1.3. G; 7.45; -. H; 9.35; 9.85. I; 2.7; 2.9. T; 2.4; 2.6. w; 6.3; 6.4. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 2 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.2 |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 15 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

## VARIANT 39 - SQUARE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; 0.7; 0.9. b; 0.2; 0.4. d; 0.59; 0.65. Ød; 1.25; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. T; 1.2; 1.4. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 40 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 35; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 41 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; 6; 6.2. B; 0.9; 1. Ød; 1.25; 1.3. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. k; 2.4; -. m; 1.4; 1.8. Øn; 4; 4.2. ØN; 0.75; -. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 42 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. e; 0.18; 0.41. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.06 + 0.007 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 43 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. e; 0.18; 0.41. f; 0.3; 0.5. Øf; 5.9; 6.1. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 +0.007 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 44 - BULKHEAD RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 10.65; 10.9. A; -; 40.1; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.3. Øf; 4; 4.2. K1; -; 8; 2 flats. K2; -; 5.4; 2 flats. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.06 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 2.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 45 - SQUARE FLANGE RECEPTACLE FOR MICRO-STRIP

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 35; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.3. d; 0.1; 0.15. d1; 0.55; 0.65. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.035 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 46 - 2-HOLE FLANGE RECEPTACLE FOR STRIP-LINE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; 2.4; 2.7. d; 0.1; 0.15. Ød; 1.25; 1.3. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. T; 2.4; 2.65. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | Not applicable |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

## VARIANT 47 - 2-HOLE FLANGE RECEPTACLE FOR STRIP-LINE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; 6.2; 6.7. Ød; 1.25; 1.3. B; 0.4; 0.65. E; 5.5; 5.8. Øf; 4; 4.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. s; 0.6; -. T; 2.4; 2.65. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | Not applicable |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

## VARIANT 48 - SQUARE FLANGE MALE RECEPTACLE FOR MICRO-STRIP

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. k; 2.3; 2.7. m; 1.4; 1.8. ØN; 0.7; 0.9. |
| --- | --- |

NOTES:

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | Not applicable |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

## VARIANT 49 - 2-HOLE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. B; 0.15; 0.25. e; 0.18; 0.41. E; 5.5; 5.8. Øf; 4; 4.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.06 + 0.006 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 50 - SQUARE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.70. e; 0.18; 0.41. f; 0.3; 0.5. Øf; 5.9; 6.1. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.06 + 0.007 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 51 - SQUARE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 40.1; Note 1. B; -; 30; Note 1. Ød; 1.25; 1.3. ØD; 5.9; 6.1. f; 0.3; 0.5. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 53 - STRAIGHT JACK, SOLDER TYPE, FOR SHF 5 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 14.2; 14.45. ØD; 10.9; 11.1. K1; -; 8; Hexagon. K2; -; 5.9; 1 flat. ØN; 5.9; -. |
| --- | --- |

NOTES:

Maximum panel thickness: 5.4mm

Components are delivered with the centre contact and insulator not mounted in the connector.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 |  |
| Maximum insertion loss | 0.06 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 7 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | F 1703.145 |  |

## VARIANT 54 - 2-HOLE FLANGE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 40.1; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.3. E; 5.5; 5.8. Øf; 4; 4.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 55 - ELBOW RECEPTACLE, ROUND FLANGE, TRIPLATE LAUNCHER

|  | Symbol; Dimensions mm. Min; Max. a1; 9.15; 9.4. a2; 9.2; 9.4. A; 0.94; 1.7. Ød; 1.7; 1.9. ØD; 14; 14.4. m; 1.95; 2.3. w; 0.15; 0.35. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.2 |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 5.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | Not applicable |  |

## VARIANT 56 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 40.1; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.5. ØD; 5.9; 6.1. f; 0.3; 0.4. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.9. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 57 - ROUND FLANGE RECEPTACLE, TRIPLATE LAUNCHER

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; 3; 3.4; After assembly. Ød; 1.7; 1.9. ØD; 10.1; 10.5. m; 2.1; 2.25. W; 0.2; 0.3. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 2 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.2 |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2.6 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | Not applicable |  |

## VARIANT 58 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.45; 7.7. A; -; 40.1; Note 1. B; -; 20; Note 1. Ød; 0.4; 0.6. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 59 - 2-HOLE FLANGE MALE RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.46MM AND EMI GASKET

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.45; 7.7. Ød; -; 0.47. ØD; -; 2.86. E; 5.41; 5.91. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. L; 7.8; 8.2. L1; 4.45; 4.7. m; 1.4; 1.8. Y; 1.55; 1.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.3 | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2.1 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 (hermetic glass seal only) | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Accept contact Ø0.46mm.

## VARIANT 60 - BULKHEAD RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.3MM

|  | Symbol; Dimensions mm. Min; Max. a; 7.05; 8.05. Ød; -; 0.31. ØD; -; 2.53. L; 7.8; 8.2. L1; 4.45; 4.7. Y; 1.55; 1.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.3 | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 1.7 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 (hermetic glass seal only) | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Accept contact Ø0.3mm.

## VARIANT 61 - BULKHEAD RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.46MM

|  | Symbol; Dimensions mm. Min; Max. a; 7.05; 8.05. Ød; -; 0.47. ØD; -; 2.86. L; 7.8; 8.2. L1; 4.45; 4.7. Y; 1.55; 1.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.3 | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 1.7 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 (hermetic glass seal only) | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Accept contact Ø0.46mm.

## VARIANT 62 - HERMETIC BULKHEAD RECEPTACLE, WITH COMPRESSION GASKET

|  | Symbol; Dimensions mm. Min; Max. a; 9.05; 10.05. Ød; -; 0.51. ØD; 5.3; 5.4. L; -; 1.67. L1; 0.62; 1.25. L4; 1.72; 2.25. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.3 | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 63 - HERMETIC BULKHEAD RECEPTACLE, WITH COMPRESSION GASKET

|  | Symbol; Dimensions mm. Min; Max. a; 9.05; 10.05. Ød; 0.45; 0.55. Ød2; 0.65; 0.95. Ød3; 1.3; 1.5. ØD; 5.3; 5.4. L; -; 4.25. L1; 0.7; -. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.3 | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 64 - HERMETIC BULKHEAD RECEPTACLE, WITH COMPRESSION GASKET

|  | Symbol; Dimensions mm. Min; Max. a; 9.05; 10.05. d; 0.1; 0.2. Ød; 0.45; 0.55. Ød2; 0.7; 0.8. Ød3; 1.3; 1.5. ØD; 5.3; 5.4. L; -; 4.25. L1; 0.7; -. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.3 | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-8 | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

## VARIANT 65 - 2-HOLE FLANGE MALE RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.3MM AND EMI GASKET

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.45; 7.65. Ød; -; 0.31. ØD; -; 2.53. E; 5.41; 5.91. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. L; 7.8; 8.2. L1; 4.45; 4.7. m; 1.4; 1.8. Y; 1.55; 1.65. |
| --- | --- |

NOTES:

Accept contact Ø0.3mm.

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.3 | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2.1 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | 10-7 (hermetic glass seal only) | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

## VARIANT 66 - BULKHEAD JACK, SOLDER-TYPE, FOR SHF 3 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 10.4; 10.8. b; 25.05; 27.1. ØD; 11.8; 12.2. K1; -; 8; Hexagon. K2; -; 5.9; 1 flat. K3; -; 11; 2 flats. |
| --- | --- |

NOTES:

Maximum panel thickness: 2.3mm

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 17 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 |  |
| Maximum insertion loss | 0.06 √ f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 7 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | F1703.143 |  |

## VARIANT 67 - BULKHEAD JACK, SOLDER-TYPE, FOR SHF 8 CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 11.8; 12.2. b; 40.85; 42.4. K1; -; 8; Hexagon. K2; -; 5.9; 1 flat. K3; -; 13; 2 flats. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.15 |  |
| Maximum insertion loss | 0.06 √ f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 25.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | F1703.148 |  |

## VARIANT 68 - STRAIGHT JACK, SOLDER-TYPE, BACK MOUNTING, FLANGE-MOUNTED, FOR SEMI-RIGID CABLE Ø2.2MM

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 5.87; 6.12. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. J; 4.6; 5. m; 1.4; 1.8. ØN; 2.25; 2.35. |
| --- | --- |

NOTES:

Maximum panel thickness: 2.3mm

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.02 √ f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 1, RG 405/U (Ø2.2mm) |  |

## VARIANT 69 - STRAIGHT JACK, SOLDER-TYPE, BACK MOUNTING, FLANGE-MOUNTED, FOR SEMI-RIGID CABLE Ø3.58MM

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 5.87; 6.12. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. J; 4.6; 5. m; 1.4; 1.8. ØN; 3.65; 3.75. |
| --- | --- |

NOTES:

Maximum panel thickness: 2.3mm

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.04 f (GHz) |  |
| Maximum insertion loss | 0.02 √ f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | KS 2, RG 402/U (Ø3.58mm) |  |

## VARIANT 70 - STRAIGHT JACK, CRIMP-TYPE, FOR 50 CIS CABLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 22.7; 24.15. ØD; 7.5; 7.9. K; -; 7; 2 flats. ØN; 2; 2.2. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.03 √ f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 0.5 | N.cm |
| Maximum weight | 4.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | 50 CIS |  |

## VARIANT 71 - ELBOW RECEPTACLE, SQUARE FLANGE (SOLID CONTACT)

|  | Symbol; Dimensions mm; Notes. Min; Max. a1; 7.9; 8.1. a2; 9.2; 9.4. A; -; 25; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 4.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 72 - SQUARE FLANGE RECEPTACLE TAB CONTACT, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; 5.7; 5.9. B; 3.13; 3.32. d; 0.11; 0.15; Tab Thickness. Ød; 0.45; 0.55. Øf; 6.05; 6.15. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.1 + 0.001 f (GHz) |  |
| Maximum voltage standing wave ratio (VSWR) (Notes 1, 2) | 1.05 + 0.005 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.05 √ f (GHz) | dB |
| Maximum insertion loss (Notes 1, 2) | 0.03 √ f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Between 0 and 5 GHz only.

## VARIANT 73 - SQUARE FLANGE RECEPTACLE, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; 35; Note 1. B; -; 20; Note 1. Ød; 1.24; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. k; 2.4; -. m; 1.4; 1.8. ØN; 0.7; 1. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 74 - 2-HOLE FLANGE RECEPTACLE, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 35; Note 1. B; -; 20; Note 1. Ød; 1.24; 1.3. E; 5.5; 5.8. Øf; 4; 4.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. k; 2.4; -. m; 1.4; 1.8. ØN; 0.7; 1. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 2.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 75 - 2-HOLE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.6. A; 4.5; 5. B; 3.05; 3.3. Ød; 0.2; 0.3. E; 5.5; 5.8. Øf; 2.1; 2.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.04 + 0.018 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 2.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 76 - SQUARE FLANGE RECEPTACLE FOR MICRO-STRIP, FRONT MOUNTING

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.6. A; 4.6; 4.9. B; 3.05; 3.3. Ød; 0.2; 0.3. Øf; 2.1; 2.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.04 + 0.018 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 77 - SQUARE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 40.1; Note 1. B; -; 30; Note 1. Ød; 1.25; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 √f(GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +155 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 78 - 2-HOLE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.6. A; -; 40.1; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.3. E; 5.5; 5.8. Øf; 4; 4.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 79 - SQUARE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. e; 0.18; 0.41. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.007 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 80 - 2-HOLE FLANGE RECEPTACLE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.6. e; 0.18; 0.41. E; 5.5; 5.8. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.06 + 0.006 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 2.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 81 - SQUARE FLANGE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 35; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 82 - 2-HOLE FLANGE RECEPTACLE, LOW RF LEAKAGE

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 7.5; 7.7. A; -; 40.1; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.3. E; 5.5; 5.8. Øf; 4; 4.2. Øg; 2.55; 2.7; 2 holes. i; 15.9; 16.1. j; 12.1; 12.3. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.003 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 3.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 83 - BULKHEAD RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.3MM

|  | Symbol; Dimensions mm. Min; Max. a; 7.05; 8.05. Ød; -; 0.31. ØD; -; 2.53. L; 7.8; 8.2. L1; 4.45; 4.7. Y; 1.55; 1.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √ f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 1.7 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | 10-8 (hermetic glass seal only) | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Accept contact Ø0.3mm.

## VARIANT 84 - BULKHEAD RECEPTACLE, WITH HERMETIC GLASS SEAL Ø0.46MM

|  | Symbol; Dimensions mm. Min; Max. a; 7.05; 8.05. Ød; -; 0.47. ØD; -; 2.86. L; 7.8; 8.2. L1; 4.45; 4.7. Y; 1.55; 1.65. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 1) | 1.06 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 √ f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 1.7 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | 10-8 (hermetic glass seal only) | atm.cm3/s |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Accept contact Ø0.46mm.

## VARIANT 85 - ELBOW RECEPTACLE, SQUARE FLANGE (SOLID CONTACT)

|  | Symbol; Dimensions mm; Notes. Min; Max. a1; 7.9; 8.1. a2; 9.2; 9.4. A; -; 25; Note 1. B; -; 20; Note 1. Ød; 1.25; 1.3. Øf; 4; 4.2. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

NOTES:

Contact and Insert dimensions A and B shall be selected by the Orderer; see Para. 1.4.1(b).

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) (Note 2) | 1.05 + 0.01 f (GHz) |  |
| Maximum insertion loss (Note 2) | 0.03 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 5.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | On centre contact only |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

## VARIANT 86 - SQUARE FLANGE RECEPTACLE, LOW VSWR, EXTENDED FREQUENCY RANGE

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 22 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.0027 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 + 0.02 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | < 0.2 | N.cm |
| Maximum weight | 2.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 87 - SQUARE FLANGE RECEPTACLE, LOW VSWR, EXTENDED FREQUENCY RANGE

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 22 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.0027 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 + 0.02 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | < 0.2 | N.cm |
| Maximum weight | 1.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 88 - SQUARE FLANGE RECEPTACLE, LOW VSWR, EXTENDED FREQUENCY RANGE

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 22 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.0027 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 + 0.02 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | < 0.2 | N.cm |
| Maximum weight | 1.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).

## VARIANT 89 - SQUARE FLANGE RECEPTACLE, LOW VSWR, EXTENDED FREQUENCY RANGE

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 22 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.0027 f (GHz) |  |
| Maximum insertion loss (Note 1) | 0.03 + 0.02 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | < 0.2 | N.cm |
| Maximum weight | 1.8 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

NOTES:

For information purposes only.

Contact Engagement and Separation Forces shall be measured on the rear contact (see Para. 2.3(a)).