Page 1 of 26

RF 	COAXIAL ADAPTORS AND CONNECTING PIECES

BASED ON TYPE SMA

ESCC Detail Specification No. 3402/003

| Issue 5 Draft A | June 2021 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2021. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| TBA | Specification updated to incorporate changes per DCR. |
| Draft 5A S.Thacker ESCC-TW 21/06/2021 | Initial draft converted Detail spec to make it consistent with the newly published, converted Generic spec: ESCC No. 3402 issue 5. Changes in this specification from the published issue 4 are mainly editorial in nature so have not been highlighted. Any actual technical changes, including those resulting from the new Generic spec, are highlighted yellow for information. |

TABLE OF CONTENTS

1	GENERAL	6

1.1	SCOPE	6

1.2	APPLICABLE DOCUMENTS	6

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	6

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	6

1.4.1	The ESCC Component Number	6

1.4.2	Component Type Variants	7

1.5	MAXIMUM RATINGS	7

1.6	PHYSICAL DIMENSIONS (SEE ALSO PARA. 3)	9

1.6.1	Connector Interface Dimensions and Connector Interface Gauge Dimensions	9

1.7	MATERIALS AND FINISHES	9

1.7.2	Type 1	9

1.7.3	Type 2	9

1.7.4	Type 3	10

1.7.5	Type 4	10

2	REQUIREMENTS	10

2.1	GENERAL	10

2.1.1	Deviations from the Generic Specification	10

2.2	MARKING	10

2.3	ENVIRONMENTAL AND MECHANICAL TESTS	11

2.4	Room Temperature Electrical Measurements	11

2.5	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	12

3	COMPONENT TYPE VARIANTS – DETAIL REQUIREMENTS	13

3.1	variant 01 – STRAIGHT Adaptor, Male - male	13

3.2	variant 02 – STRAIGHT adaptor, male - Female	14

3.3	variant 03 – STRAIGHT Adaptor, Female - Female	15

3.4	variant 04 – STRAIGHT Bulkhead adaptor, female - female	16

3.5	variant 05 – Elbow adaptor, male - female	17

3.6	variant 06 – T-adaptor, female – female/male	18

3.7	variant 07 – hermetic adaptor, female - female	19

3.8	variant 08 – T-adaptor, female – female/female	20

3.9	variant 09 – Straight flange adaptor, male - female	21

3.10	variant 10 – straight flange adaptor, female – female	22

3.11	variant 11 – STRAIGHT adaptor, male - male	23

3.12	variant 12 – STRAIGHT adaptor, male - Female	24

3.13	variant 03 – STRAIGHT Adaptor, Female - Female	25

3.14	variant 14 – STRAIGHT Bulkhead adaptor, female - female	26

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3402.

MIL-STD-348, Department of Defence Interface Standard: Radio Frequency Connector Interfaces.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

- Example: 3402003012

Detail Specification Reference: 3402003

Component Type Variant Number: 01 (as required)

Characteristic code: Material/Plating (Type 2): 2 (as required)

#### Characteristics Codes

Characteristics to be codified as part of the ESCC Component Number (as applicable) shall be as follows:

Material/Plating: Connector material and plating (see Para. 1.7) expressed by means of the following codes:

| Material/Plating | Code |
| --- | --- |
| Beryllium Copper, Gold plated, Copper underplate (Type 1) | 1 |
| Beryllium Copper, Gold plated, Nickel underplate (Type 2) | 2 |
| Amagnetic Stainless Steel, Electro-passivated (Type 3) | 3 |
| Amagnetic Stainless Steel, Gold plated (Type 4) | 4 |

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Description (Notes 1, 2) |
| --- | --- |
| 01 | Straight Adaptor, Male - Male (Note 3) |
| 02 | Straight Adaptor, Male - Female (Note 3) |
| 03 | Straight Adaptor, Female - Female (Note 3) |
| 04 | Straight Bulkhead Adaptor, Female - Female (Note 3) |
| 05 | Elbow Adaptor, Male - Female |
| 06 | T-Adaptor, Female - Female/Male |
| 07 | Hermetic Adaptor, Female - Female |
| 08 | T-Adaptor, Female - Female/Female |
| 09 | Straight Flange Adaptor, Male - Female |
| 10 | Straight Flange Adaptor, Female - Female |
| 11 | Straight Adaptor, Male – Male (Maximum Operating Temperature = +165°C) (Note 3) |
| 12 | Straight Adaptor, Male – Female (Maximum Operating Temperature = +165°C) (Note 3) |
| 13 | Straight Adaptor, Female – Female (Maximum Operating Temperature = +165°C) (Note 3) |
| 14 | Straight Bulkhead Adaptor Female – Female (Maximum Operating Temperature = +165°C) (Note 3) |

NOTES:

See Para. 3 for details.

For available connector materials and finishes, see Paras. 1.4.1.1(a) and 1.7.

Variants 11 to 14 are high temperature capability versions of Variants 01 to 04 respectively.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbol | Maximum Ratings | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Peak Power at +25°C | Pmax | 20 | kW | For duration 1µs max. |
| Power | P | 2 | kW | Notes 1, 2 |
| Nominal Impedance | Z | 50 | Ω | - |
| Operating Frequency Range | f | See Para. 3 | GHz | Note 2 |
| Operating Voltage | Vop | 335 | Vrms | - |
| Operating Temperature Range | Top | See Para. 3 | °C | - |
| Storage Temperature Range | Tstg | As per Top | °C | - |
| Coupling Nut Torque | Tq | 170 | N.cm | Coupling Proof Torque Note 3 |

NOTES:

Derate Power with respect to Operating Temperature as follows:

Derate Power with respect to Operating Frequency as follows:

Whenever a test is performed on mated pairs of connectors, the pairs shall be torqued at 100 ±20N.cm.

## PHYSICAL DIMENSIONS (SEE ALSO PARA. 3)

### Connector Interface Dimensions and Connector Interface Gauge Dimensions

SMA Male Connector Interface: compatible with series SMA pin contact interface as specified in MIL-STD-348.

SMA Female Connector Interface: compatible with series SMA socket contact interface as specified in MIL-STD-348.

SMA Male Gauge Interface: compatible with series SMA pin contact test connector interface as specified in MIL-STD-348.

SMA Female Gauge Interface: compatible with series SMA socket contact test connector interface as specified in MIL-STD-348.

## MATERIALS AND FINISHES

Materials and finishes shall be as follows (as applicable, see Para. 3):

| Type | Description |
| --- | --- |
| 1 | Beryllium Copper, Gold plated, Copper underplate (see Para. 1.7.2) |
| 2 | Beryllium Copper, Gold plated, Nickel underplate (see Para. 1.7.3) |
| 3 | Amagnetic Stainless Steel, Electro-passivated (see Para. 1.7.4) |
| 4 | Amagnetic Stainless Steel, Gold plated (see Para. 1.7.5) |

### Type 1

Shell, Coupling Nut: beryllium copper, plated gold 2.5μm minimum over copper 2.5μm minimum.

Centre Contact:

Male: Brass, plated gold 2.5μm minimum over copper 2.5μm minimum.

Female: Beryllium copper, plated gold 2.5μm minimum over copper 2.5μm minimum.

Insulator: PTFE.

Gasket, Front: Silicone.

Rear Nut: brass or copper, plated gold 2.5μm minimum over copper 2.5μm minimum.

Washers: copper alloy, plated gold 0.5μm minimum over nickel 2μm minimum.

### Type 2

Shell, Coupling Nut: beryllium copper, plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact:

Male: Brass, plated gold 1.27μm minimum over nickel 2μm minimum.

Female: Beryllium copper, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: PTFE.

Gasket, Front: Silicone.

Rear Nut: brass or copper, plated gold 0.5μm minimum over nickel 2μm minimum.

Washers: copper alloy, plated gold 0.5μm minimum over nickel 2μm minimum.

### Type 3

Shell, Coupling Nut: Amagnetic stainless steel, electro-passivated.

Centre Contact:

Male: Brass, plated gold 1.27μm minimum over nickel 2μm minimum.

Female: Beryllium copper, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: PTFE.

Gasket, Front: Silicone.

Rear Nut: stainless steel, electro-passivated.

Washers: copper alloy, plated nickel 2μm minimum.

### Type 4

Shell, Coupling Nut: Amagnetic stainless steel, plated gold 1.27μm minimum over nickel 2μm minimum.

Centre Contact:

Male: Brass, plated gold 1.27μm minimum over nickel 2μm minimum.

Female: Beryllium copper, plated gold 1.27μm minimum over nickel 2μm minimum.

Insulator: PTFE.

Gasket, Front: Silicone.

Rear Nut: stainless steel, plated gold 0.5μm minimum over nickel 2μm minimum.

Washers: copper alloy, plated gold 0.5μm minimum over nickel 2μm minimum.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests – Chart F3

Coupling Proof Torque: shall not be performed on connectors with a female contact; it shall be performed on connectors with a male contact.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component or the primary package shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## ENVIRONMENTAL AND MECHANICAL TESTS

The following requirements apply to tests performed on the connector (and contact) lot as specified in the ESCC Generic Specification:

Para. 8.1, Contact Engagement and Separation Forces:

|  | - Oversize Test Pin Test | - Maximum Diameter Test Pin Test | - Minimum Diameter Test Pin Test |
| --- | --- | --- | --- |
| - Test Pin Diameter (mm) (1) | - 0.9525 to 0.955 | - 0.94 to 0.942 | - 0.902 to 0.904 |
| - Engagement Depth (mm) (1) | - 0.76 to 1.14 | - 1.27 to 1.91 | - 1.27 to 1.91 |
| - Engagement Force (N) | - - | - 13.34 maximum | - - |
| - Separation Force (N) | - - | - - | - 0.28 minimum |

- NOTES:

Test Pins details:

Para. 8.10, Coupling Proof Torque: See Para. 1.5.

Para. 8.11, Mating and Unmating Forces: Maximum torque: 24N.cm.

Para. 8.12, Centre Contact Retention: See Para. 3.

Para. 8.13, Seal: See Para. 3.

Para. 8.19, Endurance:

During Qualification Testing: 500 cycles.

During Periodic Testing: 100 cycles

## Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance | RI | ESCC No. 3402 | 5 | - | GΩ |
| Voltage Proof Leakage Current | IL | ESCC No. 3402 See Para. 3 Note 1 | - | 2 | mA |

NOTES:

Between centre contact and shell.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified, the test methods and test conditions shall be as per the corresponding test defined in Para. 2.4 Room Temperature Electrical Measurements.

| Test Reference per ESCC No. 3402 | Characteristics and Test Conditions Ref. ESCC No. 3402 | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Random Vibration | Contact Resistance: IT = 10mA, VT = 6V Centre Contact: | Rctc | - | 3 | mΩ |
| Mechanical Shock | Contact Resistance: IT = 10mA, VT = 6V Centre Contact: | Rctc | - | 3 | mΩ |
| Temperature Cycling | Contact Resistance: IT = 10mA, VT = 6V Centre Contact: | Rctc | - | 3 | mΩ |
|  | Voltage Proof Leakage Current: | IL | Note 1 |  |  |
| Electrical Measurements at Room Temperature | Insulation Resistance: | RI | Note 1 |  |  |
|  | Voltage Proof Leakage Current: | IL | Note 1 |  |  |
|  | Contact Resistance: IT = 10mA, VT = 6V Centre Contact: Shell: | Rctc Rcts | - - | 3 2 | mΩ mΩ |
|  | VSWR: | VSWR | Note 2 |  |  |
|  | Insertion Loss: | LI | Note 2 |  |  |
| Endurance | Contact Resistance: IT = 10mA, VT = 6V Centre Contact: Shell | Rctc Rcts | - - | 4 3 | mΩ mΩ |

NOTES:

As specified in Para. 2.4.

As specified in Para. 3.

# COMPONENT TYPE VARIANTS – DETAIL REQUIREMENTS

## variant 01 – STRAIGHT Adaptor, Male - male

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 14.9; 15.1. K; -; 5.5; 2 flats. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 4.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 02 – STRAIGHT adaptor, male - Female

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 12.4; 12.65. K; 5.4; 5.5; 2 flats. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 03 – STRAIGHT Adaptor, Female - Female

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 10.1; 10.3. b; 4.4; -. K; 5.4; 5.5; 2 flats. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 1.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 04 – STRAIGHT Bulkhead adaptor, female - female

|  | Symbol; Dimensions mm; Notes. Min; Max. a; -; 18.4. b; 12.5; 12.85. K1; -; 6; 1 flat. K2; -; 8; Hex.. |
| --- | --- |

NOTES:

Maximum panel thickness: 6mm

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 05 – Elbow adaptor, male - female

|  | Symbol; Dimensions mm. Min; Max. H; 14.2; 15.9. J1; 9.1; 9.5. J2; 9.1; 9.5. J3; 12.2; 12.95. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.01 f (GHz) |  |
| Maximum insertion loss | 0.05 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 4.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 06 – T-adaptor, female – female/male

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 14.09; 14.2. H; 15.7; 16.75. J1; 9.75; 10. K; -; 7; Across flat. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | Not applicable |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | 2.8 | N.cm |
| Maximum weight | 5.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 07 – hermetic adaptor, female - female

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 12.4; 12.65. K; 5.4; 5.5; 2 flats. a; 18.5; 18.9. b; 11.7; 13.2. ØD; 12.9; 14.2. |
| --- | --- |

NOTES:

Maximum panel thickness: 4.3mm

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.015 f (GHz) |  |
| Maximum insertion loss | 0.15 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 6.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -40 to +165 | °C |
| Seal: Maximum leakage | 10-8 | atm.cm3/s |

## variant 08 – T-adaptor, female – female/female

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 14; 14.2. H; 13.25; 13.65. J1; 7.9; 8.2. K; -; 7; 2 flats. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | Not applicable |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 5.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 09 – Straight flange adaptor, male - female

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 17.05; 17.1. b; 9.45; 9.55. Øg; 2.55; 2.7; 4 holes. i; 12.6; 12.8. j; 8.59; 8.69. m; 1.4; 1.8. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +105 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 10 – straight flange adaptor, female – female

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 12.4 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 4.2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |

NOTES:

All dimensions are in mm.

## variant 11 – STRAIGHT adaptor, male - male

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 14.9; 15.1. K; -; 5.5; 2 flats. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 4.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 12 – STRAIGHT adaptor, male - Female

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 12.4; 12.65. K; 5.4; 5.5; 2 flats. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3.5 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 03 – STRAIGHT Adaptor, Female - Female

|  | Symbol; Dimensions mm; Notes. Min; Max. a; 10.1; 10.3. b; 4.4; -. K; 5.4; 5.5; 2 flats. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 1.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |

## variant 14 – STRAIGHT Bulkhead adaptor, female - female

|  | Symbol; Dimensions mm; Notes. Min; Max. a; -; 18.4. b; 12.5; 12.85. K1; -; 6; 1 flat. K2; -; 8; Hex.. |
| --- | --- |

NOTES:

Maximum panel thickness: 6mm

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 18 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.1 + 0.008 f (GHz) |  |
| Maximum insertion loss | 0.06 f (GHz) | dB |
| Voltage proof | 1000 | Vrms |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3.9 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |