Page 1 of 17

RF 	COAXIAL CONNECTORS WITH FEMALE CONTACT

BASED ON TYPE SMA 2.9

ESCC Detail Specification No. 3402/022

| Issue 6 Draft A | July 2021 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2021. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| TBA | Specification updated to incorporate changes per DCR. |
| Draft 6A S. Jeffery ESCC-TW 12/07/2021 | Initial draft converted Detail spec to make it consistent with the newly published, converted Generic spec: ESCC No. 3402 issue 5. Changes in this specification from the published Issue 5 are mainly editorial in nature, so have not been highlighted, however there are a few proposed corrections/modifications to the content of Issue 5 which are highlighted with my comments. Any actual technical changes herein, resulting from the proposed amended Generic, are highlighted yellow for information. |

TABLE OF CONTENTS

1	GENERAL	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants	5

1.5	MAXIMUM RATINGS	6

1.6	PHYSICAL DIMENSIONS (SEE ALSO PARA. 3)	7

1.6.1	Connector Interface Dimensions and Connector Interface Gauge Dimensions	7

1.7	MATERIALS AND FINISHES	8

1.7.1	Variants 01 to 05	8

1.7.2	Variant 06	8

2	REQUIREMENTS	9

2.1	GENERAL	9

2.1.1	Deviations from the Generic Specification	9

2.2	MARKING	9

2.3	ENVIRONMENTAL AND MECHANICAL TESTS	10

2.4	Room Temperature Electrical Measurements	10

2.5	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	11

3	COMPONENT TYPE VARIANTS – DETAIL REQUIREMENTS	12

3.1	variant 01 – STRAIGHT JACK, SOLDER-TYPE, FOR MICROPOROUS SEMI-RIGID CABLE Ø2.2mm	12

3.2	variant 02 – STRAIGHT JACK, SOLDER-TYPE, FOR MICROPOROUS SEMI-RIGID CABLE Ø3.58mm	13

3.3	variant 03 – SCREW JACK RECEPTACLE FOR 0.3mm DIA. PIN	14

3.4	variant 04 – SQUARE FLANGE JACK RECEPTACLE FOR 0.3MM DIA. PIN	15

3.5	variant 05 – SQUARE FLANGE JACK RECEPTACLE WITH CYLINDRICAL CONTACT	16

3.6	variant 06 – HERMETIC GLASS SEAL (FOR USE WITH VARIANTS 03, 04 ONLY)	17

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3402.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

- Example: 340202201

Detail Specification Reference: 3402022

Component Type Variant Number: 01 (as required)

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Description (Note 1) |
| --- | --- |
| 01 | Straight Jack, Solder Type, for Microporous Semi-Rigid Cable Ø2.2mm |
| 02 | Straight Jack, Solder Type, for Microporous Semi-Rigid Cable Ø3.58mm |
| 03 | Screw Jack Receptacle for Pin Ø0.3mm |
| 04 | Square Flange Jack Receptacle for Pin Ø0.3mm |
| 05 | Square Flange Jack Receptacle with Cylindrical Contact |
| 06 | Hermetic Glass Seal (for use with Variants 03, 04 only) |

NOTES:

See Para. 3 for details.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbol | Maximum Ratings | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Power | P | See Notes 1, 2 | W | - |
| Nominal Impedance | Z | 50 | Ω | - |
| Operating Frequency Range | f | See Para. 3 | GHz | Note 1 |
| Operating Voltage | Vop | 350 | Vrms | - |
| Operating Temperature Range | Top | See Para. 3 | °C | - |
| Storage Temperature Range | Tstg | As per Top | °C | - |
| Coupling Nut Torque | Tq | 170 | N.cm | Coupling Proof Torque Note 3 |

NOTES:

Derate Power with respect to Operating Frequency as follows:

- Load VSWR is better than 1.30:1.

- The part of the curve limited by multipaction takes into account a 6dB margin.

- See Para. 3 for applicability of power handling categories to the different Variants.

Derate Power with respect to Operating Temperature as follows:

Whenever a test is performed on mated pairs of connectors, the pairs shall be torqued at 80 to 115N.cm.

## PHYSICAL DIMENSIONS (SEE ALSO PARA. 3)

### Connector Interface Dimensions and Connector Interface Gauge Dimensions

#### SMA 2.9 Female Connector Interface

|  | Symbol; Dimensions mm. Min; Max. 1; 2.87; 3.27. 2; 1.88; 1.98. 3; 0.65; 0.95. 4; 2.4; 2.68. 5; 0; 0.08. ØA; 4.6; 4.63. ØB; 5.3; 5.35. ØC; ¼ - 36 UNS 2A. ØD; 2.9; 2.94. |
| --- | --- |

#### SMA 2.9 Male Gauge Interface

|  | Symbol; Dimensions mm. Min; Max. 6; 2.63; 3.25. 7; 0.9; 1.1. 8; 0; 0.08. 9; 0.49; 0.78. 10; 1.22; 1.4. ØC; ¼ - 36 UNS 2B. ØD; 2.9; 2.94. ØE; 6.6; 6.7. ØF; 4.55; 4.58. ØG; 0.92; 0.94. ØH; 0.2; 0.34. |
| --- | --- |

## MATERIALS AND FINISHES

### Variants 01 to 05

Materials and finishes shall be as follows (as applicable, see Para. 3):

Shell and coupling nut: amagnetic stainless steel, electro-passivated.

Centre Contact: beryllium copper, plated gold 1.3μm minimum over nickel 2μm minimum.

Insulator: ULTEM 1000.

Crimping or soldering elements: brass or beryllium copper, plated silver 5μm minimum over copper 0.5µm minimum.

Insert ring: brass, plated gold 1.3μm minimum over nickel 2μm minimum.

Washers: beryllium copper, plated nickel 2μm minimum.

### Variant 06

          - Shell, Centre Contact: Iron-nickel alloy, plated gold 1.3μm minimum over nickel 2μm minimum.

Insulator: Glass.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests – Chart F3

Coupling Proof Torque: shall not be performed.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component or the primary package shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## ENVIRONMENTAL AND MECHANICAL TESTS

The following requirements apply to tests performed on the connector (and contact) lot as specified in the ESCC Generic Specification:

Contact Engagement and Separation Forces:

|  | - Oversize Test Pin Test | - Maximum Diameter Test Pin Test | - Minimum Diameter Test Pin Test |
| --- | --- | --- | --- |
| - Test Pin Diameter (mm) (1) | - 0.95 to 0.955 | - 0.94 to 0.946 | - 0.89 to 0.902 |
| - Engagement Depth (mm) (1) | - 0.76 to 1.14 | - 1.27 to 1.9 | - 1.27 to 1.9 |
| - Engagement Force (N) | - - | - 13.53 maximum | - - |
| - Separation Force (N) | - - | - - | - 0.27 minimum |

- NOTES:

Test Pins details:

Coupling Proof Torque: See Para. 1.5.

Mating and Unmating Forces: Maximum torque: 23N.cm.

Centre Contact Retention: See Para. 3.

Seal: See Para. 3.

Endurance:

During Qualification Testing: 500 cycles.

During Periodic Testing: 100 cycles.

## Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance | RI | ESCC No. 3402 | 5 | - | GΩ |
| Voltage Proof Leakage Current (Voltage Proof) | IL | ESCC No. 3402 See Para. 3 Note 1 | - | 2 | mA |

NOTES:

Between centre contact and shell.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified, the test methods and test conditions shall be as per the corresponding test defined in Para. 2.4 Room Temperature Electrical Measurements.

| Test Reference per ESCC No. 3402 | Characteristics and Test Conditions Ref. ESCC No. 3402 | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Random Vibration | Contact Resistance: IT = 10mA, VT = 6V Centre contact: | Rctc | - | 3 | mΩ |
| Mechanical Shock | Contact Resistance: IT = 10mA, VT = 6V Centre contact: | Rctc | - | 3 | mΩ |
| Temperature Cycling | Contact Resistance: IT = 10mA, VT = 6V Centre contact: | Rctc | - | 3 | mΩ |
|  | Voltage Proof Leakage Current: | IL | Note 1 |  |  |
| Electrical Measurements at Room Temperature | Insulation Resistance: | RI | Note 1 |  |  |
|  | Voltage Proof Leakage Current: | IL | Note 1 |  |  |
|  | Contact Resistance: IT = 10mA, VT = 6V Centre contact: Shell: Variant 06 (Centre contact): | Rctc Rcts Rhgs | - - - | 3 2 10 | mΩ mΩ mΩ |
|  | VSWR: | VSWR | Note 2 |  |  |
|  | Insertion Loss: | LI | Note 2 |  |  |
| Endurance | Contact Resistance: IT = 10mA, VT = 6V Centre contact: Shell: Variant 06 (Centre contact): | Rctc Rcts Rhgs | - - - | 4 3 12 | mΩ mΩ mΩ |

NOTES:

As specified in Para. 2.4.

As specified in Para. 3.

# COMPONENT TYPE VARIANTS – DETAIL REQUIREMENTS

## variant 01 – STRAIGHT JACK, SOLDER-TYPE, FOR MICROPOROUS SEMI-RIGID CABLE Ø2.2mm

|  | Symbol; Dimensions mm. Min; Min. A; 13.43; 13.77. B; 15.34; 15.62. ØC; 6.19; 6.26. ØD; 7.4; 7.6. ØE; 2.25; 2.29. |
| --- | --- |
| N.B.: All dimensions are in mm. |  |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 40 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.005 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| Power Handling Category (See Para. 1.5 Note 1) | Category III |  |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3.4 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | Microporous semi-rigid cable Ø2.2mm (P/N UT 85-LL) |  |

## variant 02 – STRAIGHT JACK, SOLDER-TYPE, FOR MICROPOROUS SEMI-RIGID CABLE Ø3.58mm

|  | Symbol; Dimensions mm. Min; Min. A; 13.43; 13.77. B; 15.34; 15.62. ØC; 6.19; 6.26. ØD; 7.4; 7.6. ØE; 3.64; 3.68. |
| --- | --- |
| N.B.: All dimensions are in mm. |  |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 40 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.005 f (GHz) (Note 1) |  |
| Maximum insertion loss | 0.03 f (GHz) (Note 1) | dB |
| Voltage proof | 750 | Vrms |
| Power Handling Category (See Para. 1.5 Note 1) | Category I |  |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3.1 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Applicable |  |
| Cables used | Microporous semi-rigid cable Ø3.58mm (P/N UT 141-LL) |  |

Notes:

This value only applies up to 32GHz due to the cable.

## variant 03 – SCREW JACK RECEPTACLE FOR PIN Ø0.3mm

|  | Symbol; Dimensions mm. Min; Min. A; 8.89; 8.96. C; 10.77; 10.94. D; 0.65; 0.95. ØE; 5.3; 5.35. F; 0.09; 0.1. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 40 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.005 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| Power Handling Category (See Para. 1.5 Note 1) | Category III |  |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 2 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

## variant 04 – SQUARE FLANGE JACK RECEPTACLE FOR PIN Ø0.3mm

|  | Symbol; Dimensions mm; Notes. Min; Min. A; 7.2; 7.4. C; 10.67; 10.84. ØD; 6.15; 6.35. E; 1.4; 1.7. F; 0.055; 0.135. ØG; 2.55; 2.7; 4 holes. ØH; 15.9; 16.1. I; 12.6; 12.8; 2 places. J; 8.59; 8.69; 2 places. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 40 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.005 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| Power Handling Category (See Para. 1.5 Note 1) | Category III |  |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 3.3 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

## variant 05 – SQUARE FLANGE JACK RECEPTACLE WITH CYLINDRICAL CONTACT

|  | Symbol; Dimensions mm; Notes. Min; Max. A; 7.2; 7.4. C; 10.67; 10.84. ØD; 6.15; 6.35. E; 1.4; 1.7. F; 0.055; 0.135. ØG; 2.55; 2.7; 4 holes. ØH; 15.9; 16.1. I; 12.6; 12.8; 2 places. J; 8.59; 8.69; 2 places. ØK; 1.26; 1.28. L; 4.9; 5.1. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 40 | GHz |
| Maximum voltage standing wave ratio (VSWR) | 1.05 + 0.005 f (GHz) |  |
| Maximum insertion loss | 0.03 f (GHz) | dB |
| Voltage proof | 750 | Vrms |
| Power Handling Category (See Para. 1.5 Note 1) | Category I |  |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | 27 | N |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 5.7 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +165 | °C |
| Seal: Maximum leakage | Not applicable |  |
| Solderability | Not applicable |  |
| Cables used | Not applicable |  |

## variant 06 – HERMETIC GLASS SEAL (FOR USE WITH VARIANTS 03, 04 ONLY)

|  | Symbol; Dimensions mm. Min; Max. ØA; 1.9; 1.94. ØB; 0.28; 0.32. C; 3.1; 3.3. D; 1.3; 1.5. E; 0.6; 0.8. F; -; 0.1. |
| --- | --- |

| ELECTRICAL CHARACTERISTICS | VALUES | UNITS |
| --- | --- | --- |
| Frequency range | 0 to 40 | GHz |
| Maximum voltage standing wave ratio (VSWR) | Not applicable |  |
| Maximum insertion loss | Not applicable |  |
| Voltage proof | 750 | Vrms |
| Power Handling Category (See Para. 1.5 Note 1) | Not applicable |  |
| MECHANICAL CHARACTERISTICS | VALUES | UNITS |
| Minimum centre contact retention force (axial) | Not applicable |  |
| Minimum centre contact retention torque | Not applicable |  |
| Maximum weight | 0.03 | g |
| OTHER CHARACTERISTICS | VALUES | UNITS |
| Operating temperature range | -65 to +200 | °C |
| Seal: Maximum leakage | 10-8 | atm.cm3/s |
| Solderability | Applicable |  |
| Cables used | Not applicable |  |