Page 1 of 20

CONNECTORS, ELECTRICAL, FILTERED, RECTANGULAR,

NON-REMOVABLE SOLDER BUCKET CONTACTS

BASED ON TYPE D*J

ESCC Detail Specification No. 3405/001

| Issue 4 Draft B | November 2022 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2022. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| TBD | Specification updated to incorporate changes per DCR. |

TABLE OF CONTENTS

1	GENERAL	6

1.1	SCOPE	6

1.2	APPLICABLE DOCUMENTS	6

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	6

1.4	THE ESCC COMPONENT NUMBER AND Component Type Variants	6

1.4.1	The ESCC Component Number	6

1.4.2	Component Type Variants	6

1.4.3	Filter Arrangements	7

1.4.3.1	Filter Arrangements – Variants 01, 02 (Shell Size E)	7

1.4.3.2	Filter Arrangements – Variants 03, 04 (Shell Size A)	7

1.4.3.3	Filter Arrangements – Variants 05, 06 (Shell Size B)	8

1.4.3.4	Filter Arrangements – Variants 07, 08 (Shell Size C)	8

1.4.3.5	Filter Arrangements – Variants 09, 10 (Shell Size D)	9

1.5	MAXIMUM RATINGS	9

1.6	PHYSICAL DIMENSIONS	10

1.6.1	Component Type Variants 01 and 02 (Shell Size E)	11

1.6.2	Component Type Variants 03 and 04 (Shell Size A)	11

1.6.3	Component Type Variants 05 and 06 (Shell Size B)	11

1.6.4	Component Type Variants 07 and 08 (Shell Size C)	11

1.6.5	Component Type Variants 09 and 10 (Shell Size D)	11

1.7	Contact Arrangements	12

1.8	CONTACT FUNCTIONAL DIAGRAMS	12

1.9	MATERIALS AND FINISHES	12

1.9.1	Shell	12

1.9.2	Insert	12

1.9.3	Contact Body	13

1.9.4	Contact Filter	13

1.9.5	Ground Plane	13

1.9.6	Guiding and Locking Devices	13

2	REQUIREMENTS	13

2.1	GENERAL	13

2.1.1	Deviations from the Generic Specification	13

2.1.1.1	Deviations from Production Control Applicable to Contacts – Chart F2A	13

2.1.1.2	Deviations from Screening Tests – Chart F3	13

2.1.1.3	Deviations from Qualification and Periodic Tests – Chart F4	13

2.2	MARKING	14

2.3	ENVIRONMENTAL AND MECHANICAL TESTS	14

2.4	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	15

2.4.1	Room Temperature Electrical Measurements	15

2.4.1.1	Room Temperature Electrical Measurements for Grounded Contacts	15

2.4.1.2	Room Temperature Electrical Measurements for Non-filtered Contacts	15

2.4.1.3	Room Temperature Electrical Measurements for Low Frequency Filter Contacts	15

2.4.1.4	Room Temperature Electrical Measurements for Medium, Standard and High Frequency Filter

Contacts	16

2.4.1.5	Consolidated Notes	16

2.4.2	High Temperature Electrical Measurements	17

2.4.3	Low Temperature Electrical Measurements	17

2.5	PARAMETER DRIFT VALUES	17

2.6	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	18

2.7	BURN-IN CONDITIONS	20

2.8	OPERATING LIFE conditions	20

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, and test and inspection data for the component type variants specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3405 – Connectors, Electrical, Filtered, Rectangular.

ESCC Detail Specification No. 3401/022 – Accessories for Rectangular Connectors 3401/001, 3401/002 and Connector Savers 3401/020.

MIL-DTL-24308 – Rack and Panel Connectors, Miniature.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND Component Type Variants

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 340500101100

Detail Specification Reference: 3405001

Component Type Variant Number: 01 (as required)

Filter Arrangement Code Number: 100 (as required)

### Component Type Variants

The Component Type Variants applicable to this specification are as follows:

| Type Variant | Number of Contacts | Shell Size | Contact Type | Mating Force Max (N) | Unmating Force (N) |  | Weight Max (g) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Min | Max |  |
| 01 | 9 | E | Male | 30 | 3.5 | 20 | 12 |
| 02 | 9 | E | Female | 30 | 3.5 | 20 | 13.5 |
| 03 | 15 | A | Male | 50 | 4.5 | 34 | 18.5 |
| 04 | 15 | A | Female | 50 | 4.5 | 34 | 20.5 |
| 05 | 25 | B | Male | 83 | 8 | 55 | 28 |
| 06 | 25 | B | Female | 83 | 8 | 55 | 31 |
| 07 | 37 | C | Male | 123 | 11 | 83 | 38.5 |
| 08 | 37 | C | Female | 123 | 11 | 83 | 42 |
| 09 | 50 | D | Male | 166 | 14.5 | 120 | 47 |
| 10 | 50 | D | Female | 166 | 14.5 | 120 | 51 |

### Filter Arrangements

The applicable filter arrangements and their Filter Arrangement Codes are given below, where: L = Low Frequency, M = Medium Frequency, S = Standard Frequency, H = High Frequency, NF = Non-filtered Contact and G = Grounded Contact.

Contact arrangements, which define the contact positions for each available Shell Size, are shown in Para. 1.7.

#### Filter Arrangements – Variants 01, 02 (Shell Size E)

| Filter Arrangement Code | Contact Position |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| 100 | L | L | L | L | L | L | L | L | L |
| 101 | M | M | M | M | M | M | M | M | M |
| 102 | S | S | S | S | S | S | S | S | S |
| 103 | H | H | H | H | H | H | H | H | H |
| 104 | G | NF | G | NF | G | NF | G | NF | G |
| 105 | L | L | M | S | S | L | M | M | H |

#### Filter Arrangements – Variants 03, 04 (Shell Size A)

| Filter Arrangement Code | Contact Position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 100 | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L |
| 101 | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M |
| 102 | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S |
| 103 | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H |

#### Filter Arrangements – Variants 05, 06 (Shell Size B)

| Filter Arrangement Code | Contact Position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 |  | 2 |  | 3 |  |  | 4 |  | 5 |  | 6 |  | 7 |  | 8 |  | 9 |  | 10 |  |  | 11 |  | 12 |  | 13 |  | 14 |  | 15 |  |
| 100 | L |  | L |  | L |  |  | L |  | L |  | L |  | L |  | L |  | L |  | L |  |  | L |  | L |  | L |  | L |  | L |  |
| 101 | M |  | M |  | M |  |  | M |  | M |  | M |  | M |  | M |  | M |  | M |  |  | M |  | M |  | M |  | M |  | M |  |
| 102 | S |  | S |  | S |  |  | S |  | S |  | S |  | S |  | S |  | S |  | S |  |  | S |  | S |  | S |  | S |  | S |  |
| 103 | H |  | H |  | H |  |  | H |  | H |  | H |  | H |  | H |  | H |  | H |  |  | H |  | H |  | H |  | H |  | H |  |
| 104 | L |  | L |  | L |  |  | L |  | M |  | M |  | M |  | M |  | M |  | S |  |  | S |  | H |  | H |  | L |  | L |  |
| Filter Arrangement Code | Contact Position (Continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 16 |  | 17 |  | 18 |  |  | 19 |  | 20 |  | 21 |  | 22 |  | 23 |  | 24 |  | 25 |  |  |
| 100 | L |  | L |  | L |  |  | L |  | L |  | L |  | L |  | L |  | L |  | L |  |  |
| 101 | M |  | M |  | M |  |  | M |  | M |  | M |  | M |  | M |  | M |  | M |  |  |
| 102 | S |  | S |  | S |  |  | S |  | S |  | S |  | S |  | S |  | S |  | S |  |  |
| 103 | H |  | H |  | H |  |  | H |  | H |  | H |  | H |  | H |  | H |  | H |  |  |
| 104 | L |  | L |  | M |  |  | M |  | M |  | M |  | S |  | S |  | H |  | H |  |  |

#### Filter Arrangements – Variants 07, 08 (Shell Size C)

| Filter Arrangement Code | Contact Position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| 100 | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L |
| 101 | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M |
| 102 | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S |
| 103 | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H |

| Filter Arrangement Code | Contact Position (Continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 |
| 100 | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L |
| 101 | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M |
| 102 | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S |
| 103 | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H |

#### Filter Arrangements – Variants 09, 10 (Shell Size D)

| Filter Arrangement Code | Contact Position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| 100 | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L | L |
| 101 | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M | M |
| 102 | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S | S |
| 103 | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H | H |
| 104 | L | L | L | L | L | L | M | M | M | M | M | M | S | S | H | H | H | L | L | L |

| Filter Arrangement Code | Contact Position (Continued) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |  | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 |
| 100 | L | L | L | L | L | L | L | L | L | L | L |  | L | L | L | L | L | L | L | L | L |
| 101 | M | M | M | M | M | M | M | M | M | M | M |  | M | M | M | M | M | M | M | M | M |
| 102 | S | S | S | S | S | S | S | S | S | S | S |  | S | S | S | S | S | S | S | S | S |
| 103 | H | H | H | H | H | H | H | H | H | H | H |  | H | H | H | H | H | H | H | H | H |
| 104 | L | L | M | M | M | M | M | S | S | S | H |  | H | H | L | L | L | L | L | L | M |
| Filter Arrangement Code | Contact Position (Continued) |  |  |  |  |  |  |  |  |  |  |
|  | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| 100 | L | L | L | L | L | L | L | L | L | L |
| 101 | M | M | M | M | M | M | M | M | M | M |
| 102 | S | S | S | S | S | S | S | S | S | S |
| 103 | H | H | H | H | H | H | H | H | H | H |
| 104 | M | M | M | M | S | S | S | H | H | H |

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| Rated/Working Voltage Low Frequency Medium, Standard, High Frequency Non-filtered | UR | 100 200 300 | V | Note 1 |
| DC Rated Current | IR | 5 | A |  |
| Capacitor AC Rated Current | IRac | 250 | mArms |  |
| Operating Temperature Range | Top | -55 to +125 | °C | Tamb |
| Storage Temperature Range | Tstg | -65 to +125 | °C |  |
| Soldering Temperature | Tsol | +260 | °C | Note 2 |

NOTES:

This characteristic is not applicable to grounded contacts and for filtered contacts the following voltage derating applies:

- The Rated Voltage shall be derated linearly from 100%UR at Tamb = +25°C to 50%UR at Tamb = +125°C.

Duration 10 seconds maximum and the same contact shall not be resoldered until 3 minutes have elapsed.

## PHYSICAL DIMENSIONS

Tables of dimensions for all Component Type Variants are given in Subparagraphs 1.6.1 to 1.6.5 below.

All dimensions are in millimetres except for dimension “U”, which is in °.

The underlined dimensions in the tables are critical to ensure intermateability.

NOTES:

This dimension only applies to connectors with male contacts.

This dimension only applies to connectors with female contacts.

### Component Type Variants 01 and 02 (Shell Size E)

| Type Variant | Symbol/ Dim. | A | B | C | D | E | G | H | J | K | L | ØS | T | U° | V | W | X | Y | ØZ | Øp | ØQ | r | s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | Min | 30.43 | 16.79 | 24.87 | 8.23 | 12.17 | 19.02 | 10.46 | 0.51 | 5.82 | 0.89 | 2.92 | 2.59 | 9 | 0 | 4.03 | - | 4.5 | 0.99 | 1.1 | 1.45 | 1.85 | 2.4 |
|  | Max | 31.19 | 17.04 | 25.12 | 8.48 | 12.93 | 19.53 | 10.97 | 1.02 | 6.13 | 1.52 | 3.2 | 2.69 | 11 | 0.4 | - | 22 | - | 1.04 | 1.15 | 1.51 | 2.15 | - |
| 02 | Min | 30.43 | 16.21 | 24.87 | 7.77 | 12.17 | 19.02 | 10.46 | 0.51 | 5.87 | 0.89 | 2.92 | 2.46 | 9 | - | 3.63 | - | 4.5 | 1.07 | 1.1 | 1.45 | 1.85 | 2.4 |
|  | Max | 31.19 | 16.46 | 25.12 | 8.03 | 12.93 | 19.53 | 10.97 | 1.02 | 6.3 | 1.52 | 3.2 | 2.62 | 11 | - | - | 22 | - | 1.14 | 1.15 | 1.51 | 2.15 | - |

### Component Type Variants 03 and 04 (Shell Size A)

| Type Variant | Symbol/ Dim. | A | B | C | D | E | G | H | J | K | L | ØS | T | U° | V | W | X | Y | ØZ | Øp | ØQ | r | s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 03 | Min | 38.76 | 25.12 | 33.2 | 8.23 | 12.17 | 27.25 | 10.46 | 0.51 | 5.82 | 0.89 | 2.92 | 2.59 | 9 | 0 | 4.03 | - | 4.5 | 0.99 | 1.1 | 1.45 | 1.85 | 2.4 |
|  | Max | 39.52 | 25.37 | 33.45 | 8.48 | 12.93 | 27.76 | 10.97 | 1.02 | 6.13 | 1.52 | 3.2 | 2.69 | 11 | 0.4 | - | 22 | - | 1.04 | 1.15 | 1.51 | 2.15 | - |
| 04 | Min | 38.76 | 24.54 | 33.2 | 7.77 | 12.17 | 27.25 | 10.46 | 0.51 | 5.87 | 0.89 | 2.92 | 2.46 | 9 | - | 3.63 | - | 4.5 | 1.07 | 1.1 | 1.45 | 1.85 | 2.4 |
|  | Max | 39.52 | 24.79 | 33.45 | 8.03 | 12.93 | 27.76 | 10.97 | 1.02 | 6.3 | 1.52 | 3.2 | 2.62 | 11 | - | - | 22 | - | 1.14 | 1.15 | 1.51 | 2.15 | - |

### Component Type Variants 05 and 06 (Shell Size B)

| Type Variant | Symbol/ Dim. | A | B | C | D | E | G | H | J | K | L | ØS | T | U° | V | W | X | Y | ØZ | Øp | ØQ | r | s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 05 | Min | 52.65 | 38.84 | 46.91 | 8.23 | 12.17 | 41.02 | 10.46 | 0.51 | 5.69 | 1.05 | 2.92 | 2.59 | 9 | 0 | 3.81 | - | 4.5 | 0.99 | 1.1 | 1.45 | 1.85 | 2.4 |
|  | Max | 53.42 | 39.09 | 47.17 | 8.48 | 12.93 | 41.53 | 10.97 | 1.24 | 6.13 | 1.78 | 3.2 | 2.69 | 11 | 0.6 | - | 22 | - | 1.04 | 1.15 | 1.51 | 2.15 | - |
| 06 | Min | 52.65 | 38.25 | 46.91 | 7.77 | 12.17 | 41.02 | 10.46 | 0.51 | 5.87 | 0.89 | 2.92 | 2.46 | 9 | - | 3.63 | - | 4.5 | 1.07 | 1.1 | 1.45 | 1.85 | 2.4 |
|  | Max | 53.42 | 38.51 | 47.17 | 8.03 | 12.93 | 41.53 | 10.97 | 1.02 | 6.3 | 1.52 | 3.2 | 2.62 | 11 | - | - | 22 | - | 1.14 | 1.15 | 1.51 | 2.15 | - |

### Component Type Variants 07 and 08 (Shell Size C)

| Type Variant | Symbol/ Dim. | A | B | C | D | E | G | H | J | K | L | ØS | T | U° | V | W | X | Y | ØZ | Øp | ØQ | r | s |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 07 | Min | 68.94 | 55.3 | 63.37 | 8.23 | 12.17 | 57.45 | 10.46 | 0.51 | 5.69 | 1.05 | 2.92 | 2.59 | 9 | 0 | 3.81 | - | 4.5 | 0.99 | 1.1 | 1.45 | 1.85 | 2.4 |
|  | Max | 69.7 | 55.55 | 63.63 | 8.48 | 12.93 | 57.96 | 10.97 | 1.24 | 6.13 | 1.78 | 3.2 | 2.69 | 11 | 0.6 | - | 22 | - | 1.04 | 1.15 | 1.51 | 2.15 | - |
| 08 | Min | 68.94 | 54.71 | 63.37 | 7.77 | 12.17 | 57.45 | 10.46 | 0.51 | 5.87 | 0.89 | 2.92 | 2.46 | 9 | - | 3.63 | - | 4.5 | 1.07 | 1.1 | 1.45 | 1.85 | 2.4 |
|  | Max | 69.7 | 54.97 | 63.63 | 8.03 | 12.93 | 57.96 | 10.97 | 1.02 | 6.3 | 1.52 | 3.2 | 2.62 | 11 | - | - | 22 | - | 1.14 | 1.15 | 1.51 | 2.15 | - |

### Component Type Variants 09 and 10 (Shell Size D)

| Type Variant | Symbol/ Dim. | A | B | C | D | E | G | H | J | K | L | ØS | T | U° | V | W | X | Y | ØZ | Øp | ØQ |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 09 | Min | 66.55 | 52.68 | 60.99 | 10.95 | 14.99 | 55.07 | 13.31 | 0.51 | 5.69 | 1.05 | 2.92 | 2.59 | 9 | 0 | 3.81 | - | 4.5 | 0.99 | 1.1 | 1.45 |
|  | Max | 67.31 | 52.93 | 61.24 | 11.33 | 15.75 | 55.58 | 13.82 | 1.24 | 6.13 | 1.78 | 3.2 | 2.69 | 11 | 0.6 | - | 22 | - | 1.04 | 1.15 | 1.51 |
| 10 | Min | 66.55 | 52.3 | 60.99 | 10.62 | 14.99 | 55.07 | 13.31 | 0.51 | 5.87 | 0.89 | 2.92 | 2.46 | 9 | - | 3.63 | - | 4.5 | 1.07 | 1.1 | 1.45 |
|  | Max | 67.31 | 52.55 | 61.24 | 10.87 | 15.75 | 55.58 | 13.82 | 1.02 | 6.3 | 1.52 | 3.2 | 2.62 | 11 | - | - | 22 | - | 1.14 | 1.15 | 1.51 |
| Type Variant | Symbol/ Dim. | r | s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 09 | Min | 1.85 | 2.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Max | 2.15 | - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 | Min | 1.85 | 2.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Max | 2.15 | - |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## Contact Arrangements

The contact arrangements for the available Shell Sizes are shown below (front view of Male Insert):

NOTES:

The contact locations are in accordance with MIL-DTL-24308 specification sheets and they shall not be checked during procurement.

Both sides of the inserts shall be marked with the minimum marking shown.

## CONTACT FUNCTIONAL DIAGRAMS

| Functional diagram (equivalent circuit) for Low Frequency Filter Contact |  |
| --- | --- |
| Functional diagram (equivalent circuit) for Medium, Standard and High Frequency Filter Contacts |  |
| Functional diagram (equivalent circuit) for Non-filtered Contact | Not applicable |
| Functional diagram (equivalent circuit) for Grounded Contact |  |

## MATERIALS AND FINISHES

### Shell

The shell shall be made of brass with gold plating of thickness ≥ 1.27µm over copper underplating of thickness ≥ 2.54µm. The gold shall be of purity 99% or higher with a hardness of 130 to 200 knoop.

### Insert

The insert shall be made of glassfibre-filled diallylphthalate resin.

### Contact Body

The contact body shall be made of copper alloy with gold plating of thickness ≥ 1.27µm over nickel underplating of thickness ≥ 2µm.

The plating thickness in the solder bucket shall be ≥ 0.2µm of gold over ≥ 0.8µm of nickel.

### Contact Filter

Capacitor: ceramic dielectric.

Ferrite: sintered iron oxide.

### Ground Plane

The ground plane shall be made of copper alloy with gold plating of thickness ≥ 2.5µm over copper underplating of thickness ≥ 1µm.

### Guiding and Locking Devices

The materials and finishes of guiding and locking devices are specified in ESCC Detail Specification No. 3401/022.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Production Control Applicable to Contacts – Chart F2

Operating Life Test for Filter Elements: Not applicable to non-filtered and grounded contacts.

Burn-in: Not applicable to non-filtered and grounded contacts.

Parameter Drift Value Measurements: Not applicable to non-filtered and grounded contacts.

High and Low Temperatures Electrical Measurements: Not applicable to non-filtered and grounded contacts.

#### Deviations from Screening Tests – Chart F3

Burn-in: Not applicable to non-filtered and grounded contacts.

Parameter Drift Value Measurements: Not applicable to non-filtered and grounded contacts.

High and Low Temperatures Electrical Measurements: Not applicable to non-filtered and grounded contacts.

#### Deviations from Qualification and Periodic Tests – Chart F4

Operating Life: Not applicable to non-filtered and grounded contacts.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component or its primary package shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

Contact identification, as shown in Para. 1.7.

The ESCC Component Number.

Traceability information.

## ENVIRONMENTAL AND MECHANICAL TESTS

The following requirements apply to tests performed on the connector lot, including (where applicable) the connector’s component parts, as specified in the ESCC Generic Specification:

Para. 8.5, Contact Capability

The following weights and dimensions apply:

Pick-up weight: 28.35g with a pin diameter of 0.99 to 0.993mm

Drop weight: 226.8g with a pin diameter of 1.039 to 1.04mm

Insertion depth: 4mm

Para. 8.18, Insert Retention (In Shell)

An axial load of 42.8N/cm2 shall be applied to the insert.

Para. 8.20, Engagement And Separation Forces

The following dimensions and engagement and separation forces apply:

|  | Diameter (mm) |  | Engagement Force (N) Max | Separation Force (N) |  |
| --- | --- | --- | --- | --- | --- |
|  | Min | Max |  | Min | Max |
| Max Ø Test Pin | 1.039 | 1.04 | 3.33 | - | 2.22 |
| Min Ø Test Pin | 0.99 | 0.993 | - | 0.28 | - |

Para. 8.21, Oversize Pin Exclusion

The following dimensions and applied force apply:

Test pin diameter: 1.166mm minimum, 1.17mm maximum.

Applied force: 3.33N

Para. 8.22, Probe Damage

The following dimensions and bending moment apply:

Probe diameter: 1.007mm minimum, 1.033mm maximum.

Bending moment: 5.65N.cm at the end of the probe.

Para. 8.23, Solderability

Size A soldering iron shall be used.

Para. 8.25, Contact Retention (In Insert)

A force of 40N shall be applied to the engagement end of each contact to be tested.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3°C. Consolidated Notes follow the Room Temperature Electrical Measurements tables.

#### Room Temperature Electrical Measurements for Grounded Contacts

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Contact Resistance at Low Level | Rcl | ESCC No. 3405 | - | 6 | mΩ |
| Ground Resistance | Rcg | ESCC No. 3405 | - | 3 | mΩ |

#### Room Temperature Electrical Measurements for Non-filtered Contacts

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance | Ri | ESCC No. 3405 Test voltage = 500V | 5 | - | GΩ |
| Voltage Proof | VP | ESCC No. 3405 Test current = 500µA | 1250 | - | V |
| Contact Resistance at Low Level | Rcl | ESCC No. 3405 | - | 6 | mΩ |
| Contact Resistance at Rated Current | Rcr | ESCC No. 3405 | - | 5 | mΩ |

#### Room Temperature Electrical Measurements for Low Frequency Filter Contacts

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance | Ri | ESCC No. 3405 Test voltage = 100V | 5 | - | GΩ |
| Voltage Proof | VP | ESCC No. 3405 Test current = 500µA | 300 | - | V |
| Contact Resistance at Low Level | Rcl | ESCC No. 3405 | - | 8.5 | mΩ |
| Contact Resistance at Rated Current | Rcr | ESCC No. 3405 | - | 6 | mΩ |
| Capacitance | C | ESCC No. 3405 Test voltage: 0.1Vrms Frequency: 1 ±0.1kHz | - | 50000 | pF |
| Insertion Loss (Note 1) | IL | ESCC No. 3405 Frequency: 1MHz Frequency: 2MHz (Note 2) Frequency: 10MHz Frequency: 30MHz (Note 2) Frequency: 100MHz | 13 19 30 40 45 | - - - - - | dB |

#### Room Temperature Electrical Measurements for Medium, Standard and High Frequency Filter Contacts

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance | Ri | ESCC No. 3405 Test voltage = 100V | 10 | - | GΩ |
| Voltage Proof | VP | ESCC No. 3405 Test current = 500µA | 500 | - | V |
| Contact Resistance at Low Level | Rcl | ESCC No. 3405 | - | 8.5 | mΩ |
| Contact Resistance at Rated Current | Rcr | ESCC No. 3405 | - | 6 | mΩ |
| Capacitance Medium Frequency Standard Frequency High Frequency | C | ESCC No. 3405 Test voltage: 0.1Vrms Frequency: 1 ±0.1kHz | 4000 2300 500 | 12000 5000 1300 | pF |
| Insertion Loss, Medium Frequency Filter Contacts (Note 1) | ILM | ESCC No. 3405 Frequency: 1MHz Frequency: 2MHz (Note 2) Frequency: 10MHz Frequency: 30MHz (Note 2) Frequency: 100MHz | 2 5 13 28 50 | - - - - - | dB |
| Insertion Loss, Standard Frequency Filter Contacts (Note 1) | ILS | ESCC No. 3405 Frequency: 2MHz Frequency: 10MHz (Note 2) Frequency: 30MHz Frequency: 100MHz (Note 2) Frequency: 500MHz | 2 8 20 41 55 | - - - - - | dB |
| Insertion Loss, High Frequency Filter Contacts (Note 1) | ILH | ESCC No. 3405 Frequency: 10MHz Frequency: 30MHz (Note 2) Frequency: 100MHz Frequency: 500MHz (Note 2) Frequency: 1GHz | 2 8 25 45 50 | - - - - - | dB |

#### Consolidated Notes

Measurements shall be made with no voltage or current applied.

Only measured during Qualification Testing per Chart F4 of ESCC Generic Specification No. 3405.

### High Temperature Electrical Measurements

The measurements shall be performed at Tamb = +125 (+0 -5)°C.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance Low, Medium Frequency Standard, High Frequency | Ri | ESCC No. 3405 Test voltage = 100V | 50 1000 | - - | MΩ |

### Low Temperature Electrical Measurements

The measurements shall be performed at Tamb = -55 (+5 -0)°C.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance Low, Medium Frequency Standard, High Frequency | Ri | ESCC No. 3405 Test voltage = 100V | 5 10 | - - | GΩ |

## PARAMETER DRIFT VALUES

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.4.1, Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic where specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Drift Value Δ | Units |
| --- | --- | --- | --- |
| Insulation Resistance Drift | ΔRi/Ri | -50 | % |
| Capacitance Drift | ΔC/C | ±20 | % |

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified the test methods and test conditions shall be as per the corresponding test defined in Para. 2.4.1, Room Temperature Electrical Measurements.

| Test Reference per ESCC No. 3405 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Wiring | Insertion Loss | IL | See Para. 2.4.1 |  | dB |
| Climatic Sequence | Insulation Resistance at +55°C (Note 1) | Ri | Note 2 | - | MΩ |
|  | Insertion Loss | IL | See Para. 2.4.1 |  | dB |
|  | Capacitance | C | See Para. 2.4.1 |  | pF |
|  | Insulation Resistance | Ri | See Para. 2.4.1 |  | GΩ |
|  | Voltage Proof | VP | See Para. 2.4.1 |  | V |
| High Temperature Storage |  |  |  |  |  |
| Initial Measurements | Contact Resistance at Low Level | Rcl | See Para. 2.4.1 |  | mΩ |
|  | Ground Resistance | Rcg | See Para. 2.4.1 |  | mΩ |
|  | Capacitance | C | See Para. 2.4.1 |  | pF |
|  | Insulation Resistance | Ri | See Para. 2.4.1 |  | GΩ |
| Final Measurements | Contact Resistance at Low Level | Rcl | See Para. 2.4.1 |  | mΩ |
|  | Contact Resistance at Low Level Drift | ΔRcl | - | 3 | mΩ |
|  | Ground Resistance | Rcg | See Para. 2.4.1 |  | mΩ |
|  | Capacitance | C | See Para. 2.4.1 |  | pF |
|  | Capacitance Drift | ΔC/C | See Para. 2.5 |  | % |
|  | Insertion Loss | IL | See Para. 2.4.1 |  | dB |
|  | Insulation Resistance | Ri | See Para. 2.4.1 |  | GΩ |
|  | Insulation Resistance Drift | ΔRi/Ri | See Para. 2.5 |  | % |
|  | Voltage Proof | VP | See Para. 2.4.1 |  | V |
| Resistance to Soldering Heat | Insulation Resistance | Ri | See Para. 2.4.2 |  | MΩ |
|  | Insertion Loss | IL | See Para. 2.4.1 |  | dB |
| Pin Bending Test | Capacitance | CM & Cm | See Para. 2.4.1 |  | pF |
|  | Insulation Resistance | Ri | See Para. 2.4.1 |  | GΩ |
| Rapid Change of Temperature | Insertion Loss | IL | See Para. 2.4.1 |  | dB |
|  | Capacitance | C | See Para. 2.4.1 |  | pF |
|  | Insulation Resistance | Ri | See Para. 2.4.1 |  | GΩ |
|  | Voltage Proof | VP | See Para. 2.4.1 |  | V |
| Endurance |  |  |  |  |  |
| Initial Measurements | Contact Resistance at Low Level | Rcl | See Para. 2.4.1 |  | mΩ |
|  | Ground Resistance | Rcg | See Para. 2.4.1 |  | mΩ |
|  | Capacitance | C | See Para. 2.4.1 |  | pF |
|  | Insulation Resistance | Ri | See Para. 2.4.1 |  | GΩ |
| Final Measurements | Contact Resistance at Low Level | Rcl | See Para. 2.4.1 |  | mΩ |
|  | Contact Resistance at Low Level Drift | ΔRcl | - | 3 | mΩ |
|  | Ground Resistance | Rcg | See Para. 2.4.1 |  | mΩ |
|  | Insertion Loss | IL | See Para. 2.4.1 |  | dB |
|  | Capacitance | C | See Para. 2.4.1 |  | pF |
|  | Capacitance Drift | ΔC/C | See Para. 2.5 |  | % |
|  | Insulation Resistance | Ri | See Para. 2.4.1 |  | GΩ |
|  | Insulation Resistance Drift | ΔRi/Ri | See Para. 2.5 |  | % |
|  | Voltage Proof | VP | See Para. 2.4.1 |  | V |
| Operating Life |  |  |  |  |  |
| Initial Measurements | Capacitance | C | See Para. 2.4.1 |  | pF |
|  | Insulation Resistance | Ri | See Para. 2.4.1 |  | GΩ |
| Intermediate and Final Measurements (Note 3) | Insulation Resistance | Ri | See Para. 2.4.2 |  | MΩ |
|  | Insertion Loss | IL | See Para. 2.4.1 |  | dB |
|  | Capacitance | C | See Para. 2.4.1 |  | pF |
|  | Capacitance Drift | ΔC/C | See Para. 2.5 |  | % |
|  | Insulation Resistance | Ri | See Para. 2.4.1 |  | GΩ |
|  | Insulation Resistance Drift | ΔRi/Ri | See Para. 2.5 |  | % |
|  | Voltage Proof at 90% of rated voltage | VP | Note 4 | - | V |

NOTES:

Measured immediately after Damp Heat test.

10% of the applicable limit specified in Para. 2.4.2.

Intermediate measurements, performed after 1000 ±24 hours, are applicable to Qualification Testing, and to Periodic Testing for renewal of qualification after lapse.

90% of the limit specified in Para. 2.4.1.

## BURN-IN CONDITIONS

| Characteristics | Symbols | Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +125 (+0 -3) | °C |
| Applied Voltage | VA | See Para. 1.5 Applied between contact and ground | V |

## OPERATING LIFE conditions

The conditions shall be as specified in Para. 2.7.