Page 1 of 17

RESISTORS, HEATERS, FLEXIBLE, SINGLE AND DOUBLE LAYER

ESCC Detail Specification No. 4009/002

| Issue 13 Draft A | January 2023 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2023. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| 1548 TBD | Specification upissued to incorporate changes per DCR. |
| Issue 13 Draft A S.Thacker 12/01/2023 | Initial Draft mark-up specification implementing changes proposed by RICA in DCR1548 & DCRTBD (Ref. email dated 12/01/23). All changes from issue 12 are highlighted yellow for information (see Pages: 14, 15, 16, 17). |

TABLE OF CONTENTS

1	General	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants and Range of Components	6

1.4.3	Manufacturer Specific Heater Identification	8

1.5	MAXIMUM RATINGS	9

1.6	PHYSICAL DIMENSIONS AND HEATER OUTLINE	10

1.6.1	Heater Thickness	12

1.7	MATERIALS AND FINISHES	12

1.7.1	Heater Resistive Element	12

1.7.2	Strip Heater Bridging Tabs	12

1.7.3	Protective Coating	12

1.7.4	Terminal Leads, and Connecting Wires for Module Heaters	12

2	REQUIREMENTS	12

2.1	GENERAL	12

2.1.1	Deviations from the Generic Specification	13

2.1.1.1	Deviations from Screening Tests - Chart F3	13

2.2	MARKING	13

2.3	ROBUSTNESS OF TERMINATIONS	13

2.4	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	14

2.4.1	Room Temperature Electrical Measurements	14

2.4.2	High and Low Temperatures Electrical Measurements	14

2.5	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	15

2.6	BURN-IN CONDITIONS	16

2.7	OPERATING LIFE CONDITIONS	16

APPENDIX A	17

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 4009.

MIL-P-46112: Military Specification for Polyimide Plastic Sheet and Strip.

ASTM-D5213: Standard Specification for Polymeric Resin Film for Electrical Insulation and Dielectric Applications.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 400900201A1234

Detail Specification Reference: 4009002

Component Type Variant Number: 01 (as required)

Manufacturer Specific Heater Identification: A1234 (as applicable; see Para. 1.4.3) where:

- A:		 First letter of the applicable Manufacturer’s name

- 1234: A unique 4 digit number, sequentially allocated by the applicable Manufacturer to a specific heater design.

### Component Type Variants and Range of Components

The component type variants and range of components applicable to this specification are as follows:

| Variant | Heater Type and Construction (Note 1) | Terminal Leads |  | Resistance Range Rn (Ω) | Resistance Tolerance (±%) | Heating Area S (cm2) (Note 1) | Resistance Density (Ω/cm2) | Temperature Coefficient (10-6/°C) | Weight Max (g) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Wire Gauge (AWG) | Configuration (Note 2) |  |  |  |  |  |  |
| 01 | Individual, Strip or Module, Single Layer | 20 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 02 | Individual, Strip or Module, Single Layer | 22 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 03 | Individual, Strip or Module, Single Layer | 24 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 04 | Individual, Strip or Module, Single Layer | 26 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 05 | Individual, Strip or Module, Single Layer | 28 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 06 | Individual, Strip or Module, Single Layer | 30 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 07 | Individual, Strip or Module, Single Layer | 20 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 08 | Individual, Strip or Module, Single Layer | 22 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 09 | Individual, Strip or Module, Single Layer | 24 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 10 | Individual, Strip or Module, Single Layer | 26 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 11 | Individual, Strip or Module, Single Layer | 28 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 12 | Individual, Strip or Module, Single Layer | 30 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 13 | Individual, Strip or Module, Single Layer | 20 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 14 | Individual, Strip or Module, Single Layer | 22 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 15 | Individual, Strip or Module, Single Layer | 24 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 16 | Individual, Strip or Module, Single Layer | 26 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 17 | Individual, Strip or Module, Single Layer | 28 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 18 | Individual, Strip or Module, Single Layer | 30 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 19 | Individual, Strip or Module, Single Layer | 20 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 20 | Individual, Strip or Module, Single Layer | 22 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 21 | Individual, Strip or Module, Single Layer | 24 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 22 | Individual, Strip or Module, Single Layer | 26 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 23 | Individual, Strip or Module, Single Layer | 28 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 24 | Individual, Strip or Module, Single Layer | 30 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 25 | Individual or Module, Double Layer | 20 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 26 | Individual or Module, Double Layer | 22 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 27 | Individual or Module, Double Layer | 24 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 28 | Individual or Module, Double Layer | 26 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 29 | Individual or Module, Double Layer | 28 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 30 | Individual or Module, Double Layer | 30 | Straight | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 31 | Individual or Module, Double Layer | 20 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 32 | Individual or Module, Double Layer | 22 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 33 | Individual or Module, Double Layer | 24 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 34 | Individual or Module, Double Layer | 26 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 35 | Individual or Module, Double Layer | 28 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 36 | Individual or Module, Double Layer | 30 | UTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 37 | Individual or Module, Double Layer | 20 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 38 | Individual or Module, Double Layer | 22 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 39 | Individual or Module, Double Layer | 24 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 40 | Individual or Module, Double Layer | 26 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 41 | Individual or Module, Double Layer | 28 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 42 | Individual or Module, Double Layer | 30 | JTP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 43 | Individual or Module, Double Layer | 20 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 44 | Individual or Module, Double Layer | 22 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 45 | Individual or Module, Double Layer | 24 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 46 | Individual or Module, Double Layer | 26 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 47 | Individual or Module, Double Layer | 28 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |
| 48 | Individual or Module, Double Layer | 30 | STP | 1 ≤ Rn < 3 | 5, 10 | Note 3 | ≤ 200 | 175 | Note 4 |
|  |  |  |  | 3 ≤ Rn ≤ 5000 | 2, 3, 5, 10 |  |  |  |  |

NOTES:

See Para. 1.6.

UTP = Unjacketed Twisted Pair; JTP = Jacketed Twisted Pair; STP = Shielded Twisted Pair.

The Heating Area, S, shall be:

Individual or Module, single layer heaters: 0.2 to 1300cm2

Strip, single layer heaters: 3.2 to 312cm2

Individual or Module, double layer heaters: 1.6 to 1300cm2

The maximum weight of the heaters, excluding the weight of the terminal leads, shall be:

Individual, strip or Module, single layer heaters: 50mg/cm2

Individual or Module, double layer heaters: 75mg/cm2

- The weight of the terminal leads shall be as specified in the applicable wire ESCC Detail Specification.

### Manufacturer Specific Heater Identification

A Heater Design Drawing shall be produced by the Manufacturer after negotiation with the Orderer and shall be held under configuration control by the Manufacturer who will allocate a unique Manufacturer Specific Heater Identification sequentially when a request for a heater is received.

Each Heater Design Drawing shall include the following information:

The heater outline and dimensions as required by Para. 1.6 Physical Dimensions and Heater Outline herein. This shall include details of the heater type and construction, and the terminal leads configurations including the angle of exit of each terminal lead with respect to the heater body.

The ESCC Component Number for the heater, including the Manufacturer Specific Heater Identification (see Para. 1.4).

The terminal leads’ ESCC Detail Specification and Component Number.

The heater electrical information as follows:

Resistance value Rn and tolerance by circuit at Tamb = +22 ±3°C.

Maximum rated power in still air at Tamb = +25°C.

Track width and spacing with tolerances.

Cover material (if fitted).

Pressure sensitive tape (if fitted).

Dimensions of pre-form, if required.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| Rated Power Density | Pn | 0.54 | W/cm2 | Notes 1, 2, 3, 4 |
| Operating Temperature Range | Top | -65 to +200 | °C | Tamb Note 3 |
| Storage Temperature Range | Tstg | -65 to +200 | °C | - |
| Rated Voltage | UR | Pn.Rn.S | V | Notes 2, 3, 5 |

NOTES:

With heater suspended in still air at Tamb = +25°C.

Actual rated power shall be specified in the applicable Heater Design Drawing. It shall be determined from the Heating Area (S) in cm2.

Rated Power Density shall be derated against temperature and Heating Area as follows:

Pn = 0.54W/cm2 for Tamb ≤ +25°C. Derate to 0W/cm2 at 0.00308W/cm2/°C for +25°C < Tamb ≤ +200°C.

Pn = 0.54W/cm2 for 0.2cm2 ≤ Heating Area (S) ≤ 300cm2. Derate to 0.3W/cm2 at 0.00024W/cm2/cm2 for 300cm2 < Heating Area (S) ≤ 1300cm2.

The following design rule shall be applied:

Area of Heater Resistive Element  Heating Area (S) = 50 ±10%

Rn is the nominal resistance.

## PHYSICAL DIMENSIONS AND HEATER OUTLINE

The general physical dimensions and heater layout shall be as follows (see Note 11). The heater type, construction, physical dimensions and heater layout applicable to a specific heater will be specified in the Heater Design Drawing held by the Manufacturer.

Individual (& Strip, MODULE) HEATER

Strip Heater (Note 2)

Example with 3 individual heaters

MODULE Heater (Note 3)

Example with 3 individual heaters

| Symbol | Dimensions mm |  | Tolerance | Heater Type | Remarks |
| --- | --- | --- | --- | --- | --- |
|  | Min | Max |  |  |  |
| A | 6 | 590 | ±0.5mm or (Note 4) ±0.5% | Individual |  |
| A | 6 | 15 |  | Strip | Note 2 |
| A1 to A5 (5) | 8 | 200 |  | Module | Note 3 |
| B | 8 | 600 | ±0.5mm or (Note 4) ±0.5% | Individual |  |
| B1 to B5 (5) | 300 | 590 |  | Strip | Note 2 |
| B1 to B5 (5) | 10 | 590 |  | Module | Note 3, 6 |
| C | 300 | - | ±10% | Individual, Strip & Module |  |
| E | 2 | - | ±0.5mm | Individual, Strip & Module | Note 7 |
| F | 0.4 | - | - | Individual, Strip & Module | Notes 8, 9 |
| G | 610 | 2500 | ±0.5mm or (Note 4) ±0.5% | Strip | Note 10 |
| H1 to H4 (5) | 100 | 1000 | ±15mm or (Note 4) ±10% | Module | Note 3 |
| S | Note 1 |  | - | Individual, Strip & Module | Note 1 |

NOTES:

The Heating Area, S, is defined as the total area of the heater excluding the peripheral margin, and the terminal lead and/or bridging tab connection areas. The acceptable limits of S are specified in Para. 1.4.2.

- For Strip and Module heaters, the Heating Area is the total for all the individual heaters connected together.

A Strip heater is made up of 2 to 5 individual, single layer heaters connected together in series by means of bridging tabs welded to each individual heater’s connection pads. All individual heaters in the Strip heater shall have the same width, dimension A, but may have different lengths, dimension Bn.

A Module heater is made up of 2 to 5 individual, single or double layer heaters connected together in series by means of wires of length Hn welded to each individual heater’s connection pads. All individual heaters in a single Module heater shall either be single layer or double layer. Each individual heater of the Module heater may have any geometric shape (i.e. not limited to rectangular shapes) with different dimensions An and Bn, where An and Bn represent the overall dimensions of each individual heater.

Whichever is greater.

As applicable depending on the quantity of individual heaters in the Strip or Module heater.

The sum of all the individual dimensions Bn of a Module Heater shall not exceed 1500mm.

Terminal lead spacing shall be measured at the terminal lead connection area. Terminal leads may exit the terminal lead connection area at any angle. The terminal leads may be located on any side of the heater.

- There may be more than 2 terminal leads (for multiple resistive element and double layer heaters).

Peripheral margin dimension.

Perforated holes in the peripheral margin are allowed provided that the distance between the edge of the hole and the heater resistive element or connection pad is equal to, or greater than, dimension F.

The total length of the Strip heater.

In any case, the final feasibility of a particular heater design is established according to the limiting characteristics of the materials used and the production process, and shall be decided on by the Manufacturer; not all the configurations are possible.

### Heater Thickness

The maximum thicknesses for single layer and double layer heaters are as follows:

0.25mm maximum for Individual, Strip or Module, single layer heaters.

0.4mm maximum for Individual or Module, double layer heaters.

These maximum limits do not apply over the terminal lead and/or bridging tab connection areas.

## MATERIALS AND FINISHES

### Heater Resistive Element

The heater resistive element shall be made of flexible nickel/chromium/iron alloy (76/16/8 Inconel).

### Strip Heater Bridging Tabs

Bridging Tabs used to connect individual heaters to form a Strip heater shall be made of flexible nickel/chromium/iron alloy (76/16/8 Inconel). They shall be electrically welded to the heater resistive elements.

### Protective Coating

Heater resistive elements, terminal leads connections and, for Strip heaters, bridging tabs connections shall be completely coated with Polyimide Polymer/FEP in accordance with MIL-P-46112 (as superseded by ASTM-D5213).

### Terminal Leads, and Connecting Wires for Module Heaters

Terminal leads and connecting wires for Module heaters shall be made of multi-strand silver-plated copper in accordance with ESCC Generic Specification No. 3901. The applicable Heater Design Drawing shall specify the wire ESCC Detail Specification and the wire ESCC Component Number. The wire gauge shall be as specified in Para. 1.4.2 Component Type Variants and Range of Components herein. Terminal leads and connecting wires shall be electrically welded to the heater resistive element.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests - Chart F3

For heaters where the Heater Design Drawing requires the application of cover material or pressure sensitive tape the following additional steps shall be performed during Screening Tests:

An additional 100% External Visual Inspection shall be performed after successful completion of High and Low Temperatures Electrical Measurements.

After successful completion of the additional External Visual Inspection the cover material and/or pressure sensitive tape shall be applied.

- For heaters which already have a backing foil applied the additional External Visual Inspection shall not be performed.

For heaters where the Heater Design Drawing requires pre-forming this shall be performed on successful completion of Screening Tests. The pre-forming operation shall be performed at a temperature not exceeding the maximum storage temperature. An additional External Visual Inspection shall be performed after the pre-forming operation to check for any damage.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component or its primary package shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## ROBUSTNESS OF TERMINATIONS

The test conditions for robustness of terminations, tested as specified in the ESCC Generic Specification, shall be as follows and apply to a single terminal lead at a time.

| Wire Gauge (AWG) | 20 | 22 | 24 | 26 | 28 | 30 |
| --- | --- | --- | --- | --- | --- | --- |
| Pull Strength (N) | 45 | 36 | 22 | 13 | 9 | 4.5 |
| Duration (s) minimum | 5 | 5 | 5 | 5 | 5 | 5 |

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

The measurements shall be performed at room, high and low temperatures.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3ºC.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Resistance | RA | ESCC No. 4009 ±2% Tolerance ±3% Tolerance ±5% Tolerance ±10% Tolerance | 0.98 Rn 0.97 Rn 0.95 Rn 0.9 Rn | 1.02 Rn 1.03 Rn 1.05 Rn 1.1 Rn | Ω |
| Insulation Resistance | RI | ESCC No. 4009 | 1000 | - | MΩ |
| Voltage Proof Leakage Current | IL | ESCC No. 4009 Test Voltage = 1000Vrms | - | 6 or (1) 12 | mA μA/cm2 |

NOTES:

Whichever is greater based on the Heating Area, S.

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | Test Method and Conditions (Note 1) | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Resistance change between -65 (+3 -0)°C and +22 ±3°C | ΔRA/RA(L) | ESCC No. 4009 | 0 | -1.57 | % |
| Resistance change between +200 (+0 -3)°C and +22 ±3°C | ΔRA/RA(H) | ESCC No. 4009 | 0 | +3.17 | % |

NOTES:

Measurements shall be performed during Chart F3 Screening Tests on a sample of 3 components. In the event of any failure a 100% inspection shall be performed.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3ºC.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.4.1 Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic where specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Test Reference per ESCC No. 4009 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min. | Max. |  |
| Robustness of Terminations Initial Measurements | Resistance ±2% Tolerance ±3% Tolerance ±5% Tolerance ±10% Tolerance | RA | 0.98 Rn 0.97 Rn 0.95 Rn 0.9 Rn | 1.02 Rn 1.03 Rn 1.05 Rn 1.1 Rn | Ω |
| Final Measurements | Resistance ±2% Tolerance ±3% Tolerance ±5% Tolerance ±10% Tolerance Change in Resistance | RA RA/RA | 0.98 Rn 0.97 Rn 0.95 Rn 0.9 Rn - | 1.02 Rn 1.03 Rn 1.05 Rn 1.1 Rn ±1 | Ω % |
| Climatic Sequence Initial Measurements | Resistance ±2% Tolerance ±3% Tolerance ±5% Tolerance ±10% Tolerance | RA | 0.98 Rn 0.97 Rn 0.95 Rn 0.9 Rn | 1.02 Rn 1.03 Rn 1.05 Rn 1.1 Rn | Ω |
| Final Measurements | Resistance ±2% Tolerance ±3% Tolerance ±5% Tolerance ±10% Tolerance Change in Resistance Insulation Resistance Voltage Proof Leakage Current (Test Voltage: 1000Vrms) | RA RA/RA RI IL | 0.98 Rn 0.97 Rn 0.95 Rn 0.9 Rn - 1000 - | 1.02 Rn 1.03 Rn 1.05 Rn 1.1 Rn ±2 - 6 or (1) 12 | Ω % MΩ mA μA/cm2 |
| Operating Life Initial Measurements (0 hours) | Resistance ±2% Tolerance ±3% Tolerance ±5% Tolerance ±10% Tolerance | RA | 0.98 Rn 0.97 Rn 0.95 Rn 0.9 Rn | 1.02 Rn 1.03 Rn 1.05 Rn 1.1 Rn | Ω |
| Intermediate Measurements (1000 hours) | Resistance ±2% Tolerance ±3% Tolerance ±5% Tolerance ±10% Tolerance Change in Resistance | RA RA/RA | 0.98 Rn 0.97 Rn 0.95 Rn 0.9 Rn - | 1.02 Rn 1.03 Rn 1.05 Rn 1.1 Rn ±1.4 | Ω % |
| Final Measurements (2000 hours) | Resistance ±2% Tolerance ±3% Tolerance ±5% Tolerance ±10% Tolerance Change in Resistance (related to 0 hours) Insulation Resistance Voltage Proof Leakage Current (Test Voltage: 1000Vrms) | RA RA/RA RI IL | 0.98 Rn 0.97 Rn 0.95 Rn 0.9 Rn - 1000 - | 1.02 Rn 1.03 Rn 1.05 Rn 1.1 Rn ±2 - 6 or (1) 12 | Ω % MΩ mA μA/cm2 |

NOTES:

Whichever is greater based on the Heating Area, S.

## BURN-IN CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Heater Temperature | THTR | +200 (Note 1) | °C |
| Power Density | Pn | 0.54 (Note 1) | W/cm2 |

NOTES:

Voltage shall be applied until either the specified heater temperature or the power density is reached.

## OPERATING LIFE CONDITIONS

The conditions shall be as specified for Burn-in in Para. 2.6.

## APPENDIX A

AGREED DEVIATIONS FOR RICA (I)

| ITEMS AFFECTED | DESCRIPTION OF DEVIATIONS |
| --- | --- |
| Para. 2.1.1 Deviations from the Generic Specification: Qualification and Periodic Tests - Chart F4 | The specified period for the Endurance Subgroup shall be 24 months. |
|  | During Qualification and Periodic Testing, on completion of the Endurance Subgroup testing, 3 components selected at random from the test samples, shall be tested for Resistance change between +200 (+0 -3)°C and +22 ±3°C, ΔRA/RA(H), in accordance with Para. 2.4.2 of this specification. No failures are allowed. |
| Para. 2.1.1.1 Deviations from Screening Tests - Chart F3 | (a) Heaters with cover material applied: Where cover material has been applied during the heater lamination process, the deviations specified in Para. 2.1.1.1(a) of this specification are not applicable. |
|  | Serialisation: A 100% serialisation, performed prior to Screening Tests with subsequent Electrical Measurements at Room, High and Low Temperatures performed read and record, is optional at the Manufacturer’s discretion. |
|  | Para. 8.2, Rapid Change of Temperature: Mounting: Heaters may be mounted using any suitable method. Data Points: Electrical measurements during and after testing are optional at the Manufacturer’s discretion. |
|  | Para. 8.3, Overload: Data Points: Electrical measurements after testing are optional at the Manufacturer’s discretion. |
|  | Para. 8.4, Burn-in: Under Test Conditions, amend the duration for Burn-in to be 120 (+48 -0) hours |
| Para. 2.2 Marking | When marking of the heater body is not possible, the marking may be performed on a Polyimide + PSA label applied to the heater leads. |
| Para. 2.4.2 High and Low Temperatures Electrical Measurements | Resistance change between +200 (+0 -3)°C and +22 ±3°C, ΔRA/RA(H), shall be guaranteed but not tested. This test shall be replaced by a test performed at +150°C instead of +200°C as follows: Characteristics; Symbols; Test Method and Conditions (Note 1); Limits; Units. Min; Max. Resistance change between +150 (+0 -3)°C and +22 ±3°C; ΔRA/RA(H); ESCC No. 4009; 0; +2.29; %. |