Page 1 of 14

RESISTOR, FIXED, CHIP, METAL FOIL

BASED ON TYPE SMP-PW, SMS-PW, SMT-PW

ESCC Detail Specification No. 4001/027

| Issue 8 Draft A | July 2023 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2023. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| TBA | Specification upissued to incorporate changes per DCR. |

TABLE OF CONTENTS

1	General	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.1.1	Characteristics Codes	5

1.4.2	Component Type Variants and Range of Components	6

1.5	MAXIMUM RATINGS	7

1.6	PHYSICAL DIMENSIONS	8

1.7	FUNCTIONAL DIAGRAM	8

1.8	MATERIALS AND FINISHES	8

1.8.1	Terminations	8

2	REQUIREMENTS	9

2.1	GENERAL	9

2.1.1	Deviations from the Generic Specification	9

2.1.1.1	Deviations from Production Control - Chart F2	9

2.1.1.2	Deviations from Screening Tests - Chart F3	9

2.1.1.3	Deviations from Qualification and Periodic Tests - Chart F4	10

2.1.1.4	Deviations from Data Documentation	10

2.2	MARKING	10

2.3	OVERLOAD	10

2.4	RESISTANCE TO SOLDERING HEAT	10

2.5	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	11

2.5.1	Room Temperature Electrical Measurements	11

2.5.2	High and Low Temperatures Electrical Measurements	11

2.6	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	12

2.7	BURN-IN CONDITIONS	13

2.7.1	No-Bias Burn-in Conditions for 100% Testing	13

2.7.2	Biased Burn-in Conditions for Screening Sub-lot Samples	13

2.8	OPERATING LIFE CONDITIONS	13

APPENDIX A	14

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 4001.

A-QS-009, Isabellenhütte Test Procedure for Infrared Measurement.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 4001027031R5D

Detail Specification Reference: 4001027

Component Type Variant Number: 03 (as required)

Characteristic code: Resistance Value (1.5Ω): 1R5 (as required)

Characteristic code: Resistance Tolerance (±0.5%): D (as required)

#### Characteristics Codes

Characteristics and/or ratings to be codified as part of the ESCC Component Number shall be as follows:

Resistance Value expressed by means of the following codes. The unit quantity shall be ohm (Ω):

| Resistance Value (Ω) | Code |
| --- | --- |
| 0.00XX | R00XX |
| 0.0XX | R0XX |
| 0.XX | RXX |
| X.X | XRX |

Resistance Tolerance expressed by the following codes in accordance with ESCC Basic Specification No. 21700:

| Tolerance (± %) | Code Letter |
| --- | --- |
| 0.5 | D |
| 1 | F |

### Component Type Variants and Range of Components

The component type variants and range of components applicable to this specification are as follows:

| Variant Number | Type | Style (Note 1) | Resistance Range Rn |  | Tolerance (±%) | Temperature Coefficient TC (±10-6/°C) | Value Series | Weight max (g) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min () | Max () |  |  |  |  |
| 01 | SMP-PW | 2010 | 0.010 | 1.0 | 0.5, 1 | Note 2 | E12 | 0.06 |
| 02 | SMS-PW | 2512 | 0.010 | 1.0 | 0.5, 1 | Note 2 | E12 | 0.1 |
| 03 | SMT-PW | 2817 | 0.010 | 2.0 | 0.5, 1 | Note 2 | E12 | 0.15 |
| 04 | SMP-PW | 2010 | 0.0050 | 0.0090 | 0.5, 1 | Note 3 | E12 | 0.06 |
| 05 | SMS-PW | 2512 | 0.0030 | 0.0090 | 0.5, 1 | Note 3 | E12 | 0.1 |
| 06 | SMT-PW | 2817 | 0.0040 | 0.0090 | 0.5, 1 | Note 3 | E12 | 0.15 |

NOTES

See Para. 1.6.

Temperature Coefficient, TC, requirements for Variants 01, 02, 03 with Rn ≥ 10m:

| Temperature Coefficient TC (± 10-6/°C) | Requirement |
| --- | --- |
| -100, +0 | over Tamb = -55°C to +22°C |
| ±60 | over Tamb = +22°C to +170°C |
| ±50 | over Tamb = +22°C to +60°C |

Temperature Coefficient, TC, requirements for Variants 04, 05, 06 with Rn < 10m:

| Temperature Coefficient TC (± 10-6/°C) | Requirement |
| --- | --- |
| -140, +0 | over Tamb = -55°C to +22°C |
| ±100 | over Tamb = +22°C to +170°C |
| ±100 | over Tamb = +22°C to +60°C |

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Variant Number | Style | Symbols | Limits | Units | Remarks |
| --- | --- | --- | --- | --- | --- | --- |
| Rated Dissipation | 01, 04 02, 05 03, 06 | 2010 2512 2817 | Pn | 1 2 3 | W | Note 1 Note 2 Note 1 |
| Rated Voltage | All | All | UR | (Pn x Rn) | V | Note 3 |
| Isolation Voltage | All | All | UI | 200 | V | - |
| Operating Temperature Range | All | All | Top | -55 to +170 | °C | Tamb |
| Storage Temperature Range | All | All | Tstg | -55 to +170 | °C | - |
| Soldering Temperature | All | All | Tsol | +260 | °C | Note 4 |

NOTES:

At Tamb ≤ +130ºC. For Tamb > +130ºC, derate linearly to 0W at Tamb = +170ºC.

At Tamb ≤ +120ºC. For Tamb > +120ºC, derate linearly to 0W at Tamb = +170ºC.

Rn = rated resistance.

Duration 5 seconds maximum.

## PHYSICAL DIMENSIONS

| Variant Number | Style | Dimensions (mm) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A |  | B |  | C |  | D |  |
|  |  | Min | Max | Min | Max | Min | Max | Min | Max |
| 01, 04 | 2010 | 4.98 | 5.18 | 2.44 | 2.64 | 0.5 | 0.9 | 0.6 | 1.2 |
| 02, 05 | 2512 | 6.15 | 6.55 | 2.95 | 3.15 | 0.6 | 1 | 0.6 | 1.2 |
| 03, 06 | 2817 | 6.9 | 7.3 | 4.1 | 4.3 | 0.6 | 1 | 0.6 | 1.2 |

## FUNCTIONAL DIAGRAM

## MATERIALS AND FINISHES

### Terminations

The termination and finish shall be electroplated tin-lead Sn60 to a maximum thickness of 20µm.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Production Control - Chart F2

Infrared Measurement

- Add Infrared Measurement in accordance with A-QS-009 prior to Dimension Check.

- Applied Voltage:

Variants 01, 04: √(12Pn x Rn)

Variants 02, 05: √(7.5Pn x Rn)

Variants 03, 06: √(6.7Pn x Rn)

Duration: 10ms

#### Deviations from Screening Tests - Chart F3

Screening Sub-lot Samples Selection, Mounting and Testing

- A quantity of 20 components shall be selected at random from the production lot and mounted in accordance with ESCC Generic Specification No. 4001 Para. 8.7. These mounted Screening Sub-lot samples shall be serialised and subjected to tests and inspections of Chart F3 in parallel with the 100% testing. For each test requiring electrical measurements the results shall be recorded against component serial number. These samples shall not form part of the delivery lot.

Para. 8.1 Overload

- Overload shall only be performed on the mounted Screening Sub-lot samples. Resistance shall be measured as specified in Para. 2.5.1 Room Temperature Electrical Measurements both before and after Overload. Change in Resistance shall be calculated, recorded against component serial number, and shall not exceed the limit as follows:

Change in Resistance during Overload: ±0.3% max.

Para. 8.2, Non-Linearity: Not applicable.

Para. 8.3.2, Room Temperature Electrical Measurements

- For the testing of the mounted Screening Sub-lot samples, Resistance shall be recorded against component serial number both before and after Burn-in. Change in Resistance shall be calculated, recorded against component serial number, and shall not exceed the limit as follows:

Change in Resistance during Burn-in: ±0.3% max.

Para. 8.4 Burn-in

- For the 100% Testing during Chart F3, burn-in in accordance with Para. 2.7.1 No-Bias Burn-in of this specification shall be performed with a duration of 24 (+24, -0) hours.

- For the testing of the mounted Screening Sub-lot samples, burn-in in accordance with Para. 2.7.2 Biased Burn-in of this specification shall be performed with a duration of 168 (+24 -0) hours.

Para. 6.4 Check for Lot Failure

- 0 failures are allowed during all Chart F3 testing of the mounted Screening Sub-lot samples. In the event of any failure the lot shall be considered as failed.

#### Deviations from Qualification and Periodic Tests - Chart F4

Para. 8.3.1.2, Insulation Resistance: Not applicable.

Para. 8.11, Robustness of Terminations: Not applicable.

#### Deviations from Data Documentation – Section 9

Para. 9.6, Screening Tests Data: For each test performed on the mounted Screening Sublot samples requiring electrical measurements, the results shall be recorded against component serial number. Component drift calculations shall be recorded for each specified test against component serial number.

## MARKING

The marking of all components delivered to this specification shall be in accordance with the requirements of ESCC Basic Specification No. 21700. When the component is too small to accommodate all of the marking specified, as much as space permits shall be marked and the marking information, in full, shall accompany each component in its primary package.

The information to be marked and the order of precedence, shall be as follows:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number.

Traceability information.

## OVERLOAD

The test conditions for Overload, tested as specified in the ESCC Generic Specification, shall be as follows:

Voltage	:

- Variants 01, 04: √(5Pn x Rn)

- Variants 02, 03, 05, 06: √(3.5Pn x Rn)

Duration	: 	5s.

## RESISTANCE TO SOLDERING HEAT

The test conditions for Resistance to Soldering Heat, tested as specified in the ESCC Generic Specification, shall be as follows:

Temperature	:	 +260°C

Duration	:	 5 (+0 -1)s

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3ºC.

| Characteristics | Symbols | ESCC 4001 Test Method and Conditions | Tolerance (± %) | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Resistance | RA | Para. 8.3.1.1 | 0.5 | 0.995 Rn | 1.005 Rn | Ω |
|  |  |  | 1 | 0.99 Rn | 1.01 Rn |  |

### High and Low Temperatures Electrical Measurements

For Variants 01, 02, 03 with Rn ≥ 10m:

| Characteristics | Symbols | ESCC 4001 Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Resistance Change between -55 (+3 -0)ºC and +22 ±3°C | RA/RA | Para. 8.3.1.1, Note 1 TC = -100 x 10-6/ºC: | -0.8 | +0 | % |
| Resistance Change between +22 ±3ºC and +170 (+0 -3)°C | RA/RA | Para. 8.3.1.1, Note 1 TC = ±60 x 10-6/ºC: | -0.906 | +0.906 | % |
| Resistance change between +22 ±3°C and +60 ±3°C | RA/RA | Para. 8.3.1.1, Note 2 TC = ±50 x 10-6/ºC: | -0.22 | +0.22 | % |

For Variants 04, 05, 06 with Rn < 10m:

| Characteristics | Symbols | ESCC 4001 Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Resistance Change between -55 (+3 -0)ºC and +22 ±3 oC | RA/RA | Para. 8.3.1.1, Note 1 TC = -140 x 10-6/ºC: | -1.12 | +0 | % |
| Resistance Change between +22 ±3ºC and +170 (+0 -3)ºC | RA/RA | Para. 8.3.1.1, Note 1 TC = ±100 x 10-6/ºC: | -1.51 | +1.51 | % |
| Resistance change between +22 ±3ºC and +60 ±3ºC | RA/RA | Para. 8.3.1.1, Note 2 TC = ±100 x 10-6/ºC: | -0.44 | +0.44 | % |

NOTES:

The measurements shall be performed on all the mounted Screening Sub-lot samples.

The measurements shall be performed on a sample of 5 components selected from the total production lot.

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

The components shall be mounted as specified in the ESCC Generic Specification.

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3ºC.

Unless otherwise specified the test methods and test conditions shall be as per the corresponding test defined in Para. 2.5.1 Room Temperature Electrical Measurements.

| Test Reference per ESCC No. 4001 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Rapid Change of Temperature |  |  |  |  |  |
| Initial Measurement | Resistance | RA | Record Values |  |  |
| Final Measurement | Change in Resistance | RA/RA | 0.2 |  | % |
| Vibration |  |  |  |  |  |
| Initial Measurement | Resistance | RA | Record Values |  |  |
| Final Measurement | Change in Resistance | RA/RA | 0.2 |  | % |
| Resistance to Soldering Heat |  |  |  |  |  |
| Initial Measurement | Resistance | RA | Record Values |  |  |
| Final Measurement | Change in Resistance | RA/RA | 0.1 |  | % |
| Solderability |  |  |  |  |  |
| Initial Measurement | Resistance | RA | Record Values |  |  |
| Final Measurement | Change in Resistance | RA/RA | 0.1 |  | % |
| Climatic Sequence |  |  |  |  |  |
| Initial Measurements (Procedure 1) | Resistance (after drying) | RA | Record Values |  |  |
| Final Measurements | Change in Resistance | RA/RA | 0.3 |  | % |
|  | Insulation Resistance | - | Not applicable |  | - |
| Operating Life |  |  |  |  |  |
| Initial Measurement (0 hour) | Resistance | RA | Record Values |  |  |
| Intermediate Measurements (1000 hours) | Change in Resistance | RA/RA | 0.3 |  | % |
| Final Measurements (2000 hours) | Change in Resistance | RA/RA | 0.5 |  | % |
|  | Insulation Resistance | - | Not applicable |  | - |

## BURN-IN CONDITIONS

### No-Bias Burn-in Conditions for 100% Testing

| Characteristics | Symbols | Conditions (Note 1) | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +170 (+0 -5) | °C |
| Test Voltage | VT | No Bias | V |

NOTES:

No-Bias Burn-in (High Temperature Bake) shall be performed on components that are not mounted.

No-Bias Burn-in duration shall be 24 (+24 -0) hours.

After Burn-in, the components shall be removed from the chamber and allowed to cool under normal atmospheric conditions for a minimum of 1 hour. They shall then be visually examined. There shall be no evidence of damage and marking shall still be legible.

### Biased Burn-in Conditions for Screening Sub-lot Samples

| Characteristics | Symbols | Conditions (Note 1) | Units |
| --- | --- | --- | --- |
| Terminal Temperature Variants 01, 03, 04, 06: Variants 02, 05: | Tterm | +130 (+0 -3) +120 (+0 -3) | °C |
| Test Voltage | VT | √(Pn x Rn) | V |

NOTES:

Biased Burn-in shall be performed on the applicable sample components that are mounted in accordance with ESCC Generic Specification No. 4001 Para. 8.7.

Biased Burn-in duration shall be 168 (+24 -0) hours.

After Burn-in, the components shall be removed from the chamber and allowed to cool under normal atmospheric conditions for a minimum of 1 hour. They shall then be visually examined. There shall be no evidence of damage and marking shall still be legible.

## OPERATING LIFE CONDITIONS

The conditions shall be as specified in Para. 2.7.2 for Biased Burn-in.

## APPENDIX A

AGREED DEVIATIONS FOR ISABELLENHÜTTE (D)

| Items Affected | Description of Deviations |  |  |
| --- | --- | --- | --- |
| Para. 2.1.1.1 Deviations from Generic Specification: Production Control - Chart F2 | Para. 5.2.1, Dimension Check: A 100% inspection may be performed. |  |  |
| Para. 2.1.1.3 Deviations from Generic Specification: Qualification and Periodic Tests - Chart F4 | Para. 8.3.1.3, Voltage Proof: Not applicable. Para. 8.10, Climatic Sequence: - Para. 8.10.5, Low Air Pressure: Not applicable - Para. 8.10.7, DC Load: Not applicable Para. 8.15, Permanence of Marking: Not applicable |  |  |
| Para. 1.4.1.1 Characteristics Codes: Resistance Value | Resistance Value may be expressed by means of the following codes within the ESCC Component Number. The unit quantity shall be ohm (): |  |  |
|  | Resistance Value () | Code |  |
|  | 0.00XX | R00XX |  |
|  | 0.0XX | R0XX |  |
|  | 0.XXX | RXXX |  |
|  | X.XX | XRXX |  |
| Para. 1.4.2 Component Type Variants and Range of Components | For all Variants additional resistance values, Rn, from within the specified Resistance Range, but outside of the specified Value Series, are available upon request from Isabellenhütte. |  |  |
| Para. 2.2 Marking: Marking of Tolerance on Resistance Value | The tolerance on resistance value may be marked using the actual numeric value (0.5% or 1%) instead of the specified code letter. |  |  |
| Para. 2.2 Marking: Marking of Traceability Information | The manufacturing date code may be marked using a three-digit code where: the first digit is a letter to indicate the year of manufacture in accordance with the first digit of the two-character code for year and month in a twenty-year cycle in IEC Publication No. 60062. the last two digits indicate the week of the year (i.e. 01 to 52) during which encapsulation or the final production process occurred. For example: R41 for week 41 of 2023 |  |  |