Page 1 of 17

TRANSISTORS, POWER, MOSFET, N-CHANNEL, RAD-HARD

BASED ON TYPE STRH100N6

ESCC Detail Specification No. 5205/022

| Issue 9 Draft A | October 2023 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2023. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| TBA | Specification updated to incorporate changes per DCR. |
| Draft 9A 06/10/2023 S.Thacker | Initial revision to implement STM request to change Para. 2.12.2 for Source-to-Drain Diode Forward Voltage , VSD limit. See yellow highlight for proposed change details. |

TABLE OF CONTENTS

1	General	5

1.1	Scope	5

1.2	Applicable Documents	5

1.3	Terms, Definitions, Abbreviations, Symbols and Units	5

1.4	The ESCC component number and component Type Variants	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants	5

1.5	Maximum Ratings	6

1.6	HANDLING PRECAUTIONS	7

1.7	Physical Dimensions and Terminal Identification	8

1.8	Functional Diagram	9

1.9	Materials and Finishes	9

2	Requirements	9

2.1	GENERAL	9

2.1.1	Deviations from the Generic Specification	9

2.1.1.1	Deviations from Screening Tests - Chart F3	9

2.2	WAFER LOT ACCEPTANCE	10

2.3	MARKING	10

2.4	CASE ISOLATION	10

2.5	TERMINAL STRENGTH	10

2.6	Electrical Measurements at Room, High and Low Temperatures	10

2.6.1	Room Temperature Electrical Measurements	10

2.6.2	High and Low Temperatures Electrical Measurements	11

2.6.3	Notes to Room, High and Low Electrical Measurements	12

2.7	Parameter Drift Values	12

2.8	Intermediate and End-Point Electrical Measurements	13

2.9	High Temperature reverse bias burn-in conditions	13

2.10	high temperature forward bias Burn-in Conditions	13

2.11	Operating Life Conditions	14

2.12	TOTAL DOSE RADIATION TESTING	14

2.12.1	Bias Conditions and Total Dose Level for Total Dose Radiation Testing	14

2.12.2	Electrical Measurements for Total Dose Radiation Testing	15

APPENDIX ‘A’	16

# General

## Scope

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## Applicable Documents

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 5000

MIL-STD-750, Test Methods and Procedures for Semiconductor Devices

## Terms, Definitions, Abbreviations, Symbols and Units

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## The ESCC component number and component Type Variants

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 520502201F

Detail Specification Reference: 5205022

Component Type Variant Number: 01 (as required)

Total Dose Radiation Level Letter: F (as required)

### Component Type Variants

The component type variants applicable to this specification are as follows:

| Variant Number | Based on Type | Case | Lead Material and Finish | Weight max g | Total Dose Radiation Level Letter |
| --- | --- | --- | --- | --- | --- |
| 01 | STRH100N6 | TO-254AA | S14 | 10 | F [50kRAD(Si)] |
| 02 | STRH100N6 | TO-254AA | S4 | 10 | F [50kRAD(Si)] |

The lead material and finish shall be in accordance with the requirements of ESCC Basic Specification No. 23500.

Total dose radiation level letters are defined in ESCC Basic Specification No. 22900. If an alternative radiation test level is specified in the Purchase Order the letter shall be changed accordingly.

## Maximum Ratings

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Unit | Remarks |
| --- | --- | --- | --- | --- |
| Drain-Source Voltage | VDS | 60 | V | Over Top, VGS = 0V Note 2 |
| Gate-Source Voltage | VGS | ±20 | V | Over Top |
| Drain Current | IDS | 40 | A | Continuous At Tcase ≤ +25°C and at Tcase > +100°C Note 1 |
| Drain Current (Pulsed) | IDM | 320 | A | Note 2 |
| Power Dissipation | Ptot | 176 | W | At Tcase ≤ +25°C Note 1 |
| Avalanche Energy (Single Pulse) | EAS | 954 280 | mJ | VDS = 40V, IA = 40A Tj = +25 ±3°C Tj = +110 (+0 -5)°C |
| Avalanche Energy (Repetitive Pulse) | EAR | 24 7.7 | mJ | VDS = 40V, IA = 40A, f = 100kHz, Duty Cycle = 10% Tj = +25 ±3°C Tj = +110 (+0 -5)°C |
| Operating Temperature Range | Top | -55 to +150 | °C | Note 3 |
| Junction Temperature | Tj | +150 | °C |  |
| Storage Temperature Range | Tstg | -55 to +150 | °C | Note 3 |
| Soldering Temperature | Tsol | +260 | °C | Note 4 |
| Thermal Resistance, Junction-to-Heat Sink | Rth(j-s) | 0.71 | °C/W | Note 5 |
| Thermal Resistance, Junction-to-Ambient | Rth(j-a) | 48 | °C/W | Note 2 |

NOTES:

IDS and Ptot ratings are in accordance with Rth(j-s). The maximum theoretical ID limit at Tcase > +25°C can be obtained by using the following formula (ID is limited by the package and device construction):

- Where rDS(on) at Tj(max) = 30mΩ.

- For Tcase > +25°C, the Power Dissipation derates linearly to 0W at Tcase = +150°C.

Safe Operating Area applies as follows:

Maximum Safe Operating Area

For Variants with hot solder dip lead finish all testing and any handling performed at Tamb > +125°C shall be carried out in a 100% inert atmosphere.

Duration 10 seconds maximum at a distance of not less than 1.5mm from the device body and the same lead shall not be resoldered until 3 minutes have elapsed.

Package is mounted on an infinite heatsink.

## HANDLING PRECAUTIONS

These components are susceptible to damage by electrostatic discharge. Therefore, suitable precautions shall be employed for protection during all phases of manufacture, testing, shipment and any handling.

These components are categorised as Class 2 per ESCC Basic Specification No. 23800 with a Minimum Critical Path Failure Voltage of 2550 Volts.

The TO-254AA package contains Beryllium Oxide (BeO) and therefore it must not be ground, machined, sandblasted or subjected to any mechanical operation which will produce dust. The case must not be subjected to any chemical process (e.g. etching) which will produce fumes.

## Physical Dimensions and Terminal Identification

Metal Flange Mount Package (TO-254AA) – 3 lead

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 13.59 | 13.84 |  |
| B | 13.59 | 13.84 |  |
| C | 20.07 | 20.32 |  |
| D | 6.3 | 6.7 |  |
| E | 1 | 1.35 |  |
| ØF | 3.5 | 3.9 |  |
| G | 16.89 | 17.4 |  |
| H | 6.86 BSC |  |  |
| ØI | 0.89 | 1.14 | 2 |
| J | 3.81 BSC |  |  |
| K | 3.81 BSC |  |  |
| L | 12.95 | 14.5 |  |
| ØM | 3.05 Typical |  | 2 |
| N | - | 0.71 | 2 |
| R1 | - | 1 | 3 |
| R2 | 1.65 Typical |  | 4 |

NOTES:

The terminal identification is specified by the component’s geometry. See Para. 1.8 Functional Diagram for the terminal connections.

3 places.

Radius of heatsink flange corner, 4 places.

Radius of body corner, 4 places.

## Functional Diagram

NOTES:

The case is not connected to any lead.

## Materials and Finishes

Materials and finishes shall be as follows:

Case

- The case shall be hermetically sealed and have a metal body. The leads pass through ceramic eyelets brazed into the frame and the lid shall be welded.

Leads

- As specified in Para. 1.4.2.

# Requirements

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests - Chart F3

Verification of Safe Operating Area

- The Safe Operating Area shall be verified by performing the ΔVSD test specified in Para. 2.6.1 Room Temperature Electrical Measurements (Thermal Resistance, Junction-to-Heat Sink).

A High Temperature Forward Bias test shall be performed instead of Power Burn-in.

## WAFER LOT ACCEPTANCE

A SEM inspection shall be performed as specified in the ESCC Generic Specification.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component shall be:

The ESCC Qualified Component symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

Warning sign for Beryllium Oxide.

## CASE ISOLATION

Case Isolation shall be performed as specified in the ESCC Generic Specification and as follows:

Test Conditions:

- Test voltage: 500Vdc

- Duration of application of test voltage: 1s

- Points of application of test voltage: between case metal tab and all terminals connected together.

- Maximum leakage current: 10nA

## TERMINAL STRENGTH

The test conditions for terminal strength, tested as specified in the ESCC Generic Specification, shall be as follows:

Test Condition: A, tension, with an applied force of 10N for a duration of 10s.

## Electrical Measurements at Room, High and Low Temperatures

Electrical measurements shall be performed at room, high and low temperatures. Consolidated notes are given after the tables in Para. 2.6.3.

### Room Temperature Electrical Measurements

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Drain-to-Source Breakdown Voltage | V(BR)DSS | 3407 | VGS = 0V, ID = 1mA Bias condition C | 60 | - | V |
| Gate-to-Source Leakage Current 1 | IGSS1 | 3411 | VGS = 20V, VDS = 0V Bias condition C | - | 100 | nA |
| Gate-to-Source Leakage Current 2 | IGSS2 | 3411 | VGS = -20V, VDS = 0V Bias condition C | -100 | - | nA |
| Drain Current | IDSS | 3413 | VDS = 48V, VGS = 0V Bias condition C | - | 10 | µA |
| Gate-to-Source Threshold Voltage | VGS(th) | 3403 | VDS VGS ID=1mA | 2 | 4.5 | V |
| Static Drain-to-Source On Resistance | rDS(on) | 3421 | VGS = 12V, ID = 40A Note 1 | - | 13.5 | mΩ |
| Source-to-Drain Diode Forward Voltage | VSD | 4011 | VGS = 0V, ISD = 40A Note 1 | - | 1.5 | V |
| Thermal Resistance, Junction-to-Heat Sink | Rth(j-s) | 3161 | Note 2 | - | 0.71 | ºC/W |
| Input Capacitance | Ciss | 3431 | VGS = 0V, VDS = 25V f = 1MHz | 3900 | 5900 | pF |
| Output Capacitance | Coss | 3453 |  | 860 | 1300 | pF |
| Reverse Transfer Capacitance | Crss | 3433 |  | 300 | 470 | pF |
| Total Gate Charge | Qg | 3471 | VGS = 12V, VDS = 30V ID = 40A | 100 | 160 | nC |
| Gate-to-Source Charge | Qgs |  |  | 18 | 30 | nC |
| Gate-to-Drain Charge | Qgd |  |  | 29 | 51 | nC |
| Turn-on Delay Time | td(on) | 3472 | VGS = 12V, VDS = 30V ID = 40A RG = 4.7Ω | 16 | 40 | ns |
| Rise Time | tr |  |  | 60 | 260 | ns |
| Turn-off Delay Time | td(off) |  |  | 50 | 120 | ns |
| Fall Time | tf |  |  | 60 | 160 | ns |
| Reverse Recovery Time | trr | 3473 | VDS = 48V, ISD = 40A di/dt = 100A/µs Tj = +25 ±3°C | 320 | 480 | ns |

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | MIL-STD-750 Test Method | Test Conditions Note 3 | Limits |  | Units |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |  |
| Gate-to-Source Leakage Current 1 | IGSS1 | 3411 | VGS = 20V, VDS = 0V Bias condition C Tcase = +125 (+0-5)°C | - | 200 | nA |
| Gate-to-Source Leakage Current 2 | IGSS2 | 3411 | VGS = -20V, VDS = 0V Bias condition C Tcase = +125 (+0-5)°C | -200 | - | nA |
| Drain Current | IDSS | 3413 | VDS = 48V, VGS = 0V Bias condition C Tcase = +125 (+0-5)°C | - | 100 | µA |
| Gate-to-Source Threshold Voltage | VGS(th) | 3403 | VDS  VGS ID = 1mA Tcase = +125 (+0-5)°C | 1.5 | 3.7 | V |
|  |  |  | VDS  VGS ID = 1mA Tcase = -55 (+5-0)°C | 2.2 | 5 | V |
| Static Drain-to-Source On Resistance | rDS(on) | 3421 | VGS = 12V, ID = 40A Tcase = +125 (+0-5)°C Note 1 | - | 24 | mΩ |
| Source-to-Drain Diode Forward Voltage | VSD | 4011 | VG S= 0V, ISD = 40A Tcase = +125 (+0-5)°C Note 1 | - | 1.275 | V |

### Notes to Room, High and Low Electrical Measurements

Pulsed measurement: Pulse Width ≤ 680µs, Duty Cycle ≤ 2%.

The Rth(j-s) limit is guaranteed by performing a ΔVSD (go-no-go) test. The following test conditions and limits shall apply:

VDS = 5V

ID = 32A

Ical = 29mA

tpulse = 20ms

tcal = 50µs

VSD = 60mV minimum, 200mV maximum.

            - Read and record measurements shall be performed on a sample of 5 components with 0 failures allowed. Alternatively a 100% inspection may be performed.

## Parameter Drift Values

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.6.1 Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value Δ | Absolute |  |  |
|  |  |  | Min | Max |  |
| Gate-to-Source Leakage Current 1 | IGSS1 | ±50 or (1) ±100% | - | 100 | nA |
| Gate-to-Source Leakage Current 2 | IGSS2 | ±50 or (1) ±100% | -100 | - | nA |
| Drain Current | IDSS | ±4 or (1) ±100% | - | 10 | µA |
| Gate-to-Source Threshold Voltage | VGS(th) | ±5% | 2 | 4.5 | V |
| Static Drain-to-Source On Resistance | rDS(on) | ±10% | - | 13.5 | mΩ |

NOTES:

- Whichever is the greater referred to the initial value.

## Intermediate and End-Point Electrical Measurements

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.6.1 Room Temperature Electrical Measurements.

The limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- |
|  |  | Min | Max |  |
| Drain Current | IDSS | - | 10 | µA |
| Gate-to-Source Threshold Voltage | VGS(th) | 2 | 4.5 | V |
| Static Drain-to-Source On Resistance | rDS(on) | - | 13.5 | mΩ |

## High Temperature reverse bias burn-in conditions

HTRB Burn-in shall be performed in accordance with MIL-STD-750, Test Method 1042, Test Condition A with the following conditions:

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | °C |
| Drain-to-Source Voltage | VDS | 48 | V |
| Gate-to-Source Voltage | VGS | 0 | V |
| Duration | t | 240 minimum | Hours |

## high temperature forward bias Burn-in Conditions

HTFB Burn-in shall be performed in accordance with MIL-STD-750, Test Method 1042, Test Condition B with the following conditions:

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | °C |
| Drain-to-Source Voltage | VDS | 0 | V |
| Gate-to-Source Voltage | VGS | 16 | V |
| Duration | t | 48 minimum | Hours |

## Operating Life Conditions

Operating Life shall consist of High Temperature Reverse Bias in accordance with MIL-STD-750, Test Method 1042, Test Condition A, followed by High Temperature Forward Bias in accordance with MIL-STD-750, Test Method 1042, Test Condition B. The test conditions are as follows:

High Temperature Reverse Bias Conditions

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | °C |
| Drain-to-Source Voltage | VDS | 48 | V |
| Gate-to-Source Voltage | VGS | 0 | V |
| Duration | t | 1000 minimum | Hours |

High Temperature Forward Bias Conditions

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +150 (+0 -5) | °C |
| Drain-to-Source Voltage | VDS | 0 | V |
| Gate-to-Source Voltage | VGS | 16 | V |
| Duration | t | 1000 minimum | Hours |

## TOTAL DOSE RADIATION TESTING

All lots shall be irradiated in accordance with ESCC Basic Specification No. 22900, standard dose rate (window 1: 3.6kRAD to 36kRAD per hour).

### Bias Conditions and Total Dose Level for Total Dose Radiation Testing

The following bias condition (worst-case) shall be used for Total Dose Radiation Testing at Tamb = 22 ±3°C:

With VGS bias = +15V and VDS = 0V during irradiation.

The total dose level applied shall be as specified in Para. 1.4.2 or in the Purchase Order.

### Electrical Measurements for Total Dose Radiation Testing

Prior to irradiation testing the devices shall have successfully met Room Temperature Electrical Measurements specified herein (Para. 2.6.1).

Unless otherwise stated the measurements shall be performed at Tamb = 22 ±3°C.

Unless otherwise specified the test methods and test conditions shall be as per the corresponding test defined in Para. 2.6.1 Room Temperature Electrical Measurements.

The parameters to be measured during irradiation testing, on completion of irradiation testing, after 24 hours anneal at Room Temperature and after 168 hours anneal at +100 ±3°C are shown below.

| Characteristics | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Values Δ | Absolute |  |  |
|  |  |  | Min | Max |  |
| Drain-to-Source Voltage Note 1 | VDSS | -15% Note 2 | N/A |  | V |
| Gate-to-Source Leakage Current 1 | IGSS1 | +1.5 | - | 100 | nA |
| Gate-to-Source Leakage Current 2 | IGSS2 | -1.5 | -100 | - | nA |
| Drain Current | IDSS | +10 | - | 10 | µA |
| Gate-to-Source Threshold Voltage | VGS(th) | -60% / +25% | 2 | 4.5 | V |
| Static Drain-to-Source On Resistance | rDS(on) | ±15% | - | 13.5 | mΩ |
| Source-to-Drain Diode Forward Voltage | VSD | ±15% | - | 1.5 | V |

NOTES:

Drain-to-Source Voltage measurements shall be made in accordance with MIL-STD-750, Test Method 3405, with VGS = 0V and ID = 1mA.

Referred to an initial Drain-to-Source Voltage measurement made prior to the commencement of Total Dose Radiation Testing.

## APPENDIX ‘A’

AGREED DEVIATIONS FOR STMICROELECTRONICS (F)

| ITEMS AFFECTED | DESCRIPTION OF DEVIATIONS |
| --- | --- |
| Para. 2.1.1, Deviations from the Generic Specification: Para. 8, Test Methods and Procedures | For qualification and qualification maintenance, or procurement of qualified or unqualified components, the following replacement test method specifications shall be used instead of the following ESCC Basic Specifications: No. 20500, External Visual Inspection: replaced by MIL-STD-750 Test Method 2071. No. 20900, Radiographic Inspection of Electronic Components: replaced by MIL-STD-750 Test Method 2076. No. 21400, Scanning Electron Microscope Inspection of Semiconductor Dice: replaced by MIL-STD-750 Test Method 2077. |
| Para. 2.1.1.1, Deviations from the Generic Specification: Deviations from Screening Tests - Chart F3 | Solderability is not applicable unless specifically stipulated in the Purchase Order. |
| Para. 2.6.1, Room Temperature Electrical Measurements | The AC characteristics Ciss, Coss, Crss, Qg, Qgs, Qgd, td(on), tr, td(off), tf and trr may be considered guaranteed but not tested if successful pilot lot testing has been performed on the wafer lot in accordance with STMicroelectronics “wafers acceptation” internal procedure as specified in the PID, which includes AC (Ciss, Coss, Crss, Qg, Qgs, Qgd, td(on), tr, td(off), tf and trr) characteristic measurements per the Detail Specification. A summary of the pilot lot testing shall be provided if required by the Purchase Order. |

ADDITIONAL DATA – STMICROELECTRONICS (F)

Derating for Space Application

- These components are susceptible to Single Event Gate Rupture when operated in a spece environment unless the following derating is applied during their use:

- VDS ≤ 100V when VGS = 0V

- VDS ≤ 80V when VGS = -2V

- VDS ≤ 25V when VGS = -15V

NOTES:

The derating for space application was originally obtained under the following Test conditions:

Ion used 	= Kr

LET 		= 32.1MeV/(mg/cm²)

Energy	= 768MeV

Range 	= 94μm