Page 1 of 12

CAPACITOR FILTERS, L-TYPE, ELECTROMAGNETIC INTERFERENCE SUPPRESSION, SURFACE MOUNT

BASED ON TYPE SFCMS 35 TYPE L

ESCC Detail Specification No. 3008/036

| Issue 5 Draft A | January 2024 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2024. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| tbc | Specification upissued to incorporate changes per DCR. |

TABLE OF CONTENTS

1	General	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants	5

1.5	MAXIMUM RATINGS	6

1.6	PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION	6

1.7	functional diagram	7

1.8	MATERIALS AND FINISHES	7

1.8.1	Terminal Material and Finish	7

2	REQUIREMENTS	7

2.1	GENERAL	7

2.1.1	Deviations from the Generic Specification	7

2.1.1.1	Deviations from Screening Tests - Chart F3	7

2.1.1.2	Deviations from Qualification and Periodic Tests – Chart F4B	8

2.2	MARKING	8

2.3	solderability	8

2.4	robustness of terminations	8

2.5	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	9

2.5.1	Room Temperature Electrical Measurements	9

2.5.2	High and Low Temperatures Electrical Measurements	10

2.6	PARAMETER DRIFT VALUES	10

2.7	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	11

2.8	BURN-IN CONDITIONS	12

2.9	OPERATING LIFE CONDITIONS	12

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3008.

MIL-STD-202, Test Methods for Electronic and Electrical Component Parts.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 300803601

Detail Specification Reference: 3008036

Component Type Variant Number: 01 (as required)

### Component Type Variants

The Component Type Variants applicable to this specification are as follows:

| Type Variant (Note 1) | Capacitance C (pF) | Case Description and Terminal Configuration (Note 2) | Weight Max. (g) |
| --- | --- | --- | --- |
| 01 | 17600 | Non-hermetically sealed case, surface mount, with square section body terminal and round section tab terminals | 2 |
| 02 | 680 |  |  |

NOTES:

The Insertion Loss characteristics applicable to each Component Type Variant are as follows:

| Type Variant | Insertion Loss IL (dB) With No Current Applied |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1MHz | 10MHz | 100MHz | 1GHz |
| 01 | 10 | 30 | 50 | 70 |
| 02 | - | - | 26 | 55 |

See Para. 1.6 for physical dimensions.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| Rated DC Voltage | UR | 100 | V | Note 1 |
| Voltage Drop | Vdr | 100 | mV |  |
| DC Resistance | Rs | 10 | mΩ |  |
| Rated Current | IR | 10 | A | DC and Low Frequency |
| Operating Temperature Range | Top | -55 to +125 | °C | Tamb |
| Storage Temperature Range | Tstg | -55 to +125 | °C |  |
| Soldering Temperature | Tsol | +280 | °C | Note 2 |

NOTES:

The addition of DC applied voltage and ripple voltage shall never exceed the rated DC voltage.

Duration 6 seconds maximum and the same terminal shall not be resoldered until 3 minutes have elapsed.

## PHYSICAL DIMENSIONS AND TERMINAL IDENTIFICATION

| Symbols | Dimensions (mm) |  | Remarks |
| --- | --- | --- | --- |
|  | Min | Max |  |
| A | 1.9 | 2.6 |  |
| B | 0.7 | 0.9 |  |
| C | 3.5 | 3.6 | Note 2 |
| D | 3.5 | 3.6 | Note 3 |
| L | 11.3 | 12.7 |  |
| M | 7.4 | 7.6 |  |

NOTES:

For both tab terminals, only the end-faces are solder plated. A dark line shall be marked on one of the faces of the body terminal close to the capacitor tab terminal.

This dimension applies to the square-section body terminal (ground), over dimension M. The face of the component which bears the terminal identification marking shall also bear the marking defined in Para. 2.2.

This dimension (diameter) applies to both tab terminals and the insulation.

## functional diagram

## MATERIALS AND FINISHES

### Terminal Material and Finish

The body terminal shall be silver plated brass. The end tab terminals’ material shall be brass with each end-face plated with Sn5Pb95.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Screening Tests - Chart F3

Rapid Change of Temperature: The components may not be mounted.

Vibration: Not applicable.

Room Temperature Electrical Measurements: Voltage Drop shall be replaced by a DC resistance measurement performed in accordance with MIL-STD-202, Test Method 303.

Room, High and Low Temperature Electrical Measurements: Insertion Loss is considered destructive and therefore components so measured cannot be delivered as flight models. Insertion Loss may be measured after External Visual Inspection, and any parts rejected during External Visual Inspection or Radiographic Inspection, but electrically acceptable after Burn-in, may be used. Components subjected to Insertion Loss Measurements shall not be submitted to Check for Lot Failure, see Paras. 2.5.1 and 2.5.2.

#### Deviations from Qualification and Periodic Tests – Chart F4B

Where Insertion Loss measurements are required during Qualification Testing and Periodic Testing for renewal of qualification after lapse, they shall be made at only four test frequencies. See Paras. 1.4.2 and 2.5.1.

Intermediate and End-Point Electrical Measurements: Voltage Drop shall be replaced by a DC resistance measurement performed in accordance with MIL-STD-202, Test Method 303.

Resistance to Soldering Heat: May be performed prior to Vibration. Capacitance shall be measured instead of Insertion Loss.

Accelerated Damp Heat: Voltage Proof and Insertion Loss shall not be measured.

Operating Life, Intermediate and Final Measurements: Insertion Loss measurements shall not be performed.

Robustness of Terminations: The components shall be mounted on a suitable substrate. After mounting, examination shall be made for good tinning as evidenced by flowing of the solder with wetting of the terminals. A force of 10N shall be applied normal to the line joining the terminals and in a plane parallel to the substrate, for a duration of 10 ±1s. The components shall then be examined and there shall be no evidence of damage or loosening of the components from the substrate.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component or its primary package shall be:

Terminal identification (see Para. 1.6).

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para. 1.4.1).

Traceability information.

## solderability

The test conditions for Solderability shall be as specified in the ESCC Generic Specification and as follows:

Test Ta, Method 2

Temperature: +275 ±5°C.

## robustness of terminations

The tab terminals of the components are rigid.

The test conditions for Robustness of Terminations shall be as specified in the ESCC Generic Specification. Required deviations are as described in Para. 2.1.1.2.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

The measurements shall be performed at room, high and low temperatures.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| DC Resistance | Rs | MIL-STD-202, Test Method 303 | - | 10 | mΩ |
| Voltage Proof | VP | ESCC No. 3008 | 250 | - | V |
| Insulation Resistance | Ri | ESCC No. 3008 | 10 | - | MΩ |
| Insertion Loss |  | ESCC No. 3008 |  |  |  |
|  | IL1 | f = 1MHz (Notes 2, 3) | Note 1 | - | dB |
|  | IL2 | f = 10MHz (Notes 2, 4, 5) | Note 1 | - | dB |
|  | IL3 | f = 100MHz (Notes 2, 4, 5) | Note 1 | - | dB |
|  | IL4 | f = 1GHz (Notes 2, 4, 5) | Note 1 | - | dB |
| Capacitance | C | ESCC No. 3008 f = 1000 ±100Hz V = 0.1 to 1.2Vrms | Note 1 | - | pF |

NOTES:

See Para. 1.4.2.

For Qualification Testing and Periodic Testing for renewal of qualification after lapse, measurements shall be made with a load current of zero and, during either Subgroup 1B or Subgroup 1C of Chart F4B in the Generic Specification, measurements shall also be made at the rated current specified in Para. 1.5.

For Screening and Periodic Testing for extension of qualification, IL1 is guaranteed but not tested.

For Screening, measurements shall be made on a sample of 5 components. If one or more samples fail, another 5 samples shall be selected and measured. Lot failure will occur if any of these additional samples fail. Samples shall be mounted on a suitable substrate prior to the measurements.

For Screening and Periodic Testing for extension of qualification, IL2, IL3 and IL4 shall be measured with no load current applied.

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Insulation Resistance | Ri | ESCC No. 3008 Tamb = +125 (+0 -5)°C (Note 1) | 1 | - | MΩ |
| Insertion Loss |  | ESCC No. 3008 Tamb = +125 (+0 -5)°C and Tamb = -55 (+5 -0)°C |  |  |  |
|  | IL2 | f = 10MHz (Notes 3, 4) | Note 2 | - | dB |
|  | IL3 | f = 100MHz (Notes 3, 4) | Note 2 | - | dB |
|  | IL4 | f = 1GHz (Notes 3, 4) | Note 2 | - | dB |

NOTES:

Insulation Resistance measurements shall only be performed immediately on completion of Burn-in, as specified in the ESCC Generic Specification. Insulation Resistance shall not be measured again during testing per Chart F3 of the ESCC Generic Specification.

See Para. 1.4.2.

Measurements shall be made on a sample of 5 components. If one or more samples fail, another 5 samples shall be selected and measured. Lot failure will occur if any of these additional samples fail. Samples shall be mounted on a suitable substrate prior to the measurements.

Measurements shall be made with no load current applied.

## PARAMETER DRIFT VALUES

The measurements shall be performed at Tamb = +22 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.5.1, Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic where specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Drift Value Δ | Units |
| --- | --- | --- | --- |
| Change in Capacitance | ΔC/C | ±10 | % |

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified, test methods and test conditions shall be as per the corresponding test defined in Para. 2.5.1, Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic where specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Test Reference per ESCC No. 3008 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Low Air Pressure | Voltage Proof | VP | 125% UR | - | V |
| Overload | DC Resistance | Rs | - | Note 1 | mΩ |
|  | Insulation Resistance | Ri | Note 1 | - | MΩ |
| Damp Heat | Insulation Resistance (applied voltage = 1.5V) | Ri | Note 2 | - | MΩ |
| Vibration | Insertion Loss (applied current = minimum required for monitoring purposes) | IL | Note 3 | - | dB |
| Resistance to Soldering Heat | Insulation Resistance | Ri | Note 1 | - | MΩ |
|  | Capacitance | C | Note 3 | - | pF |
| Shock | Insertion Loss | IL | Note 3 | - | dB |
| Accelerated Damp Heat | Insulation Resistance | Ri | Note 2 | - | MΩ |
| Operating Life |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 3 | - | pF |
| Intermediate Measurements (at 500 hours (Note 4)) / (at 1000 hours (Note 5)) | Insulation Resistance at +125 (+0 -3)°C | Ri | Note 6 | - | MΩ |
|  | After recovery |  |  |  |  |
|  | Capacitance | C | Note 3 | - | pF |
|  | Change in Capacitance | ΔC/C | - | ±10 | % |
|  | Voltage Proof | VP | 90% UR | - | V |
|  | Insulation Resistance | Ri | Note 7 | - | MΩ |
| Final Measurements (at 1000 or 2000 hours (Note 8)) | Insulation Resistance at +125 (+0 -3)°C | Ri | Note 6 | - | MΩ |
|  | After recovery |  |  |  |  |
|  | Capacitance | C | Note 3 | - | pF |
|  | Change in Capacitance | ΔC/C | - | ±10 | % |
|  | Voltage Proof | VP | 90% UR | - | V |
|  | Insulation Resistance | Ri | Note 7 | - | MΩ |
| Robustness of Terminations | DC Resistance | Rs | - | Note 1 | mΩ |

NOTES:

See Para. 2.5.1.

> 10% of the value given in Para. 2.5.1.

See Para. 1.4.2.

500 hours is applicable to Qualification Testing, Periodic Testing for extension of qualification and to Periodic Testing for renewal of qualification after lapse.

1000 hours is applicable to Qualification Testing, and to Periodic Testing for renewal of qualification after lapse.

See Para. 2.5.2.

> 50% of the value given in Para. 2.5.1.

1000 hours is applicable to Periodic Testing for extension of qualification. 2000 hours is applicable to Qualification Testing, and to Periodic Testing for renewal of qualification after lapse.

## BURN-IN CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +125 (+0 -3) | °C |
| Applied Voltage | UA | 200 | V |
|  |  | (Note 1) |  |

notes:

Between one tab terminal and the body terminal.

## OPERATING LIFE CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +125 (+0 -3) | °C |
| Applied Voltage | UA | 200 | V |
|  |  | (Note 1) |  |
| Rated Current | IR | 10 | A |

NOTES:

Between one tab terminal and the body terminal.