Page 1 of 21

CAPACITORS, FIXED, SURFACE MOUNT, DC SELF-HEALING, NON-INDUCTIVE, POLYTEREPHTALATE DIELECTRIC

BASED ON TYPE PM94S

ESCC Detail Specification No. 3006/024

| Issue 14 Draft A | September 2025 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2025. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| tba | Specification updated to incorporate changes per DCR. |

TABLE OF CONTENTS

1	GENERAL	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND Component Type Variants	5

1.4.1	The ESCC Component Number	5

1.4.1.1	Characteristics and Ratings Codes	5

1.4.2	Component Type Variants and Range of Components	6

1.4.2.1	Component Type Variants	6

1.4.2.2	Range of Components	7

1.5	MAXIMUM RATINGS	13

1.6	PHYSICAL DIMENSIONS and terminal identification	14

1.6.1	Component Type Variants 01 to 04 (PM94S)	14

1.6.2	Component Type Variants 05 to 08 (PM94NS)	15

1.7	FUNCTIONAL DIAGRAM	15

1.8	MATERIALS AND FINISHES	16

1.8.1	Lead/Terminal Material	16

1.8.2	Lead/Terminal Finish	16

2	REQUIREMENTS	16

2.1	GENERAL	16

2.1.1	Deviations from the Generic Specification	16

2.1.1.1	Deviations from Qualification and Periodic Tests – Chart F4	16

2.2	MARKING	17

2.3	Robustness of terminations	17

2.4	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	18

2.4.1	Room Temperature Electrical Measurements	18

2.4.2	High and Low Temperatures Electrical Measurements	18

2.5	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	19

2.6	BURN-IN conditions	20

APPENDIX ‘A’	21

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, and test and inspection data for the component type variants and/or the Component Type Variants and Range of Components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3006.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND Component Type Variants

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 300602402155KH

Detail Specification Reference: 3006024

Component Type Variant Number: 02 (as required)

Characteristic code: Capacitance Value (1.5µF): 155 (as required)

Characteristic code: Capacitance Tolerance (±10%): K (as required)

Rating code: Rated Voltage (250V): H (as required)

#### Characteristics and Ratings Codes

Characteristics and ratings to be codified as part of the ESCC Component Number shall be as follows:

Capacitance Value, C, expressed by means of the following codes in accordance with ESCC Basic Specification No. 21700. The unit quantity shall be picofarads (pF).

| Capacitance Value C (pF) | Code |
| --- | --- |
| XX 104 | XX4 |
| XX 105 | XX5 |
| XX 106 | XX6 |

Capacitance Tolerance expressed by the following codes in accordance with ESCC Basic Specification No. 21700:

| Tolerance (± %) | Code Letter |
| --- | --- |
| 10 | K |
| 20 | M |

DC Rated Voltage, UR, expressed by the following codes:

| DC Rated Voltage UR (V) | Code Letter |
| --- | --- |
| 50 | C |
| 63 | D |
| 100 | E |
| 200 | G |
| 250 | H |
| 400 | K |

### Component Type Variants and Range of Components

#### Component Type Variants

Eight Component Type Variants are defined in this specification and four different package types are available, as follows:

Surface mount package: Component Type Variants 01 to 04

Plug-in package with two rows of 4 terminals: Component Type Variants 05 and 06

Plug-in package with two rows of 5 terminals: Component Type Variant 07

Plug-in package with two rows of 6 terminals: Component Type Variant 08

One, two or four different sizes are therefore available for each package type. See Para. 1.6 for details.

#### Range of Components

The range of components applicable to this specification are as follows:

| DC Rated Voltage (UR) Max (V) | Capacitance Value (C) (µF) (Note 1) | AC Rated Current (IRA) Max (Arms) | Available Type Variants (See Para. 1.6) | Dimension B Max (mm) (See Para. 1.6) | Weight Max (g) |
| --- | --- | --- | --- | --- | --- |
| 50 | 2.2 | 1.3 | 01, 05 | 6 | 1 |
| 50 | 2.7 | 1.6 | 01, 05 | 6 | 1 |
| 50 | 3.3 | 1.9 | 01, 05 | 6 | 1 |
| 50 | 3.9 | 2.3 | 01, 05 | 8 | 1.3 |
| 50 | 4.7 | 2.8 | 01, 05 | 8 | 1.3 |
| 50 | 4.7 | 1.4 | 02, 06 | 6 | 1.6 |
| 50 | 5.6 | 3.3 | 01, 05 | 10 | 1.7 |
| 50 | 5.6 | 1.7 | 02, 06 | 6 | 1.6 |
| 50 | 6.8 | 4.1 | 01, 05 | 10 | 1.7 |
| 50 | 6.8 | 2.1 | 02, 06 | 6 | 1.6 |
| 50 | 6.8 | 1.9 | 03, 07 | 6 | 2.2 |
| 50 | 8.2 | 4.9 | 01, 05 | 12 | 1.9 |
| 50 | 8.2 | 2.5 | 02, 06 | 8 | 2.1 |
| 50 | 8.2 | 2.3 | 03, 07 | 6 | 2.2 |
| 50 | 10 | 6 | 01, 05 | 12 | 2 |
| 50 | 10 | 3.1 | 02, 06 | 8 | 2.1 |
| 50 | 10 | 2.9 | 03, 07 | 6 | 2.2 |
| 50 | 10 | 2.6 | 04, 08 | 6 | 2.7 |
| 50 | 12 | 3.7 | 02, 06 | 8 | 2.1 |
| 50 | 12 | 3.4 | 03, 07 | 6 | 2.2 |
| 50 | 12 | 3.1 | 04, 08 | 6 | 2.7 |
| 50 | 15 | 4.6 | 02, 06 | 10 | 2.6 |
| 50 | 15 | 4.3 | 03, 07 | 8 | 3 |
| 50 | 15 | 3.9 | 04, 08 | 6 | 2.7 |
| 50 | 18 | 5.2 | 03, 07 | 8 | 3 |
| 50 | 18 | 4.6 | 04, 08 | 8 | 3.6 |
| 50 | 22 | 6.3 | 03, 07 | 10 | 3.7 |
| 50 | 22 | 5.7 | 04, 08 | 8 | 3.6 |
| 50 | 27 | 7.8 | 03, 07 | 12 | 4.7 |
| 50 | 27 | 7 | 04, 08 | 10 | 4.6 |
| 50 | 33 | 9.5 | 03, 07 | 14 | 5.2 |
| 50 | 33 | 8.5 | 04, 08 | 12 | 5.5 |
| 50 | 39 | 10 | 04, 08 | 15 | 6.8 |
| 50 | 47 | 12.2 | 04, 08 | 15 | 6.8 |
| 63 | 1.5 | 1.7 | 01, 05 | 6 | 1 |
| 63 | 1.8 | 2.1 | 01, 05 | 6 | 1 |
| 63 | 2.2 | 2.3 | 01, 05 | 8 | 1.3 |
| 63 | 2.7 | 3.1 | 01, 05 | 10 | 1.7 |
| 63 | 3.3 | 3.8 | 01, 05 | 10 | 1.7 |
| 63 | 3.3 | 1.9 | 02, 06 | 6 | 1.6 |
| 63 | 3.9 | 4.9 | 01, 05 | 12 | 1.9 |
| 63 | 3.9 | 2.3 | 02, 06 | 6 | 1.6 |
| 63 | 4.7 | 6 | 01, 05 | 12 | 2 |
| 63 | 4.7 | 2.8 | 02, 06 | 8 | 2.1 |
| 63 | 4.7 | 2.6 | 03, 07 | 6 | 2.2 |
| 63 | 5.6 | 3.3 | 02, 06 | 10 | 2.6 |
| 63 | 5.6 | 3.1 | 03, 07 | 6 | 2.2 |
| 63 | 6.8 | 3.7 | 03, 07 | 8 | 3 |
| 63 | 6.8 | 3.4 | 04, 08 | 6 | 2.7 |
| 63 | 8.2 | 4.5 | 03, 07 | 8 | 3 |
| 63 | 8.2 | 4 | 04, 08 | 8 | 3.6 |
| 63 | 10 | 5.5 | 03, 07 | 10 | 3.7 |
| 63 | 10 | 4.9 | 04, 08 | 8 | 4.6 |
| 63 | 12 | 6.6 | 03, 07 | 12 | 4.7 |
| 63 | 12 | 5.9 | 04, 08 | 10 | 4.6 |
| 63 | 15 | 8.3 | 03, 07 | 14 | 5.2 |
| 63 | 15 | 7.4 | 04, 08 | 12 | 5.5 |
| 63 | 18 | 8.9 | 04, 08 | 15 | 6.8 |
| 63 | 22 | 10.9 | 04, 08 | 15 | 6.8 |
| 100 | 0.56 | 0.8 | 01, 05 | 6 | 1 |
| 100 | 0.68 | 1 | 01, 05 | 6 | 1 |
| 100 | 0.82 | 1.1 | 01, 05 | 6 | 1 |
| 100 | 1 | 1.8 | 01, 05 | 6 | 1 |
| 100 | 1.2 | 1.8 | 01, 05 | 8 | 1.3 |
| 100 | 1.5 | 2.2 | 01, 05 | 8 | 1.3 |
| 100 | 1.5 | 1.1 | 02, 06 | 6 | 1.6 |
| 100 | 1.8 | 2.7 | 01, 05 | 10 | 1.7 |
| 100 | 1.8 | 1.4 | 02, 06 | 6 | 1.6 |
| 100 | 2.2 | 3.5 | 01, 05 | 12 | 1.9 |
| 100 | 2.2 | 1.7 | 02, 06 | 6 | 1.6 |
| 100 | 2.2 | 1.6 | 03, 07 | 6 | 2.2 |
| 100 | 2.7 | 2.1 | 02, 06 | 8 | 2.1 |
| 100 | 2.7 | 1.9 | 03, 07 | 6 | 2.2 |
| 100 | 3.3 | 2.5 | 02, 06 | 10 | 2.6 |
| 100 | 3.3 | 2.4 | 03, 07 | 6 | 2.2 |
| 100 | 3.3 | 2.1 | 04, 08 | 6 | 2.7 |
| 100 | 3.9 | 2.8 | 03, 07 | 8 | 3 |
| 100 | 3.9 | 2.5 | 04, 08 | 6 | 2.7 |
| 100 | 4.7 | 3.4 | 03, 07 | 8 | 3 |
| 100 | 4.7 | 3 | 04, 08 | 8 | 3.6 |
| 100 | 5.6 | 4 | 03, 07 | 10 | 3.7 |
| 100 | 5.6 | 3.6 | 04, 08 | 8 | 3.6 |
| 100 | 6.8 | 4.9 | 03, 07 | 12 | 4.7 |
| 100 | 6.8 | 4.3 | 04, 08 | 10 | 4.6 |
| 100 | 8.2 | 5.9 | 03, 07 | 14 | 5.2 |
| 100 | 8.2 | 5.2 | 04, 08 | 10 | 4.6 |
| 100 | 10 | 6.4 | 04, 08 | 12 | 5.5 |
| 100 | 12 | 7.7 | 04, 08 | 15 | 6.8 |
| 200 | 0.33 | 0.6 | 01, 05 | 6 | 1 |
| 200 | 0.39 | 0.8 | 01, 05 | 6 | 1 |
| 200 | 0.47 | 1 | 01, 05 | 6 | 1 |
| 200 | 0.56 | 1.1 | 01, 05 | 8 | 1.3 |
| 200 | 0.68 | 1.4 | 01, 05 | 8 | 1.3 |
| 200 | 0.82 | 1.7 | 01, 05 | 10 | 1.7 |
| 200 | 0.82 | 0.8 | 02, 06 | 6 | 1.6 |
| 200 | 1 | 2.1 | 01, 05 | 12 | 1.9 |
| 200 | 1 | 1 | 02, 06 | 6 | 1.6 |
| 200 | 1.2 | 1.2 | 02, 06 | 8 | 2.1 |
| 200 | 1.2 | 1.1 | 03, 07 | 6 | 2.2 |
| 200 | 1.5 | 1.5 | 02, 06 | 8 | 2.1 |
| 200 | 1.5 | 1.4 | 03, 07 | 6 | 2.2 |
| 200 | 1.8 | 1.8 | 02, 06 | 10 | 2.6 |
| 200 | 1.8 | 1.7 | 03, 07 | 8 | 3 |
| 200 | 1.8 | 1.6 | 04, 08 | 6 | 2.7 |
| 200 | 2.2 | 2.1 | 03, 07 | 8 | 3 |
| 200 | 2.2 | 2 | 04, 08 | 6 | 2.7 |
| 200 | 2.7 | 2.6 | 03, 07 | 10 | 3.7 |
| 200 | 2.7 | 2.4 | 04, 08 | 8 | 3.6 |
| 200 | 3.3 | 3.2 | 03, 07 | 10 | 3.7 |
| 200 | 3.3 | 3 | 04, 08 | 10 | 4.6 |
| 200 | 3.9 | 3.8 | 03, 07 | 12 | 4.7 |
| 200 | 3.9 | 3.5 | 04, 08 | 10 | 4.6 |
| 200 | 4.7 | 4.6 | 03, 07 | 14 | 5.2 |
| 200 | 4.7 | 4.3 | 04, 08 | 12 | 5.5 |
| 200 | 5.6 | 5.1 | 04, 08 | 15 | 6.8 |
| 250 | 0.22 | 0.8 | 01, 05 | 6 | 1 |
| 250 | 0.27 | 1.0 | 01, 05 | 6 | 1 |
| 250 | 0.33 | 1.2 | 01, 05 | 6 | 1 |
| 250 | 0.39 | 1.4 | 01, 05 | 8 | 1.3 |
| 250 | 0.47 | 1.7 | 01, 05 | 8 | 1.3 |
| 250 | 0.47 | 0.8 | 02, 06 | 6 | 1.6 |
| 250 | 0.56 | 2 | 01, 05 | 10 | 1.7 |
| 250 | 0.56 | 0.9 | 02, 06 | 6 | 1.6 |
| 250 | 0.68 | 2.4 | 01, 05 | 12 | 1.9 |
| 250 | 0.68 | 1.1 | 02, 06 | 6 | 1.6 |
| 250 | 0.82 | 1.3 | 02, 06 | 8 | 2.1 |
| 250 | 1 | 1.6 | 02, 06 | 8 | 2.1 |
| 250 | 1 | 1.5 | 03, 07 | 6 | 2.2 |
| 250 | 1 | 1.3 | 04, 08 | 6 | 2.7 |
| 250 | 1.2 | 2 | 02, 06 | 10 | 2.6 |
| 250 | 1.2 | 1.8 | 03, 07 | 8 | 3 |
| 250 | 1.2 | 1.6 | 04, 08 | 6 | 2.7 |
| 250 | 1.5 | 2.5 | 02, 06 | 10 | 2.6 |
| 250 | 1.5 | 2.3 | 03, 07 | 8 | 3 |
| 250 | 1.5 | 2 | 04, 08 | 6 | 2.7 |
| 250 | 1.8 | 2.7 | 03, 07 | 10 | 3.7 |
| 250 | 1.8 | 2.4 | 04, 08 | 8 | 3.6 |
| 250 | 2.2 | 3.4 | 03, 07 | 10 | 3.7 |
| 250 | 2.2 | 3 | 04, 08 | 8 | 3.6 |
| 250 | 2.7 | 4.1 | 03, 07 | 12 | 4.7 |
| 250 | 2.7 | 3.6 | 04, 08 | 10 | 4.6 |
| 250 | 3.3 | 5 | 03, 07 | 14 | 5.2 |
| 250 | 3.3 | 4.4 | 04, 08 | 10 | 4.6 |
| 250 | 3.9 | 5.3 | 04, 08 | 12 | 5.5 |
| 250 | 4.7 | 6.3 | 04, 08 | 15 | 6.8 |
| 400 | 0.1 | 0.8 | 01, 05 | 6 | 1 |
| 400 | 0.12 | 1 | 01, 05 | 8 | 1.3 |
| 400 | 0.15 | 1.2 | 01, 05 | 8 | 1.3 |
| 400 | 0.18 | 1.4 | 01, 05 | 8 | 1.7 |
| 400 | 0.22 | 1.7 | 01, 05 | 10 | 1.7 |
| 400 | 0.22 | 0.8 | 02, 06 | 6 | 1.6 |
| 400 | 0.27 | 1 | 02, 06 | 6 | 1.6 |
| 400 | 0.33 | 1.2 | 02, 06 | 8 | 2.1 |
| 400 | 0.39 | 1.4 | 02, 06 | 8 | 2.1 |
| 400 | 0.47 | 1.7 | 02, 06 | 10 | 2.6 |
| 400 | 0.47 | 1.6 | 03, 07 | 6 | 2.2 |
| 400 | 0.56 | 1.9 | 03, 07 | 8 | 3 |
| 400 | 0.68 | 2.3 | 03, 07 | 8 | 3 |
| 400 | 0.68 | 2 | 04, 08 | 6 | 2.7 |
| 400 | 0.82 | 2.8 | 03, 07 | 10 | 3.7 |
| 400 | 0.82 | 2.5 | 04, 08 | 8 | 3.6 |
| 400 | 1 | 3.4 | 03, 07 | 12 | 4.7 |
| 400 | 1 | 3 | 04, 08 | 10 | 4.6 |
| 400 | 1.2 | 4 | 03, 07 | 14 | 5.2 |
| 400 | 1.2 | 3.6 | 04, 08 | 10 | 4.6 |
| 400 | 1.5 | 4.5 | 04, 08 | 12 | 5.5 |
| 400 | 1.8 | 5.4 | 04, 08 | 15 | 6.8 |

NOTES:

            - Two Capacitance Tolerances are available:

- ±20% for E6 Series Capacitance Values

- ±10% for E12 Series Capacitance Values

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| DC Rated Voltage | UR | 50, 63, 100, 200, 250, 400 | V | Notes 1, 2 |
| DC Category Voltage | UC | Note 2 | V | Over Top |
| AC Rated Voltage | UA | 35%UR | Vrms | 50/60Hz |
| AC Rated Current | IRA | See Para. 1.4.2.2 | Arms | 100kHz |
| Operating Temperature Range | Top | -55 to +125 | °C | Tamb |
| Storage Temperature Range | Tstg | -55 to +125 | °C |  |
| Soldering Temperature | Tsol |  | °C |  |
| Variants 01 to 04 Variants 05 to 08 |  | +215 +260 |  | Note 3 Note 4 |

NOTES:

As required; See Para. 1.4.2.2.

The DC Rated Voltage is also the DC Category Voltage when Top is between -55°C (Tamb) and +100°C (Tamb). For Top > +100°C (Tamb) only the DC Category Voltage applies, and derating must be observed as follows:

UC derates linearly from 100%UC at Tamb = +100°C to 80%UC at Tamb = +125°C.

For reflow (vapour phase) soldering following a pre-heating phase at +130°C minimum:

Temperature: +215°C

Duration: 20s minimum, 40s maximum.

For Solderability and Resistance to Soldering Heat:

Maximum soldering time: 5s

Minimum soldering distance: 1mm

## PHYSICAL DIMENSIONS and terminal identification

### Component Type Variants 01 to 04 (PM94S)

| Type Variant | Dimensions (mm) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A Max | D Max | d1 |  | d2 |  | F |  | L |  |
|  |  |  | Min | Max | Min | Max | Min | Max | Min | Max |
| 01 | 10.7 | 10.7 | 5.5 | 6.5 | 1.3 | 1.7 | 4.8 | 5.2 | 1.5 | 2.5 |
| 02 | 15.5 | 11.5 | 5.5 | 6.5 | 1.3 | 1.7 | 4.8 | 5.2 | 1.5 | 2.5 |
| 03 | 16.5 | 15.5 | 7.5 | 8.5 | 1.8 | 2.2 | 6.8 | 7.2 | 1.5 | 2.5 |
| 04 | 18.5 | 17 | 7.5 | 8.5 | 1.8 | 2.2 | 6.8 | 7.2 | 1.5 | 2.5 |

NOTES:

The limits of Dimension B are defined in Para. 1.4.2.2.

Terminal identification: A voltage polarity “+” symbol shall be marked on the top of the body to indicate the terminal which should be connected to the highest potential.

### Component Type Variants 05 to 08 (PM94NS)

(For illustrative purposes Type Variant 08 with 2 rows of 6 terminals is shown)

| Type Variant | Dimensions (mm) |  |  |  |  |  |  |  |  |  |  | No. of terminals per row |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A Max | D Max | X |  | b |  | c |  | e |  | L Min |  |
|  |  |  | Min | Max | Min | Max | Min | Max | Min | Max |  |  |
| 05 | 10.7 | 10.7 | 7.75 | 8.75 | 0.15 | 0.35 | 0.4 | 0.6 | 2.34 | 2.74 | 5 | 4 |
| 06 | 15.5 | 11.5 | 13.5 | 14.5 | 0.15 | 0.35 | 0.4 | 0.6 | 2.34 | 2.74 | 5 | 4 |
| 07 | 16.5 | 15.5 | 13.5 | 14.5 | 0.15 | 0.35 | 0.4 | 0.6 | 2.34 | 2.74 | 5 | 5 |
| 08 | 18.5 | 17 | 14.74 | 15.74 | 0.15 | 0.35 | 0.4 | 0.6 | 2.34 | 2.74 | 5 | 6 |

notes:

The limits of Dimension B are defined in Para. 1.4.2.2.

Terminal identification: A voltage polarity “+” symbol shall be marked on the top of the body to indicate the row of terminals which should be connected to the highest potential.

## FUNCTIONAL DIAGRAM

NOTES:

These capacitors are not polarised; however, marking includes the voltage polarity symbol (see Para. 1.6) which should be respected in use.

For Component Type Variants 05 to 08, each row of terminals is connected internally to a single electrode.

## MATERIALS AND FINISHES

### Lead/Terminal Material

The Lead/Terminal Material shall be brass with silver plating.

### Lead/Terminal Finish

The Lead/Terminal Finish shall be either Type 3 or Type 4 in accordance with the requirements of ESCC Basic Specification No. 23500.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Qualification and Periodic Tests – Chart F4

Mounting (Applicable to Component Type Variants 01 to 04 only):

- Prior to the commencement of Chart F4 Subgroups 1A, 1B, 1C and 2, components may be mounted on a suitable substrate in accordance with IEC Publication No. 60384-1, Clause 5.5. After mounting, a force of 10N shall be applied normal to a line joining the terminals and, in a plane parallel to the substrate for a duration of 10s. There shall be no evidence of damage or loosening of the components from the substrate.

Operating Life:

Test Condition (c), Applied voltage: 1.25×UC, where UC is the applicable Category Voltage per Paras. 1.5 and 1.4.2.2.

Robustness of Terminations: For Component Type Variants 01 to 04 Robustness of Terminations shall be performed as follows:

- The components shall be mounted on a suitable substrate. After mounting, examination shall be made for good tinning as evidenced by flowing of the solder with wetting of the terminations. A force of 10N shall be applied normal to the line joining the terminals and, in a plane parallel to the substrate, for a duration of 10 ±1s. The components shall then be examined and there shall be no evidence of damage or loosening of the components from the substrate.

Solderability: For Component Type Variants 01 to 04 Solderability shall be performed in accordance with IEC Publication No. 60068-2-58. The following details shall apply:

Test Conditions:

- Temperature: +215 ±3°C

- Immersion time: 3 ±0.3s

- Immersion distance: 1mm minimum, 2mm maximum.

Resistance to Soldering Heat: For Component Type Variants 01 to 04 Resistance to Soldering Heat shall be performed in accordance with IEC Publication No. 60068-2-58. The following details shall apply:

Test Conditions:

- Temperature: +215 ±3°C

- Immersion time: 40s

- Immersion distance: 1mm minimum, 2mm maximum.

Data Points: As defined in Resistance to Soldering Heat in the Generic Specification.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component or its primary package shall be:

Terminal Identification (see Para. 1.6).

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number (see Para 1.4.1).

Traceability information.

## Robustness of terminations

The terminals of the components are rigid.

For Component Type Variants 05 to 08 the test conditions for Robustness of Terminations shall be as specified in the ESCC Generic Specification and as follows:

Test Ua1, tensile:

- Applied force: 10N

- Duration: 7.5 ±2.5s

All terminals of the component shall be tested.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

Electrical measurements shall be performed at room, high and low temperatures.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3°C.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Capacitance | C | ESCC No. 3006 Test Frequency = 1kHz | Note 1 | Note 2 | µF |
| Tangent of Loss Angle | tgδ | ESCC No. 3006 Test Frequency = 1kHz | - | 10×10-3 | - |
| Insulation Resistance, Dielectric | RI | ESCC No. 3006 |  |  |  |
|  |  | C ≤ 0.33µF; UR > 100V C > 0.33µF; UR > 100V C > 0.33µF; UR ≤ 100V (Notes 3, 4) | 7500 2500 1250 | - - - | MΩ MΩ.µF MΩ.µF |
| Voltage Proof, Terminal-to-Terminal | VP | ESCC No. 3006 | 1.6×UR (Note 4) | - | V |

NOTES:

Capacitance Value of the component minus the applicable Tolerance (see Para. 1.4.2.2).

Capacitance Value of the component plus the applicable Tolerance (see Para. 1.4.2.2).

Measuring voltage applied during Insulation Resistance shall be as follows:

For UR ≤ 100V: V = UR

For 100V < UR ≤ 400V: V = 100V

For the applicable Rated Voltage (UR) see Para. 1.4.2.2.

### High and Low Temperatures Electrical Measurements

| Characteristics | Symbols | Test Method and Conditions (Notes 1 and 2) | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Temperature Coefficient | ΔC/C | ESCC No. 3006 Test Frequency = 1kHz |  |  |  |
|  |  | Tamb = -55 (+3 -0)°C: | -10 (Note 3) | - | % |
|  |  | Tamb = +125 (+0 -3)°C: | - | +18 (Note 3) | % |

NOTES:

For Component Type Variants 01 to 04, the measurements shall be performed on a sample of 6 components from each manufacturing lot, mounted onto a suitable substrate, with 0 failures allowed. Any failures shall result in lot failure. This test is considered to be destructive and therefore components so tested shall not form part of the delivery lot.

For Component Type Variants 05 to 08, the measurements shall be performed on a sample of 6 components from each manufacturing lot with 0 failures allowed. In the event of any failure a 100% inspection may be performed.

The Temperature Coefficient limits are with respect to the capacitance at +22 ±2°C (reference point temperature).

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified, the test methods and test conditions shall be as per the corresponding test defined in Para. 2.4.1, Room Temperature Electrical Measurements.

| Test Reference per ESCC No. 3006 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Resistance to Soldering Heat |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 1 |  | µF |
| Final Measurements | Capacitance | C | Note 1 |  | µF |
|  | Change in Capacitance | ΔC/C | -3 | +3 | % |
|  | Insulation Resistance, Dielectric | RI |  |  |  |
|  | C ≤ 0.33µF; UR > 100V C > 0.33µF; UR > 100V UR ≤ 100V |  | Note 1 Note 1 Note 1 | - - - | MΩ MΩ.µF MΩ.µF |
|  | Tangent of Loss Angle | tgδ | - | Note 1 | - |
| Temperature Coefficient | Temperature Coefficient (Note 2) | ΔC/C | Note 3 |  | % |
| Rapid Change of Temperature |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 1 |  | µF |
| Final Measurements | Capacitance | C | Note 1 |  | µF |
|  | Change in Capacitance | ΔC/C | -3 | +3 | % |
|  | Tangent of Loss Angle | tgδ | - | Note 1 | - |
| Climatic Sequence |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 1 |  | µF |
| Final Measurements | Capacitance | C | Note 1 |  | µF |
|  | Change in Capacitance | ΔC/C | -3 | +3 | % |
|  | Tangent of Loss Angle | tgδ | - | Note 4 | - |
|  | Voltage Proof, Terminal-to-Terminal | VP | Note 1 | - | V |
|  | Insulation Resistance, Dielectric | RI |  |  |  |
|  | C ≤ 0.33µF; UR > 100V C > 0.33µF; UR > 100V UR ≤ 100V |  | Note 6 Note 6 Note 6 | - - - | MΩ MΩ.µF MΩ.µF |
| Operating Life |  |  |  |  |  |
| Initial Measurements | Capacitance | C | Note 1 |  | µF |
| Intermediate Measurements (1000 hours) | Capacitance | C | Note 1 |  | µF |
|  | Change in Capacitance | ΔC/C | -5 | +5 | % |
| Final Measurements (1000 or 2000 hours) (Note 7) | Capacitance | C | Note 1 |  | µF |
|  | Change in Capacitance | ΔC/C | -5 | +5 | % |
|  | Tangent of Loss Angle | tgδ | - | Note 4 | - |
|  | Insulation Resistance, Dielectric | RI |  |  |  |
|  | C ≤ 0.33µF; UR > 100V C > 0.33µF; UR > 100V UR ≤ 100V |  | Note 6 Note 6 Note 6 | - - - | MΩ MΩ.µF MΩ.µF |

NOTES:

As specified in Para. 2.4.1.

The test method and test conditions shall be as specified in Para. 2.4.2.

As specified in Para. 2.4.2.

1.5× the limit specified in Para. 2.4.1.

1.6×UR or 200V, whichever is greater. For the applicable Rated Voltage (UR) see Para. 1.4.2.2.

50% of the limit specified in Para. 2.4.1.

1000 hours is applicable to Periodic Testing for extension of qualification. 2000 hours is applicable to Qualification Testing, and to Periodic Testing for renewal of qualification after lapse.

## BURN-IN conditions

| Characteristics | Symbols | Conditions (Note 1) | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +125 (+0 -3) | °C |
| Test Voltage | VT | 1.25×UC (Note 2) | V |

NOTES:

On completion of Burn-in the components shall be removed from the burn-in chamber and allowed to cool, under normal atmospheric conditions, for recovery for 24 ±2 hours.

UC is the applicable Category Voltage; see Paras. 1.5 and 1.4.2.2.

## APPENDIX ‘A’

AGREED DEVIATIONS FOR EXXELIA TECHNOLOGIES (F)

| Items Affected | Description of Deviations |
| --- | --- |
| Para. 2.1.1, Deviations from the Generic Specification: Deviations from Screening Tests – Chart F3 | If a particular lot is required to undergo testing in accordance with Chart F4 then the results of the Temperature Coefficient measurements, made during High and Low Temperatures Electrical Measurements on a sample of 6 randomly-selected capacitors, shall be recorded. |
| Para. 2.1.1.1, Deviations from Qualification and Periodic Tests – Chart F4 | Robustness of Terminations shall be performed as follows: The components shall be mounted on a suitable substrate. After mounting, examination shall be made for good tinning as evidenced by flowing of the solder with wetting of the terminations. A force of 10N shall be applied normal to the line joining the terminals and, in a plane parallel to the substrate, for a duration of 10 ±1s. The components shall then be examined and there shall be no evidence of damage or loosening of the components from the substrate. |
|  | The Temperature Coefficient measurements recorded during Screening Tests may be used in lieu of the performance of the Temperature Coefficient test as specified in Subgroup 2B of Chart F4. |