VARIANT 03 - Square Flange Female Receptacle, for PCB TRANSMISSION Line WITH Ø PIN 1.73mm

| NOTES: - Accept pin Ø 1.73mm. | Symbols; Dimensions (mm); Remarks. Min; Max. A; 19.9; 20.3. B; 13.6; 13.8. C; 4.4; 4.6. D; 21.9; 22.2. E; 4.15; 4.35. F; 13.45; 13.55. G; 18.9; 19.1. ØH; 3.1; 3.3; 4 Holes. ØJ; 24.3; 24.6. |
| --- | --- |

| Electrical Characteristics | Values | Units |
| --- | --- | --- |
| Voltage Standing Wave Ratio (VSWR) | ≤ 1.1 + 0.02 x f (GHz) | - |
| RF Insertion loss | ≤ 0.04 x f (GHz) | dB |
| RF Leakage | ≤ -115 + f (GHz) | dBi |
| Voltage Proof | ≥ 1500 | Vrms |
| Corona | ≥ Pco at 1GHz (see Table 1(b)) | W |
| Multipaction Threshold (pulse width 20µs; PRF: 1kHz) | ≥ 2000 at 1GHz | Wpp |
| Mechanical Characteristics | Values | Units |
| Centre Contact Retention Force (Axial) | ≥ 27.2 | N |
| Centre Contact Retention Torque (Rotational) | ≥ 2.8 | N.cm |
| Cable Retention Force (Axial) | Not applicable | N |
| Cable Retention Torque (Rotational) | Not applicable | N.cm |
| Weight | ≤ 16 | g |
| Other Characteristics | Values | Units |
| Operating Temperature Range | -65 to +165 | °C |
| Leakage Rate (Panel-sealed Connectors) | Not applicable | - |
| Leakage Rate (Hermetic Sealed Connector) | Not applicable | - |
| Solderability | Not Applicable | - |
| Soldering Proof | Not Applicable | - |
| Cables used | Not applicable | - |