Page 1 of 20

LOAD, RF, COAXIAL, TYPE SMA, DC - 22GHz

ESCC Detail Specification No. 3403/006

| Issue 6 Draft A | June 2026 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# Legal Disclaimer and Copyright

European Space Agency, Copyright © 2026. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| TBA | Specification upissued to incorporate changes per DCR. |
| Draft 6A 17/6/26 S.Thacker | DCR raised on behalf of Radiall (ref email dated 17/6/26 & NRB action per 2CRAD601). Changes are highlighted yellow; see Paras.2.7, 2.8. Note: some minor editorial changes are also included. |

TABLE OF CONTENTS

1	General	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants and Range of Components	6

1.5	MAXIMUM RATINGS	6

1.6	PHYSICAL DIMENSIONS	7

1.6.1	SMA Male Load, Body Type S – Variant 03	7

1.6.2	SMA Female Load – Variant 04	7

1.6.3	SMA Male Load, Body Type C – Variants 05 and 06	8

1.6.4	Interface Dimensions	9

1.6.5	Mating Gauge Dimensions	12

1.7	MATERIALS AND FINISHES	15

2	REQUIREMENTS	15

2.1	GENERAL	15

2.1.1	Deviations from the Generic Specification	15

2.1.1.1	Deviations from Qualification and Periodic Tests - Chart F4	15

2.2	MARKING	16

2.3	CONTACT ENGAGEMENT AND SEPARATION FORCES TEST	16

2.4	COUPLING PROOF TORQUE TEST	16

2.5	MATING AND UNMATING FORCES TEST	16

2.6	ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES	17

2.6.1	Room Temperature Electrical Measurements	17

2.6.2	High and Low Temperatures Electrical Measurements	17

2.7	PARAMETER DRIFT VALUES	17

2.8	INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS	18

2.9	BURN-IN CONDITIONS	20

2.10	OPERATING LIFE CONDITIONS	20

# General

## SCOPE

This specification details the ratings, physical and electrical characteristics and test and inspection data for the component type variants and/or the range of components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

(a) ESCC Generic Specification No. 3403.

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 340300603

Detail Specification Reference: 3403006

Component Type Variant Number: 03 (as required)

### Component Type Variants and Range of Components

The component type variants and range of components applicable to this specification are as follows:

| Variant Number | Connector Type | Body Material and Finish | Maximum RF Power PRF (W) | Weight max (g) |
| --- | --- | --- | --- | --- |
| 03 | SMA Male, Body Type S | Passivated Stainless Steel | 2 | 5 |
| 04 | SMA Female | Passivated Stainless Steel | 2 | 5 |
| 05 | SMA Male, Body Type C | Beryllium Copper, Copper Underplate, Gold Plated | 1 | 3.5 |
| 06 | SMA Male, Body Type C | Beryllium Copper, Nickel Underplate, Gold Plated | 1 | 3.5 |

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| RF Power | PRF | Note 1 | W | Tamb ≤ +70°C, Note 2 |
| Peak Power | PP | 100 | W | duration 1μs 1% duty cycle |
| DC Power | PDC | Note 1 | W | Tamb ≤ +70°C |
| Impedance | Z | 47.5 to 52.5 | Ω | - |
| Frequency Range | fop | DC to 22 | GHz | - |
| RF Leakage | E | -90 | dBi | - |
| Operating Temperature Range | Top | -55 to +125 | °C | Tamb |
| Storage Temperature Range | Tstg | -55 to +125 | °C | - |
| Coupling Nut Torque Range | Tq | 80 to 120 | N.cm | Note 3 |

NOTES:

The maximum rated RF Power is specified in Para. 1.4.2 Component Type Variants and Range of Components.

- The maximum rated DC Power is the same value as the maximum rated RF Power.

RF Power shall be derated against Operating Temperature as follows: For Tamb > +70°C, derate linearly to 25% of RF Power at +125°C

Coupling Proof Torque: 170N.cm

## PHYSICAL DIMENSIONS

### SMA Male Load, Body Type S – Variant 03

| Symbols | Dimensions mm |  |
| --- | --- | --- |
|  | Min | Max |
| A | - | 7.7 |
| B | 8.5 | 9.5 |
| C | - | 16.5 |
| D | 1.9 | 2.3 |

### SMA Female Load – Variant 04

| Symbols | Dimensions mm |  |
| --- | --- | --- |
|  | Min | Max |
| A | - | 7.7 |
| C | - | 14.3 |
| D | 1.9 | 2.3 |

### SMA Male Load, Body Type C – Variants 05 and 06

| Symbols | Dimensions mm |  |
| --- | --- | --- |
|  | Min | Max |
| A | 6.2 | 6.4 |
| B | 7.8 | 8 |
| C | - | 12 |
| D | - | 10.5 |

### Interface Dimensions

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| a | 5.54 | - |  |
| b | 4.32 | - |  |
| c | 0.38 | 1.14 |  |
| d | 4.597 | 4.67 |  |
| e | 5.28 | 5.49 |  |
| g | 1.88 | 1.98 |  |
| h | 0 | 0.2 |  |
| j | 0 | 0.25 |  |
| k | 2.92 | - |  |
| l | - | 0.04 | Radius |

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| a | - | 3.43 |  |
| b | 2.54 | - |  |
| c | 0.38 | 1.14 |  |
| d | - | 4.592 |  |
| e | 6.35 | - |  |
| f | - | 0.08 | Radius or 45° chamfer |
| g | 0 | 0.2 |  |
| h | 0 | 0.25 |  |
| j | - | 2.54 |  |
| k | 0.38 | - |  |
| l | 0.9 | 0.94 |  |
| m | 1.27 | - |  |
| n | - | 0.38 |  |
| p | 3.17 | - |  |
| q | - | - |  |
| r | 7.84 | 8 | Hexagon |
| s | - | 9.2 |  |

### Mating Gauge Dimensions

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| a | 3.81 | - |  |
| b | 1.88 | 1.98 |  |
| c | 0 | 0.08 | Contact recess |
| D | 5.28 | 5.49 |  |
| g | 4.6 | 4.67 |  |
| h | 4.1 | 4.13 |  |
| j | 0.13 | 0.23 | 2 or more slots |
| k | 1.27 | 1.29 |  |
| m | 0.72 | 0.84 | After closing |
| n | 0.38 | 1.14 |  |
| p | 0 | 0.05 | Insert recess |
| u | 2.54 | - |  |
| t | 0.94 | 0.99 |  |
| v | 1.91 | 2.41 |  |
|  | - | 0.25 | 45° Chamfer |
|  | 0.99 | 1.19 | 45° Chamfer |

NOTES:

No fillet permitted. Radial undercut 0.2mm maximum deep x 0.89mm maximum long permitted.

| Symbols | Dimensions mm |  | Notes |
| --- | --- | --- | --- |
|  | Min | Max |  |
| a | 3.71 | 4.32 |  |
| b | 2.59 | 3.35 |  |
| D | 6.48 | 6.73 |  |
| g | 4.34 | 4.59 |  |
| h | 4.1 | 4.13 |  |
| j | - | 0.38 | Flat |
| k | 1.27 | 1.29 |  |
| n | 0.64 | 1.14 |  |
| p | 0 | 0.05 | Insert recess |
| r | 0 | 0.08 | Contact recessed |
| R | - | 0.08 | Radius |
| s | 2.03 | 2.29 |  |
| t | 0.9 | 0.93 |  |
| U | 2.03 | - |  |
|  | - | - | 45 ± 3° Chamfer |

## MATERIALS AND FINISHES

Materials and finishes shall be as follows:

For Variants 03 and 04:

Shell: Amagnetic Stainless Steel, electro-passivated.

Coupling Nut: Amagnetic Stainless Steel, electro-passivated.

Centre Contact: Beryllium Copper, with nickel underplate (2μm minimum) and gold plating (1.3μm minimum). Measurements of plating thickness shall be performed inside the female centre contact at a maximum distance of 0.4mm from the end and on the male centre contact on pin diameter Øt (see Para. 1.6.5).

Inserts: PTFE

Gaskets: Silicone rubber.

For Variant 05:

Shell, Coupling Nut, Centre Contact: Beryllium copper, with copper underplate (2.5µm minimum) and gold plating (2.5µm minimum). Measurements of plating thickness on the centre contact shall be performed on pin diameter Øl (see Para. 1.6.4).

Inserts: PTFE

Gaskets: Silicone rubber.

For Variant 06:

Shell, Coupling Nut, Centre Contact: Beryllium copper, with nickel underplate (2µm minimum) and gold plating (2.5µm minimum). Measurements of plating thickness on the centre contact shall be performed on pin diameter Øl (see Para. 1.6.4).

Inserts: PTFE

Gaskets: Silicone rubber.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Qualification and Periodic Tests - Chart F4

Residual Magnetism: is not applicable to Variants 03, 04 and 06.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700 and as follows.

The information to be marked on the component shall be:

The ESCC qualified components symbol (for ESCC qualified components only).

The ESCC Component Number.

Traceability information.

## CONTACT ENGAGEMENT AND SEPARATION FORCES TEST

Ref. Contact Engagement and Separation Forces in the ESCC Generic Specification.

Oversize Test Pin:

Pin diameter	: 0.9525/0.955mm

Insertion depth : 0.76/1.14mm

Maximum Diameter Test Pin:

Pin diameter	: 0.94/0.942mm

Engagement depth	: 1.27/1.91mm

Engagement force: 1360g maximum

Minimum Diameter Test Pin:

Pin diameter	: 0.902/0.904mm

Separation depth: 1.27/1.91mm

Separation force: 28.4g minimum

## COUPLING PROOF TORQUE TEST

Ref. Coupling Proof Torque in the ESCC Generic Specification.

Coupling Proof Torque: 170N.cm.

## MATING AND UNMATING FORCES TEST

Ref. Mating and Unmating Forces in the ESCC Generic Specification.

Maximum torque during mating or unmating: 24N.cm.

## ELECTRICAL MEASUREMENTS AT ROOM, HIGH AND LOW TEMPERATURES

The measurements shall be performed at room, high and low temperatures.

### Room Temperature Electrical Measurements

The measurements shall be performed at Tamb = +22 ±3ºC.

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Voltage Standing Wave Ratio | VSWR | ESCC No. 3403 f = 0 to 18GHz f = 18 to 22GHz | - - | 1.1 1.15 | - |
| Resistance | R | f = DC | 47.5 | 52.5 | Ω |

### High and Low Temperatures Electrical Measurements

The measurements shall be performed at Tamb = +125 (+0 -3)°C and Tamb = -55 (+3 -0)°C.

| Characteristics | Symbols | Test Method and Conditions (Note 1) | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Temperature Coefficient of Resistance | TCR | DC test Reference Temperature: +25°C | - | 3 x 10-4 | Ω/Ω/°C |

NOTES:

Measurements shall be performed during Screening Tests on a sample of 2 components. In the event of any failure a 100% inspection shall be performed.

## PARAMETER DRIFT VALUES

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.6.1 Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics | Symbols | Drift Value Δ | Units |
| --- | --- | --- | --- |
| Voltage Standing Wave Ratio | ΔVSWRVSWR | ±4 | % |
| Resistance | ΔR | ±2 | Ω |

## INTERMEDIATE AND END-POINT ELECTRICAL MEASUREMENTS

Unless otherwise specified, the measurements shall be performed at Tamb = +22 ±3°C.

Unless otherwise specified, test methods and test conditions shall be as per the corresponding test defined in Para. 2.6.1 Room Temperature Electrical Measurements.

The drift values (Δ) shall not be exceeded for each characteristic where specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Test Reference per ESCC No. 3403 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Vibration Initial Measurements | Resistance Voltage Standing Wave Ratio | R VSWR | 47.5 Note 1 | 52.5 Note 1 | Ω - |
| Measurements during last cycle | Intermittent contact | - | No discontinuity > 0.5ms No open or short circuit |  | - |
| Final Measurements | Resistance Resistance Drift (from initial measurement) Voltage Standing Wave Ratio VSWR Drift (from Initial measurement) | R ΔR VSWR ΔVSWR VSWR | 47.5 - Note 1 | 52.5 ±250 Note 1 ±2 | Ω mΩ - % |
| Mechanical Shock Initial Measurements | Resistance (Note 2) Voltage Standing Wave Ratio (Note 2) | R VSWR | 47.5 Note 1 | 52.5 Note 1 | Ω - |
| Final Measurements | Resistance Resistance Drift (from initial measurement) Voltage Standing Wave Ratio VSWR Drift (from Initial measurement) | R ΔR VSWR ΔVSWR VSWR | 47.5 - Note 1 | 52.5 ±250 Note 1 ±2 | Ω mΩ - % |
| Rapid Change of Temperature Initial Measurements | Resistance Voltage Standing Wave Ratio | R VSWR | 47.5 Note 1 | 52.5 Note 1 | Ω - |
| Final Measurements | Resistance Resistance Drift (from initial measurement) Voltage Standing Wave Ratio VSWR Drift (from Initial measurement) | R ΔR VSWR ΔVSWR VSWR | 47.5 - Note 1 | 52.5 ±250 Note 1 ±2 | Ω mΩ - % |
| Climatic Sequence Initial Measurements | Resistance (Note 2) Voltage Standing Wave Ratio (Note 2) | R VSWR | 47.5 Note 1 | 52.5 Note 1 | Ω - |
| Measurements during Dry Heat | Temperature Coefficient of Resistance | TCR | - | 3 x 10-4 | Ω/Ω/°C |
| Measurements during Cold | Temperature Coefficient of Resistance | TCR | - | 3 x 10-4 | Ω/Ω/°C |
| Final Measurements | Resistance Resistance Drift (from initial measurement) Voltage Standing Wave Ratio VSWR Drift (from Initial measurement) | R ΔR VSWR ΔVSWR VSWR | 47.5 - Note 1 | 52.5 ±250 Note 1 ±2 | Ω mΩ - % |
| Operating Life Initial Measurements | Resistance (Note 2) Voltage Standing Wave Ratio (Note 2) | R VSWR | 47.5 Note 1 | 52.5 Note 1 | Ω - |
| Final Measurements | Resistance Resistance Drift (from initial measurement) Voltage Standing Wave Ratio VSWR Drift (from Initial measurement) | R ΔR VSWR ΔVSWR VSWR | 47.5 - Note 1 | 52.5 ±2 Note 1 ±4 | Ω Ω - % |
| RF Leakage | RF leakage f = 0 to 22GHz | E | -90 | - | dBi |
| Peak Power Final Measurements | Resistance Voltage Standing Wave Ratio | R VSWR | 47.5 Note 1 | 52.5 Note 1 | Ω - |

NOTES:

The limits for VSWR are as specified in Para. 2.6.1 Room Temperature Electrical Measurements.

This test need not be repeated. The most recent result from the previous test may be used instead.

## BURN-IN CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +125 | °C |
| Power | Pin | 0 | W |

## OPERATING LIFE CONDITIONS

| Characteristics | Symbols | Test Conditions | Units |
| --- | --- | --- | --- |
| Ambient Temperature | Tamb | +25 | °C |
| Power Variants 03, 04: Variants 05, 06: | Pin | 2 1 | W |
| Frequency | fin | Note 1 | - |

NOTES:

Operating Life may be performed at DC, 10GHz, 18GHz or 22GHz.