PAGE 1 of 44

CRYSTAL CONTROLLED OSCILLATORS

ESCC Generic Specification No. xxxxx

| ISSUE 1 Draft D | June 2015 |
| --- | --- |

LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2015. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

- copied in whole, in any medium, without alteration or modification.

- copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |

TABLE OF CONTENTS

1.	INTRODUCTION	9

1.1.	Scope	9

1.2.	Applicability	9

2.	APPLICABLE DOCUMENTS	9

2.1.	ESCC Specifications	9

2.2.	Other APPLICABLE Documents	10

2.3.	Other reference Documents	10

2.4.	Order of Precedence	10

3.	Terms, Definitions, Abbreviations, Symbols and Units	10

4.	Requirements	11

4.1.	General	11

4.1.1.	Specifications	12

4.1.2.	Conditions and Methods of Test	12

4.1.3.	Manufacturer’s Responsibility for Performance of Tests and Inspections	12

4.1.4.	Inspection Rights	12

4.1.5.	Pre-encapsulation Inspection Witnessing	12

4.2.	Qualification approval and Qualification Maintenance Requirements on a Manufacturer	12

4.3.	Deliverable Components	12

4.3.1.	ESCC Qualified Components	12

4.3.2.	ESCC Components	12

4.3.3.	Lot Failure	13

4.4.	Marking	13

4.5.	Materials And Finishes	13

4.6.	Radiation Testing	14

4.7.	Procurement of Add-on Components and Materials	14

4.7.1.	General	14

4.7.2.	Selection of Add-on Components and Materials	14

4.7.3.	Specifications for Add-on Components	14

4.7.3.4.1.	General	15

4.7.3.4.2.	Bondability	15

4.7.3.4.3.	LAT	16

4.7.4.	Screening requirements and Lot Acceptance Test (LVT)/ Quality Conformance Inspection (QCI) Requirements for Add-on Components	17

4.7.5.	Traceability	17

4.7.6.	Changes in Add-on Components and Materials	17

4.7.7.	Documentation of Add-on Components	17

5.	Production Control	17

5.1.	General	17

5.2.	Special In-Process Controls	17

5.2.1.	Pre-encapsulation Inspection	17

5.2.2.	Dimension Check	18

5.2.3.	Weight	18

5.2.4.	RGA or Internal Vapour content	18

5.2.5.	Documentation	18

5.2.6.	Bond Strength and Die Shear Test	18

6.	Screening Tests	18

6.1.	General	18

6.2.	Failure Criteria	18

6.2.1.	Environmental and Mechanical Test Failure	18

6.2.2.	Parameter Drift Limits	18

6.2.3.	Parameter Limit Failure	18

6.2.4.	Other Failures	19

6.3.	Failed Oscillators	19

6.4.	Lot Failure	19

6.4.1.	Lot Failure during 100% Testing	19

6.4.2.	Lot Failure during Sample Testing	19

6.5.	Documentation	19

7.	Qualification, Qualification Maintenance and Lot Validation Testing	20

7.1.	Oscillator Qualification Testing (Components Qualification)	20

7.1.1.	General	20

7.1.2.	Distribution within the qualification test lot	20

7.2.	Qualification Maintenance (Periodic Testing)	20

7.3.	Lot Validation Testing	20

7.3.1.	General	20

7.3.2.	Distribution within the sample for lot validation testing	20

7.4.	Failure Criteria	20

7.4.1.	Environmental and Mechanical Test Failures	21

7.4.2.	Electrical Failures	21

7.4.3.	Other Failures	21

7.5.	Failed Oscillators	21

7.6.	Lot Failure	21

7.7.	Qualification, Qualification Maintenance and Lot Validation Testing Samples	21

7.8.	Documentation	21

8.	Test Methods and Procedures	22

8.1.	Internal Visual Inspection	22

8.2.	Bond Strength and Die Shear Test	22

8.2.1.	Bond Strength	22

8.2.2.	Die Shear	24

8.3.	scanning electron microscope inspection	24

8.4.	Radiation Test	24

8.5.	High Temperature Stabilisation Bake	24

8.6.	Temperature Cycling	24

8.7.	Particle Impact Noise Detection (PIND)	24

8.8.	Seal	24

8.8.1.	Seal, Fine Leak	24

8.8.2.	Seal, Gross Leak	24

8.9.	Electrical Measurements	25

8.9.1.	Parameter Drift Values	25

8.9.2.	High and Low Temperatures Electrical Measurements	25

8.9.3.	Room Temperature Electrical Measurements	25

8.9.4.	Intermediate and End-Point Electrical Measurements	25

8.10.	External Visual Inspection and Dimension Check	25

8.11.	Radiographic Inspection	25

8.12.	Mechanical Shocks	26

8.13.	Vibrations	26

8.13.1.	Screening (for class 1 only)	26

8.13.2.	Qualification	26

8.14.	Constant Acceleration (for class 2 only)	26

8.14.1.	Screening	26

8.14.2.	Qualification	26

8.15.	Thermal shocks	26

8.16.	Moisture Resistance	26

8.17.	Solderability	26

8.18.	Permanence of Marking	26

8.19.	Lead integrity	26

8.20.	Operating Life Test	26

8.21.	left blank	27

8.22.	Internal Water Vapour Content	27

8.23.	Scanning Electron Microscope Inspection (SEM)	27

8.24.	Frequency Ageing	27

8.25.	Destructive Physical Analysis	28

8.25.1.	Class 1	28

8.25.2.	Class 2	28

8.26.	Burn in	28

9.	Data Documentation	29

9.1.	General	29

9.1.1.	Qualification and Qualification Maintenance	29

9.1.2.	Oscillator Procurement and Delivery	29

9.1.3.	Additional Documentation	29

9.1.4.	Data Retention / Data Access	29

9.2.	Cover Sheet(s)	29

9.3.	List of Equipment used	30

9.4.	List of Test References	30

9.5.	Special In-Process Controls Data (Chart F2)	30

9.6.	Procurement, Inspection,Testing and Screening of Add-on Components and Materials Data	30

9.7.	Screening Tests Data (Chart F3)	30

9.8.	Qualification and Periodic Tests Data (Chart F4)	30

9.8.1.	Qualification Tests	30

9.8.2.	Periodic Tests for Qualification Maintenance	31

9.8.3.	Lot Validation Testing	31

9.9.	Failed Oscillators List and Failure Analysis Report	31

9.10.	Certificate of Conformity	31

10.	Delivery	31

11.	PAckaging and Dispatch	31

12.	Charts	32

12.1.	naked dice chart	32

12.2.	wafer screening chart	33

12.3.	CHART F1 General Flow For Procurement	33

12.4.	CHART F2 Production Control	34

12.5.	CHART F3 Screening Tests	35

12.6.	CHART F4 Qualification and Periodic Tests	36

13.	AppendiX – test methods	37

13.1.	Oscillator input current-power.	37

13.2.	Initial accuracy at reference temperature.	37

13.3.	Frequency-temperature stability.	37

13.4.	Frequency-voltage tolerance.	37

13.5.	Frequency-load tolerance.	38

13.6.	Phase noise (when specified).	38

13.7.	Jitter (when specified).	38

13.8.	Output voltage-power.	38

13.8.1.	Output voltage.	38

13.8.2.	Output power (sinusoidal waveform).	38

13.8.3.	Output logic voltage levels (square wave output waveform).	39

13.9.	Rise and fall times (square wave output waveform).	40

13.10.	Duty cycle (square wave output waveform).	40

13.11.	Harmonic and sub-harmonic distortion.	40

13.12.	Spurious response.	42

13.13.	Output impedance.	42

13.14.	Startup time.	43

INTRODUCTION

Scope

This specification defines the general requirements for the qualification, qualification maintenance, procurement, and delivery of Crystal Controlled Oscillator types XO, VCXO, TCXO, or mix of these types (e.g. VCTCXO etc.) for space applications. This specification contains the appropriate inspection and test schedules and also specifies the data documentation requirements.

Both classes 1 and 2 as defined in Para. 3 are covered by this specification.

Applicability

This specification is primarily applicable to the granting of qualification approval to components qualified in accordance ESCC Basic Specification No. 20100.

It is also primarily applicable to the procurement of components so qualified.

This specification may also be applied to the procurement of unqualified components, recommendations for which are given in ESCC Basic Specification No. 23100.

APPLICABLE DOCUMENTS

The following documents form part of, and shall be read in conjunction with, this specification. The relevant issues shall be those in effect on date of starting qualification or placing the Purchase Order.

ESCC Specifications

- No. 3501	Generic Specification for Quartz Crystal Units

- No. 5000	Generic Specification for Discrete Semiconductor Components

- No. 9000	Generic Specification for Integrated Circuits Monolithic

- No. 20100	Requirements for the Qualification of Standard Electronic Components for Space Application.

- No. 21300	Terms, Definitions, Abbreviations, Symbols and Units

- No. 20400	Internal Visual Inspection.

- No. 20500	External Visual Inspection

- No. 20600	Preservation, Packaging and Dispatch of ESCC Electronic 	Components.

- No. 20900	Radiographic Inspection of Electronic Components

- No. 21300	Terms, Definitions, Abbreviations, Symbols and Units.

- No. 21400	Scanning Electron Microscope Inspection of Semiconductor Dice.

- No. 21500	Calibration System Requirements

- No. 21700	General Requirements for the Marking of ESCC Components.

- No. 22600	Requirements for the Evaluation of Standard Electronic Components for Space Application

- No. 22700	Requirements and Guidelines for the ‘Process Identification Document’ (PID)

- No. 22800	ESCC Non-conformance Control System.

- No. 22900	Total Dose Steady-State Irradiation Test Method.

- No. 23100 	Recommendations on the use of the ESCC Specification System for the Evaluation and Procurement of Unqualified Components.

- No. 23500	Lead Materials and Finishes for Components for Space Application.

- No. 24600	Minimum Quality System Requirements.

- No. 24800	Resistance to Solvents of Marking, Materials and Finishes.

- No. 2xxxx 	Electrical Test Methods for Crystal Controlled Oscillators

For qualification and qualification maintenance or procurement of qualified oscillators, with the exception of ESCC Basic Specifications Nos. 20100, 21700, 22800 and 24600 where Manufacturers’ specifications are equivalent to, or more stringent than, the ESCC Basic Specifications listed above, they may be used in place of the latter, subject to the approval of the ESCC Executive.

Such replacements shall be clearly identified in the applicable Process Identification Document (PID).

For procurement of unqualified components, where Manufacturers’ specifications are equivalent to or more stringent than the applicable ESCC Basic Specifications listed above, they may be used in place of the latter subject to the approval of the Orderer.

Such replacements may be listed in an appendix to the appropriate Detail Specification at the request of the Manufacturer or Orderer, subject to the approval of the ESCC Executive.

Unless otherwise stated herein, references within the text of this specification to “Detail Specification” shall mean the relevant ESCC Detail Specification.

Other APPLICABLE Documents

- ECSS-Q-ST-70-08	The manual soldering of high-reliability electrical connections

- ECSS-Q-ST-70-38 	High-reliability soldering for surface mount and mixed technology

- ECSS-Q-ST-70-02	Thermal Vacuum Outgassing Test for the Screening of Space Materials

- ECSS-Q-ST-60	Electrical, electronic and electromechanical (EEE) components

- ECSS-Q-ST-70	Materials, mechanical parts and processes

- ECSS-Q-ST-70-11	Procurement of printed circuit boards

- ECSS-Q-ST-60-15	Radiation hardness assurance - EEE components

- MIL-STD-883	Test Methods Standard, Microcircuits

- MIL-STD-202 	Test Methods Standard, Electronic and Electrical Component Parts

- IEC 60758 (1993-04)	Synthetic Quartz crystal Specifications and guide to the use

Other (reference) Documents

- REP005, ESCC Qualified Parts List.

- REP006, ESCC Qualified Manufacturers List.

- IEC Publication No.410	Sampling Plans and Procedures for Inspection by Attributes

- IEC 60068-2-17	Basic environmental testing procedures – Test Q (Sealing)

- IEC 60068-2-20	Test methods for solderability and resistance to soldering heat

- IEC 60410	Sampling plans and procedures for inspection by attributes

Order of Precedence

For the purpose of interpretation and in case of conflict with regard to documentation, the following order of precedence shall apply:

- ESCC Detail Specification

- ESCC Generic Specification

- ESCC Basic Specifications

- Other documents, if referenced herein.

Terms, Definitions, Abbreviations, Symbols and Units

The terms, definitions, abbreviations, symbols and units specified in the ESCC Basic Specification No. 21300 shall apply. In addition following definitions apply:

Supplier:	Manufacturer of add-on components or materials.

Material:	"Materials" refer to non-electronic piece parts as: attachment, wires, cables, ribbons, carriers, mechanical parts, packages, substrate and PCBs.

Add-on components:	Add-on Components are the individual active or passive piece parts for oscillator assembly such as:

- Microcircuits and Semiconductors in die form

- Discrete Active Components (encapsulated)

- Passive Components (in die form or packaged)

- Crystal Units

Swept Crystal:	Swept crystals are crystals that have undergone a solid-state electro-diffusion purification process. Swept crystals have increased resistance to radiation.

Production Lot	A production lot consists of a single oscillator type manufactured on the 	same production line by the same production techniques according to the same oscillator design and with the same add-on components and materials incoming inspection lots during one uninterrupted period.

XO: 	The simplest and most general-purpose-type of crystal oscillator where the principal control element is the crystal unit. Because the XO employs no means of temperature control or compensation, it exhibits a frequency-temperature characteristic determined mainly by the crystal unit employed.

VCXO: 	Voltage controlled oscillator: A crystal oscillator whose output frequency can be offset or modulated by application of an external control voltage. Similar to the XO, the frequency-temperature characteristic of a VCXO is determined mainly by the crystal unit employed.

TCXO:	Temperature controlled oscillator: A crystal oscillator whose frequency-temperature deviation is reduced from that of the crystal unit by an electronic (analog or digital) means of compensation within the device. TCXO-type devices are crystal oscillators in which compensation is achieved by a temperature-dependent variation of the crystal load reactance in a manner that compensates for the crystal unit’s frequency-temperature characteristic.

Components (Oscillators) covered by this specification are identified in the technology classes as follows:

Class 1:

Oscillators using discrete technology. This technology uses exclusively discrete (encapsulated) type electronic components (including surface mount devices) assembled and interconnected on a printed circuit board or an insulating substrate.

Class 2:

Oscillators using microelectronic (hybrid) technology. This technology uses microelectronic circuit elements electrically and mechanically interconnected on an insulating substrate upon which resistors, capacitors, or conductors have been deposited, and used in hermetic package that will be backfilled with an inert gas.

Requirements

General

The requirements for the qualification of a component shall be in accordance with ESCC Basic Specification No. 20100.

The test requirements for procurement of both qualified and unqualified components (See Chart F1) shall comprise:

- Production control : add-on component procurement and screening, special in-process controls

- Screening Tests.

- Periodic Testing (for qualified components).

- Lot Validation Testing if stipulated in the Purchase Order.

Specifications

For qualification approval, qualification maintenance, procurement and delivery of components in conformity with this specification, the applicable specifications listed in Section 2 of this document shall apply in total unless otherwise specified herein or in the Detail Specification.

Conditions and Methods of Test

The conditions and methods of test shall be in accordance with this specification, the ESCC Basic Specifications referenced herein and the Detail Specification.

Manufacturer’s Responsibility for Performance of Tests and Inspections

The Manufacturer shall be responsible for the performance of tests and inspections required by the applicable specifications. These tests and inspections shall be performed at the plant of the Manufacturer of the components unless it is agreed by the ESCC Executive (for qualification, qualification maintenance, or procurement of qualified components) or the Orderer (for procurement of unqualified components), to use an approved external facility.

Inspection Rights

The ESCC Executive (for qualification, qualification maintenance) or the Orderer (for procurement of qualified components, if stipulated in the Purchase Order) reserves the right to monitor any of the tests and inspections scheduled in the applicable specifications.

Pre-encapsulation Customer Source Inspection

If stipulated in the Purchase Order, the Orderer may perform a source inspection at the Manufacturer’s facility prior to encapsulation (including, for example, performance of Internal Visual Inspection and Bond Pull and Die Shear). Details of the inspections to be performed or witnessed and the required period of notification shall be as stipulated in the Purchase Order.

Depending of the product class (1 or 2), two source inspections, before and after crystal assembly, may be necessary.

Qualification approval and Qualification Maintenance Requirements on a Manufacturer

To obtain and maintain the component qualification of a component or a family of components a Manufacturer shall satisfy the requirements of ESCC Basic Specification No. 20100.

Deliverable Components

ESCC Qualified Components

Components delivered to this specification shall be processed and inspected in accordance with the relevant Process Identification Document (PID).

ESCC Components

Each component, irrespective of qualification status, identified with an ESCC component number and delivered to this specification shall:

– be traceable to its production lot: all add-on components, process and materials used shall be traced to their respective production lot allowing a bidirectional and unequivocal relationship between add-on components, materials, processes and Oscillators.

– have satisfactorily completed all the tests required by the relevant issues of the applicable specifications.

– be produced from lots that are considered by the Manufacturer to be capable of passing all applicable tests, and sequences of tests, that are defined in Chart F4. The Manufacturer shall not knowingly supply components that cannot meet this requirement. In the event that, subsequent to delivery and prior to operational use, a component is found to be in a condition such that, demonstrably, it could not have passed these tests at the time of manufacture, this shall be grounds for rejection of the delivered lot.

Lot Failure

Oscillator lot failure may occur during Special In-Process Controls (Chart F2), Screening Tests (Chart F3) or Qualification and Periodic Tests (Chart F4).

Should such failure occur during qualification, qualification maintenance or procurement of qualified components the Manufacturer shall initiate the non-conformance procedure in accordance with ESCC Basic Specification No. 22800. The Manufacturer shall notify the Orderer and the ESCC Executive by any appropriate written means, within 5 working days, giving details of the number and mode of failure and the suspected cause. No further testing or analysis shall be performed on the failed components until so instructed by the ESCC Executive.

Should such failure occur during procurement of unqualified components the Manufacturer shall notify the Orderer by any appropriate written means within 5 working days, giving details of the number and mode of failure and the suspected cause. No further testing or analysis shall be performed on the failed components until so instructed by the Orderer. The Orderer shall inform the Manufacturer within 5 working days of receipt of notification what action shall be taken

Marking

All oscillators procured and delivered to this specification shall be marked in accordance with ESCC Basic Specification No. 21700.

Materials And Finishes

Specific requirements for materials and finishes are specified in the Detail Specification. Where a definite material or finish is not specified a material or finish shall be used so as to ensure that the component meets the performance requirements of this specification and the Detail Specification. Acceptance or approval of any constituent material or finish does not guarantee acceptance of the finished product.

Unless otherwise specified in the Detail Specification the component shall be hermetically sealed and shall have a metal body with hard glass seals or a ceramic body. The component case lid shall be welded, brazed, pre-form soldered or glass frit sealed.

All materials and finishes of the components specified in the Detail specification shall comply with the restrictions on materials specified in ESCC Basic Specification No. 22600.

Radiation Testing

For qualification, qualification maintenance radiation testing shall be performed when specified in the Detail Specification to the total dose level given according to ESCC 22900 unless relevant data are available for the add-on components. In this respect, the criteria for acceptance of data intended to substitute actual total dose radiation testing shall be specified in the PID and will be subjected to review and approval by the ESCC Executive.

For procurement as required in the Purchase Order radiation testing shall be performed to the total dose level given in the Detail Specification or to an alternative radiation verification test plan if so required in the Purchase Order.

Add-on Components and Materials REQUIREMENTS

General

For qualified Oscillators, the requirements for add-on components and materials shall be as defined in the PID and the relevant procurement specifications.

Selection of Add-on Components and Materials

The selection of add-on components and materials shall be made according to ECSS-Q-ST-70 and ECSS-Q-ST-60 requirements, and for qualified Oscillators, shall be defined in the PID.

All add-on components and materials shall be procured as traceable homogeneous lots.

For add-on components and materials, the same restrictions apply as defined in Para. 4.5.

For qualified Oscillators, the PID shall include a list of add-on components and materials which have been successfully tested according to the requirements of this specification and approved for their inclusion in the PID by the ESCC Executive

Printed Circuits Boards shall be procured according to ECSS-Q-ST-70-11 from a Manufacturer qualified by ESA according to ECSS-Q-ST-70-10..

For ESCC qualified Oscillators, add-on components shall be shall be ESCC Qualified as evidenced by a listing in the current ESCC Qualified Parts List REP005 or ESCC Qualified Manufacturers List REP006 with the exception of naked semiconductor dice add-on components selected in accordance with Para. 4.7.3.4 . If the required component types are not available, ECSS-Q-ST-60 requirements for Class 1 components shall be applied,

Pure Tin (≥ 97 %) shall not be used on internal and external elements of the oscillator; this includes under plating.

Specifications for Add-on Components

Each add-on component shall be controlled by a Detail Specification.

The procurement specification of add-on components listed in REP005 or REP006 shall be the relevant listed ESCC Detail Specification.

For qualified Oscillators, when non-qualified add-on components are used, these procurement specifications shall be:

- approved by the ESCC Executive

- subject to configuration control specified in the PID

- include requirements for procurement, storage and relife as defined in PID.

Crystals and Crystal Units

The Crystals used in the oscillators specified herein shall be swept synthetic quartz, meeting the infrared quality requirements of grade A or B of IEC60758 (1993-04). For discrete (encapsulated) crystal units (ESCC qualified), the requirements of ESCC No. 3501 shall apply. Crystals shall be assembled with at least a three-point mount.

Passive encapsulated chips

Lots shall undergo a bondability test prior use for production.

10 samples from the procured lot shall be assembled into suitable packages that simulate the assembly methods, processes and materials to be used in production.

The so assembled hybrid shall be submitted to:

- High temperature Stabilization bake in accordance with MIL-STD-883 test method 1008 condition C as a minimum.

- Thermal cycling (10°C/min max): in accordance with MIL-STD-883 test method 1010 condition B, 100 Cycles.

- Bond pull and shear tests.

If wire bonding is used, a minimum of 22 wires shall be tested.

A shear test shall then be performed on all the chips.

Results shall conform to MIL-STD-883 test method 2011 for bond pull strength if wire bonding is used, and MIL-STD-883 test method 2019 for die shear strength, with no failure allowed.

Bare Passive chips

Procured lots shall be evaluated prior to use . Successful completion of the tests below allows the procured lot to be used in production.

- Bondability:

10 samples from the procured lot shall be assembled into suitable packages that simulate the assembly methods, processes and materials to be used in production.

The so assembled hybrid shall be submitted to a bimetallic bonding ageing (1h @ 300°C) before Bond pull and shear tests.

For bond pull, a minimum of 22 wires shall be tested according to MIL-STD-883 test method 2011. A shear test shall then be performed on all the chips according to MIL-STD-883 test method 2019.

Both bond pull and shear tests shall be performed prior to and after the thermal ageing.

Naked Semiconductor Die Requirements

General

Each lot of naked semiconductor dice shall be procured according to the requirements of Chart 12.1 in order to verify:

- The ability of the dice to be bonded / Wire-bonded

- The reliability of the dice after assembly and screening.

All testing shall be performed at the component Manufacturer’s facility or at a source approved by the ESCC Executive for qualified Oscillators or by the Orderer for unqualified Oscillators.

Successful completion of those tests allows the procured lot to be used in production

Naked Semiconductor Dice Testing

Each naked semiconductor die lot shall undergo the following tests according to Chart 12.1 prior use in production as follows:

Wafer lot acceptance according to ESCC 5000 or 9000 (as applicable)

Wafer screening (Chart 12.2)

  - Electrical Measurement At Room Temperature:

DC probe measurements, with traceability and wafer mapping, shall be performed on all the procured wafers at room temperature in accordance with Room Temperature Electrical Measurements of the appropriate Detail Specification. All values obtained shall be recorded by location and all failed dice shall be clearly identified.

As an alternative to on-wafer measurements, all dice may be measured after dice separation, in accordance with Room Temperature Electrical Measurements of the appropriate Detail Specification, provided that full traceability to the wafer is maintained.

  - Electrical Measurement At High And Low Temperature:

DC probe measurements, with traceability and wafer mapping, shall be performed on all the procured wafers at high and low temperature in accordance with High And Low Temperatures Electrical Measurements of the appropriate Detail Specification. The measurements shall be performed on 5 sample dice from each wafer chosen at random and all failed dice shall be identified.

As an alternative to on-wafer measurements, 5 dice may be measured after dice separation, in accordance with High And Low Temperatures Electrical Measurements of the appropriate Detail Specification, provided that full traceability to the wafer is maintained.

A wafer shall be counted as a limit failure if one or more parameters on any of the probed 5 sample dice exceeds the limits specified in High And Low Temperature Electrical Measurements of the appropriate Detail Specification.

If a failure occurs, further measurements shall be performed to General Inspection Level I, AQL 1.0 of IEC Publication No. 60410 on the remaining dice of the wafer containing the failed dice. If a further failure occurs, the wafer shall be rejected.

  - Wafer dicing: in accordance with the PID

  - Dice visual inspection: in accordance with ESCC 20400 and the relevant paragraphs of the appropriate ancillary specification

Bondability

- 5 sample dice from the procured lot shall be assembled into suitable packages that simulate the assembly methods, processes and materials used in production.

- The so assembled hybrid shall be submitted to a bimetallic bonding ageing (1h @ 300°C) before bond pull and shear tests.

- For bond pull, a minimum of 20 wires shall be tested according to MIL-STD-883 test method 2011. A shear test shall then be performed on all the chips according to MIL-STD-883 Test method 2019 with no failure allowed

Packaged Test Sublot Testing

Dice shall be selected for assembly into suitable packages for submission to further testing as test vehicles. For qualified Oscillators dice, the packages shall be as specified in the PID,

The quantities to be selected are 18 per wafer. After post Burn-In drift calculation the lot is to be split in 2: 11 parts for radiation testing if applicable and 7 for life test.

The sample dice so encapsulated shall be submitted to the following screening and lot validation tests in accordance to ESCC No. 5000 or 9000, as applicable:

  - Encapsulation and serialization

  - Storage: 72 hours @ +125°C

  - Thermal cycling (30°C/min): in accordance with MIL-STD-883 test method 1010 condition B, 10 cycles

  - Constant acceleration: in accordance with MIL-STD-883 test method 2001 condition B, Y1 axis

  - Initial electrical test at Room, High and Low temperatures. Read and record.

  - Burn-In: 240 hours @ 125°C

  - Electrical test at Room temperatures. Read and record.

  - Parameter Drift calculation

  - Split of the test samples: 11 for radiation testing (if required) and 7 for life test (LVT)

  - Life test: 1000 hours @ 125°C

  - Electrical test at Room temperatures. Read and record.

  - Bond pull (22 wires) in accordance with MIL-STD-883 test method 2011 condition C

  - Shear test (5 dice) in accordance with MIL-STD-883 test method 2019.

Radiation test

Radiation tests (TID to be performed according to requirements of ESCC 22900, and the Detail Specification

Check for wafer lot failure according to the below table.

| Samples Number | Number of defects allowed after |  |  |  | Accumulated defects |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Electrical tests | Burn-In | Life test | Radiation | Rejection | Acceptance |
| 18 | 0/18 | 0/18 | 0/7 | 0/11 | 1 | 0 |

Screening requirements and Lot Acceptance Test (LVT)/ Quality Conformance Inspection (QCI) Requirements for Add-on Components

The minimum screening requirements shall be those of the applicable ESCC Detail Specifications or equivalent to those of the nearest ESCC Detail Specification. All screening and LVT/QCI testing shall be performed at the component manufacturer’s facility or at a source approved by the PID approval authority.

Traceability

All add-on components and materials used shall be traceable to their Oscillator production lot and their individual incoming inspection lots.

Records shall be maintained for a minimum of 10 years to provide traceability from each Oscillator serial number to the add-on production lots.

In case of materials with limited shelf life, appropriate means to verify the validity of the relevant material shall be in place. The verification and re-certification shall be in compliance with the Manufacturer’s QM plan and, for qualified Oscillators, the PID.

Changes in Add-on Components and Materials

The Manufacturer shall be responsible for controlling all add-on components and materials. Any change must be controlled and evaluated by the Manufacturer and finally approved by the ESCC Executive for qualified Oscillators or the Orderer for nonqualified oscillator. This is also applicable for documentation of all changes, additional testing of devices due to any changes and / or notification of customers and the ESCC Executive.

Documentation of Add-on Components

Documentation of procurement requirements shall be in accordance with Para.9.6.

Production Control

General

For qualified Oscillators, the full production control provisions are defined in the PID.

Unless otherwise specified in the Detail Specification, all lots of oscillators used for qualification and qualification maintenance, lot validation testing and for delivery shall be subjected to tests and inspections in accordance with Chart F2. The applicable test requirements are detailed in the paragraphs referenced in Chart F2.

Any components which do not meet these requirements shall be removed from the lot and at no future time be resubmitted to the requirements of this specification.

In case of lot failure, the manufacturer shall act in accordance with Para. 4.3.3.

Special In-Process Controls

Internal Visual Inspection

Internal visual inspection shall be performed in accordance with Para. 8.1 (for class 1 and 2).

Bond Strength and Shear (Class 2 only)

Bond Strength and Shear tests shall be performed on test samples in accordance with Para. 8.2.2. A Single failure shall be cause for lot failure. These tests are considered destructive and therefore components so tested shall not form part of the delivery lot.

Dimension Check

Dimension check shall be performed in accordance with Para 8.9 on 3 samples only. If a failure occurs, the complete lot shall be checked.

Weight

The maximum weight of the oscillators specified in the Detail Specifications shall be guaranteed but not tested.

Internal Vapour Content

Internal Water Vapour shall be performed in accordance with Para 8.20.1 on 1 sample minimum per production lot A failure shall be cause for lot failure. This test is considered destructive and therefore the component so tested shall not form part of the delivery lot.

Documentation

Documentation of Special-In Process Controls shall be in accordance with Para. 9.5.

Screening Tests

General

Unless otherwise specified in the Detail Specification, all oscillators used for qualification and qualification maintenance, lot validation testing, and for delivery, shall be subjected to tests and inspections in accordance with Chart F3 in the sequence shown. All oscillators shall be serialised prior to the tests and inspections.

Any oscillators which do not meet these requirements shall be removed from the lot And at no future time be resubmitted to the requirements of this specification.

The applicable test methods and conditions are specified in the paragraphs referenced in Chart F3.

Failure Criteria

Environmental and Mechanical Test Failure

The following shall be counted as oscillator failures:

- Oscillators which fail during tests for which the pass/fail criteria are inherent in the test method, e.g. PIND, Radiographic Inspection, Solderability, Seal, etc.

Parameter Drift Limits

The acceptable change limits are shown in Parameter Drift Values in the Detail Specification. An oscillator shall be counted as a parameter drift failure if the changes during Burn-in or Frequency Ageing are larger than the drift values () specified.

Parameter Limit Failure

An oscillator shall be counted as a limit failure if one or more parameters exceed the limits shown in Room Temperature Electrical Measurements or High and Low Temperatures Electrical Measurements in the Detail Specification.

Any oscillator which exhibits a limit failure prior to the submission to Burn-in shall be rejected and not counted when determining lot rejection.

Other Failures

An oscillator shall be counted as a failure in any of the following cases:

- Visual failure

- Mechanical failure

- Handling failure

- Lost oscillator

Failed Oscillators

An oscillator shall be considered as a failed oscillator if it exhibits one or more of the failure modes described in Para. 6.2.

Lot Failure

In the case of lot failure, the Manufacturer shall act in accordance with Para. 4.3.3.

Lot Failure during 100% Testing

If the number of oscillators failed on the basis of the failure criteria specified in Para’s. 6.2.2 and 6.2.3 exceeds 5% (rounded upwards to the nearest whole number) of the oscillators submitted to Burn-in of Chart F3, the lot shall be considered as failed.

If a lot is composed of groups of oscillators of one family defined in one Detail Specification, but separately identifiable for any reason, then the lot failure criteria shall apply separately to each identifiable group.

Lot Failure during Sample Testing

A lot shall be considered as failed if the number of allowable failures during sample testing as specified herein or in the Detail Specification is exceeded.

Unless otherwise specified, if a lot failure occurs, 100% testing may be performed but the cumulative percentage defective shall not exceed that given in Para. 6.4.1.

No failures are allowed for the Solderability test.

Documentation

Documentation of Screening Tests shall be in accordance with Para. 9.7.

Qualification, Qualification Maintenance and Lot Validation Testing

The requirements of this paragraph are applicable to the tests performed on Oscillators as part of qualification or qualification maintenance in accordance with ESCC Basic Specification No. 20100. They are also applicable to Lot Validation Testing as part of the procurement of qualified or unqualified Oscillators.

Oscillator Qualification Testing

General

Qualification testing shall be in accordance with the requirements given in Chart F4. The tests of Chart F4 shall be performed on the specified sample, chosen at random from oscillators which have successfully passed the tests in Chart F3. This sample constitutes the Qualification Test Lot.

The Qualification Test Lot is divided into subgroups of tests and all oscillators assigned to a subgroup shall be subjected to all of the tests in that subgroup, in the sequence shown. The applicable test requirements are detailed in the paragraphs referenced in Chart F4.

The conditions governing oscillator qualification testing are given in ESCC Basic Specification No. 20100.

Distribution within the Qualification Test Lot

Where a Detail Specification covers a range, or series of Oscillators that are considered similar, then the Qualification Test Lot shall be comprised of component types so selected that they adequately represent all of the various mechanical, structural and electrical peculiarities of that range or series.

The distribution shall be as specified by, or agreed with, the ESCC Executive.

Qualification Maintenance (Periodic Testing)

Qualification is maintained through periodic testing and the test requirements of Para. 7.1 shall apply. For each subgroup, the sample size and the period between successive subgroup testing shall be as specified in Chart F4. The conditions governing qualification maintenance are given in ESCC Basic Specification No. 20100.

Lot Validation Testing

For procurement of qualified Oscillators, Lot Validation Testing is not required and shall only be performed if specifically stipulated in the Purchase Order.

For procurement of unqualified Oscillators, the need for Lot Validation Testing shall be determined by the Orderer (ref. ESCC Basic Specification No. 23100).

When Lot Validation Testing is required, it shall consist of the performance of one or more of the tests or subgroup test sequences of Chart F4. The testing to be performed and the sample size shall be as stipulated in the Purchase Order.

When procurement of more than one Oscillator type is involved from a family, range or series, the selection of representative samples shall also be stipulated in the Purchase Order.

Failure Criteria

The following criteria shall apply to qualification, qualification maintenance and Lot Validation Testing.

Environmental and Mechanical Test Failures

The following shall be counted as oscillator failures:

- Oscillators which fail during tests for which the pass/fail criteria are inherent in the test method e.g. Seal, Terminal Strength, etc.

Electrical Failures

The following shall be counted as oscillator failures:

Oscillators which fail one or more of the applicable limits at each of the relevant data points specified for environmental, mechanical and endurance testing in Intermediate and End-Point Electrical Measurements in the Detail Specification.

Other Failures

An oscillator shall be counted as a failure in any of the following cases:

- Visual failure

- Mechanical failure

- Handling failure

- Lost oscillator

Failed Oscillators

An oscillator shall be considered as failed if it exhibits one or more of the failure modes detailed in Para. 7.4.

When requested by the ESCC Executive (for qualification, qualification maintenance or procurement of qualified oscillators) or the Orderer (for procurement of qualified or unqualified oscillators), failure analysis of failed oscillators shall be performed by the Manufacturer and the results provided.

Failed oscillators shall be retained at the Manufacturer’s plant until the final disposition has been agreed and certified.

Lot Failure

For qualification and qualification maintenance, the lot shall be considered as failed if one oscillator in any subgroup of Chart F4 is a failed oscillator based on the criteria given in Para. 7.4.

For procurement, the lot shall be considered as failed if one oscillator in any test specified for lot validation testing is a failed oscillator based on the criteria given in Para. 7.4.

In the case of lot failure, the Manufacturer shall act in accordance with Para. 4.3.3.

Qualification, Qualification Maintenance and Lot Validation Testing Samples

All tests of Chart F4 are considered to be destructive and therefore oscillators so tested shall not form part of the delivery lot.

Documentation

Documentation of qualification, qualification maintenance and Lot Validation Testing shall be in accordance with Para. 9.8.

Test Methods and Procedures

If a Manufacturer elects to eliminate or modify a test method or procedure, the Manufacturer is still responsible for delivering oscillators that meet all of the performance, quality and reliability requirements defined in this specification and the Detail Specification.

For a qualified component, documentation supporting the change shall be approved by the ESCC Executive and retained by the Manufacturer. It shall be copied, when requested, to the ESCC Executive.

The change shall be specified in an appendix to the Detail Specification and in the PID.

For an unqualified Oscillator, the change shall be approved by the Orderer. The change may be specified in an appendix to the Detail Specification at the request of the Manufacturer or Orderer, subject to the approval of the ESCC Executive.

Internal Visual Inspection

Class 1 oscillators shall be inspected according to ECSS-Q-ST-70-08 and ECSS-Q-ST-70-38.

The internal visual inspection of encapsulated crystal units shall be performed in accordance with ESCC Basic Specifications No. 20400 and No. 2043501.

For class 2 oscillators, internal visual shall be performed in accordance with ESCC Basic Specification No. 20400 and the following specifications:

- MIL-STD-883 Method 2017, Class K requirements, to the extent applicable to the hybrid part of the XO.

- ESCC Basic specification No. 2043501 to the extent applicable to XO Xtals.

Bond Strength and Shear Test

Applicable to class 2 oscillators only.

Special In-Process Controls

For Special In-Process Controls, bond strength test shall be performed on sample Oscillators as follows:

- The required test samples shall be selected at random from the lot of oscillators accepted after internal visual inspection.

- Quantity of internal bond wires <25: Test samples = 2, Test all bonds

- Quantity of internal bond wires >25: Test samples = 1, Test all bonds

Test Method):

- MIL-STD-883 Test method 2011

- Test condition ‘D’ for thermo-compression, ultrasonic or wedge bonding.

Individual separation force and categories shall be recorded..

Subsequently, a shear test shall be performed on the same samples in accordance with MIL-STD-883 Test Method 2019. All dice, chips or crystal studs shall be tested. Individual separation forces and categories shall be recorded.

Qualification and Periodic Tests

As defined in Chart F4 all bonds of one oscillator shall be submitted to destructive bond pull test.

Test Method:

- MIL-STD-883 Test method 2011

- Test condition ‘D’ for thermo-compression, ultrasonic or wedge bonding.

Individual separation force and categories shall be recorded..

Subsequently, a shear test shall be performed on the same sample in accordance with MIL-STD-883 Test Method 2019. All dice, chips or crystal studs shall be tested. Individual separation forces and categories shall be recorded.

Radiation

ESCC Basic Specification No. 22900 to the total dose level as specified in the Detail Specification or as stipulated in the Purchase Order.

High Temperature Stabilisation Bake

MIL-STD-883, method 1008, 24 hours at the maximum storage temperature rating specified in the Detail Specification.

Temperature Cycling

MIL-STD-883, Test method 1010, Test Condition B, and temperature extremes shall not exceed the maximum storage temperature of the Detail Specification.

Particle Impact Noise Detection (PIND)

Class 2 oscillators only. MIL-STD-883, Test method 2020, Test Condition A.

Seal

Applicable for hermetically sealed oscillators only.

Seal, Fine Leak

MIL-STD-883, Test method 1014, Test Condition A2

Seal, Gross Leak

MIL-STD-883, Test method 1014, Condition C

Electrical Measurements

When measured the oscillator performances shall be as specified in the Detail Specification. As a minimum the following parameters shall be tested:

- input current-power

- output waveform, frequency and frequency accuracy

- output voltage-level

Test conditions are specified in ESCC DRAFT 190 (2xxxx). The details of the measurements set-up shall be as specified in the Manufacturer’s documentation and shall be agreed by the ESCC Executive for qualified components, and by the Orderer for unqualified components.

All test results obtained shall be recorded against serial numbers.

Parameter Drift Values

At each of the relevant data points during Screening Tests, Parameter Drift Values shall be measured as specified in the Detail Specification. All values obtained shall be recorded against serial numbers and the parameter drifts calculated.

High and Low Temperatures Electrical Measurements

High and Low Temperatures Electrical Measurements shall be performed as specified in the Detail Specification. All values obtained shall be recorded against serial numbers.

Room Temperature Electrical Measurements

Room Temperature Electrical Measurements shall be performed as specified in the Detail Specification. All values obtained shall be recorded against serial numbers.

Intermediate and End-Point Electrical Measurements

At each of the relevant data points during Qualification, and Periodic Tests Intermediate and End-Point Electrical Measurements shall be performed as specified in the Detail Specification. All values obtained shall be recorded against serial numbers and the parameter drift calculated if specified.

External Visual Inspection and Dimension Check

External visual inspection shall be performed in accordance with ESCC Basic Specification No. 20500and relevant ancillary specification.

Dimension check (during Special In-Process Controls only) shall be performed in accordance with ESCC Basic Specification No. 20500 and the Detail Specification on a sample of 3 oscillators. In the event of any failure a 100% Dimension Check shall be performed.

Radiographic Inspection

For class 1 oscillators Radiographic inspection shall be performed in accordance with MIL-STD-883,

method 2012. Views X, Y and Z axis.

For class 2 oscillators radiographic inspection shall be performed in accordance with ESCC Basic Specification No. 20900 and the following specifications:

  - MIL-STD-883, method 2012. Views X, Y and Z axis. to the extent applicable to the hybrid part of the Oscillator.

- 	ESCC Basic specification No. 2093501 to the extent applicable to crystals

Note: the performance of the test depends on the technology. The opacity of the construction materials and the use of Aluminium bonding wires and epoxy bonding for chips may effectively prevent

radiographic identification of certain types of defects or materials from some or all possible viewing

angles.

Mechanical Shock

MIL STD 202 method 213, condition B: 1500g, 0.5ms

Random Vibration

Screening Tests (for class 1 only)

MIL-STD-202, Method 214, Condition I-B (7.56grms overall), 3 minutes per axis

Qualification and Periodic Tests

MIL-STD-202, Method 214, Condition I-F (20.71grms overall), 3 minutes per axis

Constant Acceleration (for class 2 only)

Screening

MIL-STD-883, Method 2001, condition A, Y1 only.

Qualification and Periodic Tests

MIL-STD-883, Method 2001, condition B, Y1 only.

Thermal shocks

MIL-STD-883, Test method 1011, Test Condition B, and temperature extremes shall not exceed the maximum storage temperature of the Detail Specification.

Moisture Resistance

MIL-STD-883, Test method 1004.

Solderability

For procurement lots: 3 samples. A single failure shall be cause for lot failure

Test shall be performed on all terminals according to MIL-STD-883, Test method 2003,

Solderability testing may be performed on empty packages or electrical rejects. The test samples used must be of the same package type and must have been manufactured using the same process, at the same time and have been subjected to the same screening as the packages of the delivery lot with which they are associated.

For oscillators with gold plated lead finish, de-golding operation as per ECSS-Q-ST-70-08 shall be performed prior the solderability test. Activated fluxes (RMA) may be used but shall be immediately cleaned off after dipping using an acceptable solvent.

Solderability testing is classified as destructive and therefore oscillators so tested shall not form part of the delivery lot.

Permanence of Marking

ESCC Basic Specification No. 24800.

Lead integrity

MIL-STD-883, Test method 2004, Test Condition B2, performed on 3 leads (excluding corner leads) or 10% of the leads (whichever is greater) shall be randomly selected on each oscillator.

Operating Life Test

MIL-STD-883, Test method 1005 cond B

Duration: 2000 hours.

Conditions: As specified in Operating Life in the Detail Specification.

Data Points:

As specified in Intermediate and End-point Electrical Measurements in the Detail Specification at 0 hours, 1000 and 2000  48 hours. If drift values are specified, the drift shall always be related to the 0-hour measurement.

Internal Water Vapour Content

Special In-Process Controls

MIL-STD-883, method 1018 Procedure 1.

The internal water vapour content shall not exceed 5000ppm at +100°C when tested. The values shall be recorded.

Qualification and Periodic Tests

MIL-STD-883, method 1018 Procedure 1.

The internal water vapour content shall not exceed 5000ppm at +100°C when tested. The values shall be recorded.

Frequency Ageing

Measurement of frequency ageing for crystal oscillators shall be for ageing measurement periods of 30 days or longer (unless otherwise specified). The test procedure shall be as follows:

After insertion into the oven, the crystal oscillators shall be stabilised at an ageing temperature for 48 hours prior to beginning the measurement acquisition (unless otherwise specified). The frequency of each unit shall be measured immediately after the stabilisation period, and then a minimum of four times per week at intervals of at least 20 hours for a minimum of 30 days. The frequency change between this initial frequency and all subsequent frequencies measured within the 30-day period shall not exceed the maximum frequency change specified. The measurements obtained shall be fit using the method of least squares to the function:

f(t) = A (ln(Bt + 1)) + f0

where f(t) is the frequency of the crystal oscillator, t days after the start of the ageing cycle (the time origin for measurements analysis shall be the beginning of the stabilisation period), and A, B and f0 are constants to be determined from the least squares fit.

For monotonic ageing, all the measurements shall be used for the curve fitting. If the ageing trend is not monotonic, the measurement period shall be extended to 40 days or longer after the extreme in the ageing trend, and the measurements from 12 days after the extreme is reached to the end of the ageing measurement period shall be fit to the above function (unless otherwise specified).

The total frequency change and the ageing rate at the end of the specified period shall be determined from the above equation using the constants determined from the least squares fit. The square root of the least squares fit variance of the measurements form the curve-fit function shall not exceed 5 percent of the total ageing change allowed during the test period. The following test details shall apply:

- Ageing temperature: +70°C or maximum specified operating temperature in the Detail Specification, whichever is lower

- Ageing temperature tolerance: 3°C (unless otherwise specified)

- Ageing temperature measurement tolerance (stability): 0.2°C (unless otherwise specified)

- If a projected total frequency change for a period (such as 1 year) is specified, the measurement periods shall be 30 days or longer, and a linear extrapolation shall be used form the end of the ageing measurement period using the coefficients determined from a least squares fit to the measurements, as described below..

The projected total frequency change for one year, unless otherwise specified, shall be determined by means of a linear extrapolation from the end of the ageing measurement period using the coefficients determined from the least squares fit. That is, for the total ageing measurement period of Ta, in days (where Ta is 30 days or longer).

where

If a projected total frequency change for a period other than one year is specified, the calculation shall be similarly performed, i.e., a linear extrapolation shall be used from the end of the ageing measurement period using the coefficients determined from the least squares fit.

Destructive Physical Analysis (DPA)

Class 1 Oscillators

1. External visual inspection: ESCC Basic Specification No. 20500 and relevant ancillary specification,.

2. Radiographic Inspection: MIL-STD-883 Method 2012

3. Seal test: MIL-STD-883 Method 1014 Condition A2 (Fine leaks) – Condition C (Gross leaks).

4. Decapsulation

5. Internal Visual inspection in accordance with ECSS-Q-ST-70-08 and ECSS-Q-ST-70-38

6. Microsections:

At least one micro section shall be made on each type of add-on components. The micro section shall be made on the add-on having the worst solder joint appearance identified during the visual inspection.

The microsections shall be stored for a period of at least ten years.

The ESCC executive shall have access to the microsections.

No cracked solder joints or damaged components shall be observed.

Class 2 Oscillators

DPA test sequence depends on the technology and shall be performed in conformance with the requirements of MIL-STD-1580 with deviations when specified in the Detail Specification:

1. External visual inspection: ESCC Basic Specification No. 20500 and relevant ancillary specification.

2. Radiographic Inspection: MIL-STD-883 Method 2012.

3. Seal test: MIL-STD-883 Method 1014 Condition A2 (Fine leaks) – Condition C (Gross leaks).

4. PIND test: MIL-STD-883 Method 2020 Condition A.

5. Internal Water Vapour Content: Para. 8.20.2

6. Decapsulation.

7. Internal visual inspection: ESCC Basic Specification No. 20400 and the following ancillary specifications:

- ESCC Basic specification No. 2043501 to the extent applicable to crystals.

- ESCC Basic specification No. 2049000 to the extent applicable to the hybrid part of the Oscillator.

8. SEM inspection: MIL-STD-883 Method 2018

9. Bond pull test: MIL-STD-883 Method 2011 Condition D (Pull all wires).

10. Shear test: MIL STD-883 Method 2019 (Shear all chips, dice, or crystal Studs ).

Photos of typical assemblies and possible anomalies shall be provided in the report.

Burn in

Duration: 240 hours, max operating temperature, nominal load and supply voltage as defined in the Detail Specification.

Data Documentation

General

For the qualification, qualification maintenance and procurement of each lot a data documentation package shall exist in printed or electronic form.

This package shall be compiled from:

- Cover sheet (or sheets)

- List of equipment (testing and measuring)

- List of test references

- Special in-process controls data (Chart F2)

- Add-on components procurement and testing data (Chart 12.1 and 12.2)

- Screening test data (Chart F3)

- Qualification and periodic tests data (Chart F4)

- Lot validation testing data (when applicable)

- Failed oscillators list and failure analysis report (when applicable)

- Certificate of Conformity

Items (a) to (j) inclusive shall be grouped, preferably as sub-packages and, for identification purposes, each page shall include the following information.

- ESCC Component Number

- Manufacturer’s name

- Lot identification

- Date of establishment of the document

- Page number

Whenever possible, documentation should preferably be supplied in electronic format suitable for reading using a compatible PC. The format supplied shall be legible, durable and indexed. The preferred storage media is CD-ROMS and the preferred file formats are ASCII or PDF.

Qualification and Qualification Maintenance

In the case of qualification or qualification maintenance, the items listed in Para. 9.1 (a) to (j) less item (h) are required.

Oscillator Procurement and Delivery

For all deliveries of oscillators procured to this specification, the following documentation shall be supplied:

- Cover sheet (if all of the information is not included on the Certificate of Conformity)

- Certificate of Conformity (including range of delivered serial numbers)

Additional Documentation

The Manufacturer shall deliver additional documentation containing data and reports to the Orderer, if stipulated in the Purchase Order.

Data Retention / Data Access

If not delivered, all data shall be retained by the Manufacturer for a minimum of 10 years during which time it shall be available for review, if requested, by the Orderer or the ESCC Executive(for qualified oscillators).

Cover Sheet(s)

The cover sheet(s) of the data documentation package shall include as a minimum:

- Reference to the Detail Specification, including issue and date

- Reference to the applicable Generic Specification, including issue and date

- ESCC component number and the Manufacturer’s part type number

- Lot identification

- Range of delivered serial numbers

- Number of the Purchase Order

- Radiation testing level (if applicable)

- Information relative to any additions to this specification and/or the Detail Specification

- Manufacturer’s name and address

- Location of the manufacturing plant (specify place of diffusion, assembly and test)

- Signature on behalf of Manufacturer

- Total number of pages of the data package.

List of Equipment used

A list of equipment used for tests and measurements shall be prepared. Where applicable, this list shall contain inventory number, Manufacturer’s type number, serial number, etc. This list shall indicate for which tests such equipment was used.

List of Test References

This list shall include all Manufacturer’s references or codes which are necessary to correlate the test data provided with the applicable tests specified in the tables of the Detail Specification.

Special In-Process Controls Data (Chart F2)

A test result summary shall be compiled, showing the total number of oscillators submitted to, and the total number rejected after each of the tests. For the bond-strength and shear tests, the separation forces and categories shall be recorded.

ADD-on Components Procurement and Testing Data (Para. 4.7)

A summary of the procurement details applicable to all add-on components shall be complied including:

- Procurement specification(s) number and quality level

- Manufacturer’s name and location

- Lot identification

- Certificate of Conformity

- Traceability information against Oscillator serial number

For add-on component testing in accordance with Para. 4.7.3, a test result summary shall be compiled showing the total number of each add-on component submitted to and the total number rejected after each of the tests. For each test requiring electrical measurements the results shall be recorded. For all Bond Strength and Shear tests, the separation forces and categories shall be recorded.

Screening Tests Data (Chart F3)

A test result summary shall be compiled showing the total number of oscillators submitted to and the total number rejected after each of the tests. For each test requiring electrical measurements the results shall be recorded against oscillator serial number. Oscillator drift calculations shall be recorded for each specified test against oscillator serial number.

Qualification and Periodic Tests Data (Chart F4)

Qualification Tests

A test result summary shall be compiled showing the oscillators submitted to and the number rejected after each test in each subgroup. Oscillator serial numbers for each subgroup shall be identified. For each test requiring electrical measurements, the results shall be recorded against the oscillator serial number. Where a drift value is specified during a test, the drift calculation shall be recorded against the oscillator serial number.

Periodic Tests for Qualification Maintenance

A test result summary shall be compiled showing the oscillators submitted to and the number rejected after each test in each subgroup. Oscillator serial numbers for each subgroup shall be identified. For each test requiring electrical measurements the results shall be recorded against oscillator serial number. Where a drift value is specified during a test, the drift calculation shall be recorded against the oscillator serial number.

In addition to the full test data, a report shall be compiled for each subgroup of Chart F4 to act as the most recent periodic testing summary. These reports shall include a list of all tests performed in each subgroup, the oscillator numbers and quantity of oscillators tested, a statement confirming all results were satisfactory, the date the tests were performed and a reference to the full test data.

Lot Validation Testing

For lot validation testing a test result summary shall be compiled showing the oscillators submitted to and the number rejected after each test in each subgroup (as applicable). Oscillator serial numbers for each subgroup shall be identified. For each test requiring electrical measurements the results shall be recorded against oscillator serial number. Where a drift value is specified during a test the drift calculation shall be recorded against the oscillator serial number.

Failed Oscillators List and Failure Analysis Report

The failed oscillators’ list and failure analysis report shall provide full details of:

- The reference and description of the test or measurement performed as defined in this specification and/or Detail Specification during special in-process controls, Screening Tests and Qualification and Periodic Tests.

- Traceability information including serial number (if applicable) of the failed oscillator

- The failed parameter and the failure mode of the oscillator

- Detailed failure analysis (if requested by the ESCC Executive or Orderer)

Certificate of Conformity

A Certificate of Conformity shall be established as defined in the ESCC Basic Specification No. 20100..

Delivery

For procurement, for each order, the items forming the delivery are:

- The delivery lot.

- The oscillators used for lot validation testing (when applicable), but not forming part of the delivery lot

- The relevant documentation in accordance with the requirements of Paras. 9.1.2 and 9.1.3.

In the case of a oscillator for which a valid qualification is in force, all data of all oscillators submitted to lot validation testing shall also be copied, when requested, to the ESCC Executive.

For qualification or qualification maintenance, the disposition of the Qualification Test Lot and its related documentation shall be as specified in ESCC Basic Specification No. 20100 and the relevant paragraphs of Section 9 of this specification.

PAckaging and Dispatch

The packaging and dispatch of oscillators to this specification shall be in accordance with the requirements of ESCC Basic Specification No. 20600.

Charts

naked SEMICONDUCTOR dice Testing

Wafer lot acceptance

ESCC 5000

ESCC 9000

Wafer screening

Chart 12.2

Deliverable naked dice

Screening

Power burn-in

Electrical measurements

Drift calculation

ESCC 5000

ESCC 9000

LVT

Operating life test

Electrical measurements

Bond Pull & Die Shear

ESCC 5000

ESCC 9000

Naked dice delivery

Naked dice from

Manufacturer

Assembly

As per PID

Package Test Sublot Testing

Release

Bondability

Radiation Tests

wafer screening chart

Wafer from wafer lot acceptance

Electrical measurements at room temperature

Electrical measurements at high and low temperature

Wafer dicing

Dice visual inspection

Deliverable naked dice

CHART F1	General Flow For Procurement

|  |  | Add-on Components and Materials Requirements Para. 4.7 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Production Control: - Special In Process Controls Para. 5.2 / Chart F2 |  |  |  |  |  |  |
|  |  |  | Screening Test Para. 6 / Chart F3 |  |  |  |  |  |
|  | ESCC Qualified |  |  |  |  | Not ESCC qualified |  |  |
| Lot validation testing (1) Para. 7.3 / Chart F4 |  |  |  |  |  | Lot validation testing (2) Para. 7.3 / Chart F4 |  |  |
|  |  |  | Deliverable Oscillators |  |  |  |  |  |
| Periodic Testing Para. 7.2 |  |  | Release |  |  |  |  |  |
|  |  |  | Delivery |  |  |  |  |  |

NOTES:

- Lot Validation Testing is not required for qualified components unless specifically stipulated in the Purchase Order.

- For unqualified components the need for Lot Validation Testing shall be determined by the Orderer and the required testing shall be as stipulated in the Purchase Order (ref. ESCC Basic Specification No. 23100).

CHART F2	Production Control

| Oscillator Lot Manufacturing |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Add-on Components and Materials Requirements |  |  |  |
|  |  |  |  |  |  | Para. 4.7 | Procurement and Testing |  |  |
| Special In-Process Controls |  |  |  |  |  |  |  |  |  |
| Para. 8.15.2.1 | Internal Visual Inspection (Pre-encapsulation Inspection) |  |  |  |  |  |  |  |  |
| Para.5.2.2 | Bond Strength (Pre-encapsulation Inspection) (Notes 1, 3) Die shear |  |  |  |  |  |  |  |  |
| - | Pre-seal (vacuum) bake and encapsulation |  |  |  |  |  |  |  |  |
| Para. 5.2.3 | Dimension Check (Note 1) |  |  |  |  |  |  |  |  |
| Para. 5.2.4 | Weight (Note 2) |  |  |  |  |  |  |  |  |
| Para. 5.2.5 | Internal Water Vapour Content (Notes 1, 3) |  |  |  |  |  |  |  |  |
| To CHART F3 – Screening |  |  |  |  |  |  |  |  |  |

Notes:

1) 	Performed on a sample basis

- Guaranteed but not tested

- Class 2 only

CHART F3	Screening Tests

| From Production Control (note1) |  |  |  |
| --- | --- | --- | --- |
| Para. 8.4 | High Temperature Stabilization Bake (Optional) |  |  |
| Para. 8.5 | Temperature Cycling |  |  |
| Para. 8.13.1 Para. 8.12.1 | constant acceleration (Class 2) or random vibration (Class 1) |  |  |
| Para. 8.6 | Particle Impact Noise Detection (PIND) (Class 2 only) |  |  |
| Para. 8.10 | Radiographic Inspection |  |  |
| Para. 8.8.1 | Parameter Drift Values (Initial Measurements) |  |  |
| Para. 8.23 | Burn in |  |  |
| Para. 8.8.1 | Parameter Drift Values (Initial Measurements) (Note 4) |  |  |
| Para. 8.21 | Frequency Ageing |  |  |
| Para. 8.8.1 | Parameter Drift Values (Final Measurements) (Note 4) |  |  |
| Para. 8.8.2 | Electrical Measurements at High and Low Temperature (Note 4) |  |  |
| Para. 8.8.3 | Electrical Measurements at Room Temperature (Note 2, 4) |  |  |
| Para. 6.4.1 | Check for lot failure (Note 5) |  |  |
| Para. 8.7 | Seal-Test (Fine and Gross Leak) |  |  |
| Para. 8.9 | External Visual Inspection |  |  |
| Para. 8.16 | Solderability (Note 3, 4) |  |  |
| To CHART F4 when applicable |  |  |  |

Notes:

- All oscillators shall be serialised prior to Screening Tests

- Measurements of Parameter Drift Values need not to be repeated at Room Temperature Electrical Measurements

- On sample basis

- The lot failure criteria of Para 6.4 applies to this test

- Check for lot failure shall take into account all electrical parameter failures that may occur during 	Screening Tests in accordance with Para 8.8.1, 8.8.2 and 8.8.3 subsequent to Burn-In..

CHART F4	Qualification and Periodic Tests

| 20 Oscillators (1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subgroup 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Subgroup 2 |  |  |  |  |  |  |  |  |  |  |  | Subgroup 3 |  |  |  |  |  |  |  |  |  | Subgroup 4 |  |  |  |  |  |  |  |  |  |  |  |
| Environmental and Mechanical Subgroup |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Endurance Subgroup |  |  |  |  |  |  |  |  |  |  |  | Radiation Subgroup (2) |  |  |  |  |  |  |  |  |  | Assembly / Capability Subgroup |  |  |  |  |  |  |  |  |  |  |  |
| 24 Months Period |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 12 Months Period |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 24 Months Period |  |  |  |  |  |  |  |  |  |  |  |
| 5 Oscillators |  |  |  |  |  |  | 2 Oscillators |  |  |  |  |  |  |  |  | 5 Oscillators |  |  |  |  |  |  |  |  |  |  |  | 5 Oscillators |  |  |  |  |  |  |  |  |  | 3 Oscillators (4) |  |  |  |  |  |  |  |  |  |  |  |
| Mechanical Shock |  |  |  |  |  |  | Thermal Shock |  |  |  |  |  |  |  |  | Operating Life Test |  |  |  |  |  |  |  |  |  |  |  | Radiation |  |  |  |  |  |  |  |  |  | Lead Integrity |  |  |  |  |  |  |  |  |  |  |  |
| Para.8.11 |  |  |  |  |  |  | Para. 8.14 |  |  |  |  |  |  |  |  | Para. 8.19 |  |  |  |  |  |  |  |  |  |  |  | Para. 8.3 (2) |  |  |  |  |  |  |  |  |  | Para 8.18 |  |  |  |  |  |  |  |  |  |  |  |
| Random Vibration |  |  |  |  |  |  | Moisture Resistance |  |  |  |  |  |  |  |  | Electrical Measurements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | External Visual Inspection |  |  |  |  |  |  |  |  |  |  |  |
| Para. 8.12.2 |  |  |  |  |  |  | Para. 8.15 |  |  |  |  |  |  |  |  | Para 8.8.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Para. 8.9 |  |  |  |  |  |  |  |  |  |  |  |
| Constant Acceleration |  |  |  |  |  |  | Seal Tests(Fine and Gross Leak) |  |  |  |  |  |  |  |  | Seal Tests(Fine and Gross Leak) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Permanence of Marking |  |  |  |  |  |  |  |  |  |  |  |
| Para. 8.13.2 |  |  |  |  |  |  | Para.8.7 |  |  |  |  |  |  |  |  | Para8.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Para. 8.17 |  |  |  |  |  |  |  |  |  |  |  |
| Temperature Cycling |  |  |  |  |  |  | Electrical Measurements |  |  |  |  |  |  |  |  | External Visual Inspection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | solderability |  |  |  |  |  |  |  |  |  |  |  |
| Para. 8.5 |  |  |  |  |  |  | Para 88.4 |  |  |  |  |  |  |  |  | Para. 8.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Para. 8.16 |  |  |  |  |  |  |  |  |  |  |  |
| Seal Tests (Fine and Gross Leak) |  |  |  |  |  |  | External Visual Inspection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Seal Tests(Fine and Gross Leak) |  |  |  |  |  |  |  |  |  |  |  |
| Para 8.7 |  |  |  |  |  |  | Para. 8.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Para8.7 |  |  |  |  |  |  |  |  |  |  |  |
| Electrical Measurements |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Internal Water Vapor Content |  |  |  |  |  |  |  |  |  |  |  |
| Para 8.8.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Para. 8.20.2 (3) |  |  |  |  |  |  |  |  |  |  |  |
| External Visual Inspection |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Para. 8.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DPA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Para. 8.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Notes:

- No failures are permitted

- Only applicable to Lot Validation Testing if specified in the Detail Specification and stipulated in the Purchase Order.

- For class 2 only

- Samples that have successfully completed subgroup 2 tests may be used for subgroup 4