Page 1 of 17

SWITCHES, THERMOSTATIC, BIMETALLIC, SPST WITH SNAP ACTION, OPENING OR CLOSING CONTACT, POTTED AND LEADED

BASED ON TYPE TH473

ESCC Detail Specification No. 240 (3702/002)

| Issue 1 Draft C | February 2018 |
| --- | --- |

Document Custodian: European Space Agency – see https://escies.org

# LEGAL DISCLAIMER AND COPYRIGHT

European Space Agency, Copyright © 2018. All rights reserved.

The European Space Agency disclaims any liability or responsibility, to any person or entity, with respect to any loss or damage caused, or alleged to be caused, directly or indirectly by the use and application of this ESCC publication.

This publication, without the prior permission of the European Space Agency and provided that it is not used for a commercial purpose, may be:

copied in whole, in any medium, without alteration or modification.

copied in part, in any medium, provided that the ESCC document identification, comprising the ESCC symbol, document number and document issue, is removed.

# DOCUMENTATION CHANGE NOTICE

(Refer to https://escies.org for ESCC DCR content)

| DCR No. | CHANGE DESCRIPTION |
| --- | --- |
| Draft 1A S.Thacker (TW) 05/12/2017 | Initial draft ESCC spec created based on Comepa TH473POTTING issue 2 and ESCC3702/001 Draft 6G in converted format to go with converted Generic 3702 Draft 5A. |
| Draft 1B S.Thacker (TW) 26/01/2018 | New revision of draft ESCC spec based on Draft 1A (to go with converted Generic 3702 Draft 5E) plus changes outlined in ESA/SH email dated 25/01/18, and Comepa spec TH473POTTING issue 3. Comments/questions against various points have been included in the review margin (including those against Draft 1A) for Comepa/ESA review & reply. Changes from Draft 1A are highlighted yellow for information |
| Draft 1C S.Thacker (TW) 01/02/2018 | New revision of draft ESCC spec based on Draft 1A & 1B (to go with converted Generic 3702 Draft 5E) plus changes outlined in ESA/SH email dated 25/01/18, and Comepa spec TH473POTTING issue 3; plus telecon S.Thacker/S.Hernandez 01/02/18 Comments/questions against various points have been included in the review margin (including those against Draft 1A & 1B) for Comepa/ESA review & reply. Changes from Draft 1A are highlighted yellow and from draft 1B in blue, for information |

TABLE OF CONTENTS

1	GENERAL	5

1.1	SCOPE	5

1.2	APPLICABLE DOCUMENTS	5

1.3	TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS	5

1.4	THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS	5

1.4.1	The ESCC Component Number	5

1.4.2	Component Type Variants and Range of Components	7

1.5	MAXIMUM RATINGS	9

1.6	PHYSICAL DIMENSIONS	10

1.7	FUNCTIONAL DIAGRAM	11

1.8	MATERIALS AND FINISHES	11

1.8.1	Thermostatic Switch Unit	11

1.8.2	Potting Case	11

1.8.3	Terminal Leads	11

2	REQUIREMENTS	12

2.1	GENERAL	12

2.1.1	Deviations from the Generic Specification	12

2.2	MARKING	13

2.3	Thermal Measurements	13

2.4	Electrical Measurements	14

2.5	PARAMETER DRIFT VALUES	14

2.6	INTERMEDIATE AND END-POINT THERMAL AND ELECTRICAL MEASUREMENTS	15

APPENDIX A	17

# GENERAL

## SCOPE

This specification details the ratings, physical and electrical characteristics, and test and inspection data for the component type variants and/or the Component Type Variants and Range of Components specified below. It supplements the requirements of, and shall be read in conjunction with, the ESCC Generic Specification listed under Applicable Documents.

## APPLICABLE DOCUMENTS

The following documents form part of this specification and shall be read in conjunction with it:

ESCC Generic Specification No. 3702

ESCC Detail Specification No. 3702/001

ESCC Detail Specification No. 3901/001

ESCC Detail Specification No. 3901/013

ESCC Qualified Parts List: REP005

## TERMS, DEFINITIONS, ABBREVIATIONS, SYMBOLS AND UNITS

For the purpose of this specification, the terms, definitions, abbreviations, symbols and units specified in ESCC Basic Specification No. 21300 shall apply. In addition the definitions specified in the Generic Specification shall apply.

## THE ESCC COMPONENT NUMBER AND COMPONENT TYPE VARIANTS

### The ESCC Component Number

The ESCC Component Number shall be constituted as follows:

Example: 370200201B026020HY1.0

Detail Specification Reference: 3702002

Component Type Variant Number: 01 (as required)

Characteristic code: contact type (opening): B (as required)

Characteristic code: nominal functioning temperature, TNF (+26°C): 026 (as required)

Characteristic code: nominal restoring temperature, TNR (+20°C): 020 (as required)

Characteristic code: switching temperature tolerance, Tol (±3.5°C): H (as required)

Characteristic code: grade: Y (as required)

Characteristic code: nominal terminal lead length, L (1m): 1.0 (as required)

#### Characteristics Codes

Characteristics to be codified as part of the ESCC Component Number shall be as follows:

Contact Type expressed by means of the following codes:

| Contact Type | Code |
| --- | --- |
| Switch with Opening Contact: A component where the switch contact will open when the temperature rises to a value equal to or greater than the upper of the two switching temperatures | B |
| - Switch with Closing Contact: - A component where the switch contact will close when the temperature rises to a value equal to or greater than the upper of the two switching temperatures | C |

Nominal functioning temperature, TNF, and nominal restoring temperature, TNR, each expressed by means of the following codes (see Para. 1.4.2 Note 5). The unit quantity shall be °C.

| Nominal Functioning and Restoring Temperature TNF, TNR (°C) | Code |
| --- | --- |
| +XXX | XXX |
| +XX | 0XX |
| +X | 00X |
| 0 | 000 |
| -X | 60X |
| -XX | 6XX |

The switching temperature tolerance, Tol, relative to TNF and TNR, expressed by the following codes (see Para. 1.4.2 Note 5):

| Temperature Tolerance Tol (± °C) | Code |
| --- | --- |
| 2.5 | B |
| 3.5 | H |
| 5.5 | J |

Component grade expressed by the following codes (see Para. 1.4.2 Note 5):

| Grade Description | Code |
| --- | --- |
| A switch designed to operate within the specified switching temperatures tolerance that also maintains a minimum actual temperature differential | Y |
| A switch designed to operate within the specified switching temperatures tolerance that also maintains both a minimum and a maximum actual temperature differential | Z |

The required nominal terminal lead length, L (see Para. 1.6), expressed by means of the following codes. The unit quantity shall be m.

- Note: The available resolution on lead length is ±10cm.

| Nominal Lead Length L (m) | Code |
| --- | --- |
| 0.XX | 0.X |
| X.XX | X.X |
| XX.XX | XX.X |

### Component Type Variants and Range of Components

The Component Type Variants and Range of Components applicable to this specification are as follows:

| Variant Number | Terminal Leads (Wires) |  |  | Thermal Switching Characteristics (TNF, TNR, Tol, Nom Diff, Act Diff) | Total Weight Max (g) |
| --- | --- | --- | --- | --- | --- |
|  | Wire Type & Gauge | Wire ESCC Component Number | Lead Length (m) (Note 1) |  |  |
| 01 | Polyimide insulated wire AWG26 | 390100124B (Note 2) | 0.5 to 10 | See Note 5 | See Note 4 |
| 02 | Polyimide insulated wire AWG24 | 390100125B (Note 2) | 0.5 to 10 | See Note 5 | See Note 4 |
| 03 | Polyimide insulated wire AWG22 | 390100126B (Note 2) | 0.5 to 10 | See Note 5 | See Note 4 |
| 04 | Polyimide insulated wire AWG20 | 390100127B (Note 3) | 0.5 to 10 | See Note 5 | See Note 4 |
| 05 | PTFE insulated wire AWG24 | 390101303B (Note 3) | 0.5 to 10 | See Note 5 | See Note 4 |
| 06 | PTFE insulated wire AWG20 | 390101305B (Note 3) | 0.5 to 10 | See Note 5 | See Note 4 |

NOTES:

See Para. 1.6.

In accordance with ESCC Detail Specification No. 3901/001.

In accordance with ESCC Detail Specification No. 3901/013.

The total maximum weight of the component depends on the required terminal lead length. It includes the weight of the potted thermostatic switch unit plus that of the terminal leads, as follows:

Thermostatic switch unit with potting case: 11g maximum.

Terminal leads: as specified in the ESCC Detail Specification for the applicable wire; see Notes 2 and 3 above.

The following thermal switching characteristic details apply to the available range of components (ref. ESCC 3702/001):

Grade Y:

| Nominal Functioning Temperature TNF (°C) | Nominal Restoring Temperature TNR (°C) | Temperature Tolerance Tol (*) (°C) | Nominal Temperature Differential Nom Diff (TNF - TNR) (°C) | Actual Temperature Differential Act Diff (TF – TR) (°C) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |
| -36 to -31 | -45 to -40 | 5.5 | 9 to 14 | 4 | Nom Diff + 2Tol |
| -30 to +99 | -44 to +94 | 3.5 | 5 to 14 | 4 | Nom Diff + 2Tol |
|  | -40 to +94 | 2.5 | 5 to 10 | 4 | Nom Diff + 2Tol |
| +100 to +150 | +85 to +140 | 5.5 | 10 to 15 | 4 | Nom Diff + 2Tol |

Grade Z:

| Nominal Functioning Temperature TNF (°C) | Nominal Restoring Temperature TNR (°C) | Temperature Tolerance Tol (*) (°C) | Nominal Temperature Differential Nom Diff (TNF – TNR) (°C) | Actual Temperature Differential Act Diff (TF – TR) (°C) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max |
| -35 to -31 | -45 to -41 | 5.5 | 10 | 4 | 18 |
|  |  | 3.5 |  | 4 | 14 |
|  | -44 to -40 | 3.5 | 9 | 4 | 13 |
|  | -43 to -39 | 3.5 | 8 | 4 | 12 |
|  | -42 to -38 | 3.5 | 7 | 4 | 11 |
| -30 to +99 | -40 to +89 | 3.5 | 10 | 4 | 13 |
|  |  | 2.5 |  | 4 | 12 |
|  | -39 to +90 | 3.5 | 9 | 4 | 12 |
|  |  | 2.5 |  | 4 | 11 |
|  | -38 to +91 | 3.5 | 8 | 4 | 11 |
|  |  | 2.5 |  | 4 | 10 |
|  | -37 to +92 | 3.5 | 7 | 4 | 10 |
|  |  | 2.5 |  | 4 | 9 |
|  | -36 to +93 | 3.5 | 6 | 4 | 9 |
|  |  | 2.5 |  | 4 | 8 |
|  | -35 to +94 | 3.5 | 5 | 4 | 8 |
|  |  | 2.5 |  | 4 | 7.5 |
| +100 to +150 | +90 to +140 | 5.5 | 10 | 4 | 17 |
|  |  | 3.5 |  | 4 | 13 |
|  | +91 to +141 | 3.5 | 9 | 4 | 12 |
|  | +92 to +142 | 3.5 | 8 | 4 | 11 |
|  | +93 to +143 | 3.5 | 7 | 4 | 10 |

- * Due to the physical differences between the component and the thermostatic switch unit contained within it, the switching temperatures of the component under test can vary by up to ±0.5°C compared to those of the thermostatic switch unit. Accordingly, an additional 0.5°C has be included in the specified Tol value.

## MAXIMUM RATINGS

The maximum ratings shall not be exceeded at any time during use or storage.

Maximum ratings shall only be exceeded during testing to the extent specified in this specification and when stipulated in Test Methods and Procedures of the ESCC Generic Specification.

| Characteristics | Symbols | Maximum Ratings | Units | Remarks |
| --- | --- | --- | --- | --- |
| Rated Current | IR | 4 | A | V ≤ 30V resistive 100000 switching cycles Note 1 |
| Overload Current | Ioverload | 6 | A | V ≤ 30V resistive |
| Rated Voltage | VR | 120 | V | I ≤ 0.3A resistive 100000 switching cycles Note 1 |
| Operating Temperature Range | Top | -50 to +155 | °C |  |
| Storage Temperature Range | Tstg | -55 to +160 | °C |  |

Notes:

            - Derate as follows:

## PHYSICAL DIMENSIONS

| Symbol | Dimension (mm) |  | Remarks |
| --- | --- | --- | --- |
|  | Min | Max |  |
| ØA | 15.3 | 15.5 | Thermostatic switch unit body diameter. Note 1 |
| B | 14 | 16 |  |
| ØC | 17 | 19 | Potting case diameter |
| D | 20 | 22 |  |
| E | 14.4 | 16.4 |  |
| ØF | Note 2 | Note 2 |  |
| L | L -10 | L + 10 | L = required nominal terminal lead length in mm (i.e. 500mm minimum; 10000mm maximum) Note 3 |

NOTES:

See ESCC Detail Specification No. 3702/001 for this and other switch unit dimensions.

See ESCC Detail Specification No. 3901/001 or 3901/013, as applicable, for lead diameter.

The required nominal terminal lead length, L, shall be specified by the Customer by means of the coded characteristic in the ESCC Component Number; see Para. 1.4.1. The resolution is ±10mm.

## FUNCTIONAL DIAGRAM

NOTES:

Contact opens or closes with increasing temperature (see Para. 1.4.1.1(a)).

The terminal leads are isolated from the case.

## MATERIALS AND FINISHES

### Thermostatic Switch Unit

The thermostatic switch unit shall be ESCC type 370200101 in accordance with ESCC Detail specification No. 3702/001. One or more production lots of thermostatic switch units may be used in the manufacture of a single inspection lot of the components specified herein. Traceability to each production lot of thermostatic switch units shall be maintained.

For ESCC qualified components, the thermostatic switch unit shall be ESCC qualified as evidenced by a listing in the current ESCC Qualified Parts List, REP005.

### Potting Case

The thermostatic switch unit including its terminals shall be potted in a suitable non-conductive epoxy resin moulding.

### Terminal Leads

See Para. 1.4.2.

Leads shall be soldered using Sn62Pb36Sn2 solder to the terminals of the thermostatic switch unit which shall then be covered by a suitable thermo-retractable sheath prior to potting of the thermostatic switch unit.

For ESCC qualified components, the leads shall be ESCC qualified as evidenced by a listing in the current ESCC Qualified Parts List, REP005.

# REQUIREMENTS

## GENERAL

The complete requirements for procurement of the components specified herein are as stated in this specification and the ESCC Generic Specification. Permitted deviations from the Generic Specification, applicable to this specification only, are listed below.

Permitted deviations from the Generic Specification and this Detail Specification, formally agreed with specific Manufacturers on the basis that the alternative requirements are equivalent to the ESCC requirement and do not affect the component’s reliability, are listed in the appendices attached to this specification.

### Deviations from the Generic Specification

#### Deviations from Customer Source Inspection

The Orderer may perform Pre-Encapsulation Customer Source Inspection and/or Final Source Inspection on both the components specified herein, and the related thermostatic switch unit lot(s) used in the manufacture of the components specified herein.

#### Deviations from Special In-Process Controls – Chart F2

Internal Visual Inspection: shall be performed in accordance with ESCC Basic Specification No. 20500, prior to potting, as an external visual inspection of each thermostatic switch unit plus its terminals and its seals. The inspection shall be performed both before and after application of the thermo-retractable sheath, after the terminal leads have been attached.

Serialisation: The serial number already allocated to each respective thermostatic switch unit during its own ESCC Screening Tests shall be maintained and used for all tests.

Thermal Measurements: shall be guaranteed but not tested.

#### Deviations from Screening Tests - Chart F3

Parameter Drift Values (Initial and Final Measurements): shall be guaranteed but not tested.

Rapid Change of Temperature: shall be guaranteed but not tested.

Run-in: shall be guaranteed but not tested.

Thermal Measurements: shall be guaranteed but not tested.

Creepage: shall be guaranteed but not tested.

Seal (Fine and Gross Leak): shall not be performed.

Check for Lot Failure: No failures are allowed.

#### Deviations from Qualification and Periodic Testing - Chart F4

Rapid Change of Temperature: the number of cycles shall be 500.

Seal (Fine and Gross Leak): shall not be performed (in all Subgroups).

Subgroup 1B: Damp Heat, Steady State shall be performed in Subgroup 2C, subsequent to Permanence of Marking. The other tests in Subgroup 1B (Resistance to Soldering Heat, etc.) shall not be performed.

Subgroup 2B: all the tests in Subgroup 2B (Overload, etc.) shall not be performed.

Subgroup 3: Permanence of Marking shall be performed in Subgroup 2C, subsequent to High and Low Temperature Storage (and prior to Damp Heat, Steady State). The other tests in Subgroup 3 (Solderability, etc.) shall not be performed.

#### Deviations from Data Documentation

General: For each inspection lot of components, the full data documentation package for the related production lot or lots of thermostatic switch units (see Para. 1.8.1) used in the manufacture of the delivered components shall be included in the data documentation package and be available for delivery.

Component Procurement and Delivery: either the documentation cover sheet or the Certificate of Conformity shall include the production lot(s) identification of the thermostatic switch units (see Para. 1.8.1) used in the manufacture of the delivered components.

## MARKING

The marking shall be in accordance with the requirements of ESCC Basic Specification No. 21700. The information to be marked on the potting case of the component shall be:

The ESCC Component Number (see Para. 1.4.1).

Traceability information (as a minimum: inspection lot identification).

NOTES:

The original ESCC marking on the body of the thermostatic switch unit, in accordance with ESCC Detail specification No. 3702/001, shall still be present and fully visible (including: ESCC qualified components symbol (for ESCC qualified components only), relevant ESCC Component Number, serial number, production lot number, etc.).

The serial number already allocated to each respective thermostatic switch unit during its own ESCC Screening Tests shall be maintained for each component specified herein. This serial number may not be marked on the potting case of the component.

## Thermal Measurements

| Characteristics (Note 1) | Symbols | Test Method and Conditions | Limits (Note 2) |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Functioning Temperature | TF | ESCC No. 3702. Note 3 | TNF – Tol | TNF + Tol | °C |
| Restoring Temperature | TR | ESCC No. 3702. Note 3 | TNR – Tol | TNR + Tol | °C |
| Actual Temperature Differential | Act Diff | = TF – TR | Act Diff Min | Act Diff Max | °C |

NOTES:

These characteristics are guaranteed but not tested during Charts F2 and F3 of the Generic Specification.

See Para. 1.4.2 Note 5 for characteristic limits.

The following temperature gradient shall apply for measurements:

0.33°C/ minute minimum; 1°C/ minute maximum.

## Electrical Measurements

| Characteristics | Symbols | Test Method and Conditions | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Voltage Proof | VP | ESCC No. 3702 T ≥ T2 or T ≤ T3 (Note 1) Between contacts & case: Between open contacts: | 1000 100 | - - | Vrms |
| Insulation Resistance | RI | ESCC No. 3702 T ≥ T2 or T ≤ T3 (Note 1) V = 100Vdc Between contacts & case: Between open contacts: | 500 500 | - - | MΩ |
| Contact Switching | - | ESCC No. 3702 T ≥ T2 and T ≤ T3 (Note 1) | Verify Correct Switching |  | - |
| Contact Resistance (Note 2) | RC | ESCC No. 3702 T ≥ T2 or T ≤ T3 (Note 1) | - | 50 | mΩ |

NOTES:

See the Generic Specification for definition of T2 and T3.

Contact Resistance of the thermostatic switch unit only (i.e. without leads). Contact Resistance is guaranteed but not tested during Chart F3 of the Generic Specification.

## PARAMETER DRIFT VALUES

The test methods and test conditions shall be as per the corresponding test defined in Para. 2.3 Thermal Measurements.

The drift values (Δ) shall not be exceeded for each characteristic specified. The corresponding absolute limit values for each characteristic shall not be exceeded.

| Characteristics (Note 1) | Symbols | Limits |  |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  | Drift Value Δ | Absolute |  |  |
|  |  |  | Min | Max |  |
| Functioning Temperature | TF | ±1 | TNF – Tol | TNF + Tol | °C |
| Restoring Temperature | TR | ±1 | TNR – Tol | TNR + Tol | °C |

NOTES:

These characteristics are guaranteed but not tested during Chart F3 of the Generic Specification.

## INTERMEDIATE AND END-POINT THERMAL AND ELECTRICAL MEASUREMENTS

Unless otherwise specified, the test methods and test conditions shall be as per the corresponding test defined in Para. 2.3 Thermal Measurements and Para. 2.4 Electrical Measurements.

| Test Reference per ESCC No. 3702 | Characteristics | Symbols | Limits |  | Units |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max |  |
| Rapid Change of Temperature |  |  |  |  |  |
| Initial Measurements | Functioning Temperature | TF | See Para. 2.3 |  | °C |
|  | Restoring Temperature | TR | See Para. 2.3 |  | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.3 |  | °C |
| Vibration |  |  |  |  |  |
| During Testing | No intermittent contact ≥ 1ms, or improper open or short circuit | - | - |  | - |
| Mechanical Shock |  |  |  |  |  |
| During Testing | No intermittent contact ≥ 1ms, or improper open or short circuit | - | - |  | - |
| Thermal and Electrical Measurements | Voltage Proof | VP | See Para. 2.4 |  | Vrms |
|  | Insulation Resistance | RI | See Para. 2.4 |  | MΩ |
|  | Contact Resistance (Note 1) | RC | See Para. 2.4 |  | mΩ |
|  | Functioning Temperature | TF | See Para. 2.3 |  | °C |
|  | Change of Functioning Temperature | ΔTF | -1.7 | +1.7 | °C |
|  | Restoring Temperature | TR | See Para. 2.3 |  | °C |
|  | Change of Restoring Temperature | ΔTR | -1.7 | +1.7 | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.3 |  | °C |
|  | Change of Actual Temperature Differential | ΔAct Diff | -1.7 | +1.7 | °C |
| Response Time |  |  |  |  |  |
| Initial Measurements | Functioning Temperature | TF | See Para. 2.3 |  | °C |
| During Testing | Response Time | trp | - | 20 | s |
| Operating Life (4A, 30Vdc, resistive) |  |  |  |  |  |
| Initial Measurements | Functioning Temperature | TF | See Para. 2.3 |  | °C |
|  | Restoring Temperature | TR | See Para. 2.3 |  | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.3 |  | °C |
| During Testing | Monitor for correct switching and voltage during 50% of closure period | - | - |  | - |
| Intermediate Measurements (25000 and 50000 cycles) | Functioning Temperature | TF | See Para. 2.3 |  | °C |
|  | Change of Functioning Temperature | ΔTF | -1.7 | +1.7 | °C |
|  | Restoring Temperature | TR | See Para. 2.3 |  | °C |
|  | Change of Restoring Temperature | ΔTR | -1.7 | +1.7 | °C |
| Final Measurements | Voltage Proof | VP | See Para. 2.4 |  | Vrms |
|  | Insulation Resistance | RI | See Para. 2.4 |  | MΩ |
|  | Contact Resistance (Note 1) | RC | - | 100 | mΩ |
|  | Functioning Temperature | TF | See Para. 2.3 |  | °C |
|  | Change of Functioning Temperature | ΔTF | -1.7 | +1.7 | °C |
|  | Restoring Temperature | TR | See Para. 2.3 |  | °C |
|  | Change of Restoring Temperature | ΔTR | -1.7 | +1.7 | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.3 |  | °C |
|  | Change of Actual Temperature Differential | ΔAct Diff | -1.7 | +1.7 | °C |
| High and Low Temperature Storage |  |  |  |  |  |
| Initial Measurements | Functioning Temperature | TF | See Para. 2.3 |  | °C |
|  | Restoring Temperature | TR | See Para. 2.3 |  | °C |
|  | Actual Temperature Differential | Act Diff | See Para. 2.3 |  | °C |

NOTES:

Contact Resistance of the thermostatic switch unit only (i.e. without leads). This characteristic is guaranteed but not tested during Chart F4 of the Generic Specification.

## APPENDIX A

AGREED DEVIATIONS FOR COMEPA (F)

| Items Affected | Description of Deviations |
| --- | --- |
| Para. 2.1.1, Deviations from the Generic Specification: Data Documentation | Para. 9.3, List of Equipment Used The list of equipment used shall not be delivered to the Orderer but shall available for review at Comepa’s facilities. |
| Para. 2.1.1.2, Deviations from Special In-Process Controls – Chart F2 | For components to be manufactured using ESCC thermostatic switch units held by the Manufacturer as stock with a production lot date code exceeding 2 years prior to the intended date of encapsulation, the following sequence of destructive tests shall be performed on a random sample of 2 switch units prior to Internal Visual Inspection. The tests shall be performed in accordance with the Generic Specification. In the event of any failure, the production lot of stock switch units shall be rejected. These sample switch units shall not be used to build components that form part of the delivery lot. - Solderability; Para. 8.16. - Permanence of Marking; Para. 8.17. - Robustness of Terminations; Para. 8.18. - Seal (Fine and Gross Leak); Para. 8.6. - External Visual Inspection; Para. 8.7. |
| Para. 2.1.1.3, Deviations from Screening Tests – Chart F3 | External Visual Inspection: Minor visual defects related to the encapsulation process, as specified in the Comepa PID, are considered acceptable. |
| Para. 2.1.1.4, Deviations from Qualification and Periodic Testing - Chart F4 | External Visual Inspection: Minor visual defects related to the encapsulation process, as specified in the Comepa PID, are considered acceptable. |
| Para. 2.2(a), Marking of ESCC Component Number | The marking of the ESCC Component Number on the potting case of the component may be reduced to include only the following details (see Para. 1.4.1): Example: 3702002 01 1.0m - Detail Specification Reference: 3702002 - Component Type Variant Number (e.g. 01) (as required) - Nominal terminal lead length (e.g. 1.0m) (as required) |